3GPP TSG SA WG3 (Security) Meeting #80	S3-152037

24-28 August 2015, Tallinn (Estonia)

Source:	Alcatel-Lucent

Title:	SCAS: Mapping TR.33.806 Annex D.5.3 (Vulnerability Scanning) test case to TS’s.

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution: This contribution proposes to map test cases in Annex D.5.3 – Vulnerability Scanning to the relevant sub-clauses in TS.33.117 (Generic requirements).

The first section of the contribution copies the text from TR 33.806, D.5.3 and adds Word comments explaining which part of the text is believed to be MME-specific and which is generic. It turns out that, for these clauses, there are no MME-specific parts. The current contribution maps Annex D.5.3 to TS.33.117 5.4.3.

The second section of the contribution provides a pCR to TS 33.117. Note that the Requirements Reference in the test case template used in TR 33.806 is no longer needed here since in TS 33.117 the test case immediately follows the requirement.

# Annotated text from TR 33.806

### D.5.3 Vulnerability Scanning

Test Name: TC_BVT_VULNERABILITY_SCANNING

Requirements: Requirements Reference- B.5.3	Vulnerability Scanning

Purpose:

The purpose of vulnerability scanning is to ensure that there no known vulnerabilities (or that  relevant vulnerabilities are identified and remediation plans in place to  mitigate them)  on the Network Product that can be detected by means of automatic testing tools via the Internet Protocol enabled network interfaces.

Procedure and execution steps:

Pre-Conditions:

A list of all available network services containing at least the following information shall be included in the documentation accompanying the Network Product:

•	all interfaces providing IP-based protocols,;

•	the available transport layer protocols on these interfaces;

•	their open ports and associated services;

•	and a free-form description of their purposes.

NOTE: This list is to be validated as part of the BVT port scanning activity.

Editor’s Note: how to review the list of services, for each interface, against the minimum security requirements is ffs.

Editor’s Note: how to establish a list of insecure services is ffs.

The used vulnerability scanning tool shall be capable to detect known vulnerabilities on common services. The used vulnerability information shall be reasonably recent at the time of testing.

Execution Steps

The accredited evaluator’s test lab is required to execute the following steps

1.	Execution of the suitable vulnerability scanning tool against all interfaces providing IP-based protocols of the Network Product

2.	Evaluation of the results based on their severity.

Editor’s Note: It is ffs whether this severity rating is to be established by 3GPP or rather as part of the accreditation process of the evaluator’s test lab. An example could be CVSS.

Expected Results:

The used tool(s) name, their unambiguous version (also for plug-ins if applicable), used settings, and the relevant output is evidence and shall be part of the testing documentation.

The discovered vulnerabilities (including source, example CVE ID), together with a rating of their severity, shall be highlighted in the testing documentation.

NOTE: This testing documentation is input to the vulnerability mitigation process (that may include patching). This is part of the product lifecycle management process developed by GSMA SECAG.

Expected format of evidence:

NA

# 2 pCR to TS 33.117 (generic requirements)

# ***	BEGIN OF FIRST CHANGE	***

5.4.3	Vulnerability Scanning

Requirement Name: Vulnerability Scanning

Requirement Reference: TBA

Requirement Description:

The purpose of vulnerability scanning is to ensure that there no known vulnerabilities (or that relevant vulnerabilities are identified and remediation plans in place to mitigate them) on the Network Product that can be detected by means of automatic testing tools via the Internet Protocol enabled network interfaces.

Vulnerability scanning tools may also report false positives and they shall be investigated and documented in the test report.

Editor's note; the scope for Vulnerability Scanning (i.e. application only or both application and Operating System) shall be clarified.

The test for this requirement can be carried out using a suitable tool or manually performed as described below. If a tool is used then the tester needs to provide evidence, e.g. by referring to the documentation of the tool, that the tool actually provides functionality equivalent to the steps described below.

Threat References: Denial of Service, Information Disclosure, Tampering

Security Objective references: TBA

Test case:

Test Name: TC_BVT_VULNERABILITY_SCANNING

Requirements: Requirements Reference- B.5.3	Vulnerability Scanning

Purpose:

The purpose of vulnerability scanning is to ensure that there no known vulnerabilities (or that  relevant vulnerabilities are identified and remediation plans in place to  mitigate them)  on the Network Product that can be detected by means of automatic testing tools via the Internet Protocol enabled network interfaces.

Procedure and execution steps:

Pre-Conditions:

A list of all available network services containing at least the following information shall be included in the documentation accompanying the Network Product:

•	all interfaces providing IP-based protocols,;

•	the available transport layer protocols on these interfaces;

•	their open ports and associated services;

•	and a free-form description of their purposes.

NOTE: This list is to be validated as part of the BVT port scanning activity.

Editor’s Note: how to review the list of services, for each interface, against the minimum security requirements is ffs.

Editor’s Note: how to establish a list of insecure services is ffs.

The used vulnerability scanning tool shall be capable to detect known vulnerabilities on common services. The used vulnerability information shall be reasonably recent at the time of testing.

Execution Steps

The accredited evaluator’s test lab is required to execute the following steps

1.	Execution of the suitable vulnerability scanning tool against all interfaces providing IP-based protocols of the Network Product

2.	Evaluation of the results based on their severity.

Editor’s Note: It is ffs whether this severity rating is to be established by 3GPP or rather as part of the accreditation process of the evaluator’s test lab. An example could be CVSS.

Expected Results:

The used tool(s) name, their unambiguous version (also for plug-ins if applicable), used settings, and the relevant output is evidence and shall be part of the testing documentation.

The discovered vulnerabilities (including source, example CVE ID), together with a rating of their severity, shall be highlighted in the testing documentation.

NOTE: This testing documentation is input to the vulnerability mitigation process (that may include patching). This is part of the product lifecycle management process developed by GSMA SECAG.

Expected format of evidence:

NA

***	END OF FIRST CHANGE	***