# 3GPP TSG SA WG3 (Security) Meeting #81	S3-152560

# November 9 – 14, 2015, Anaheim, CA (USA)	revision of S3-152304, 440, 379

Source:	Huawei, HiSilicon, Alcatel-Lucent, Nokia Networks

Title:	Merging of Logging Events in 5.2.3.6 and 5.2.6.3 from contributions 304, 440 and 379

Document for:	Discussion/approval

Agenda Item:	7.10

Work Item / Release:	SCAS / Rel-13

Abstract of the contribution:  Commenting contribution on Merge of logging event sections 5.2.3.6 and 5.2.6.3 in TS 33.117.

# Introduction

S3-152304 proposes to merge two sections related to security event logging requirements in TS.33.117. This commenting contribution adds entries for 5 missing entries under the Description heading.

# Proposed pCR

***	BEGIN OF FIRST CHANGE	***

##### 5.2.3.6.1	Security event logging

Requirement Name: Security event logging

Requirement Reference: TBA

Requirement Description: Security events shall be logged together with a unique system reference (e.g. host name, IP or MAC address) and the exact time the incident occurred. For each security event, the log entry shall include user name and/or timestamp and/or performed action and/or result and/or length of session and/or values exceeded and/or value reached.

IETF RFC 3871 [4], section 2.11.10 specifies the minimum set of security events. Each vendor shall document what security events the product logs so it can be verified by testing.

In particular, it shall be possible to log the following events (which are intended to be supported by the network product and which can be enabled by default at manufacturing time or at a later time by the operator):

| EventTypes | Description | Event data to be logged |
| --- | --- | --- |
| Incorrect login attempts | Records any user incorrect login attempts to the network product | • Username, • Source (IP address) if remote access • Timestamp |
| Administrator access | Records any access attempts to accounts that have system privileges. | • Username, • Timestamp, • Length of session, • Source (IP address) if remote access |
| Account administration | Records all account administration activity, i.e. configure, delete, enable, and disable. | • Administrator username, • Administered account, • Activity performed (configure, delete, enable and disable) • Timestamp |
| Resource Usage | Records events that have been triggered when system parameter values such as disk space, CPU load over a longer period have exceeded their defined thresholds. | • Value exceeded, • Value reached (Here suitable threshold values shall be defined depending on the individual system.) • Timestamp |
| Configuration change | Changes to configuration of the network device | • Change made • Username |
| Reboot/shutdown/crash | This event records any action on the network device that forces a reboot or shutdown OR where the network device has crashed. | • Action performed (reboot, shutdown, etc.) • Username (for intentional actions) • Timestamp |
| Interface status change | Change to the status of interfaces on the network device (e.g. shutdown) | • Interface name and type • Status (shutdown, missing link, etc.) • Timestamp |

In addition, optionally it shall be possible to log also the following event (if supported):

| EventTypes | Description | Event data to be logged |
| --- | --- | --- |
| Change of group membership or accounts | Any change of group membership for accounts | • Administrator username, • Administered account, • Activity performed (group added or removed) • Timestamp. |

Threat References: TBA

Security Objective references: tba.

Test case:

Test Name: TC_SECURITY_EVENT_LOGGING

## Purpose:

To verify that the network product correctly logs all required security event types.

## Procedure and execution steps:

## Pre-Conditions:

- The following information shall be provided by the documentation accompanying the network product:

- The log where the event is recorded (e.g. in the Security Log rather than, for instance, System log) and how it can be accessed (e.g. the complete path).

- If the event type is enabled by default or how to enable it.

- What O&M services can be used on the Network Product in the configuration according to the pre-requisites for testing in clause 5.1 and how to use them.

- The tester has the needed administrative privileges to sufficiently perform the tests

- If needed for testing specific O&M services, a tester machine is available.

## Execution Steps

## For each O&M service perform the following test steps

- The Tester sequentially triggers each security event listed in the requirement, while covering each option detailed in the individual security event descriptions.

- The Tester verifies whether the security events, and their individual options, were correctly logged. In particular it is verified whether they include at least the event data specified as required to be logged.

## Expected Results:

## All security events are appropriately logged, including all required event data.

## Expected format of evidence:

The testing report contains the following information for each security event:

- List of O&M services

- Commands executed per O&M services

- The relevant parts of the logs in appropriate form (e.g. file, screenshot)

- Test result (Passed or not)

***	NEXT CHANGES	***

#### 5.2.6.3 	VOID

##### 5.2.6.3.1	VOID

***	END OF CHANGES	***