3GPP TSG-SA WG3 Meeting #81 	S3-152565

9-13 November 2015, Anaheim (USA) 		Revision of S3-152329

Source:	China Mobile, TeliaSonera

Title:	pCR for 33.860: Preventing unauthorized modification of user data in cellular IOT

Document for:	Discussion and Approval

Agenda Item:	7.13

Work Item / Release:	FS_EASE_IoT/Rel-13

Abstract of the contribution:  The pCR identifies that integrity protection should be applied to user data in cellular IOT in order to prevent unauthorized modification.

# Introduction

The SA3# 79 has identified that the signalling and the user data shall be encrypted using the strongest common encryption algorithm between CIoT UE and SGSN for confidentiality in S3-151504: Proposed key issue on eavesdropping for EASE_IoT.  Whether the integrity of user data should be protected or not remains an open issue.

# pCR

***	BEGIN OF FIRST CHANGE	***

## 5.x	Key Issue #x: unauthorized modification of user data

### 5.x.1	Key issue details

It is well known that the encryption does not ensure the integrity of the plaintext. This is especially true for a stream cipher. An attacker usually knows the frame structure of user data and the meaning of each data segment, such as header and payload of the CoAP protocol, which is a standardized application protocol in IoT [X]. It is not so difficult for an attacker to launch a meaningful modification by altering one or more bits in the encrypted user data without breaking stream cipher.

### 5.x.2	Security threats

User data in IoT are usually modelled as low throughput and long traffic inter-arrival time [Y]. The applications of IoT are usually used for the remote information retrieve or remote system control. This is quite different from the user data in real time communication whose traffic arrival is constant and continuous. Real time communication is tolerant to the packet loss or data modification because this usually does not change the content of a real time communication [Z]. A user at most perceives a noise in this case. Also, in IoT, user data can more appropriately be viewed as “signalling data”, with respect to information classification. Therefore, while an attacker’s interest, opportunity, and non-detectability to modify human-generated user data can generally be viewed as low, this need not be the case for modifying IoT user data in several use cases. Thus we cannot deduce that the integrity of user data in CIoT shall not be protected only because integrity of user data is not required in 2G, 3G and 4G networks. The latter were designed for mainly human-generated user data The modification of user data in the CIoT scenario is judged to be a more valid threat than that of the modification of human-originated user data.

### 5.x.3	Security requirements

User data can be discarded at the link layer to avoid the delivery to the upper layer for further processing if data modification is detected at the link layer. This can save the energy consumption since the upper layer doesn’t need to process the compromised data packet.

Considering the efficiency, IoT applications may rely on the security schemes in the network layer to ensure data confidentiality and integrity.

Editor’s note: It is FFS whether integrity protection of user data shall be supported as an optional-to-use feature at the link layer.

***	END OF FIRST CHANGE	***

***	BEGIN OF SECOND CHANGE	***

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 45.820: “Cellular System Support for Ultra Low Complexity and Low Throughput Internet of Things”.

[3]	3GPP TR 23.060: "General Packet Radio Service (GPRS)".

[4]	3GPP TS 43.020: "Security related network functions"

[5]	3GPP TS 35.215: "Confidentiality and Integrity Algorithms UEA2 & UIA2; Document 1: UEA2 and UIA2 specifications"

[6]	3GPP TS 35.201: " Specification of the 3GPP Confidentiality and Integrity Algorithms; Document 1: f8 and f9 Specification "

[X] 	Z. Shelby, K. Hartke, and C. Bormann: The Constrained Application Protocol (CoAP). RFC 7252, June 2014.

[Y]	GP-140681: Traffic model for Cellular IoT (update of GP-140560)

[Z]	3GPP TR 33. 821: Rationale and track of security decisions in Long Term Evolved (LTE) RAN / 3GPP System Architecture Evolution (SAE)

***	END OF SECOND CHANGE	***