3GPP TSG SA WG3 (Security) Meeting #84	S3-160908

25-29 July 2016 Chennai (India)		revision of S3-13abcd

Source:	GSMA FSAG

Title:	Protecting UE network capabilities from ‘bidding down attack.

Document for:	Discussion / Information

Agenda Item:	6.4

# Liaison Statement

| Liaison Statement Title: | Protecting UE network capabilities from ‘bidding down attack’ |
| --- | --- |
| Security Classification: | Confidential |

| Source Meeting Information |  |  |
| --- | --- | --- |
| Meeting Number | Meeting Date | Meeting Location |
| FSAG#29 | 14th March 2016 | Conference Call |

| Document Details |  |  |
| --- | --- | --- |
| Document Number: | Creation Date: | Document Author: |
| FSAG Doc 29_001 | 14th March 2016 | R. D’Amico & L. Costa, Telecom Italia |
| Originating GSMA Source: | Deadline for response: | Liaison Statement Contact |
| Fraud and Security Architecture Group (FSAG) | May 2016 | jmoran@gsma.com |

| Action Required by Recipient |  |
| --- | --- |
| Internal Recipients: | GSMA Networks Group |
| External Recipients: | 3GPP SA3 |

# Summary

## The GSMA’s Fraud and Security Architecture Group (FSAG) wishes to ask the members of 3GPP SA3 to consider the feasibility and potential impacts of the solutions proposed below for the protection of mobile device network capabilities from bidding down attacks.

# Introduction and Background

An October 2015 research paper ‘Practical attacks against privacy and availability in 4G/LTE mobile communication systems’ (http://arxiv.org/pdf/1510.07563v1.pdf) describes a number of attacks feasible against/on LTE networks.

In the attack identified in the paper as ‘Denying selected services (D3)’, an active attacker modifies messages exchanged between the eNodeB and UE to deny the target UE from utilizing some network services. Specifically, the paper states the following:

‘The UE initiates an “Attach Request” message to the eNodeB and this message is intercepted by the attacker. The message contains “Voice domain preference and UE’s usage setting” which informs the network about UE’s voice calling capabilities. The attacker removes these capabilities from this unprotected message and adds “Additional update type - SMS only” before forwarding it to the network. The network accepts this message and executes AKA protocol with the UE to complete Attach procedure. However at this step, the MME configures UE’s profile with the received (modified) capabilities, thereby allowing only SMS and data services.

When there is an incoming call for UE, the MME rejects it and informs the cause to the subscriber who is calling. On the other hand, if UE tries to make an outgoing voice call, the network rejects this request and informs the cause. This is an example of a bidding down attack. The denial is persistent since the attack is effective even after the attacker has moved away.

However, the user can recover from the attack by restarting the UE or moving to another TA.’

This attack scenario is possible because during an Attach procedure, only the UE’s security capabilities are sent back to it for confirmation after security activation in order to protect against bidding down attacks (this is specified in TS33.401). The network sends an integrity protected message including the list of supported security algorithms previously received from the UE.

However, there is no similar confirmation of the network’s capabilities.

FSAG proposes to extend the matching history principle to all parameters, including the network capabilities.

Two possible solutions are identified and described in the following section.

# Solution 1: Attach Request and Security Mode Command

The UE includes in the ‘Attach Request’ message the information to let the network know that it supports the protection feature against bidding down attacks of the network capabilities.  If the network is upgraded to support this feature then the “network capabilities” are replayed back to the UE in an integrity protected way.

As specified in TR 24.301 v13.4.0 the Information Element Identifiers (IEI) F and 5D specify the network capabilities of UE, while the IEI D specifies the device properties.

A possible way is to use, for example, bit 2 of the IEI D (Device properties) to indicate that the UE is able to process the integrity protection on network capabilities (bit 2 set to ‘1’).  In this case the IEI D (Device Properties) shall not be optional but shall be mandatory. When the network receives an Attach Request, it shall check the bit 2 in the IEI D and if this bit is set to 1, then in the Security mode Command message (see TR 24.301) the UE’s network capabilities will also be included, e.g. in the IE Replayed UE security capabilities or adding a new dedicated IE.

In the first case the actual IE “Replayed UE security capabilities” shall be extended; let us call it ‘Replayed UE security and network capabilities’. Alternatively, a completely new IE can be specified for this issue.

However, since the Attach request is unprotected (no encryption nor integrity protection) an active attacker can modify it to indicate to the network that the device is old and unable to support network capabilities verification.

To counter this attack case, the alternative is that the network (supporting such a feature) always sends back to the UE the received “network capabilities” i.e. inside a new IE in the Security mode command.

If the UE supports the new feature it shall verify that network capabilities have not been modified during the radio transmission.

If the UE is old, and therefore unable to process the new IE, it should jump over the unknown information elements and analyse any following information elements (as specified in TS 24.301 V13.4.0 clause “9.9. Other information elements” and in TS 24.007 V13.0.0 clause 11.2.4	Non-imperative part of a standard L3 message). In any case the behaviour shall be verified.

# Solution 2: Security mode command and Security Mode Complete

In this second proposal, the Attach Request message is not modified.

In the Security Mode Command, using a specific bit, the network notifies to UE its capability to support this new protection feature (i.e. a bit of the IE ‘Spare half octet’ set to ‘1’).

If the UE supports this new feature, (i.e. it is able to understand this bit), then in the Security Mode Complete message the UE provides again its network capabilities (in a protected way as this message is protected), so the network can verify if they match the previous one.

For the terminal, nothing is changed.

In this proposal, it is necessary to identify if there is a bit in the Security Mode Command message that can be used to achieve this goal. Otherwise, a completely new IE, or a revision of an already existing IE, inside a Security Mode Complete message, needs to be specified.

# Action for 3GPP SA3

This proposal aims to extend the matching history principle also to UE’s network capabilities.

In clause 3 and 4, two possible ways to address this issue have been proposed.

FSAG kindly asks 3GPP SA3 to analyse this proposal in order to better understand the impacts of the solutions proposed and to identify the best way to integrate in the specifications the protection of UE network capabilities to prevent denial of service attacks.

# Next FSAG Group meetings

Please find below the upcoming schedule of conference calls on which FSAG plans to discuss this topic.

FSAG#30	21st March 2016	14:00 to 15:30 (UK time)

FSAG#31	4th April 2016 		14:00 to 15:30 (UK time)

FSAG#32	18th April 2016 	14:00 to 15:30 (UK time)

FSAG#33	9th May 2016 		14:00 to 15:30 (UK time)

FSAG#34	23rd May 2016 	14:00 to 15:30 (UK time)

# Attachment

Please find below a link to the presentation on the GSMA Infocentre.

# Contact

In case of further questions and/or feedback to the attached document, these can be directed to James Moran, GSMA FSAG Group Director.