# 3GPP TSG SA WG3 (Security) Meeting #86	S3-170161

# 6-10 February 2017, Sophia Antipolis (France)

# Source:	NEC

# Title:	pCR to TR 33.899 Security procedure for NextGen networks

# Document for:	Approval

Agenda Item: 	8.4.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes the Security mode procedure as a solution to Security Area #1 “Architectural aspects of NextGen security area“

# 1	Introduction

The Security mode procedure used in the current SAE/LTE specification is not directly applicable for NextGen system because

- Security implications will vary due to introduction of network slice

- Multiple levels of Security handling function introduced in NextGen system like

  - Authentication Credential Repository and Processing Function (ARPF) – Co-located with UDM

  - Authentication Server Function (AUSF)

  - Security Anchor Function (SEAF) – Co-located with AMF

  - Security Context Management Function (SCMF) – Co-located with AMF

- MME functionality is divided into many components like

  - AMF – Access and Mobility management Function

  - SMF – Session Management Function

This document proposes security mode procedures for NextGen networks to establish NAS (MM and SM), UP and AS Security.

# 2	Proposal

#### **************BEGIN OF CHANGES************

#### 5.1.4.zz	Solution #1.xx Security mode command procedure for NextGen network

#### 5.1.4.zz.1	Introduction

This solution addresses the security mode procedure for NextGen network.

#### 5.1.4.zz.2	Solution details

Security mode procedure in the NextGen consists of NAS security mode procedure, AS security mode procedure, NAS-SM security mode procedure and UP security mode procedure. The reason for introduction of UP SM procedure is to establish user plane security between UE and UPF and the introduction of NAS-SM Security Mode procedure is to establish security for messages between UE and SMF.

This solution makes following assumptions:

- AKA procedure precedes Security Mode procedure.

##### 5.1.4.zz.2.1 NAS security mode command procedure

Figure 5.1.4.zz.2.1-1 shows the detailed procedure to establish NAS Security UE and AMF.

- After the UE is authenticated, the AMF derives KNAS_MM from KSEAF.

- The AMF then request the algorithm to establish NAS security from the SPCF.

- The SPCF selects a suitable algorithm based on the security capabilities of the UE and sends the algorithm information to the AMF in the Security Algorithm Response message.

- The AMF uses the algorithm selected by the SPCF to derive KNAS_MMint and KNAS_MMenc, the integrity and encryption keys for NAS Security.

- The AMF then sends the NAS Security Mode Command(KSI, Replayed UE Security Capabilities, Replayed NSSAI, NAS Algorithms, NAS-MAC) to the UE

- The UE derives KNAS_MM, KNAS_MMint and KNAS_MMenc keys using the parameters received from the AMF.

- The UE sends the integrity protected and encrypted NAS Security Mode Complete (NAS-MAC) to the AMF.

AUSF

NG (R)AN

UPF

SPCF

UDM/ ARPF

UE

SMF

AMF

2. Security Algorithm Request(UE Security Capabilities, NSSAI)

1. KNAS_MM

Derivation

5. NAS Security Mode Command(KSI, Replayed UE Security Capabilities, Replayed NSSAI, NAS Algorithms, NAS-MAC)

3. Security Algorithm Response (NAS Integrity Algorithm, NAS Encryption Algorithm)

4. KNAS_MMint, KNAS_MMenc

Derivation

6b. KNAS_MMint, KNAS_MMenc

Derivation

7. NAS Security Mode Complete(NAS-MAC)

AKA Procedure

6a. KNAS_MM

Derivation

Figure 5.1.4.zz.2.1-1 Message flow for NAS security mode command procedure of non-roaming scenario

##### 5.1.4.zz.2.2 AS security procedure

Figure 5.1.4.zz.2.2-1 shows the security procedure to establish AS Security between the UE and NG (R)AN.

- The AMF derives KAN from KSEAF.

- The AMF sends the Initial Context Setup Request(KAN, Accepted NSSAI, UE Security Capabilities) to the NG (R)AN.

- The NG (R)AN selects the algorithms for UP and RRC keys derivation and derives KUPint, KUPenc, KRRCint and KRRCenc.

- The NG (R)AN sends the integrity protected AS Security Mode Command(AS algorithm ID, AS algorithm distinguisher, MAC) to the UE.

- The UE derives the KAN and corresponding RRC and UP integrity and encryption keys.

- The UE then sends the AS SM Complete command along with MAC to authenticate the AS SMC message.

- The NG (R)AN then sends the Initial Context Setup Response to the AMF.

AUSF

NG (R)AN

UPF

SPCF

UDM/ ARPF

UE

SMF

AMF

1. KAN

Derivation

2. Initial Context Setup Request(KAN, Accepted NSSAI, UE Security Capabilities)

6. AS Security Mode Complete (MAC)

3. KUPint, KUPenc

KRRCint, KRRCenc

Derivation

4. AS Security Mode Command(AS Algorithm ID, Algorithm distinguisher, MAC, Parameters)

7. Initial Context Setup Response

5. KUPint, KUPenc

KRRCint, KRRCenc

Derivation

Figure 5.1.4.zz.2.2-1 Message flow for AS security mode command procedure of non-roaming scenario

##### 5.1.4.zz.2.3 NAS-SM security mode command procedure

Figure 5.1.4.zz.2.3-1 shows the procedure to establish NAS-SM Security between UE and SMF.

- The UE sends the Service Request to the CN(AMF) along with its Temp ID, security capabilities and SM-NSSAI.

- The AMF then selects the suitable SMF based on the SM-NSSAI and the UE’s security capabilities.

- The AMF derives the KNAS_SM from KSEAF.

- The AMF forwards the Service Request with the KNAS_SM to the selected SMF.

- If the SMF does not have the subscription data for the UE, it sends the Subscription Data Request to the UDM.

- The UDM sends UE’s subscription information to the SMF in the Subscription Data Response.

- The SMF requests the algorithm for generating the NAS_SM integrity and encryption keys from the SPCF.

- The SPCF sends the algorithm ID and distinguisher in the Security Algorithm Response to the SMF.

- The SMF derives KNAS_SMint, KNAS_SMenc keys using the algorithms selected by the SPCF.

- The SMF sends the NAS-SM Security Mode Command to the UE to the SMF via the AMF.

- The UE derives KNAS_SM, KNAS_SMint and KNAS_SMenc using the parameters it receives.

- The UE then sends the NAS-SM Security Mode Complete(NAS_SM-MAC) to the SMF via the AMF.

AUSF

NG (R)AN

UPF

SPCF

UDM/ ARPF

UE

SMF

AMF

2. SMF Selection

12. NAS_SM Security Mode Complete (NAS-SM_MAC)

3. KNAS_SM

Derivation

10. NAS_SM Security Mode Command(NAS Algorithms, NAS-SM_MAC, Parameters)

1. Service Request (UE Identity, SM-NSSAI, UE Security Capabilities, KSISEAF)

7. Security Algorithm Request(UE Security Capabilities, SM-NSSAI)

8. Security Algorithm Response (NAS_SM Integrity Algorithm, NAS_SM Encryption Algorithm, SM-NSSAI)

4. Service Request (UE Identity, SM-NSSAI, UE Security Capabilities, KNAS_SM)

9. KNAS_SMenc, KNAS_SMint

Derivation

11b. KNAS_SMenc, KNAS_SMint

Derivation

11a. KNAS_SM

Derivation

6. Subscription Data Response(UE Identity, UE Subscription Data)

5. Subscription Data Request(UE Identity)

Figure 5.1.4.zz.2.3-1 Message flow for UP security mode command procedure of non-roaming scenario

##### 5.1.4.zz.2.4 UP security mode command procedure

Figure 5.1.4.zz.2.4-1 shows the procedure to establish UP Security between UE and UPF. The UP Security Mode procedure is initiated when the UE requests to access a service (and  a PDU session needs to be established).

- The SMF selects the appropriate UPF for the requested service.

- The SMF derives KUP, the key for UP protection.

- The SMF sends requests the algorithm for generating the UP keys from the SPCF.

- The SPCF sends the algorithm ID and distinguisher in the Security Algorithm Response to the SMF.

- The SMF derives Ksessint, Ksessenc keys using the algorithms selected by the SPCF.

- The SMF sends the UP Security Mode Command to the UE via the AMF.

- The UE derives KUP, Ksessint and Ksessenc using the parameters it receives.

- The UE then sends the UP Security Mode Complete(NS-MAC) to the SMF via the AMF.

- The SMF then sends the Service Accept(Ksessint, Ksessenc) to the UPF to establish UP Security between UE and UPF.

AUSF

NG (R)AN

UPF

SPCF

UDM/ ARPF

UE

SMF

AMF

3. Security Algorithm Request(UE Security Capabilities, SM-NSSAI)

2. KUP

Derivation

6. UP Security Mode Command(KSI, UP Algorithms, NS-MAC, Parameters)

4. Security Algorithm Response (UP Integrity Algorithm, UP Encryption Algorithm)

5. KSessint, KSessenc

Derivation

7. KUP, KSessint, KSessenc

Derivation

8. UP Security Mode Complete(NS-MAC)

9. Service Accept(KSessint, KSessenc, Timeout)

1. UPF Selection

##### Figure 5.1.4.zz.2.3-1 Message flow for UP security mode command procedure of non-roaming scenario

#### 5.1.4.zz.3	Evaluation

#### FFS

#### ***************END OF CHANGES*************