3GPP TSG SA WG3 (Security) Meeting #86bis	    S3-170812

27-31 March 2017, Busan (Korea)

Source:	NEC

Title:	pCR to TR 33.899: Intra AMF, Intra SMF, Inter NG RAN handover without Xn interface

Document for:	Approval

Agenda Item:	5.1.1

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes the security procedure of Intra AMF, Intra SMF, Inter NG RAN handover without Xn interface for Security Area #1.

#### Introduction

#### This pCR proposes a solution for Intra AMF, Intra SMF, Inter NG RAN handover (without Xn interface) in NextGen network.

#### Proposal

***** Start of Change *****

#### 5.1.4.x	Solution #1.x Intra AMF, Intra SMF, Inter NG RAN handover without Xn interface

#### 5.1.4.x.1	Introduction

This solution proposes the handover procedure for Intra AMF, Intra SMF, Inter NG RAN (without Xn interface) scenario.

#### 5.1.4.x.2	Solution details

#### Figure 5.1.4.x.2-1 shows the procedure for Intra AMF, Intra SMF, Inter NG RAN handover without Xn interface.

Figure 5.1.4.x.2-1: Intra AMF, Intra SMF, Inter NG RAN handover without Xn interface.

The procedure for Intra AMF, Intra SMF, Inter NG RAN handover without Xn interface is as follows.

- If the handover is required, the UE sends the Measurement Report to the source NG (R)AN.

- The source NG (R)AN evaluates the report and makes the handover decision.

- The source NG (R)AN sends the Handover Required (UE Security Capabilities, Handover Restriction List, NSSAI) command to the AMF.

- The AMF sends the Handover Request (UE Security Capabilities, Handover Restriction List, NSSAI) to the target NG (R)AN.

- The target NG (R)AN checks if the NSSAI can be supported or not.

- If the UE qualifies for accessing the slice via the new NG (R)AN, the target NG (R)AN refreshes the AN keys (derives KAN* and the subsequent RRC and UP integrity and encryption keys).

- It then sends the Handover Request ACK to the AMF.

- The AMF forwards the Handover Request ACK to the source NG (R)AN.

- The source NG (R)AN sends the Handover Command to the UE.

- Using the parameter sent in the Handover Command, the UE derives the KAN* and the subsequent RRC and UP integrity and encryption keys.

- The UE sends the Handover Complete command to the target NG (R)AN.

#### 5.1.4.x.3	Evaluation

#### FFS

***** End of Change *****