# 3GPP TSG SA WG3 (Security) Meeting #87	S3-171154

# 15-19 May 2017 Ljubljana (Slovenia)	revision of S3-17xabc

# Source:	CATT

# Title:	Questions and interim agreements for KI #8.3

# Document for:	Discussion and Approval

Agenda Item:	8.3.8 Network slicing security

1	Decision/action requested

This contribution adds questions and interim agreements for key issue #8.3: Security on UEs' access to slices

2	References

N/A

3	Rationale

This contribution proposes a list of questions that may lead to interim agreements, addressing key issue #8.3: Security on UEs’ access to slices, for TR33.899.

4	Detailed proposal

It is proposed to introduce the following changes to the TR 33.899.

* * * Start of changes * * * *

E.8 	Questions and Interim Agreements for security area #8

E.8.3	Questions and Interim Agreements for Security on UEs’ access to slices

E.8.3.x   Shall UE authentication be supported in slices?

E.8.3.x.1 Description of Question

This question addresses whether UE authentication should be supported in slices in addition to the primary authentication performed outside of slices.

E.8.3.x.2 Interim Agreement

UE authentication in slices is mandatory to support and optional to use. It is used for establishing a PDU session for an UE in a slice instance. In case UE authentication in a slice instance is required, an UE can establish a PDU session only after it passed the authentication in the slice instance.

E.8.3.y   Howl UEs being authenticated in slices?

E.8.3.y.1 Description of Question

This question addresses what methods shall be supported for UE authentication in slices..

E.8.3.y.2 Interim Agreement

Secondary authentication mechanism shall be used for UE authentication in slices.

* * * End of Changes * * *