3GPP TSG SA WG3 (Security) Meeting #87	    S3-171183

15-19 May 2017, Ljubljana

Source:	NEC

Title:	pCR to TS 33.501: Intra NG (R)AN handover

Document for:	Approval

Agenda Item:	7.4

Work Item / Release:	FS_NSA / Rel-14

Abstract: This pCR proposes security procedure of Intra NG (R)AN handover to TS 33.501.

#### Introduction

#### This pCR proposes to add the security procedure for Intra NG (R)AN handover to clause 8.3 Security handling in mobility of TS 33.501.

#### Proposal

***** Start of Change *****

## 8.3	Security handling in mobility

## 8.3.1	Security handling during Intra NG (R)AN handover

Figure 8.3.1-1 shows the security procedure for Intra NG (R)AN handover.

AMF

NG (R)AN

1. Handover Decision

UE

5.Handover Complete

3.Handover Command

SMF

2.Derive KAN*

4.Derive KAN*

Figure 8.3.1-1 Security procedure of Intra NG (R)AN handover

- When handover within the NG (R)AN is required, the NG (R)AN shall make the handover decision.

- The RAN shall refresh the AN keys (derives KAN* and the subsequent RRC and UP integrity and encryption keys).

- NG (R)AN shall send the handover command to the UE

- Using the parameter sent in the Handover Command, the UE shall derive the KAN* and the subsequent RRC and UP integrity and encryption keys.

- UE shall then send the Handover Complete message to NG (R)AN.

      - Key handling during Intra NG (R)AN handover

## When the NG (R)AN decides to perform an intra-NG (R)AN handover it shall derive KAN* as in Annex Y depending on the following criteria: the NG (R)AN shall use the NH for deriving KAN* if an unused {NH, NCC} pair is available in the NG (R)AN (this is referred to as a vertical key derivation), otherwise if no unused {NH, NCC} pair is available in the NG (R)AN, the NG (R)AN shall derive KAN* from the current KAN (this is referred to as a horizontal key derivation).

The NG (R)AN shall use the KAN* as the KAN after handover. The NG (R)AN shall send the NCC used for KAN* derivation to UE in HO Command message.

8.3.1.2	Algorithm Selection during Intra NG (R)AN handover

It is not required to change the AS security algorithm during intra-NG (R)AN handover. If the UE does not receive any selection of new AS security algorithms during an intra-NG (R)AN handover, the UE continues to use the same algorithms as before the handover (see TS 36.331 [xx]).

***** End of Change *****