# 3GPP TSG SA WG3 (Security) Meeting #87	S3-171259

# 15 – 19 May, 2017, Ljubljana, Slovenia

# Source:	Huawei, Hisilicon

# Title:	Use of legacy USIM and NextGen ME in solution #7.12

# Document for:	Approval, Discussion

Agenda Item:	8.3.7 Subscription privacy

# 1	Decision/action requested

A variant to solution #7.12 Mechanisms of Pseudo-IMSI for hiding long-term identifier is proposed.

# 2	References

[1]       3GPP TR 33.899 "Study on security aspects of the Next Generation System".

# 3	Rationale

There is a history of allowing legacy smart card in new technology mobile phones. For that reason, a UE comprised of NextGen ME and a legacy, LTE USIM is a possible scenario in 5G.

A variant of solution #7.12 in TR 33.899 -- where a separate key inside NextGen ME is used instead of the master K for computing the mobile subscriber's pseudonym MSPN -- could support this scenario.

# 4	Detailed proposal

**** Begin Change ****

#### 5.7.4.12.a	Solution #7.12.a: ME-based variant of solution #7.12

There is a history of allowing legacy smart card in new technology mobile phones. For that reason, a UE comprised of NextGen ME and a legacy, LTE USIM is a possible scenario in 5G. The solution described in this clause is a variant of solution #7.12 that could support that scenario.

##### 5.7.4.12.a.2	Solution details

In this variant to solution #7.12 it is proposed to use a separate key Kp inside NextGen ME instead of the master K in the USIM for computing the mobile subscriber's pseudonym MSPN. Thus, Kp becomes a long-term secret key shared between the ME and the HSS and all computation as shown in solution #7.12 would be then done with Kp.

##### 5.7.4.12.a.3	Evaluation

Evaluation as given in 5.7.4.12.a.3 applies, with the following additional point:

- Solution #7.12 should be considered more secure than this variant, because it is in general easier to tamper with ME than to tamper with UICC.

**** End Change ****