# 3GPP TSG SA WG3 (Security) Meeting #88	S3-172082

# 7 – 11 August, 2017, Dali, PR China	revision of S3-171946

# Source:	Nokia

# Title:	LI compliance when applying subscriber privacy

# Document for:	Approval

Agenda Item:	7.3 / TS 33.501

# 1	Decision/action requested

Subscriber privacy must not impact LI compliance. It is proposed to add a new section related to LI following the agreements in TR 33.899 clause E.7.2.1.2.

# 2	Rationale

This contribution is a update to S3-171946 to reflect offline comments that the SUPI can be calculated from H(SUPI*,SUPI) by brute force and would therefore compromise privacy. The intention of this proposal is to have a very slim variant to fulfil LI.

# 3	pCR

Note to Editor: text shows text from 1946 for transparency of the original proposal. All text is new!

<<< START OF CHANGES >>>

### 6.8.X	LI compliance when applying subscriber identifier privacy

Assumption: UE must work as specified and has no functional additions while the HPLMN UDM may be assumed to be cheating. Thus, the underlying assumption of the solution is that the UE is not cheating or cooperating in cheating with the UDM.

The UE conceals SUPI together with a random value R using a public key encryption method, e.g. the concatenation of SUPI||R is the plain text input to a public key encryption method. Let’s call the encrypted value SUPI*. Further the UE creates a hash of SUPI and R as the expected hash, i.e. hxres= H(SUPI,R) and provides SUPI* and hxres within the registration message to the VPLMN. Let’s call hxres an identity proof information, which VPLMN can use for confirming that HPLMN has not cheated, once VPLMN has received the needed input from the HPLMN. The hash function is publicly known. Note that R is not sent along by the UE and, hence, not known to an outside attacker. Note further that the HPLMN cannot, by assumption, control R and hxres, and, hence, cannot find a pre-image of hxres with a false SUPI.

HPLMN decrypts SUPI* and thus obtains SUPI and R. When the VPLMN receives from HPLMN the SUPI and R, it can quickly calculate hres=H(SUPI,R). By comparing hres=hxres the VPLMN (AMF) knows that HPLMN has returned the SUPI (under the above assumption) which the UE sent in a concealed way.

The advantage of having the UE sent hxres in addition to SUPI* is that the HPLMN needs to return only a random value R in addition to the SUPI and VPLMN can fulfil LI by light-weight calculation of the hash hres.

<<< END OF CHANGE >>>