# 3GPP TSG-SA WG3 Meeting #95-BIS 	S3-191929

# Sapporo (Japan), 24-28 June 2019	revision S3-19abcd

# Source:	Huawei, HiSilicon

# Title:	Conclusions to KI #1 on Slice Authentication

# Document for:	Approval

Agenda Item:	8.11

# 1	Decision/action requested

This contribution proposes a pCR for the conclusion for KI #1 of TR 33.813

# 2	References

[1]

# 3	Rationale

With the agreement reached in SA2 on slice authentication procedure in R16 TS23.501/TS23.502, it is proposed to conclude the SA3 study on KI#1 on slice authentication,

# 4	Detailed proposal

pCR

***	BEGINNING of CHANGES	***

## 8.1	Conclusions for key issue #1 - Authentication for access to specific Network Slices

For KI#1, it is concluded that Solution #2 in clause 7.2, with the following essential features, shall be the basis of normative work:

- Slice Authentication is optional and it is performed after Primary Authentication

- AMF confirms, locally or based on ARPF/UDM, whether slice authentication is required for each S-NSSAI

- The credential used in slice authenticatin is different from that used in Primary Authenticaiton

- Slice Authentication is based on EAP framework where AMF takes the role of the passing through Authenticator

- Varied EAP authentication methods are supported and determined between AAA and UE

***	END of CHANGES	***