3GPP TSG-SA WG3 Meeting #96Ad-Hoc 	S3-193451

# ChongQing (China), 14-18 October 2019												revision of S3-19xabc

# Source:	Huawei, Hisilicon

# Title:	Resolving the ENs in KI#3.1

# Document for:	Approval

Agenda Item:	5.14

1	Decision/action requested

This document proposes to resolve the Editor’s Notes in the KI#3.x of TR 33.846.

2	References

[1]    3GPP TS 33.501 "Security architecture and procedures for 5G system"

[2]	   3GPP TS 29.503 "5G System; Unified Data Management Services”

3	Rational

According to SA3#96 meeting, four Editor’s Notes were added to Key Issue #3.X of TR33.846:

Editor’s Note: Align with the objective of the study needs to be clarified.

Editor’s Note: Key issue details are FFS.

Editor’s Note: security threats are FFS.

Editor’s Note: Potential security requirements are FFS.

These issues listed in the above enditor’s notes will be resolved in this contribution.

For the first Editor’s Note: Align with the objective of the study needs to be clarified:

Currently, TS 33.501 [1] clause 6.1.4 specifies that AUSF will inform the UDM about the result and timestamp of a successful authentication run. However, in case of UE-initiated deregistration procedure or network-initiated deregistration or SMC failure after AKA during the registration procedure, UDM still keeps the record of  UE’s authentication result. If the authentication result is not removed, the purpose of preventing fraudulent Nudm_UECM_Registration request attack by increasing the home controlled authentication in 5G is NOT fully achieved, and the fraudulent AMF may steal the UE data from the UDM.

Therefore, there are still vulnerabilities in the current authentication procedure, and the corresponding security mechanisms need to be studied in the FS_AUTH_ENH study item. The key Issue on fraudulent attack due to expired authentication result in the UDM is align with the objective of the FS_AUTH_ENH study item.

For the second Editor’s Note: Key issue details are FFS:

More Key issue details are introduced as following:

Security issue 1:

If the authentication result is not removed after the UE deregistration or SMC failure, the AMF can always send the fraudulent Nudm_UECM_Registration Request to UDM pretending the UE is already connected to it while in fact the UE is not. So the fraudulent Nudm_UECM_Registration request attack still exists and is not mitigated, which means that the purpose of preventing certain types of fraud by increasing the home controlled authentication in 5G is NOT fully achieved.

Security issue 2:

If the authentication result is not removed after UE deregistration or SMC failure, fraudulent Nudm_UECM_Update service may also be utilized by the malicious AMF to tamper UE related information stored in the UDM, which may cause DOS attacks, e.g. if the Radio capabilities for E-UTRAN/NG-RAN FDD and/or TDD is disabled by the a fraudulent update message, the UE will not be able to use these capabilities to visit the network service.

Apart from the Nudm_UECM_Update service, other subsequent procedures (e.g. Nudm_SDM_Get service operation) can also be used by the fraudulent AMF to steal the UE data from the UDM if the authentication result is not removed after deregistration, which means data leakage.

For the third Editor’s Note: security threats are FFS:

Security threats are introduced as following:

In case of UE-initiated deregistration procedure or network-initiated deregistration or SMC failure after AKA during the registration procedure, fraudulent service request can be initated by an attacker to tamper, or steal the UE data, or subscribe the service from the UDM if the authentication result stored in the UDM is not removed. For example, fraudulent Nudm_UECM_ Registration service request can be sent by the attacker, and accepted by the UDM to store the AMF ID as the current AMF which is serving for the UE. Fraudulent Nudm_SDM_Get service may be used by the attacker to steal the data from the UDM.

For the fourth Editor’s Note: Potential security requirements are FFS:

Potential security requirements are introduced as following:

The fradulent attack due to expired authentication result in the UDM shall be mitigated.

4	Detailed proposal

***	BEGIN CHANGES	***

5.3.X	Key Issue #3.X: Key issue on fraudulent attack due to expired authentication result in the UDM

5.3.X.1	Key issue details

TS 33.501 clause 6.1.4 specifies that the authentication result will be stored by the UDM after the successful authentication, and that it can be used to link authentication confirmation to the subsequent procedures, in order to prevent certain types of fraud, e.g. fraudulent Nudm_UECM_Registration Request for registering the subscriber's serving AMF in UDM while the subscriber is not actually present in the visited network.

However, in the case of UE-initiated deregistration procedure or network-initiated deregistration or SMC failure after AKA during the registration procedure, UDM still has the authentication result, which may cause security issues:

Security issue 1:

If the authentication result is not removed after the UE deregistration or SMC failure, the malicious AMF can always send the fraudulent Nudm_UECM_Registration Request to UDM pretending the UE is already connected to it while in fact the UE is not. So the fraudulent Nudm_UECM_Registration request attack still exists and is not mitigated, which means that the purpose of preventing certain types of fraud by increasing the home controlled authentication in 5G is NOT fully achieved.

Security issue 2:

If the authentication result is not removed after UE deregistration or SMC failure, fraudulent Nudm_UECM_Update service may also be utilized by the malicious AMF to tamper UE related information stored in the UDM, which may cause DOS attacks, e.g. if the Radio capabilities for E-UTRAN/NG-RAN FDD and/or TDD is disabled by the a fraudulent update message, the UE will not be able to use these capabilities to visit the network service.

Apart from the Nudm_UECM_Update service, other subsequent procedures (e.g. Nudm_SDM_Get service operation) can also be used by the malicious AMF to steal the UE data from the UDM if the authentication result is not removed after deregistration, which means data leakage.

5.3.X.2	Security threats

In case of UE-initiated deregistration procedure, or network-initiated deregistration, or SMC failure after AKA during the registration procedure, the fraudulent service request can be initated by an attacker to tamper, or steal the UE data, or subscribe the service from the UDM if the authentication result stored in the UDM is not removed. For example, fraudulent Nudm_UECM_ Registration service request can be sent by the attacker, and accepted by the UDM to store the AMF ID as the current AMF which is serving for the UE. Fraudulent Nudm_SDM_Get service may be used by the attacker to steal the data from the UDM.

5.3.X.3	Potential security requirements

The fradulent attack due to expired authentication result in the UDM shall be mitigated.

***	END OF CHANGES	***