|  | INTERNATIONAL TELECOMMUNICATION UNION TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2017-2020 |  |  | SG11-TD988 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | STUDY GROUP 11 |  |  |
|  |  |  |  | Original: English |  |  |
| Question(s): |  |  | 2/11 | Geneva, 16-25 October 2019 |  |  |
| TD |  |  |  |  |  |  |
| Source: |  |  | Editor |  |  |  |
| Title: |  |  | Output – draft Recommendation ITU-T Q.SR-Trust “Signalling requirements and architecture for interconnection between trustable network entities” (Geneva, 16-25 October 2019) |  |  |  |
| Purpose: |  |  | Information |  |  |  |
| Contact: |  | Minrui Shi China Telecom China |  |  | Tel: + 86 21 50440056 E-mail: shimr@sttri.com.cn |

| Keywords: | Trust, requirement, reference point, signalling |
| --- | --- |
| Abstract: | This document is the output draft of Recommendation Q.SR-Trust "Signalling requirements and architecture for interconnection between trustable network entities", as agreed at the Q2/11 meeting, on 16-25 October 2019. |

The following text represents output document of draft Recommendation Q.SR-Trust "Signalling requirements and architecture for interconnection between trustable network entities", as agreed at the Q2/11 meeting, on 16-25 October 2019.

This document is based on SG11-TD37/WP1 (the output of the Q2/11 meeting on June 2019), the outcomes of the discussion of the Q2/11 meeting (16-25 October 2019), including the contributions C316, C335 and final agreements.

| Document Number | Source | Title | Meeting results |
| --- | --- | --- | --- |
| C316 | China Telecom | Proposals for clause 9 "procedures for interconnection between trustable network entities" of Q.SR-trust | Accept with modification |
| C335 | Vaulto Technologies | Proposals for clauses 7 ,8 and 9 of Q.SR-trust | Accept with modification |

Draft Recommendation ITU-T Q.SR-Trust

Signalling requirements and architecture for interconnection
between trustable network entities

# Scope

This draft Recommendation presents the signalling architecture and requirement for interconnection between trustable network entities in support of existing and emerging networks. Based on the architecture, it specifies the interfaces and signalling requirements between the functional entities. It also presents procedures to be applied for the signalling, security consideration, etc.

# References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

| [ITU-T X.509] | Recommendation ITU-T X.509(2016) Information technology – Open Systems Interconnection–The Directory: Public-key and attribute certificate frameworks |
| --- | --- |
| [ITU-T X.1163] | Recommendation ITU-T X.1163(2015) Security requirements and mechanisms of peer-to-peer-based telecommunication networks |

# Definitions

- Terms defined elsewhere

3.1.1 authority [X.509] An entity responsible for the issuance of certificates or of revocation lists.

3.1.2 certification authority (CA) [X.509]: An authority trusted by one or more entities to create and digital sign public-key certificates. Optionally the certification authority may create the subjects' keys.

3.1.3 cross-certificate [X.509]: A certification authority (CA) certificate where the issuer and the subject are different CAs. CAs issue cross-certificates to other CAs as a mechanism to authorize the subject CA's existence

3.1.4 hash function[X.509]: A (mathematical) function which maps data of arbitrary size into data of a fixed size called a digest.

3.1.5 trust [X.1163] the relationship between two entities where each one is certain that the other will behave exactly as it expects

- Terms defined in this Recommendation

3.2.1 security domain: a set of objects or entities of whose security policy can be administered by one organization

3.2.2 signalling security gateway (SSGW): an entity on the borders of the security domains that terminates and initiates secure native signalling/protocols, relays signalling traffic between security domains, configures security parameters or protocols and can perform a security policy management function.

# Abbreviations and acronyms

| AKA | Authentication and key Agreement |
| --- | --- |
| CA | Certificate Authority |
| CLI | Calling Line Identification |
| CSP | Communications Service Providers |
| IAM | Initial Address Message |
| IMSI | International Mobile Subscriber Identity |
| ISUP | ISDN User Part |
| LS | Local Switch |
| MAP | Mobile Application Part |
| NE | Network Entity |
| NNI | Network-Network interface |
| PKI | Public Key Infrastructures |
| SP | Signalling Point |
| SIP | Session Initiation Protocol |
| SMS | Short Message Service |
| SSGW | Signalling Security Gateway |
| STP | Signalling Transit Point |
| TM | Tandem switch |
| UE | User Equipment |
| UNI | User-Network Interface |

# Conventions

TBD

# Overview of interconnection between trustable network entities

Communications service providers (CSPs) have traditionally owned network infrastructure, including access network, core network and service network. UE are connected though UNI which is regarded as untrusted party by network. Many security requirements have been considered and satisfied on the UNI, such as authentication and key agreement (AKA) mechanism which provides authentication and authorization.  The internal network entities of the CSPs are connected though NNI and the relation between network entities are regarded as trusted based on the closedness and isolation of the network. Connection between network entities of different CSPs is also trusted based on the commercial contracts or agreements rather than security technologies. Security measures and policies on NNI are usually not implemented due to this trusted relationship. Nowadays, telecommunications network is more and more open. Customer’s entities are accessed to network through various interfaces and protocols which are used for NNI, e.g. SIP, No.7 signalling and Diameter. Under such circumstances, signalling for controlling and management maybe abused, resulting theft, fraud of sensitive information related to the user, such as CLI, IMSI and location etc. Furthermore, SMS and calls of user will be under surveillance illegally.

Thus, trust relationship should be built between network entities based on appropriate mechanism. The behaviour and results between entities should be predictable, traceable and controllable.

Editor’s Note：The introduction of PKI may be put into the Appendix. FFS.

# Introduction of PKI

Public key cryptography is a cryptographic technique that enables entities to securely communicate on an insecure public network, and reliably verify the identity of an entity via digital signatures. A public key infrastructure (PKI) is a system for the creation, storage, and distribution of digital certificates which are used to verify that a particular public key belongs to a certain entity. PKIs help establish the identity of devices, network entities and services – enabling controlled access to systems and resources, protection of data, and accountability in transactions.

PKI management of public-keys is usually based on the certificate standard X.509 which provides verification of ownership of a private-key by some external entity (certificate authority). The X.509 certificate is defined as a data structure that binds public-key values to subjects (e.g., domain names). The binding is asserted by trusted Certificate Authorities (CA) digitally signing each certificate. The CA may base this assertion by profoundly validating the identification of the private certificate holder

According to X.509, the digital signature signer goes through the following procedure:

1)	The signer creates a hash digest over the PKI/PMI data using a secure hashing algorithm.

2)	The hash digest is then supplemented by additional information in preparation for generation of the digital signature for improved security and for padding the hash digest to a length required by the asymmetric cryptographic function. For the RSA algorithms, that supplementation can be the addition of some information to the hash digest and in some cases, to perform yet another hashing operation. For the DSA and ECDSA signature algorithms, additional domain parameters are added.

3)	The result from item 2) together with the private key of the signer and the use of a specific algorithm result in a bit string that together with the used algorithm constitute the digital signature.

4)	The signature is appended to data to be signed.

Having received the data, the recipient (validator) goes through a similar procedure:

1)	The validator goes through the same procedure as in steps 1) and 2) above, and if the received data is unmodified, the result will be the same as for the signer. If not, the next step will fail.

2)	From the result from item 1) together with the public key of the signer, the bit string of the signature and the use of an associated algorithm, the digital signature is evaluated as either valid or invalid.

If the digital signature proves valid, the validator has ensured that the data has not been modified and that the signer is in the position of the private key that corresponds to the public key used by the validator, i.e., the digital signature provides insurance of data integrity and authentication of the signer.

If the digital signature proves invalid, either the data has been modified or the signing private key does not correspond to the public key used by the validator.

# Cross-certification

When two independent CA hierarchies need to be connected or a sub-CA needs to be created, cross-certification is involved. Cross-certification allows entities from the different hierarchically-certified organizations to access entities in the other organization, and to verify the digital signature of entities from another organization. Different model can be applied to the cross-certification.

# Peer to peer model

Figure 6.1 shows the peer to peer model. In this cross-certifications mode, authorities directly recognise each other. Peer-to-peer cross-certification must occur between CAs. Each CA needs to create an initial trust matrix from its certiﬁcate store and saves it locally. When an authority A chooses to trust an authority B, the authority A signs the certificate of the authority B and distributes the new certificate (B's certificate signed by A) locally.

Figure 6.1 Peer to peer model

# root CA model

Figure 6.2 shows the root CA model. In a strict hierarchy, the CA at the top of the hierarchy is the root CA. The trusted root CA will issue a certificate to subordinate CAs. Depending on the relevant policy, those CAs may certify other CAs. CAs trust each other because the higher CA that certifies it is trusted. Only the root CA must be trusted on its own.

Figure 6.2 Root CA model

# bridge CA model

Figure 6.3 shows the bridge CA model. The bridge CA model is based around a central (bridging) CA which cross-certifies with each CA. It functions as a communication channel between each of the CAs.

This combines aspects of both the root model and the peer to peer model. The bridge CA acts like a bridge between the authorities.  It only requires one pair of cross-certifications for each CA, rather than need to know about each other in a fully meshed system.

Figure 6.3 Bridge CA model

# Architecture for interconnection between trustable network entities

This clause describes the architecture, functional entities and interfaces for interconnection between trustable network entities

# Reference architecture

Figure 7-1: reference architecture of interconnection between trustable network entities

Figure 7-1 shows the reference architecture of interconnection between trustable networks. The CA of a security domain issues certificates to the SSGWs in the domain for communication between domains. When SSGW of the security domain A establishes a secure connection with the SSGW of the domain B, they shall be able to authenticate each other by cross-certificate. NE is the origination or destination of signalling message. The message which should be routed or transited to the other security domain will be delivered to the SSGW. The SSGW signs the messages using the certificate issued by CA and routes the messages to another security domain over the Sa-interface.

Note: security mechanism employed is optional in the interaction between NEs within the security domain.

# Functional entities

# CA

CA issues end entity digital certificates which contain a public key and the identity of the owner to the SSGWs/NEs within a particular security domain. The certificate is also a confirmation or validation by the CA that the public key contained in the certificate belongs to the entity noted in the certificate. CA binds public keys with respective SSGWs/NEs, accepts requests for digital certificates and authenticates the SSGWs/NEs making the request. The binding is established through the registration and issuance process. Depending on the assurance level of the binding, this may be carried out by software at a CA or under human supervision.

# Bridge CA

The bridge CA is a system that facilitates an entity accepting certificates issued by another entity for a transaction. The bridge CA servers as a hub allowing the entity to create a trust path from its security domain back to another security domain of the entity that issued the certificate. Bridge CA provides interoperability by managing tables including policies and procedures created by CAs.

Note: bridge CA is not mandatory, and entities in a PKI may use other methods to determine trust.

The existence of a trusted bridge CA will greatly improve the overall security posture of the system.

Editor’s note: it is recommended that ITU (or another organization licensed by ITU) establish the bridge CA function to ensure the integrity of the PKI key generation process.

# SSGW

SSGW is an entity on the border of security domain and used for communication between two security domains. The security policies are specified in the SSGW. SSGW is responsible for whether protection shall be applied and enforcing security policies towards external domain.

SSGW initials request to CA for issuing or validating certificate. All outgoing messages are protected by the SSGW belong to the domain, including signature, encryption, etc.. Then, the messages are transited to the destination domain. Security of all incoming messages from another domain should be validated by the SSGW, including authentication, validate, decryption and etc. After the messages are validated by a SSGW of the destination domain, the SSGW shall deliver the message to the destination NE. If the message does not comply with the security policy, it will be blocked or discarded by the SSGW.

SSGW shall not be impact message routing.  Therefore, SSGW are stateless at protected level that mean no states of signalling procedure are maintained in the SSGW since the request messages and corresponding response message maybe transited by different SSGW.

SSGW may be standalone or integrated with NE.

# NE

NE is the original or destination of the messages. It generates messages, e.g. ISUP message, MAP message，and receives, interprets and processes the received message. If NE combined with SSGW, NE would have the responsibility enforcing security policies towards other security domains.

# Reference points

# Sa reference point

Sa reference point is located between SSGWs, NEs or SSGW and NE. This reference point is for providing security interconnection, to support protection of domain. The Sa-interface covers all signalling between domains and is mandatory for connecting between operators. The implementation of Sa-interface between SSGW and NE or NEs is optional which is mean that it depends on security policy of operator.

# Sb reference point

Sb reference point is located between SSGW and NE or between NEs within the same security domain. The Sb-interface is optional for implementation.

Sb-interface is legacy interface without security connection, which means that neither NE nor SSGW need mutual authentication each other.

The SSGW shall maintain explicit policy configuration for each NE allowed to communicate with when Sb interface is implemented.

# Sc reference point

Sc reference point is located between SSGW/NE and CA. This reference point is for distribution and validation certificates for NE and SSGW. CA issues certificates to NEs for communication between NEs and between NE and SSGW within the responsible domain . The procedures of Sc are referred to ITU-T Recommendation X.509.

# Sd reference point

Sd reference point is located between CAs in different security domains, this reference point is used in the case there is no bridge CA entity in the architecture to cross validate signatures across different security domains. In the case there is no bridge CA, each individual CA will need to publish its pubic key to other CA in other security domains and subscribe to other CAs publications of public keys.

Editor’s note: we strongly recommend that a centralized trusted bridge CA will be a mandatory part of the reference architecture, thus eliminating the need for this reference point.

# Signalling requirements for interconnection between trustable network entities

# General requirements

In order to build a certification path, the CA must interface with other CAs in different security domains and with the optional bridge CA for cross-certification PKI. And build a certification path to be used in cross-domain digital signature validation.

For signature creation / validation, the SSGW / NE must interface with the CA in order to validate the digital signature of inbound and outbound signalling messages.

For session establishment between NEs (either TCAP, ISUP or Diameter based sessions) the SSGW must interface with other SSGW in different security domains and establish an integrity validation process that precedes the session creation between the NEs. This integrity validation process must be transparent to the NEs and require no SW or HW changes in the NEs, in order to ensure backwards compatibility while providing security.

The PKI process must comply with IETF X.509 standard.

The key management of the PKI must be secured according to the latest EU GDPR directives for key management and secure storage.

# Requirements for SSGW

The signalling between SSGWs (security domains) shall provide:

-	signalling data integrity;

-	data origin authentication;

Each SSGW has keys associated with the entity for signing and validation by the SSGW received. Digital signature derive from the key and hash of data shall be included in the sending message for the receiver to validate the sender. Execution of signalling process at receive side occurs only when the sender is validated.

The Sa reference point should allow information exchange as follows:

| Information element | Description | Category |
| --- | --- | --- |
| Original information | The original payload should be verified by receiver | M |
| Digital signature | the special algorithm result from private key and data (original information) hashing |  |

The information elements are sent together with other IEs received from the NEs, e.g. IE1, IE2 in the message on the Sa interface.

Figure 8-1 shows the information flow between the SSGWs through the Sa interface.

Figure 8-1 – Information flow

This procedure is used to push digital signature information from the sender SSGW to the other SSGWs in another security domain. This information flow occurs on the connection between the two SSGWs that afford trustable signalling traffic.

# Procedures for interconnection between trustable network entities

# CA High level functions

# CA PKI key-pair provisioning

The CA function shall create and store its own PKI key pair using a secure method as warranted in the signalling requirements section above. The private key will be securely stored on the CA function or another compliant secure alcove local or external to the CA function. The public key will be published to other CAs via the bridge CAs in the certification path building function.

# Build CA certification path

# Local certification path

Every CA shall sign and validate the digital signatures of messages internal to its own security domain, in this case the certification path will be 0, i.e. the CA will validate signatures without requiring cross-validation from the bridge CA / other CAs in other security domains.

# Cross-domain certification path

In the case of outbound signalling messages to other security domains (i.e. other PLMNs) the CA will build a certification path to the other security domain via the bridge CA or directly via peer-CAs if there is no bridge CA present. The certification path is the chaining of public keys of every peer-CA or bridge CA in the path between the two security domains. Once the path is built the CA will store the chained keys and publish them to the CA in the other security domain.

# Generation and validation of signatures in the same security domain

In the same security domain, the digital signature will be based on the domain parameters and the CA’s private key. The validation of the digital signature will be performed using the security domain parameters and the CA’s public key.

# Generation of signatures cross security domains

In the cross-security domain scenario, the digital signature will be based on the same domain parameters as in the intra-security domain scenario, however, the public key will be built according to the certification path between the two security domains. The validation of of the digital signature will be performed using the security domain parameters and the chained public key (chained according to the CAs in the certification path).

## 9.2 Security policy of SSGW

Security policy shall be specified and maintained by the SSGW for a particular network. The requirements of policy are required as follows:

The policy shall define if it allows traffic without signature from another security domain as well as traffic with signature.

The policy shall define actions, such as passing, block or de-signature, to be applied for each security domain the SSGW communicate with. When pass mode is applied, signalling messages are transferred transparently.

The policy should define each security domain that is allowed to communicate with.

## 9.3 signalling procedures of SSGW

SSGW performs message signing, signature verify, signature remove, passing unmodified and blocking  depending on signature, the message's origin, the message's destination and the message's direction.

If the message in coming from NEs and the destination is outside the security domain, which means signalling traffic is from inside to outside, the SSGW shall decode the original received message, added signature, re-package and send the signed message to another SSGW according to the security policy.

If the message is coming from SSGW in another security domain and the destination is NEs in the same security domain which means signalling traffic is from outside to inside, the SSGW shall verify the signature. If the signature is validated, the message shall be transited to the NEs and the signature shall be removed, otherwise the message shall be blocked according to the security policy.

If the message is coming from another SSGW and the destination is another security domain, which means signalling traffic is from outside to outside, the SSGW shall not verify the signature and pass the message transparently according to the security policy.

Figure 9-1 signalling procedures of SSGW

# Security considerations

TDB

_______________________