3GPP TSG-SA3 Meeting # 97	S3-194347

# Reno,US, 18 - 22 November 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.515 | CR | 0004 | rev | - | Current version: | 16.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for clean-up and alignment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G |  |  |  |  |  | Date: |  |  | 2019-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The requriement reference (TS 33.501) in clause 4.2.2.1.3 is not in the reference list and needs to be added. The requirement description in sub-clause 4.2.2.1.3 needs to be updated according to 33.501 v15.6.0. The sub-clause 4.2.7 is not necessary and not aligned with the structure in TS 33.117 R16, hence needs to be removed. The heading of sub-clauses in 4.3.6 is not aligned with that in TS 33.117 R16. The clause numbers of TR 33.926 referenced in the test cases need to be updated. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added the entry for TS 33.501 in the reference list. - Updated the requirement description in sub-clause 4.2.2.1.3 based on TS 33.501 v15.6.0. - Updated the clause numbers of TR 33.926 referenced in the test cases for alignment. - Removed sub-clause 4.2.7. - Updated the heading of sub-clause 4.3.6 for alignment with that in TS 33.117 R16. - Other editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing reference and mis-alignment |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.1.1, 4.2.2.1.2, 4.2.2.1.3, 4.2.2.1.4, 4.2.7, 4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of the 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 23.501: "System Architecture for the 5G System".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 23.060: "General Packet Radio Service".

[4]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U) ".

[6]	3GPP TS 32.255: "Charging Management; 5G Data Connectivity Domain Charging".

[7]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 33.501 (Release 15): "Security architecture and procedures for 5G system".

*************** Start of the 2nd Change ****************

## 4.2 	SMF-specific security functional adaptations of requirements and related test cases

### 4.2.1	Introduction

Present clause contains SMF-specific security functional adaptations of requirements and related test cases.

### 4.2.2	Security functional requirements on the SMF deriving from 3GPP specifications and related test cases

#### 4.2.2.1	Security functional requirements on the SMF deriving from 3GPP specifications

##### 4.2.2.1.1	Priority of UP security policy

Requirement Name: Priority of UP security policy

Requirement Reference: TS 23.501 [1], clause 5.10.3

Requirement Description: "User Plane Security Policy from UDM takes precedence over locally configured User Plane Security Policy." as specified in TS 23.501 [1], clause 5.10.3

Threat References: TR 33.926 [4], clause J.2.2.1 Non-compliant UP security policy handling

Test Case:

Test Name: TC_UP_POLICY_PRECEDENCE_SMF

Purpose:

Verify that the user plane security policy from the UDM takes precedence at the SMF under test over locally configured user plane security policy.

Pre-Conditions:

Test environment with AMF and UDM may be simulated.

Both UDM and SMF under test are configured with UP security policy, and the UP security policies are different.

There is no Session Management Subscription data in SMF.

Execution Steps

1)	The tester triggers PDU session establishment procedure by sending Nsmf_PDUSession_CreateSMContext Request message to the SMF.

2)	The SMF under test retrieves the Session Management Subscription data using Nudm_SDM_Get service from UDM, where the Session Management Subscription data includes the user plane security policy stored in UDM.

3)	The tester captures the Namf_Communication_N1N2MessageTransfer message sent from the SMF under test to the AMF.

Expected Results:

There is a Security Indication IE in the N2 SM information contained in the Namf_Communication_N1N2MessageTransfer message, which is the same with the UP security policy configured in the UDM.

Expected format of evidence:

Evidence suitable for the interface, e.g., Screenshot containing the operational results.

##### 4.2.2.1.2	TEID uniqueness

Requirement Name: TEID uniqueness.

Requirement Reference: TS 23.501[1], clause 5.8.2.3.1, TS 29.281 [5], clause 5.1, TS 23.060 [3], clause 14.6

Requirement Description:

"Allocation and release of CN Tunnel Info is performed when a new PDU Session is established or released. This functionality is supported either by SMF or UPF, based on operator's configuration on the SMF" as specified in TS 23.501[1], clause 5.8.2.3.1.

"Tunnel Endpoint Identifier (TEID): This field unambiguously identifies a tunnel endpoint in the receiving GTP-U protocol entity. The receiving end side of a GTP tunnel locally assigns the TEID value the transmitting side has to use" as specified in TS 29.281[5], clause 5.1.

"The TEID is a unique identifier within one IP address of a logical node." as specified in TS 23.060 [3], clause 14.6.

Threat References: TR 33.926 [4], clause J.2.2.2, Failure to assign unique TEID for a session.

TEST CASE:

Test Name: TC_TEID_ID_UNIQUENESS_SMF

Purpose:

Verify that the TEID generated by SMF under test for each new GTP tunnel is unique per PDU session.

Pre-Conditions:

Test environment is set up with a UPF, which may be real or simulated. The tester is able to trace traffic between the SMF under test and the UPF. The SMF under test is configured to generate the TEIDs.

Execution Step

1)	The tester intercepts the traffic between the UPF and the SMF under test.

2)	The tester triggers the maximum number of concurrent N4 session establishment requests that the SMF under test can handle.

3)	The tester captures the N4 session establishment requests sent from the SMF under test to the UPF and verifies that the F-TEID created in each generated request is unique.

Expected Results:

The F-TEID set in each different N4 session establishment request is unique.

Expected format of evidence:

Files containing the triggered GTP messages (e.g. pcap trace).

##### 4.2.2.1.3	Security functional requirements on the SMF checking UP security policy

Requirement Name: UP security policy check.

Requirement Reference: TS 33.501 [x], clause 6.6.1

Requirement Description:

"The SMF shall verify that the UE's UP security policy received from the target ng-eNB/gNB is the same as the UE's UP security policy that the SMF has locally stored. If there is a mismatch, the SMF shall send its locally stored UE's UP security policy of the corresponding PDU sessions to the target gNB. This UP security policy information, if included by the SMF, is delivered to the target ng-eNB/gNB in the Path-Switch Acknowledge message. The SMF shall logging capabilities for this event and may take additional measures, such as raising an alarm. "

Threat References: TR 33.926 [4], clause J.2.2.4, Unchecked UP security policy

TEST CASE:

Test Name: TC_UP_SECURITY_POLICY _SMF

Purpose:

Verify that the SMF checks the UP security policy that is sent by the ng-eNB/gNB during handover.

Pre-Conditions:

The SMF under test is preconfigured with a UE UP security policy.

Execution Step1. The tester sends the Nsmf_PDUSession_SMContextUpate Request message to the SMF under test. A UE UP security policy different than the one preconfigured at the SMF under test is included in the Request message.

2. The tester captures the Nsmf_PDUSession_SMContextUpdate Response message sent from the SMF under test.

Expected Results:

. The preconfigured UE security policy is contained in the ‘n2SmInf’ IE in the captured Response message.

Expected format of evidence:

Files containing the triggered GTP messages (e.g. pcap trace).

##### 4.2.2.1.4	Charging ID Uniqueness

Requirement Name: Charing ID uniqueness.

Requirement Reference: TS 32.255 [6], clause 5.1.2

Requirement Description:

"-	The SMF shall support PDU session charging using service based interface.

-	The SMF shall collect charging information per PDU session for UEs served under 3GPP access and non-3GPP access.

- Every PDU session shall be assigned a unique identity number for billing purposes per PLMN. (i.e. the Charging Id). "

Threat Reference: TR 33.926 [4], clause J.2.2.3, "Failure to assign unique Charging ID for a session"

TEST CASE:

Test Name: TC_CHARGING_ID_UNIQUENESS_SMF

Purpose:

Verify that the charging ID generated by the SMF for each PDU session is unique.

Pre-Conditions:

Test environment is set up with a Charging Function (CHF), which may be real or simulated, and the SMF under test. The tester is able to capture the traffic between the SMF under test and the CHF.

Execution Step

1)	The tester intercepts the traffic between the SMF under test and the CHF.

2)	The tester triggers the establishment of the maximum number of concurrent PDU sessions that the SMF under test can handle.

3)	The tester captures each Charging Data Request [initial] sent from the SMF under test to the CHF, and verifies the charging ID contained in the ‘PDU Session Charging Information’ IE in each Charging Data Request [initial] is unique.

Expected Results:

The charging ID in each Charging Data Request [initial] is unique.

Expected format of evidence:

Files containing the Charging Data Request [initial] messages (e.g. pcap trace).

### 4.2.3	Technical Baseline

#### 4.2.3.1	Introduction

The present clause provides baseline technical requirements.

#### 4.2.3.2	Protecting data and information

##### 4.2.3.2.1	Protecting data and information – general

There are no SMF-specific additions to clause 4.2.3.2.1 of TS 33.117 [2].

##### 4.2.3.2.2	Protecting data and information – unauthorized viewing

There are no SMF-specific additions to clause 4.2.3.2.2 of TS 33.117 [2].

##### 4.2.3.2.3	Protecting data and information in storage

There are no SMF-specific additions to clause 4.2.3.2.3 of TS 33.117 [2].

##### 4.2.3.2.4	Protecting data and information in transfer

There are no SMF-specific additions to clause 4.2.3.2.4 of TS 33.117 [2].

##### 4.2.3.2.5	Logging access to personal data

There are no SMF-specific additions to clause 4.2.3.2.5 of TS 33.117 [2].

#### 4.2.3.3	Protecting availability and integrity

There are no SMF-specific additions to clause 4.2.3.3 of TS 33.117 [2].

#### 4.2.3.4	Authentication and authorization

There are no SMF-specific additions to clause 4.2.3.4 of TS 33.117 [2].

#### 4.2.3.5	Protecting sessions

There are no SMF-specific additions to clause 4.2.3.5 of TS 33.117 [2].

#### 4.2.3.6	Logging

There are no SMF-specific additions to clause 4.2.3.6 of TS 33.117 [2].

### 4.2.4	Operating Systems

There are no SMF-specific additions to clause 4.2.4 of TS 33.117 [2].

### 4.2.5	Web Servers

There are no SMF-specific additions to clause 4.2.5 of TS 33.117 [2]

### 4.2.6	Network Devices

There are no SMF-specific additions to clause 4.2.6 of TS 33.117 [2].

### 4.2.7	Void

*************** Start of the 3rd Change ****************

## 4.3	SMF-specific adaptations of hardening requirements and related test cases

### 4.3.1	Introduction

The present clause contains SMF-specific adaptations of hardening requirements and related test cases.

### 4.3.2	Technical Baseline

There are no SMF-specific additions to clause 4.3.2 of TS 33.117 [2].

### 4.3.3	Operating Systems

There are no SMF-specific additions to clause 4.3.3 of TS 33.117 [2].

### 4.3.4	Web Servers

There are no SMF-specific additions to clause 4.3.4 of TS 33.117 [2].

### 4.3.5	Network Devices

There are no SMF-specific additions to clause 4.3.5 of TS 33.117 [2].

### 4.3.6	Network Functions in service-based architecture

There are no SMF-specific additions to clause 4.3.6 of TS 33.117 [2].

*************** End of the Changes ****************