3GPP TSG-SA3 Meeting #97 	S3-194610

# Reno (Nevada), 18-22 November 2019						                             was S3-194400

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.501 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing ENs in annex X in the living document for 5WWC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CableLabs, Charter Communications, Rogers Communications, Nokia, Nokia Shanghai Bell, Lenovo, Motorola Mobility, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2019-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Removing editor notes, and correcting some minor errors |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed two ENs, corrected errors in Figure X.3-1., and some minor editorial changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the procedure in Annex X would be neither complete nor accurate. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************* Start of change  **************************

Annex X (Informative):
Authentication for non-5G capable devices behind residential gateway

# X.1 	General

This annex describes the authentication procedure, using EAP-TLS as an example, for Non-5G Capable (N5GC) devices behind Residential Gateways (RGs) in an isolated 5G networks (i.e., roaming is not considered) with wireline access. The registration procedure of N5GC devices is described in TS 23.316 [xx].

X.2 	Baseline for using non-5G capable devices with 5GC

N5GC devices lack some key 5G capabilities, including NAS and the derivation of 5G key hierarchy. Those devices exist in wireline networks and need to be able to access the converged 5G core. The authentication of N5GC devices can be based on additional EAP methods other than EAP-AKA’. The procedure in X.3 uses EAP-TLS in Annex B as an example, but it differs from the Annex B in the following:

a) the W-AGF creates the registration request on behalf of the N5GC device,

b) AMF/SEAF does not send any 5G related parameters (including ngKSI and ABBA) to the N5GC device, and

c) Neither the N5GC device nor the AUSF derive any 5G related keys after EAP authentication.

X.3		Authentication Procedure

Figure X.3-1 shows the details of the authentication procedure as part of the initial registration procedure specified in TS 23.316 [xx] following the principles listed in clause X.2. It uses EAP-TLS as an example, but other EAP methods can also be supported.

NOTE1: W-AGF acts on behalf of N5GC device during the registration process.

NOTE2: The link between the N5GC device and the RG, and between the RG and the W-AGF can be any data link (L2) that supports EAP encapsulation.

Figure X.3-1: Registration and authentication of a non-5G capable device to the 5GC

In the following, the procedure for registration and authentication of an N5GC device to the 5GC are is described:

1.  The N5GC device connects to W-AGF via a RG which is configured as a layer 2 bridge. W-AGF is associated with a 5GC and acts on behalf of the N5GC device during its registration process.

2a. W-AGF initiates EAP authentication procedure by sending an EAP request/Identity to the N5GC device via the RG, which acts as an L2 bridge device and forwards the ethernet frame to the N5GC device. The EAP message is encapsualted inside an L2 frame (e.g., EAPOL).

2b. In response, the N5GC device sends back an EAP response/Identity including its Network Access Identifier (NAI) in the form of username@realm.

NOTE 3: If subscription identifier privacy protection is supported, the "username" part must be is either omitted or "anonymous".

3.  W-AGF creates a registration request on behalf of the N5GC device with an indication of "N5GC device". The SUPI of the N5GC device is in the form of NAI, and W-AGF constructs SUCI from SUPI using the NULL scheme.

Editor's Note: SUPI/SUCI related aspects need to be clarified.

4a. W-AGF selects the AMF/SEAF.

4b. W-AGF sends to AMF/SEAF a registration request on behalf of the N5GC device. The registration request includes the NAI SUCI, wireline network name if available, and a device capability indicator (e.g., the device is non-5G capable).

4c. The AMF/SEAF selects the AUSF based the SUCI in the received registration request and sends a Nausf_UEAuthentication_Authenticate Request message to the AUSF. It contains the SUCI of the N5GC device, and an indicator of the N5GC device.AMF/SEAF sends an UE authentication request to the AUSF.

5a. The AUSF sends a Nudm_UEAuthentication_Get Request to the UDM. It contains the SUCI of the N5GC device.AUSF sends an UE authentication Get Request to UDM.

5b. The UDM invokes the SIDF to map the SUCI to the SUPI and selects an authentication method, e.g., EAP-TLS, based on the SUPI.UDM selects an authentication method, e.g., EAP-TLS.

5c. The UDM sends a Nudm_UEAuthentication_Get Response to the AUSF, which contains the SUPI of the N5GC device and an indicator of the selected authentication method, e.g., EAP-TLS. UDM responds to AUSF with the selected authentication method.

Steps 6-14.

The selected authentication method, e.g., EAP-TLS, is executed between the AUSF and the N5GC device with AMF/SEAF and W-AGF functioning as the transparent EAP authenticator. Upon a successful EAP authentication, EAP-related keying materials including MSK and EMSK are derived by both the AUSF and N5GC device.

Editor's Note: Steps 6-14 need to be specified further.

NOTE 4: Steps 6a-14c describe the exchange of EAP-TLS between the AUSF and the N5GC device, based on TLS 1.2 without subscription identifier privacy protection. To support subscription identifier privacy protection in TLS 1.2, clause 2.1.4 in RFC 5216 applies. TLS 1.3 provides native protection of privacy since the client certificate is sent in an encrypted format.

6a.  The AUSF starts EAP-TLS by sending to the AMF/SEAF a Nausf_UEAuthentication_Authenticate Response message containing an EAP-Request message of EAP-type=EAP-TLS with the Start (S) bit set, denoted as EAP-Request/EAP-TLS [TLS start].

6b.	The AMF/SEAF forwards the EAP-Request/EAP-TLS [TLS start] in the Authentication Request message to the W-AGF. The AMF/SEAF does not send ngKSI or ABBA to W-AGF based on the indicator of an N5GC device in this step and all subsequent messages to W-AGF.

6c. After receiving the EAP-Request/EAP-TLS [TLS start] message from AMF/SEAF, the W-AGF forwards the EAP-Request/EAP-TLS [TLS start] message to the N5GC device in an L2 message.

7a. After receiving the EAP-Request/EAP-TLS [TLS start] message from the W-AGF, the N5GC device replies to the W-AGF with an EAP-Response/EAP-TLS message whose data field encapsulates a TLS client_hello message. Such EAP-Response message, denoted as EAP-Response/EAP-TLS [client_hello], is encapsulated in an L2 message.

7b.	The W-AGF forwards the EAP-Response/EAP-TLS [client_hello] to the AMF/SEAF in an Authentication Response message.

7c.	The AMF/SEAF forwards the EAP-Response/EAP-TLS [client_hello] message to AUSF in a Nausf_UEAuthentication_Authenticate Request message.

8a.	The AUSF replies to the AMF/SEAF with EAP-Request/EAP-TLS message whose data field encapsulates a TLS server_hello message, a TLS server certificate message, a TLS server_key_exchange message, a TLS client certificate_request message, and a TLS server_hello_done message. Such EAP-Request message, denoted as EAP-Request/EAP-TLS [server_hello], is encapsulated in a Nausf_UEAuthentication_Authenticate Response message.

8b.	The AMF/SEAF forwards the EAP-Request/EAP-TLS [server_hello] message to the W-AGF in an Authentication Request message.

8c. The W-AGF forwards the EAP-Request/EAP-TLS [server_hello] to the N5GC device in an L2 message.

9.	The N5GC device authenticates the AUSF by validating the server certificate included in the EAP-Request/EAP-TLS [server_hello] message received in step 8c. The N5GC device needs to be provisioned with certificates of trust anchor to validate the AUSF server certificate. In addition, the N5GC device also needs to be provisioned with its own client certificate.

10a. If the TLS server authentication is successful, then the N5GC device replies to the W-AGF with EAP-Response/EAP-TLS in an L2 message. The data field of the EAP-Response/EAP-TLS message contains a TLS client certificate message, a TLS client_key_exchange message, a TLS certificate_verify message, a TLS change_cipher_spec message, and TLS finished message. This EAP-Response message is denoted as EAP-Response/EAP-TLS [client_certificate].

10b. The W-AGF forwards to the AMF/SEAF the EAP-Response/EAP-TLS [client_certificate] in an Authentication Response message.

10c. The AMF/SEAF forwards the EAP-Response/EAP-TLS [client_certificate] message to the AUSF in a Nausf_UEAuthentication_Authenticate Request message.

11.	The AUSF authenticates the N5GC device by verifying the client certificate received in the EAP-Response/EAP-TLS [client_certificate] message. Among other validations, the AUSF verifies that the client certificate must is be issued by a certificate authority trusted by the AUSF. If the client certificate is verified successfully, the AUSF continues to step 12a. Otherwise the AUSF returns an EAP-failure message. The AUSF needs to be provisioned with certificates of trust anchor to verify client certificates.

12a. The AUSF sends to the AMF/SEAF an EAP-Request/EAP-TLS message with its data field encapsulating a TLS change_cipher_spec message and a TLS server finished. This EAP-Request message, denoted as EAP-Request/EAP-TLS [server_finished], is encapsulated in a Nausf_UEAuthentication_Authenticate Response message.

12b. The AMF/SEAF forwards EAP-Request/EAP-TLS [server_finished] message to the W-AGF in an Authentication Request message.

12c. The W-AGF forwards EAP-Request/EAP-TLS [server_finished] message to the N5GC device in an L2 message.

13a. The N5GC sends to the W-AGF an EAP-Response/EAP-TLS message with its data field set to empty, denoted as EAP-Response/EAP-TLS [empty], in an L2 message

13b. The W-AGF forwards to the AMF/SEAF the EAP-Response/EAP-TLS [empty] message in an Authentication Response message.

13c. The AMF/SEAF forwards the EAP-Response/EAP-TLS [empty] message to the AUSF in a Nausf_UEAuthentication_Authenticate Request message.

14a. The AUSF sends to the AMF/SEAF an EAP-Success message along with the SUPI in a Nausf_UEAuthentication_Authenticate Response message. AUSF does not perform the derivation of KAUSF nor KSEAF based on the indicator of an N5GC device, nor send KSEAF to AMF/SEAF.

14b. The AMF/SEAF forwards to the W-AGF the EAP-Success message in an Authentication Result message or a Security Mode Command message.

14c. The W-AGF forwards to the N5GC device the EAP-Success message in an L2 message, and the authentication procedure is finished.

NOTE5:  Neither the AUSF nor the N5GC device performs further 5G related key derivation from EMSK, since neither 5G AS nor 5G NAS security are used in this setting.

15. The AMF sends Registration Accept message to the W-AGF.

******************* End of change  **************************