| 3GPP TS 33.535 V0.2.0 (2019-11) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Authentication and key management for applications; based on 3GPP credential in 5G （AKMA） (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	6

4	Architecture for Authentication and Key Management for Applications (AKMA)	7

4.1	Reference model	7

4.2	Network elements	7

4.2.1	AAnF	7

4.3	Interface description	7

4.3.1	Reference point Ua*	7

4.4	Security requirements and principles for AKMA	8

5	Key Management	8

5.1	AKMA key hierarchy	8

5.2	AKMA key lifetimes	9

6	AKMA Procedures	9

6.1	Deriving AKMA key during UE registration	9

6.2	Deriving AKMA Application key for a specific AF	10

Annex <X> (informative): Change history	11

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

1	Scope

The present document specifies the security features and mechanisms to support authentication and key management aspects for applications based on subscription credential(s) in 5G system as defined in 33.501[2].

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AAnF	AKMA Anchor Function

AF	Application Function

AMF	Access and Mobility Management Function

AUSF	AUthentication Server Function

NEF	Network Exposure Function

4	Architecture for Authentication and Key Management for Applications (AKMA)

Editor’s Note: Titles and Numberings of sub clauses could be adjusted according to the content.

4.1	Reference model

Figure 4.1-1 shows a fundamental network model of AKMA, as well as the interfaces between them.

Figure 4.1-1: Fundamental Network Model for AKMA

NOTE: Figure 4.1-1 shows the case where AAnF is deployed as a standalone function. Deployments can choose to collocate AAnF with AUSF or with NEF according to operators’ deployment scenarios.

The AKMA service requires a new logical entity: AKMA Anchor Function (AAnF).

AAnF is the anchor function in the HPLMN that generates the key material to be used between the UE and the AF and maintains UE AKMA contexts to be used for subsequent bootstrapping requests.

4.2	Network elements

Editor’s Note: This clause will describe the functionalities and requirements of each entities in the reference model.

4.2.1	AAnF

AAnF enables the AKMA anchor key (KAKMA) derivation for AKMA service. Before invoking AKMA service, UE shall have successfully registered to the 5G core, which results in KAUSF being stored at the AUSF and the UE after a successful 5G primary authentication.

4.3	Interface description

The following interfaces are involved in AKMA network architecture:

- Nnef: Service-based interface exhibited by NEF.

- Nausf: Service-based interface exhibited by AUSF.

- Naanf: Service-based interface exhibited by AAnF.

- Naf: Service-based interface exhibited by AF.

The AAnF interacts with the AUSF and the AF using Service-Based Interfaces.

4.3.1	Reference point Ua*

The reference point Ua* carries the application protocol, which is secured using the key material agreed between UE and AAnF as a result of successful AKMA procedures.

Editor’s Note: It is expected that the existing GBA based Ua protocols are reused with necessary adaptations as the Ua* for AKMA. The changes required for Ua* are FFS.

4.4	Security requirements and principles for AKMA

The following security requirements are applicable to AKMA:

-    AKMA shall reuse the same UE subscription and the same credentials used for 5G access.

-    AKMA shall reuse the 5G primary authentication procedure and methods (both 5G AKA and EAP AKA’ shall be supported) for the sake of implicit authentication for AKMA services.

-    AAnF’s SBI interface to AUSF shall be confidentiality, integrity and replay protected.

-     The interface between AAnF and AF shall be confidentiality, integrity and replay protected.

-    The application key shall be provided with a maximum lifetime. When the application key lifetime is expired, it shall be renegotiated.

Editor’s Note: Further security requirements regarding roaming and other aspects will be added.

5	Key Management

Editor’s Note: Sub clauses will be added according to the conclusions made in TR 33.835.

5.1	AKMA key hierarchy

Editor’s Note: This clause needs further details.

The key hierarchy (see Figure 5.1-1) includes the following keys: KAUSF, KAKMA, KAF. KAUSF is generated by AUSF as specified in clause 6 of TS 33.501 [2].

Keys for AAnF:

KAKMA is a key derived by ME and AUSF from KAUSF.

Keys for AF:

KAF is a key derived by ME and AAnF from KAKMA.

KAKMA and KAF are derived according to the procedures of clause 6.1 and 6.2.

Figure 5.1-1: AKMA Key Hierarchy

5.2	AKMA key lifetimes

The anchor key KAKMA shall use the implicit lifetime and the application key KAF shall use explicit lifetimes based on operator’s policy. In case that a new anchor key KAKMA is established, the application key KAF can continue to be used until their lifetimes expire. When the KAF lifetime expires, a new application key is established based on the anchor key KAKMA. Because of the implicit lifetime for anchor key KAKMA, key negotiation procedure is only applicable for application key KAF.

Editor’s Note: the meaning of explicit lifetime and implicit lifetime is FFS.

6	AKMA Procedures

6.1	Deriving AKMA key during UE registration

There is no separate authentication of the UE to support AKMA functionality. Instead, it reuses the 5G primary authentication procedure executed during the UE Registration to authenticate the UE. A successful 5G primary authentication results in KAUSF being stored at the AUSF and the UE.

Figure 6.1-1 Deriving AKMA root key during UE registration

The UE and the AUSF shall generate the AKMA Anchor Key (KAKMA) and the associated key Identifier from KAUSF as part of the UE Registration procedure.

The KAKMA key identifier identifies the KAKMA key of the UE from which other AKMA keys are derived.

Editor’s Note: Derivation of KAKMA is FFS.

Editor’s Note: Format and derivation of KAKMA key identifier and its association with UE identifier is FFS. Whether the key identifier is generated during the primary authentication or as needed, is FFS.

Since AKMA keys are based on KAUSF from primary authentication run, the AKMA keys can only be refreshed by running a fresh primary authentication.

6.2	Deriving AKMA Application key for a specific AF

Figure 6.2-1 AF Key generation from KAKMA

When the UE initiates communication with the AKMA AF, it shall include the derived AKMA key identifier in the message (cf. clause 6.1).

If the AApF does not have an active context associated with the key identifier, then the AF sends a request to AAnF with the key identifier to request application function specific AKMA keys for the UE.  The AF also includes its identity (AF Id) in the request.

If the AAnF is in possession of the AF specific key (KAF), it responds to the AF with the KAF key. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the AKMA key identifier.

If KAKMA is available in AAnF, it shall derive the AF specific AKMA key (KAF) from KAKMA and respond to the AF with KAF and lifetime.

If KAKMA is not available, the AAnF shall send a request to the AUSF to obtain the KAKMA key specific to the UE. It includes the AKMA key identifier in the request. The AUSF shall respond with the KAKMA key identified by the key identifier. The AAnF derives the AF specific key (KAF) from KAKMA and responds to the AF with KAF and lifetime.

Editor’s Note: Derivation of the AF specific KAF is FFS.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | SA3 #96adhoc | S3-193817 |  |  |  | TS skeleton based on S3-193769; Scope is based on S3-193770; Other content including S3-193841, S3-193772 | 0.1.0 |
| 2019-11 | SA3 #97 | S3-194640 |  |  |  | Updates based on S3-194340, S3-194160, S3-194641, S3-194642, S3-194643, S3-194341, S3-194644, S3-194645, S3-194229, S3-194156 | 0.2.0 |