3GPP TSG-SA3 Meeting #99e 	  S3-200978

# e-meeting, 11 – 15 May 2020	revision of S3-20wxyz

# Source:	ZTE

# Title:	Editorial correction to solution#12

# Document for:	Approval

Agenda Item:	5.5

1	Decision/action requested

This contribution proposes editorial correction to solution #12 of TR 33.813.

2	References

[1]	3GPP TR 33.813 “Study on Security Aspects of Enhanced Network Slicing (Release 16)”

3	Rationale

The figure in solution#12 is updated, and some literal errors is corrected.

4	Detailed proposal

******	BEGIN CHANGES	******

7.12	Solution #12 Privacy protection of NSSAI

### 7.12.1	Introduction

This solution addresses the Key Issue #6 Confidentiality protection of NSSAI and home control.

This solution is based on the notion of temporary S-NSSAI (T-S-NSSAI) as in solution #8. In this solution, the AMF generates the T-S-NSSAIs based on the S-NSSAIs supported by the PLMN and stores the list of mapping between the S-NSSAIs and the T-S-NSSAI in the AMF. The NG-RAN gets and stores a list of mapping of S-NSSAI and T-S-NSSAI supported by the PLMN from the AMF in a NG Setup Response during an NG Setup procedure. UE are offered a temporary NSSAIs (T-S-NSSAIs) instead of cleartext S-NSSAIs in registration procedure. After that, to preserve the privacy, when UE use the T-S-NSSAI to access the slice, AMF will generate a new T-S-NSSAI for the allowed S-NSSAI and allocate for the UE.

The T-S-NSSAI needs to be updated based on predefined policies when it is used to ensure the privacy.

### 7.12.2		Solution details

Figure 7.12.2-1 illustrates this solution

NG-RAN

AMF

UE

Provisioning

1.Registration has been performed successfully and 5G NAS established. NG-RAN gets and stores a list of mapping of S-NSSAI and T-S-NSSAI supported by the PLMN

2. Identify the T-S-NASSAIs for allowed S-NSSAIs for the UE. Obtains the {allowed S-NSSAIs,T-S-NSSAIs}tuple.

3.Registration accept.({allowed S-NSSAIs,  T-S-NSSAIs}tuple...)

Utilizing and  Re-provisioning

4. Slice Registration Request(T-S-NSSAI).NG-RAN identify the allowed S-NSSAI from the T-S-NSSAI and select the target AMF.

5. Generate and allocate the new T-S-NASSAI for the allowed S-NSSAI for the UE based on predefined policies. Cancel the old T-S-NSSAI. Store the {allowed S-NSSAI, new T-S-NSSAI }tuple.

6. Slice Registration Accept({allowed S-NSSAI, new T-S-NSSAI}tuple...)

Figure 7.12.2-1: Privacy protection of NSSAI

- The AMF generates the T-S-NSSAIs based on the S-NSSAIs supported by the PLMN and stores the list of mapping of the S-NSSAIs and the T-S-NSSAI in the AMF.

1.	The UE has registered successfully to a PLMN and 5G NAS security context has been created, and NG-RAN gets and stores a list of mapping of S-NSSAI and T-S-NSSAI supported by the PLMN from the AMF in a NG Setup Response during an NG Setup procedure .

2.	In the registration procedure, AMF identifies T-S-NSSAIs according to allowed S-NSSAIs based on the mapping of S-NSSAIs and T-S-NSSAIs. Then, AMF obtains the {allowed S-NSSAIs, T-S-NSSAIs} tuple.

3.	In the registration/service accept message, {allowed S-NSSAIs, T-S-NSSAIs} tuple will be sent to UE instead of cleartext S-NSSAIs.

4.	In a slice registration request message, UE transmits T-S-NSSAI to NG-RAN, NG-RAN identify the allowed S-NSSAI from the T-S-NSSAI based on the mapping of the NSSAI and T-NSSAI in the NG-RAN, and select the target AMF.

5. 	AMF calculates a new T-S-NSSAI according to the allowed S-NSSAI based on the predefined policies. Then AMF cancel the old T-S-NSSAI, and stores the {allowed S-NSSAI, new T-S-NSSAI} tuple.

6.	In the Slice registration Accept message, the new T-S-NSSAI will be sent to UE.

7. AMF indicate NG-RAN to update the stored mapping of NSSAI and old T-NSSAI with the new mapping of  NSSAI and new T-NSSAI.

Editor’s Note: Idle mode mobility need to be addressed and solved.

### 7.12.3	Evaluation

The solution #12 addresses the key issue #6, the solution introduce a notion of temporary T-S-NSSAI similar as introduced in existing Solution #8. The AMF generates the T-S-NSSAIs based on the S-NSSAIs supported by the PLMN and stores the list of mapping between the S-NSSAIs and the T-S-NSSAI in the AMF. The NG-RAN gets and stores a list of mapping of S-NSSAI and T-S-NSSAI supported by the PLMN from the AMF. The UE gets and maintains a list of mapping between allowed S-NSSAIs and T-S-NSSAIs. The T-S-NSSAI will be updated based on the predefined policies when it is used, and accordingly the allowed S-NSSAI and T-S-NSSAI mapping relationship will also be updated. This solution will allow UE to use the updated T-S-NSSAI based on the predefined policies, thus achieving enhanced privacy protection.

*********	END CHANGES	*********