3GPP TSG SA WG3LI	Tdoc S3LI02_054

January 29 - 31, 2002

Amsterdam, the Netherlands

-- RegTP explanatory note:

-- This Module is only an example, the definition of the parameters is not yet finalized in 3GPP SA3 LI

HI2-LIB-Libary

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

-- Security DomainId

securityDomainId OBJECT IDENTIFIER	::= { ccitt (0) identified-organization (4) etsi (0) securityDomain (2)}

-- Security Subdomains

fraudSubDomainId OBJECT IDENTIFIER			::= {securityDomainId fraud (1)}

lawfulInterceptSubDomainId OBJECT IDENTIFIER		::= {securityDomainId lawfulIntercept (2)}

-- LawfulIntercept Subdomains

hi1DomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId hi1 (0)}

hi2DomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId hi2 (1)}

hi3DomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId hi3 (2)}

himDomainId OBJECT IDENTIFIER				::= {lawfulInterceptSubDomainId him (3)}

-- HI1 Subdomains

hi1NotificationOperations OBJECT IDENTIFIER	::= {hi1DomainId notificationOperations (1)}

-- HI3 Subdomains

hi3CircuitLISubDomainId OBJECT IDENTIFIER	::= {hi3DomainId circuitLI (1)}

hi3TETRALISubDomainId OBJECT IDENTIFIER	::= {hi3DomainId tETRALI (2)}

-- For further study

hi3GPRSLISubDomainId OBJECT IDENTIFIER	::= {hi3DomainId gPRSLI (3)}

-- For further study

hi3CCLinkLISubDomainId OBJECT IDENTIFIER	::= {hi3DomainId cclinkLI (4)}

hi3GSMLISubDomainId OBJECT IDENTIFIER 	::= {hi3DomainId gSMLI (5)}

-- For further study

END -- SecurityDomainDefinitions

HI2-LIB-Module { ccitt (0) identified-organization (4) etsi (0) securityDomain (2) lawfulIntercept (2) hi2 (1)  version2 (2)}

-- the real object identifer need to be assigned

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

EXPORTS

CallContentLinkCharacteristics,

CalledPartyNumber,

CallingPartyNumber,

CommunicationIdentifier,

DataNodeAddress,

GA-Point,

GA-PointWithUnCertainty,

GA-Polygon,

GeographicalCoordinates,

GPRSCorrelationNumber,

GPRSEvent,

GPRSOperationErrorCode,

GPRS-parameters,

GSMLocation,

Intercepted-Call-State,

IPAddress,

IP-value,

ISUP-parameters,

ISUP-SS-parameters,

LawfulInterceptionIdentifier,

LocalTimeStamp,

Location,

MAP-SS-Invoke-Components,

maxNrOfPoints,

National-Parameters,

Network-Element-Identifier,

Network-Identifier,

PartyInformation,

SciDataMode,

Services-Data-Information,

Services-Information,

SimpleIndication,

SMS-report,

Standard-Supplementary-Services,

Supplementary-Services,

TetraLocation,

TimeStamp,

UMTSLocation,

X25Address,

Qos;

-------------------------- PARAMETERS FORMATS ----------------------------------

-- the following parameters need to be enhanced after finalization of the specification by 3GPP SA3 LI

CommunicationIdentifier		::= SEQUENCE

{

communication-Identity-Number	[0] OCTET STRING (SIZE (1 .. 8)) OPTIONAL,

--Temporary Identifier of an intercepted call to uniquely identify an intercepted call

--within the node (free format). This parameter is mandatory if there is associated

--information sent over HI3interface (CClink, data,..) or when

--CommunicationIdentifier is used for IRI other than IRI-Report-recor

--This parameter was called call-Identity-Number in Ed.1 of the document

network-Identifier		[1] Network-Identifier,

...

}

--NB : The same "CommunicationIdentifier" value is sent :

--with the HI3 information  for correlation purpose between  the IRI and the

--information sent on the HI3 interfaces (CCLink, data, ..)

--with each IRI associated to a same intercepted call for correlation purpose between

--the different IRI

Network-Identifier		::= SEQUENCE

{

operator-Identifier		[0] OCTET STRING (SIZE (1 .. 5)),

--it's a notification of the NWO/AP/SvP in ASCII- characters

--the parameter is mandatory.

network-Element-Identifier	[1] Network-Element-Identifier OPTIONAL,

...

}

Network-Element-Identifier	::= CHOICE

{

e164-Format			[1] OCTET STRING    (SIZE (1 .. 25)),

--E164 address of the node in international format. Coded in the same format as the

--calling party number  parameter of the ISUP (parameter part : [5])

x25-Format			[2] OCTET STRING    (SIZE (1 .. 25)),

--X25 address

iP-Format			[3] OCTET STRING    (SIZE (1 .. 25)),

--IP address

dNS-Format			[4] OCTET STRING    (SIZE (1 .. 25)),

--DNS address

...

}

CC-Link-Identifier		::= 	OCTET STRING (SIZE (1..8))

--Depending on a network option, this parameter may be used to identify a CClink

--in case of multiparty calls.

TimeStamp 				::= CHOICE

{

localTime			[0] LocalTimeStamp,

utcTime				[1] UTCTime

}

--The UTC Time is an ASN1 universal class and its format is the one defined

--in case b) of  the ASN1 recommendation [33] (year month day

--hour minutes seconds)

LocalTimeStamp				::= SEQUENCE

{

generalizedTime					[0] GeneralizedTime,

--The generalized Time format is an ASN1 universal class and its format is the

--one defined in case a) of the ASN1 recommendation [33], b) (year

--month day hour minutes seconds)

winterSummerIndication			[1] ENUMERATED

{

notProvided(0),

winterTime(1),

summerTime(2),

...

}

}

PartyInformation 			::= SEQUENCE

{

party-Qualifier 	[0]  ENUMERATED

{

originating-Party(0),

--In this case, the partyInformation parameter provides the identities related to

--the originating party and all information provided by this party.

--This parameter provides also all the information concerning the redirecting

--party when a forwarded call reaches a target.

terminating-Party(1),

--In this case, the partyInformation parameter provides the identies related to

--the terminating party and all information provided by this party.

forwarded-to-Party(2),

--In this case, the partyInformation parameter provides the identies related to

--the forwarded to party and parties beyond this one and all information

--provided by this parties, including the call forwarding reason .

gPRS-Target(3),

...

},

partyIdentity 		[1] SEQUENCE

{

imei				[1] OCTET STRING (SIZE (8)) OPTIONAL,

--See MAP format [32]

tei					[2] OCTET STRING (SIZE (1..15)) OPTIONAL,

--ISDN-based Terminal Equipment Identity

imsi				[3] OCTET STRING (SIZE (3..8)) OPTIONAL,

--See MAP format [32] International Mobile

--Station Identity E.212 number beginning with Mobile Country Code

callingPartyNumber	[4] CallingPartyNumber OPTIONAL ,

--The calling party format is used to transmit the identity of a calling party

calledPartyNumber	[5] CalledPartyNumber OPTIONAL ,

--The called party format is used to transmit the identy of  a called party or

--a forwarded to party.

msISDN				[6] OCTET STRING (SIZE (1..9)) OPTIONAL,

-- MSISDN of the target, encoded in the same format as the AddressString

-- parameters defined in MAP format document ref [32], § 14.7.8

...

},

services-Information	[2] Services-Information OPTIONAL,

--This parameter is used to transmit all the information concerning the

--complementary information associated to the basic call

supplementary-Services-Information	[3] Supplementary-Services OPTIONAL,

--This parameter is used to transmit all the information concerning the

--activation/invocation of supplementary services during a call or out-of call not

--provided by the previous parameters.

services-Data-Information [4] Services-Data-Information OPTIONAL,

-- This parameter is used to transmit all the information concerning the complementary

-- information associated to the basic data call

...

}

CallingPartyNumber		::= CHOICE

{

iSUP-Format			[1] OCTET STRING (SIZE (1..25)),

--Encoded in the same format as the calling party number (parameter field)

--of the ISUP (see [5])

dSS1-Format			[2] OCTET STRING (SIZE (1..25)),

--Encoded in the format defined for the value part of the Calling party number

--inf. ele. of DSS1 protocol [6]. The DSS1 Information

--element identifier and the DSS1 length are not included .

...

}

CalledPartyNumber		::= CHOICE

{

iSUP-Format			[1] OCTET STRING (SIZE (1..25)),

--Encoded in the same format as the called party number (parameter field)

--of the ISUP (see [5])

mAP-Format			[2] OCTET STRING (SIZE (1..25)),

--Encoded as AddressString of the MAP protocol [32]

dSS1-Format			[3] OCTET STRING (SIZE (1..25)),

--Encoded in the format defined for the value part of the Called party number inf.

--ele. Of DSS1 protocol [6] . The DSS1 Information element

--identifier and the DSS1 length are not included .

...

}

Location	::= SEQUENCE

{

e164-Number		[1] OCTET STRING  (SIZE (1 .. 25)) OPTIONAL,

--coded in the same format as the ISUP location number  (parameter

--field) of the ISUP (see [5])

globalCellID		[2] OCTET STRING  (SIZE (5..7)) OPTIONAL,

--see MAP format (see [32])

tetraLocation		[3] TetraLocation OPTIONAL,

rAI			[4] OCTET STRING (SIZE (6)) OPTIONAL,

-- the Routeing Area Identifier is coded in accordance with the § 10.5.5.15 of

-- document ref [41] without the Routing Area Identification IEI (only the

-- last 6 octets are used)

gsmLocation		[5] GSMLocation OPTIONAL,

umtsLocation		[6] UMTSLocation OPTIONAL,

sAI					[7] OCTET STRING  (SIZE (7)) OPTIONAL,

-- format:	PLMN-ID	3 octets (no. 1 - 3),

--			LAC		2 octets (no. 4 - 5),

--			SAC		2 octets (no. 6 - 7)

--			(according to 3GPP TS 25.413)

...

}

TetraLocation	::= CHOICE

{

ms-Loc	[1] SEQUENCE

{

mcc				[1]  INTEGER (0..1023),

--16 bits  ETS [40]

mnc				[2] INTEGER (0..1023),

--14 bits  ETS [40]

lai				[3] INTEGER (0..65535),

--14 bits  ETS [40]

ci				[4] INTEGER  OPTIONAL

},

-- (to be completed)

ls-Loc	[2] INTEGER

-- (to be confirmed and completed)

}

GSMLocation 	::= CHOICE

{

geoCoordinates 	[1] SEQUENCE

{

latitude	[1]	PrintableString (SIZE(7..10)),

-- format : 	XDDMMSS.SS

longitude	[2]	PrintableString (SIZE(8..11))

-- format : 	XDDDMMSS.SS

},

-- format : 	XDDDMMSS.SS

-- 				X 			: N(orth), S(outh), E(ast), W(est)

--				DD or DDD 	: degrees (numeric characters)

--				MM			: minutes (numeric characters)

--				SS.SS 		: seconds, the second part (.SS) is optionnal

-- Example :

--			latitude short form		N502312

--			longitude long form		E1122312.18

utmCoordinates	[2] SEQUENCE

{

utm-East 	[1] PrintableString (SIZE(10)),

utm-North 	[2] PrintableString (SIZE(7))

-- example	utm-East	32U0439955

--		utm-North	   5540736

},

utmRefCoordinates  	[3] PrintableString (SIZE(13)),

-- example	32UPU91294045

wGS84Coordinates  	[4] OCTET STRING (SIZE(7..10))

-- format is as defined in GSM 03.32; polygon type of shape is not allowed.

}

UMTSLocation ::= CHOICE {

point					[1]		GA-Point,

pointWithUnCertainty	[2]		GA-PointWithUnCertainty,

polygon					[3]		GA-Polygon,

...

}

GeographicalCoordinates ::= SEQUENCE {

latitudeSign			ENUMERATED { north, south },

latitude				INTEGER (0..8388607),

longitude				INTEGER (-8388608..8388607),

...

}

GA-Point ::= SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

GA-PointWithUnCertainty ::=SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

uncertaintyCode				INTEGER (0..127)

}

maxNrOfPoints				INTEGER ::= 15

GA-Polygon ::= SEQUENCE (SIZE (1..maxNrOfPoints)) OF

SEQUENCE {

geographicalCoordinates		GeographicalCoordinates,

...

}

CallContentLinkCharacteristics		::= SEQUENCE

{

cCLink-State			[1] CCLink-State OPTIONAL,

--current state of the CCLink

release-Time			[2] TimeStamp OPTIONAL,

--date and time of the release of the Call Content Link.

release-Reason			[3] OCTET STRING (SIZE(2)) OPTIONAL,

--Release cause coded in [31] format.

lEMF-Address			[4] CalledPartyNumber OPTIONAL,

--Directory number used to route the call toward the LEMF.

...

}

CCLink-State	::= ENUMERATED

{

setUPInProcess(1),

callActive(2),

callReleased(3),

lack-of-resource(4),

--the lack-of-resource state is sent when a CC Link cannot

--be established because of lack of resource at the MF level

...

}

Intercepted-Call-State			::= ENUMERATED

{

idle(1),

--When the intercept call is released, the state is IDLE and the reason is provided

--by the release-Reason-Of-Intercepted-Call parameter.

setUpInProcees(2),

--The setup of the call is in process

connected (3),

--The answer has been received

...

}

Services-Information			::= SEQUENCE {

iSUP-parameters			[1] ISUP-parameters OPTIONAL,

dSS1-parameters-codeset-0	[2] DSS1-parameters-codeset-0 OPTIONAL,

...

}

ISUP-parameters			::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "OCTET STRING" contains one additional ISUP parameter TLV coded not already

--defined in

--the previous parameters. The Tag value is the one given in Recommendation [5] .

--The Length and the Value are coded in accordance with the parameter definition

--in recommendation

--[5]. Hereafter are listed the main parameters. However other parameters may be added :

--Transmission medium requirement : format defined in recommendation [5]

--This parameter can be provided with the "Party Information" of the  "calling party"

--Transmission medium requirement prime : format defined in recommendation [5]

--This parameter can be provided with the "Party Information" of the "calling party"

DSS1-parameters-codeset-0		::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "OCTET STRING" contains one DSS1 parameter of the codeset 0. The parameter

--is coded as

--described in recommendation [6] (The DSS1 Information element identifier

--and the DSS1 length

--are included). Hereafter are listed the main parameters (However other

--parameters may be added) :

--Bearer capability : this parameter may be repeated. Format defined in recommendation [6]

--This parameter can be provided with the "Party Information" of the "calling party",

--"called party"  or "forwarded to party".

--High Layer Compatibility : this parameter may be repeated. Format defined in

-- recommendation [6].

--This parameter can be provided with the "Party Information" of the "calling party",

--"called party"  or " forwarded to party".

--Low Layer capability : this parameter may be repeated. Format defined in

-- recommendation [6].

--This parameter can be provided with the "Party Information" of the "calling party",

--"called party"  or "forwarded to party".

Supplementary-Services			::= SEQUENCE

{

standard-Supplementary-Services		[1] Standard-Supplementary-Services OPTIONAL ,

non-Standard-Supplementary-Services	[2] Non-Standard-Supplementary-Services OPTIONAL,

other-Services				[3] Other-Services OPTIONAL,

...

}

Standard-Supplementary-Services		::= SEQUENCE

{

iSUP-SS-parameters				[1] ISUP-SS-parameters OPTIONAL,

dSS1-SS-parameters-codeset-0	[2] DSS1-SS-parameters-codeset-0 OPTIONAL,

dSS1-SS-parameters-codeset-4	[3] DSS1-SS-parameters-codeset-4 OPTIONAL,

dSS1-SS-parameters-codeset-5	[4] DSS1-SS-parameters-codeset-5 OPTIONAL,

dSS1-SS-parameters-codeset-6	[5] DSS1-SS-parameters-codeset-6 OPTIONAL,

dSS1-SS-parameters-codeset-7	[6] DSS1-SS-parameters-codeset-7 OPTIONAL,

dSS1-SS-Invoke-components		[7] DSS1-SS-Invoke-Components OPTIONAL,

mAP-SS-Parameters				[8] MAP-SS-Parameters OPTIONAL,

mAP-SS-Invoke-Components		[9] MAP-SS-Invoke-Components OPTIONAL,

...

}

Non-Standard-Supplementary-Services	::= SET SIZE (1..20) OF  CHOICE

{

simpleIndication		[1] SimpleIndication,

sciData				[2] SciDataMode,

...

}

Other-Services				::= SET SIZE (1..50) OF OCTET STRING (SIZE (1..256))

--reference manufacturer manuals

ISUP-SS-parameters			::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

-- It must be noticed this parameter is retained for compatibility reasons.

-- It is recommended not to use it in new work but to use ISUP-parameters parameter.

--each "OCTET STRING" contains one additional ISUP parameter TLV coded not already defined in

--the previous parameters. The Tag value is the one given in recommendation [5] .

--The Length and the Value are coded in accordance with the parameter

--definition in recommendation

-- [5]. Hereafter are listed the main parameters. However other parameters may be added :

--Connected Number : format defined in recommendation [5]

--This parameter can be provided  with the " Party Information" of the

--"called party" or  "forwarded to party"

--RedirectingNumber : format defined in recommendation [5]

--This parameter can be provided with the " Party Information" of the

--"originating party"

--Original Called Party Number : format defined in recommendation [5]

--This parameter can be provided with the " Party Information" of the

--"originating party"..

--Redirection information : format defined in recommendation [5]

--This parameter can be provided with the "Party Information" of the

--"originating party" , "forwarded to party" or/and "Terminating party"

--Redirection Number : format defined in recommendation [5]

--This parameter can be provided with the "Party Information"  of the

--"forwarded to party" or "Terminating party"

--Call diversion information: format defined in recommendation [5]

--This parameter can be provided with the "Party Information" of the

--"forwarded to party" or "Terminating party" .

--Generic Number : format defined in recommendation [5]

--This parameter can be provided with the "Party Information"of the

--"calling party", "called party"  or "forwarded to party".

--This parameters are used to transmit additional identities (additional ,calling party

--number, additional called number, ...)

--Generic Notification : format defined in recommendation [5]

--This parameter may be provided with the "Party Information" of the

--"calling party", "called party"  or "forwarded to party".

--This parameters transmit the notification to the other part of the

--call of the supplementary

--services activated or invoked by a subscriber during the call.

--CUG Interlock Code : format defined in recommendation [5]

--This parameter can be provided with the "Party Information" of the

--"calling party".

DSS1-SS-parameters-codeset-0	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "OCTET STRING" contains one DSS1 parameter of the codeset 0.

--The parameter is coded as

--described in recommendation [6] (The DSS1 Information element identifier

--and the DSS1 length

--are included). Hereafter are listed the main parameters (However other

--parameters may be added) :

--Calling Party Subaddress : Format defined in recommendation  [6].

--This parameter can be provided with the "Party Information" of the

--"calling party".

--Called Party Subaddress  : Format defined in recommendation [6].

--This parameter can be provided with the "Party Information" of the

--"calling party", .

--Connected Subaddress. : Format defined in recommendation  (see [14]).

--This parameter can be provided with the "Party Information" of the

--"called party"  or "forwarded to party".

--Connected Number  : Format defined in recommendation  (see [14]).

--This parameter can be provided with the "Party Information" of  the

--"called party"  or "forwarded to party".

--Keypad facility  : Format defined in recommendation  [6].

--This parameter can be provided with the "Party Information" of the

--"calling party",  "called party"  or "forwarded to party"

--Called Party Number : format defined in recommendation [5]

--This parameter could be provided with the "Party Information" of the

--"calling party"

--when target is the originating party; it contains the dialled digits

--before modification

--at network level (e.g. IN interaction, translation, etc ...)

DSS1-SS-parameters-codeset-4	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "OCTET STRING" contains one DSS1 parameter of the codeset 4.

--The parameter is coded as

--described in the relevant recommendation .

DSS1-SS-parameters-codeset-5	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "OCTET STRING" contains one DSS1 parameter of the codeset 5.

--The parameter is coded as

--described in the relevant national recommendation .

DSS1-SS-parameters-codeset-6	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "OCTET STRING" contains one DSS1 parameter of the codeset 6.

--The parameter is coded as

--described in the relevant local network recommendation .

DSS1-SS-parameters-codeset-7	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "octet string" contains one DSS1 parameter of the codeset 7.

--The parameter is coded as

--described in the relevant user specific recommendation .

DSS1-SS-Invoke-Components	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "octet string" contains one DSS1 Invoke or Return Result component.

--The invoke or return result component is coded as

--described in the relevant DSS1 supplementary service recommendation.

--Invoke or Return Result component (BeginCONF)  : reference [19]

--Invoke or Return Result component (AddCONF) :  reference [19]

--Invoke or Return Result component (SplitCONF) : reference [19]

--Invoke or Return Result component (DropCONF) : reference [19]

--Invoke or Return Result component (IsolateCONF) : reference [19]

--Invoke or Return Result component (ReattachCONF) : reference [19]

--Invoke or Return Result component (PartyDISC) : reference [19]

--Invoke or Return Result component (MCIDRequest) : reference [16]

--Invoke or Return Result component (Begin3PTY) : reference [20]

--Invoke or Return Result component (End3PTY) : reference [20]

--Invoke or Return Result component (ECTExecute) : reference [25]

--Invoke or Return Result component (ECTInform) : reference [25]

--Invoke or Return Result component (ECTLinkIdRequest) : reference [25]

--Invoke or Return Result component (ECTLoopTest) : reference [25]

--Invoke or Return Result component (ExplicitECTExecute) : reference [25]

--Invoke or Return Result component (ECT : RequestSubaddress) : reference [25]

--Invoke or Return Result component (ECT : SubaddressTransfer) : reference [25]

--Invoke or Return Result component (CF : ActivationDiversion) : reference [21]

--Invoke or Return Result component (CF : DeactivationDiversion) : reference [21]

--Invoke or Return Result component (CF : ActivationStatusNotification) : reference [21]

--Invoke or Return Result component (CF : DeactivationStatusNotification) : reference [21]

--Invoke or Return Result component (CF : InterrogationDiversion) : reference [21]

--Invoke or Return Result component (CF : InterrogationServedUserNumber) : reference [21]

--Invoke or Return Result component (CF : DiversionInformation) : reference [21]

--Invoke or Return Result component (CF : CallDeflection) : reference [21]

--Invoke or Return Result component (CF : CallRerouteing) : reference [21]

--Invoke or Return Result component (CF : DivertingLegInformation1) : reference [21]

--Invoke or Return Result component (CF : DivertingLegInformation2) : reference [21]

--Invoke or Return Result component (CF : DivertingLegInformation3) : reference [21]

--other invoke or return result components ...

MAP-SS-Invoke-Components	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "octet string" contains one MAP Invoke or Return Result component.

--The invoke or return result component is coded as

--described in the relevant MAP supplementary service recommendation.

MAP-SS-Parameters	::= SET SIZE (1..256) OF OCTET STRING (SIZE (1..256))

--each "octet string" contains one MAP Parameter. The parameter is coded as

--described in the relevant MAP supplementary service recommendation.

SimpleIndication			::= ENUMERATED {

call-Waiting-Indication(0),

--the target has received a call waiting indication for this call

add-conf-Indication(1),

--this call has been added to a conference

call-on-hold-Indication(2),

--indication that this call is on hold

retrieve-Indication(3),

--indication that this call has been retreived

suspendIndication(4),

-- indication that this call has been suspended

resume-Indication(5),

-- indication that this call has been resumed

answer-Indication(6),

--indication that this call has been answered

... }

SciDataMode		::= OCTET STRING (SIZE (1..256))

SMS-report		::= SEQUENCE

{

communicationIdentifier				[1] CommunicationIdentifier,

-- used to uniquely identify an intercepted call : the same used for the

-- relevant IRI

-- called CallIdentifier in Ed.1 of the document

timeStamp				[2] TimeStamp,

--date and time of the report. The format is

--the one defined in case a) of the ASN1 recommendation [33].

--(year month day hour minutes seconds)

sMS-Contents			[3] SEQUENCE

{

initiator				[1] ENUMERATED

{

--party which sent the  SMS

target(0),

server(1),

undefined-party(2),

...

},

transfer-status		[2] ENUMERATED

{

succeed-transfer(0), --the transfer of the SMS message succeeds

not-succeed-transfer(1),

undefined(2),

...

} OPTIONAL,

other-message		[3] ENUMERATED

{

--in case of terminating call, indicates if the server will send

--other SMS

yes(0),

no(1),

undefined(2),

...

} OPTIONAL,

content			[4] OCTET STRING (SIZE (1 .. 270)) ,

--Encoded in the format defined for the SMS mobile

...

}

}

LawfulInterceptionIdentifier	::= OCTET STRING (SIZE (1..25))

--It is recommended to use ASCII characters in

--"a"..."z", "A"..."Z", "-", "_", ".", and "0"..."9"

--For sub-address option only "0"..."9" shall be us

National-Parameters		::= SET SIZE (1..40) OF OCTET STRING (SIZE (1..256))

--Content defined by national law

GPRSCorrelationNumber ::= OCTET STRING (SIZE(8))

GPRSEvent ::= ENUMERATED

{

pDPContextActivation(1),

startOfInterceptionWithPDPContextActive(2),

pDPContextDeactivation(4),

gPRSAttach (5),

gPRSDetach (6),

cellOrRAUpdate (10),

sMS (11),

...

}

-- see ref [42]

Services-Data-Information ::= SEQUENCE

{

gPRS-parameters [1] GPRS-parameters OPTIONAL,

...

}

GPRS-parameters ::= SEQUENCE

{

pDP-address-allocated-to-the-target 	[1] DataNodeAddress OPTIONAL,

aPN 					[2] OCTET STRING (SIZE(1..100)) OPTIONAL,

pDP-type 				[3] OCTET STRING (SIZE(2)) OPTIONAL,

...

}

GPRSOperationErrorCode ::= OCTET STRING (SIZE(2))

-- refer to standard [41] for values(GMM cause or SM cause parameter).

DataNodeAddress ::= CHOICE

{

ipAddress 	[1] IPAddress,

x25Address 	[2] X25Address,

...

}

IPAddress ::= SEQUENCE

{

iP-type [1] ENUMERATED

{

iPV4(0),

iPV6(1),

...

},

iP-value [2] IP-value,

...

}

IP-value ::= CHOICE

{

iPBinaryAddress	[1] OCTET STRING (SIZE(4..16)),

iPTextAddress	[2] IA5String (SIZE(7..45)),

...

}

X25Address ::= OCTET STRING (SIZE(1..25))

Qos ::= OCTET STRING (SIZE(3))

-- The Quality Of Service parameter is coded in accordance with the § 10.5.6.5 of

-- document ref [9] without the Quality of service IEI and Length of

-- quality of service IE (only the last 3 octets are used).  This parameter

-- is used to report reliability class, delay class, precedence class, peak

-- throughput, and mean throughput.

END -- OF HI2Operations