3GPP TSG-SA3 LI Meeting #14 	Tdoc S3LI04_140r1

Poa de Varzim, Portugal, 19 – 21 July 2004

# Working Draft - Interception for WLAN Interworking

## Spec:			3GPP TS 33.107

## Release:		Rel-6

## Source: 		SA3-LI

## Document for:		Information Only.

## Summary

This contribution is the latest working document for LI of WLAN Interworking.  It is being provided for information only.  The LI for WLAN interworking is a work item for SA3-LI for Release 6 and SA3-LI has been actively developing a solution for this area and expects to have stable requirements by our October meeting.

*** FIRST CHANGE ********

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	ETSI TS 101 331: "Telecommunications security; Lawful Interception (LI); Requirements of Law Enforcement Agencies".

[2]	ETSI ES 201 158: "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[4]	GSM 01.33: "Lawful Interception requirements for GSM".

[5]	GSM 02.33: "Lawful Interception - stage 1".

[6]	GSM 03.33: "Lawful Interception - stage 2".

[7]	3GPP TS 33.106: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A: "Lawfully Authorised Electronic Surveillance".

[9]	IETF RFC 2806: "URLs for Telephone Calls ".

[10]	3GPP TS 23.060: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Handover interface for Lawful Interception".

[12]	3GPP TS 21.905: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

[13]	3GPP TS 23.234: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

*** NEXT CHANGE ********

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [12] and the following apply.

Network Based Interception: Interception that is invoked at a network access point regardless of Target Identity.

Subject Based Interception: Interception that is invoked using a specific Target Identity.

Target Identity: A technical identity that uniquely identifies a target of interception. One target may have one or several identities.

Interception Area: is a subset of the network service area comprised of a set of cells which defines a geographical zone.

Location Dependent Interception: is interception of a target mobile within a network service area that is restricted to one or several Interception Areas (IA).

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [12] and the following apply:

3GPP MS	3rd Generation Mobile Communication System

3G GGSN	3rd Generation Gateway GPRS Support Node

3G GSN		3rd Generation GPRS Support Node (GGSN/SGSN)

3G MSC		3rd Generation Mobile Switching Center

3G SGSN	3rd Generation Serving GPRS Support Node

3G UMSC	3rd Generation Unified Mobile Switching Centre

AAA	Authentication, Authorization, and Accounting

ADMF	Administration Function

CC	Content of Communication

DF	Delivery Function

ECT	Explicit Call Transfer

GPRS	General Packet Radio Service

HI	Handover Interface

IA	Interception Area

ICEs	Intercepting Control Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, PDG, AAA server)

IMS	IP Multimedia Core Network Subsystem

INEs	Intercepting Network Elements (,3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, MGW, PDG, AAA server)

IP	Internet Protocol

IRI	Intercept Related Information

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

PDG	Packet Data Gateway

RA	Routing Area

RAI	Routing Area Identitiy

SAI	Service Area Identity

TEL URL	"tel" URL, as defined in [9]

*** NEXT CHANGE ********

# 4	Functional architecture

The following figures contain the reference configuration for the lawful interception. The circuit-switched configuration is shown in figure 1a. The packet-switched configuration is shown in figure 1b. Intercept configurations for HLR and IMS are shown in figures 1c and 1d. The WLAN interworking configuration is shown in figure 1e.  The various entities and interfaces are described in more detail in the succeeding subclauses.

Figure 1a: Circuit switched intercept configuration

Figure 1b: Packet Switched Intercept configuration

Figure 1c: HLR Intercept configuration

Figure 1d: IMS-CSCF Intercept configuration

Figure 1e: WLAN Interworking Intercept configuration

The reference configuration is only a logical representation of the entities involved in lawful interception and does not mandate separate physical entities. This allows for higher levels of integration.

Regional Mediation Functions, which may be transparent or part of the administration and delivery functions, are used to convert information on the HI1, HI2 and HI3 interfaces in the format described in various national or regional specifications. For example, if ES 201 671 or J-STD-25 is used, then the adaptation to HI1, HI2 and HI3 will be as defined in those specifications.

There is one Administration Function (ADMF) in the network. Together with the delivery functions it is used to hide from the 3G ICEs that there might be multiple activations by different Law Enforcement Agencies (LEAs) on the same target. The administration function may be partitioned to ensure separation of the provisioning data from different agencies.

See the remaining sections of this document for definitions of the X1_1, X1_2, X1_3, X2 and X3 interfaces.

Interception at the Gateways is a national option.

For figure 1a DF3 is responsible for two primary functions:

-	Call Control (Signalling) for the Content of Communication (CC); and

-	Bearer Transport for the CC.

HI3 is the interface towards the LEMF. It must be able to handle the signalling and the bearer transport for CC. LEMF can be located within the 3G network or can be in any other network.

For figure 1a, 1b, and 1e, the HI2 and HI3-interfaces represent the interfaces between the LEA and two delivery functions. The delivery functions are used:

-	to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2 (based on IAs, if defined);

-	to distribute the Content of Communication (CC) to the relevant LEA(s) via HI3 (based on IAs, if defined).

For figures 1c and 1d the HI2 interface represents the interface between the LEA and the delivery function. The delivery function is used to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2.

NOTE 1:	With reference to figure 1c, CC interception does not apply to HLR.

NOTE 2:	For IMS, figure 1d relates to the provision of IRI for SIP messages handled by the CSCF. Interception of CC for this case can be done at the GSN under a separate activation and invocation, according to the architecture in Figure 1b (see also section 7.A.1).

*** NEXT CHANGE ********

# 5	Activation, deactivation and interrogation

Figure 2 is an extraction from the reference intercept configuration shown in figure 1which is relevant for activation, deactivation and interrogation of the lawful interception.

Figure 2: Functional model for Lawful Interception activation, deactivation and interrogation

In addition to the typical 3G ICEs  functional entities, a new functional entity is introduced - the ADMF - the Lawful Interception administration function. The ADMF:

-	interfaces with all the LEAs that may require interception in the intercepting network;

-	keeps the intercept activities of individual LEAs separate;

-	interfaces to the intercepting network.

Every physical 3G ICE is linked by its own X1_1-interface to the ADMF. Consequently, every single 3G ICE performs interception (activation, deactivation, interrogation as well as invocation) independently from other 3G ICEs. The HI1-interface represents the interface between the requester of the lawful interception and the Lawful administration function; it is included for completeness, but is beyond the scope of standardisation in this document.

The target identities for 3GPP MS CS and PS interception at the SGSN, GGSN, 3G MSC Server and 3G GMSC Server can be at least one of the following: IMSI, MSISDN or IMEI.

NOTE 1:	Some communication content during a mobility procedure may not be intercepted when interception is based on MSISDN (only PS interception) or IMEI. The use of the IMSI does not have this limitation. For the availability of the target identities IMSI, MSISDN and IMEI (PS interception), refer to [10].

The target identities for multi-media at the CSCF can be one or more of the following: SIP URL or TEL URL. Other identities are for further study.

The target identities for 3GPP WLAN Interworking interception at the PDG and AAA Server can be at least one of the following: IMSI or NAI.

In case of location dependent interception the following network/national options exist:

-	target location versus Interception Areas (IAs) check in the 3G ICEs and Delivery Functions (DFs);

-	target location versus IAs check in the DFs (physical collocation of the DFs to the 3G ICEs may be required by national law).

NOTE 2:	The IA is previously defined by a set of cells. From the location of the target this set of cells permits to find the relevant IA.

NOTE 3:	It is not required that the 3G GMSC or the 3G GGSN are used for interception when Location Dependent Interception is invoked and the location of the target is not available.

Editors' note:	Location dependent intercept at the CSCF is for further study.

Editors' note:	Location dependent intercept for the 3G MSC Server and SSGN is for further study.

The ADMF shall be able to provision P-CSCFs independently from S-CSCFs. If both P-CSCFs and S-CSCFs are administered within the network for intercept, redundant multi-media IRI may be presented to the agency as a result.

## 5.1	Activation

Figures 3,4 and 5 show the information flow for the activation of Lawful Interception.

### 5.1.1	X1_1-interface

The messages sent from the ADMF to the 3G ICEs (X1_1-interface) contain the:

-	target identities (MSISDN, IMSI, IMEI, SIP URL or TEL URL) (see notes 4 and 5);

-	information whether the Content of Communication (CC) shall be provided (see note 1);

-	address of Delivery Function 2 (DF2) for the intercept related information (see note 2);

-	address of Delivery Function 3 (DF3) for the intercepted content of communications (see note 3);

-	IA in case of location dependent interception.

NOTE 1:	As an option, the filtering whether intercept product and/or intercept related information has to be provided can be part of the delivery functions. (Note that intercept product options do not apply at the CSCF, HLR, and AAA server). If the option is used, the corresponding information can be omitted on the X1_1-interface, while "information not present" means "intercept product and related information has to be provided" for theICE. Furthermore the delivery function which is not requested has to be "pseudo-activated", in order to prevent error cases at invocation.

NOTE 2:	As an option, only a single DF2 is used by and known to every 3G ICE. In this case the address of DF2 can be omitted.

NOTE 3:	As an option, only a single DF3 is used by and known to every 3G ICE (except at the CSCFs, HLR, and AAA server). In this case the address of DF3 can be omitted.

NOTE 4:	Since the IMEI is not available, interception based on IMEI is not applicable at the 3G Gateway. Moreover, in case the IMEI is not available, interception based on IMEI is not applicable at 3G ICEs.

NOTE 5:	Interception at the CSCFs is based upon either SIP URL or TEL URL. SIP URL and TEL URL as target identities are not supported by the other ICEs.

If after activation subsequently Content of Communications (CC) or Intercept Related Information (IRI) has to be activated (or deactivated) an "activation change request" with the same identity of the target is to be sent.

Figure 3: Information flow on X1_1-interface for Lawful Interception activation

Interception of a target can be activated on request from different LEAs and each LEA may request interception via a different identity. In this case, each target identity on which to intercept will need to be sent via separate activation messages from ADMF to the 3G ICEs on the X1_1-interface. Each activation can be for IRI only, or both CC and IRI.

When several LEAs request activation on the same identity then the ADMF determines that there are existing activations on the identity. In this case, the ADMF may (as an implementation option) send an additional activation message to the 3G ICEs. When the activation needs to change from IRI only to CC and IRI an activation change message will be sent to the 3G ICEs.

In case of a secondary interception activation only the relevant LEAs will get the relevant IRIs.

### 5.1.2	X1_2-interface (IRI)

For the activation of IRI the message sent from the ADMF to the DF contains:

-	the target identity;

-	the address for delivery of IRI (= LEMF address);

-	which subset of information shall be delivered;

-	a DF2 activation identity, which uniquely identifies the activation for DF2 and is used for further interrogation or deactivation, respectively;

-	the IA in case of location dependent interception;

-	the warrant reference number if required by national option.

If a target is intercepted for several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.

Figure 4: Information flow on X1_2-interface for Lawful Interception activation

### 5.1.3	X1_3-interface (CC)

For the activation of intercepted Content of Communications the message sent from the ADMF to the Delivery Function contains:

-	the target identity;

-	the address of delivery for CC (= LEMF address);

-	a DF3 activation identity, which uniquely identifies the activation for DF3 and is used for further interrogation or deactivation, respectively;

-	the IA in case of location dependent interception;

-	the warrant reference number if required by national option.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single activation of delivery is necessary for each combination of LEA and identity.

Figure 5: Information flow on X1_3-interface for Lawful Interception activation

## 5.2	Deactivation

Figures 6,7 and 8 show the information flow for the deactivation of the Lawful interception.

### 5.2.1	X1_1-interface

The messages sent from the ADMF to the 3G ICEs for deactivation contain:

-	the  target identity;

-	the possible relevant IAs in case of location dependent interception.

Figure 6: Information flow on X1_1-interface for Lawful Interception deactivation

If interception of a target has been activated via different identities then a separate deactivation message will need to be sent from the ADMF to the 3G ICEs for each identity.

When several LEAs requested activation on the same identity and subsequently request deactivation then the ADMF determines that there are remaining activations on the identity. In this case, the ADMF will not send a deactivation message to the 3G ICEs except when the activation needs to change from CC and IRI to IRI only. In that case an activation change message will be sent to the 3G ICEs.

### 5.2.2	X1_2-interface (IRI)

The messages sent from the ADMF to Delivery Function 2 for the deactivation of the Intercept Related Information contain:

-	a DF2 activation id, which uniquely identifies the activation to be deactivated for DF2.

If a target is intercepted by several LEAs and/or several identities simultaneously, a single deactivation is necessary for each combination of LEA and identity.

Figure 7: Information flow on X1_2-interface for Lawful Interception deactivation

### 5.2.3	X1_3-interface (CC)

For the deactivating the delivery of the CC the messages from the ADMF to DF3 contain:

-	a DF3 activation id, which uniquely identifies the activation to be deactivated for DF3.

Figure 8: Information flow on X1_3-interface for Lawful Interception deactivation

## 5.3	Interrogation

Interrogation provides the current status of the interception activation in the system. Interrogation of all activations for a given LEA is an ADMF function.

### 5.3.1	Interrogation of the 3G ICEs

Figure 9 shows the information flow for the interrogation of the Lawful Interception. It shall be possible to interrogate:

-	a specific activation at each relevant 3G ICEs;

-	all activations at each relevant 3G ICEs.

As result of the interrogation the activation status and data are returned.

Figure 9: Interrogation of the Lawful Interception (3G ICEs)

### 5.3.2	Interrogation of Delivery Functions

Figure 10 shows the information flow for the interrogation of the Lawful Interception. It shall be possible to interrogate:

-	a specific activation at a DF;

-	all activations at a DF for a given target identity;

-	all activations at a DF.

As result of the interrogation the activation status and data are returned.

Figure 10: Interrogation of the Lawful Interception (Delivery Functions)

*** NEXT CHANGE – Add New Section 9 ********

# 9	Invocation of Lawful Interception for WLAN Interworking Packet Data services

Figure shows the extract from the reference configuration which is relevant for the invocation of the Lawful Interception of the packet data 3GPP WLAN Interworking network.

Figure 18: Functional model for Packet Data WLAN Interworking Network Lawful Interception invocation

The HI2 and HI3 interfaces represent the interfaces between the LEA and two delivery functions. Both interfaces are subject to national requirements. They are included for completeness, but are beyond the scope of this specification. The delivery functions are used:

-	to convert the information on the X2-interface to the corresponding information on the HI2 interface;

-	to distribute the intercept related information to the relevant LEA(s);

-	to distribute the intercept product to the relevant LEA(s).

In case a Packet Data communication is selected based on several identities (IMSI) of the same target, the PDG will deliver CC and IRI only once to the DF2 and DF3. DF2 and DF3 will then distribute the information to the relevant LEAs.

For the delivery of the CC and IRI the PDG provides correlation number and target identity to the DF2 and DF3 which is used there in order to select the different LEAs where the product shall be delivered.

Interception at the WAG is for further study.

Generation and use of correlation numbers is for futher study.

## 9.1	Provision of Intercept Product - Short Message Service

For Further Study.

## 9.2	Provision of Intercepted Content of Communications – Packet data WLAN Interworking services

The access method for the delivering of Packet Data WLAN Interworking Intercept Product is based on duplication of packets without modification at the PDG. The duplicated packets with additional information in the header, as described in the following sections, are sent to DF3 for further delivery.

Figure 20: Configuration for interception of Packet Data WLAN Interworking product data

### 9.2.1	X3-interface

In addition to the intercepted content of communications, the following information needs to be transferred from the PDG to the DF3 in order to allow the DF3 to perform its functionality:

-	target identity;

-	correlation number; - for further study

-	time stamp - optional;

-	direction (indicates whether T-PDU is MO or MT) - optional;

-	the target location (if available). – for further study

## 9.3	Provision of Intercept Related Information

Intercept Related Information (Events) are for further study.

Figure 21 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the PDG or the AAA Server sends the relevant data to the DF2.

See section 7A for multi-media Intercept Related Information produced at the CSCF.

AAA Server

PDG

Other

Party

DF2

Intercept

Subject

LEMF

Figure 21: Provision of Intercept Related Information

### 9.3.1	X2-interface

The following information needs to be transferred from the PDG, or the AAA server to the DF2 in order to allow a DF2 to perform its functionality:

-	target identity (IMSI or NAI, if available);

-	events and associated parameters as defined in section 9.3.2 and 9.4 may be provided;

-	the target location (if available);

-	Correlation number;

-	Quality of Service (QoS) identifier.

The IRI should be sent to DF2 using a reliable transport mechanism.

### 9.3.2	Structure of the events

The set of events are for further study.

A set of fields as shown below is used to generate the events. The events transmit the information from PDG or AAA server to DF2. This set of fields as shown below can be extended in the PDG or  AAA server, if this is necessary as a national option. DF2 can extend this information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

Table 2: Information Events for Packet Data Event Records

Observed NAI NAI of the target subscriber (monitored subscriber). For further study.

Observed IMSI IMSI of the target subscriber (monitored subscriber).

Event type Description which type of event is delivered.

Event date Date of the event generation in the PDG or the AAA server.

Event time Time of the event generation in the PDG or the AAA server. Timestamp shall be generated relative to PDG or AAA server internal clock.

WLAN UE remote address The WLAN remote address of the target subscriber. Note that this address might be dynamic.

W-Access Point Name The W-APN of the access point.

Location Information Location Information is for further study.

Correlation Number The correlation number is for further study.

Network Element Identifier Unique identifier for the element reporting the ICE.

Initiator The initiator of the request either the network or the WLAN UE.

## 9.4	Packet Data related events

### For further study.