Work Item: DTS/LI-00058

Draft ETSI DTS 102 677 V0.1.2 (2009-09)

Technical Specification

Lawful Interception (LI);

Dynamic Triggering;

Dynamic Triggering Content of Communication Interception

Reference

<dts/li-00058>

Keywords

<Security, Interception, Dynamic Triggering>

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2009.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM, TIPHONTM, the TIPHON logo and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.
LTE™ is a Trade Mark of ETSI currently being registered
for the benefit of its Members and of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://portal.etsi.org/tb/status/status.asp

If you find errors in the present document, please send your comment to one of the following services:
http://portal.etsi.org/chaircor/ETSI_support.asp

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2009.

All rights reserved.

DECTTM, PLUGTESTSTM, UMTSTM, TIPHONTM, the TIPHON logo and the ETSI logo are Trade Marks of ETSI registered for the benefit of its Members.
3GPPTM is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners.
LTE™ is a Trade Mark of ETSI currently being registered
for the benefit of its Members and of the 3GPP Organizational Partners.
GSM® and the GSM logo are Trade Marks registered and owned by the GSM Association.

Contents

Intellectual Property Rights	5

Foreword	5

1	Scope	6

2	References	6

2.1	Normative references	6

2.2	Informative references	7

3	Definitions and abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Overview of Dynamic Triggering	9

4.1	Single Operator Reference Model (Common MF & CCTF)	9

4.2	Single Operator Reference Model (Multiple MFs and/or CCTFs)	10

4.3	Multiple Operator Reference Model (Single Legal Jurisdiction)	11

5	Architecture and Scenarios	12

5.1	Single Operator Scenarios	12

5.1.1	Message Flows	13

5.2	Multiple Operator Scenarios (Single Legal Jurisdiction)	14

5.2.1	Message Flows	14

6	Triggering Parameters	15

6.1	IP Addresses	16

6.2	Communications Resource IDs	16

6.3	  Transport Layer 2 Identifiers	16

6.4	  SIP URI	16

6.5	Dynamic Triggering Correlation Number (DTCN)	16

6.6	Warrant ID	17

6.7	Security Related Information	17

6.8	Destination IP-CAN(s) Operator Identity	17

6.9	Originating Service Operator Identity	17

6.10 Target Identity	18

7	Triggering Interface Details	18

7.1	Internal Triggering Command Interfaces	18

7.1.1. Content of Communications Triggering Interface (CCTI)	18

7.1.2 Content of Communications Control Interface (CCCI)	18

7.1.3 Content of Communications Triggering Interface Gateway Initiating (CCTIGI)	18

7.1.4 Content of Communications Triggering Interface Gateway Receiving (CCTIGR)	18

7.2	External Triggering Interfaces	18

7.2.1 Content of Communications Triggering Function Gateway Inter Domain 1 (CCTFGID1)	19

7.2.2 Content of Communications Triggering Function Gateway Inter Domain 2 (CCTFGID2)	19

7.3	LEMF Correlation Interfaces	19

7.3.1	TO1a	19

7.3.2	TO1b	19

7.3.3	TR1a	19

7.3.4	TR1b	20

8 DT Commands and Messages	20

8.1 Command Types	20

8.1.1 Activation	20

8.1.2 Invocation	20

8.1.3 Combined Activation and Invocation	21

8.1.4 Deactivation	21

8.1.5 Revocation	21

8.1.6 Keep Alive	21

8.2 Response Types	21

8.2.1 Acknowledged	21

8.2.2 Rejected	22

8.3 LEMF Message Types	22

8.3.1 Begin	22

8.3.2 Update IP-CAN	22

8.3.3 End	22

9	Security	23

Annex A (Normative): TBC (If Required)	24

Annex B (Informative): Cross Legal Jurisdiction Dynamic Triggering	24

B.1 Cross Legal Jurisdiction Dynamic Triggering	24

Annex Z (Informative): Change Request History	26

History	27

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://webapp.etsi.org/IPR/home.asp).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Technical Committee Lawful Interception (LI).

EDITOR’S NOTE: Areas for further study are marked in yellow and will be resolved or removed from the TS prior to initial publication.

EDITOR’S NOTE: Text marked in RED is used for editorial or placeholdering purposes only and will be removed prior to initial publication.

# 1	Scope

The present document specifies the requirements for lawful interception where lawful interception of the communication cannot rely on a fixed, or known in advance relationship between identities and addresses used in different domains (e.g. service and transport domain), for determination of the traffic to be intercepted within each domain  (i.e. traffic to be delivered as Intercept Related Information (IRI) across Handover Interface port 2 (HI2) and traffic to be delivered as Content of Communication (CC) across Handover Interface port 3 (HI3)). As this fixed relationship becomes less commonplace or difficult to reconcile, dynamic methods of lawful interception invocation are required. The dynamic methods defined in the present document define the relationship between "target identity" aware domains (e.g. service domain) and other domains (e.g. transport domain).

The present document defines an architecture for achieving dynamic invocation of Content of Communication. The functions defined in the present document are intended to be re-usable in any generic service domain and transport network scenario requiring the use of dynamic activation of lawful interception.

The present document assumes the necessary legal frameworks are in place to allow the use of dynamic triggering in both single and multiple operator domains. Any legal issues concerning the use of dynamic triggering are outside the scope of this document.

# 2	References

References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- Non-specific reference may be made only to a complete document or a part thereof and only in the following cases:

- if it is accepted that it will be possible to use all future changes of the referenced document for the purposes of the referring document;

- for informative references.

Referenced documents which are not found to be publicly available in the expected location might be found at http://docbox.etsi.org/Reference.

NOTE:	While any hyperlinks included in this clause were valid at the time of publication ETSI cannot guarantee their long term validity.

## 2.1	Normative references

The following referenced documents are indispensable for the application of the present document. For dated references, only the edition cited applies. For non-specific references, the latest edition of the referenced document (including any amendments) applies.

[1]

[2]

[3]

[4]

[5]

## 2.2	Informative references

The following referenced documents are not essential to the use of the present document but they assist the user with regard to a particular subject area. For non-specific references, the latest version of the referenced document (including any amendments) applies.

Not applicable.

or

[i1]

[i2]

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Activation (Warrant):	The applying of a Warrant to a specific LI node by the admin system or CCTF, providing the authorisation to perform interception.

Content of Communication Triggering Function (CCTF):

The logical function responsible for controlling the invocation and revocation of CC LI.

Deactivation:	The cessation of lawful interception in an LI node directly as a result of actions by the admin system in relation to a Warrant.

Dynamic Triggering: 	The dissemination of an interception obligation for a certain target’s communication between network elements. Typically these network elements will belong to the same network, and be under the control of a single network operator. Alternatively, the interception obligation may pass from one network operator to another network operator within the same jurisdiction. The network elements may have been supplied by different vendors.

Dynamic Triggering Command:	A message sent between functions involved in dynamic triggering to invoke or revoke CC LI for a specific communication. Command can be either invocation or revocation.

Gateway Triggering Originating:	The logical security gateway function responsible for communicating with the TTP and/or networks receiving the triggering commands. In addition the gateway is responsible for hiding the other internal dynamic triggering network functions (e.g. CCTF and TOF) from any entity external to the originating operator’s network.

Gateway Triggering Receiving:	The logical security gateway function responsible for receiving triggering commands from the TTP and/or networks originating the triggering commands. In addition the gateway is responsible for hiding the other internal dynamic triggering network functions (e.g. CCTF and TRF) from any entity external to the receiving operator’s network.

Invocation: 	The entering of the target ID in the network element(s) that will be involved in the target communication. For IRI interception this will be directly as a result of actions by the admin system in relation to a Warrant. For CC this is as a result of a dynamic triggering command being received and is communication session specific.

IP CAN:	IP-Connectivity Access Network. The collection of network entities and interfaces that provides the underlying IP transport connectivity between the UE and the Service Domain (e.g. IMS). An example of an "IP-Connectivity Access Network" is GPRS.

Law Enforcement Agency (LEA):	Organization authorized by a lawful authorization based on a national law to receive the results of telecommunications retained data.

Provisioning (Warrant):	The initial entering of the target in the admin system directly in relation to a specific Warrant. This includes Warrant start/end, date/time.

Revocation: 	The termination of interception at an LI node resulting from a dynamic triggering revocation command being received.

NOTE:	This is the deleting of the IP Addresses or media stream IDs from the CC LI node which were provided in a Dynamic Triggering Invocation command.

Service Domain:	The logical domain within an operator which is responsible for communications service control. The 3GPP IP Multimedia Subsystem (IMS) is an example of such a Service Domain.

Transport Domain:	The physical domain(s) used to provide the communications service users with access to the Service Domain and provide End to End connectivity between users of a communication service. The term Transport Domain can include both Core and Access networks and is not specifically limited to IP CANs.

Triggering Origination Function:	The network function, node or element responsible for originating the Dynamic triggering command, in response to user communication events.

Triggering Receiving Function:	The network function, node or element receiving the Dynamic Triggering command and responsible for invoking or revoking of Content of Communication interception. The command may be received from either TOF or CCTF and the Triggering Receiving Function is not necessarily the actual node/function which will perform the physical CC interception.

Trusted Third Party:	The Trusted Third Party (TTP) provides a DT Command authorisation and checking function. The TTP exists in its own legal domain and is authorised by the responsible national  authority (directly or on behalf of) as part of the national legal framework required to support dynamic triggering.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3GPP	3rd Generation Partnership Project

AF	Administration Function

ASN.1	Abstract Syntax Notation One

CAN	Connectivity Access Network

CC	Content of Communication

CCCI	Content of Communication Control Interface

CCTF	Content of Communication Triggering Function

CCTI	Content of Communication Triggering Interface

CS	Circuit Switched

CSCF	Call Session Control Function

CSP	Communication Service Provider

DSL	Digital Subscriber Line

DSA	Digital Signature Algorithm

DSS	Digital Signature Standard

DT	Dynamic Triggering

DTC	Dynamic Triggering Command

DTCN	Dynamic Triggering Correlation Number

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

GTO	Gateway Triggering Originating

GTR	Gateway Triggering Receiving

HI	Handover Interface

IIDTI	Internal Inter Domain Triggering Interface

IMEI	International Mobile Equipment Identity

IMPI	IMS Private Identity

IMS	IP Multimedia Subsystem

IMSI	International Mobile Subscriber Identity

IMPU	IMS Public Identity

IP	Internet Protocol

IP CAN	IP-Connectivity Access Network

IPSec	Internet Protocol Security

IRI	Intercept Related Information

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

L2TP	Layer 2 Tunnelling Protocol

NAT	Network Address Translation

PS	Packet Switched

SBC	Session Border Controller

TISPAN	Telecommunications and Internet converged Services and Protocols for Advanced Networking

TOF	Triggering Origination Function

TTP	Trusted Third Party

TRF	Triggering Receiving Function

UMTS	Universal Mobile Telecommunication System

VLAN	Virtual Local Area Network

VoIP	Voice over IP

# 4	Overview of Dynamic Triggering

EDITOR’S NOTE – This section should contain overview information on dynamic triggering based on the TISPAN generic CCTF architecture etc. Scenario issues and specific implementation architectural issues should not go in this section.

## 4.1	Single Operator Reference Model (Common MF & CCTF)

Figure 4.1 shows an enhanced version of the basic Lawful Interception Architectural model as used by TC LI and ETSI TISPAN WG7. Figure 4.1 represents a simple Dynamic Triggering (DT) scenario where all entities involved in dynamic triggering are located in the same single operator’s network and the network has a common Mediation Function (MF) and Content of Communication Trigger Function (CCTF) for all IP-Connectivity Access Network (IP-CAN) and Service domains.

The Content of Communication Triggering Interface (CCTI) reference point passes information from the Triggering Origination Function (TOF) to the Content of Communication Triggering Function (CCTF). The CCTF is the logical function responsible for the co-ordination and control of CC Dynamic Triggering. The CCTF passes triggering information to the Triggering Receiving Function (TRF) over the Content of Communication Control Interface (CCCI) reference point, in order to intercept a specific user communication session. The TOF and the TRF are the logical functions responsible for Originating and Receiving Dynamic Triggering commands.

Figure 4.1: Dynamic Triggering Single Operator Common MF and CCTF Reference Model

Since the TOF, CCTF and TRF are all in the same operator’s network, the CCTF is able to implicitly trust that dynamic triggering commands received from the TOF are lawfully authorised, so there is no Warrant administration interface (INI1b) from the Administration Function (AF) required at the CCTF in this scenario.

While the reference model in figure 4.1 shows the TOF as a sub function of the Intercepted Related Information-Internal Intercept Function (IRI-IIF), the present document does not specifically require that the TOF, is actually an IRI interception function. For the purposes of the present document the TOF can be considered to be any node involved in the control or provision of a user service which is able to output the information necessary to achieve dynamic triggering. However, in most scenarios it is expected that the TOF will in practice be part of the IRI-IIF.

Editors Note:- TO1b is a logical reference point only. We may wish to transport this over the HIx interface but this will required work in other specifications.

## 4.2	Single Operator Reference Model (Multiple MFs and/or CCTFs)

Figure 4.2 represents a Dynamic Triggering scenario where all entities involved in dynamic triggering are located in the same single operator’s network but each IP-CAN and Service domain (e.g. IMS domain) has a separate MF and CCTF.

The significant difference from the single operator scenario in figure 4.1 is inclusion of the Gateway Triggering Origination (GTO) / Gateway Triggering Receiving (GTR) nodes. Since triggering is occurring within a single operator’s network the GTO and the GTR communicate directly over the CCTFGID1 interface, as it is assumed that there is implicit trust for Surveillance Order purposes between the CCTFs.

If the operator has a common CCTF and Separate MFs then the GTO and GTR (plus associated interfaces), would be omitted and a single CCTF would interface to both the TOF and TRF as per Figure 4.1.

If the operator has a common MF and Separate CCTFs then the TR1a and TR1b interfaces are not required.

Figure 4.2: Dynamic Triggering Single Operator Separate CCTF and MF Reference Model

4.3	Multiple Operator Reference Model (Single Legal Jurisdiction)

Figure 4.3 shows how the basic single operator model in figure 4.2 is extended to cover scenarios involving multiple operators.

Figure 4.3: Dynamic Triggering Multiple Operator Reference Model(Single Legal Jurisdiction)

The multiple operator scenario is an extension of the complex single operator scenario in Figure 4.2. In multiple operator scenarios, national regulations may require regulatory involvement in checking or explicitly authorising Dynamic Triggering commands received over the CCTFGID reference points.

The Trusted Third Party (TTP) provides a DT Command authorisation and checking function. The TTP exists in its own legal domain within Figure 4.3 and is authorised by the responsible national  authority (directly or on behalf of) as part of the national legal framework required to support dynamic triggering.

The CCTF in Operator 1’s Network may communicate with the CCTF in Operator Network 2 directly via CCTFGID1 or via the Trusted Third Party (TTP) over the CCTFGID2 and CCTFGID3 reference points.

# 5	Architecture and Scenarios

## 5.1	Single Operator Scenarios

TBC

### 5.1.1	Message Flows

This clause provides a high level message flow for Dynamic Triggering in a single CSP’s network where there is a common MF and CCTF for both the service domain and the IP-CAN. The flow in figure 5.1 assumes that there is implicit trust in terms of the Warrant between the AF, TOF, CCTF and TRF such that there is no requirement to pre-activate a Warrant in the TRF prior to Invocation of CC LI.

The message flow in figure 5.1 is a simplified message diagram. It is intended to show the relationship between Dynamic Triggering specific LI messages and Warrant steps required to invoke CC interception. For simplicity, messages such as the full HI2 and HI3 message flows are not shown in full in Figure 5.1.

LEA

AF

MF

CSP

AF

CCTF

LEMF

IRI-IIF

(TOF)

CC-IIF

(TRF)

Provision

IRI Activation & Invocation

Target  Communication  Start

CCTI

CC Activation & Invocation

HI2

CCCI

TO1a

TO1b

CC Activation & Invocation

Target CC Intercept Start (INI3)

HI3

CCTI

CC De-Activation & Revocation

CCCI

CC De-Activation & Revocation

INI2

Target  Communication End

Surveillance OrderWarrant Expiry

IRI De-Activation & Revocation

Figure 5.1: Single Network Operator Dynamic Triggering Simplified Message Flow

## 5.2	Multiple Operator Scenarios (Single Legal Jurisdiction)

TBC

### 5.2.1	Message Flows

This clause provides a high level message flow for Dynamic Triggering in a single legal jurisdiction. The flow in figure 5.2 assumes the TTP is required to validate Dynamic Triggering requests being sent to CSP2 from CSP1. The IRI De-Activation and Revocation steps are not shown in Figure 5.2 as the process is the same as Figure 5.1 In addition for simplicity the GTO and GTR gateways are not shown in the figure 5.2.

LEA

AF

MF

CSP1

AF

CCTF

LEMF

IRI-IIF CSP1

(TOF)

CC-IIF CSP2

(TRF)

Provision

IRI Activation & Invocation

Target  Communication  Start

CCTI

CC Invocation

INI2 & TO1a

CCCI

CC Activation & Invocation

Target CC Intercept Start (INI3)

HI3

CCTI

Target  Communication  End

CC Revocation

CSP1

CSP1

MF

AF

CSP2

CCTF

CSP2

TTP

CSP2

Provision

CC Activation

HI2 & TO1b

CC Invocation

TR1a

TR1b

CCCI

CC Revocation

CC Revocation

CC Invocation

CC De-Activation & Revocation

Figure 5.2: Multiple Network Operator Dynamic Triggering Simplified Message Flow

Figure 5.3 shows the high level message flow involved if a communication changes IP-CAN during an active session. For the purposes of figure 5.3, Activation and Invocation of IRI in the TOF has already been achieved and CC interception has already been started in CSP2 using the message flows in figure 5.2. HI2 and HI3 events are not shown in figure 5.3.

MF

CCTF

LEMF

CSP1

IRI-IIF

(TOF)

CSP2

CC-IIF

(TRF)

Target Communication IP-CAN change for on-going LI session

CC CSP3 Invocation

CSP1

CSP1

MF

CCTF

CSP2

TTP

CSP2

CSP3

CSP3

CC-IIF

CSP3

CCTF

(TRF)

MF

CCTI

TO1a

CC CSP3 Invocation

TO1b

CCTI

CC CSP3 Invocation

CC CSP3 Activation & Invocation

TR1a CSP3

TR1b CSP3

CC CSP2 Revocation

CC CSP2 Revocation

CCCI

CCCI

CC CSP2 Revocation

CC CSP2 De-Activation & Revocation

TO1a

TO1b

Figure 5.3: Multiple Network Operator Dynamic Triggering Change of IP-CAN Message Flow

# 6	Triggering Parameters

This clause defines the parameters and other information which must be transported by the dynamic triggering commands in order to allow invocation of CC interception.

The following high level parameters shall be included in each Dynamic Triggering Command where applicable;

- IP Address(s) and Port Number(s)

- Media Stream ID(s) or Media Resource ID(s)

- Transport Layer 2 Identifiers

- SIP URL

- Dynamic Triggering Correlation Number (DTCN)

- Warrant ID

- Security Related Information

- Destination IPCAN Operator(s) Identity

- Originating IPCAN Operator Identity

- Target Identity

- National Parameters [Editors Note:- Delete if none identified before TS is published]

EDITOR’S NOTE:- Depending on the eventual architecture and scenarios agreed for inclusion in the TS, it may be necessary to specify multiple sets of Parameters relative to the specific interfaces within the dynamic triggering architecture over which the triggering commands are passing.

## 6.1	IP Addresses

One or more IPv4 or IPv6 addresses relating to a specific user communication, as used by the transport network. This is not necessarily the IP address as seen by the signalling domain server (e.g. CSCF in IMS) as network domain security or network architecture may impose one or more layers of NAT.

In addition to the IP address, it may be necessary to provide 1 or more port numbers to ensure only the specific communication required is intercepted in cases where an IP address is shared between one or more users and/or the interception Warrant specifically applies to a specific service.

NOTE:	The IP address and associated port number are unlikely to be known to the requesting party but are to be returned from the current hosting party.

## 6.2	Communications Resource IDs

In some scenarios, it may not be possible to identify the transport domain IP address and port number associated with the specific communication of interest or it may be desirable to use other methods of linking between the signal and transport domains. In this case a Media Stream ID or Media Resource ID could be used to provide the necessary information about which communication to intercept at the CC interception function.

## 6.3			Transport Layer 2 Identifiers

One or more transport layer 2 identities relating to a specific user communication, as used by the transport network. Such parameters include Mac Addresses, VLAN IDs, Stacked VLAN Identities and L2TP Identities.

## 6.4			SIP URI

One or more SIP URIs as used by the Service domain to request or control resources in the IP-CAN. Such SIP URI scenarios include triggering interception in Application Servers or Session Border Controllers (SBC) in an NGN.

## 6.5	Dynamic Triggering Correlation Number (DTCN)

A unique Dynamic Triggering Correlation Number (DTCN) is required for the LEMF to associate the IRI with the 1 or more CC streams produced as the result of dynamic triggering. This DTCN shall be included in all HI2 and HI3 messages sent to the LEMF in addition to any other correlation information provided by the underlying LI functionality.

The triggering command originating network TOF shall be responsible for generating all DTCNs to be used in conjunction with dynamic triggering. The DTCN must be unique for each communication in the originating network such that if multiple communications are in progress the LEMF can uniquely identify all IRI and CC for each communication from any other communication which is in progress, A single communication may result in multiple IP-CAN CC streams which will share a common DTCN.

The network receiving a Dynamic Triggering command shall ensure that all CC intercepted as the result of a Dynamic Triggering command shall contain the received DTCN in all HI3 messages sent to the LEMF. Any HI2 information generated locally by the IP-CAN shall use existing correlation mechanisms to associate the local HI2 with the HI3 in the IP-CAN.

EDITORS Note:- CR required for TS 102 232 to add Dynamic Triggering Corrleation Number to HI2 and HI3 interfaces in TS 102 232.

## 6.6	Warrant ID

In multiple network scenarios, it is necessary for the TTP and/or the receiving IP-CAN CCTF/AF to verify that it has legal authorisation to accept the DTC from the TOF. The method (e.g., electronic or manual) used to verify the legal authorisation is a national option.

The Warrant ID is a globally unique reference number issued by the LEA.

The LEA is required to issue the same Warrant ID to both the Service provider (e.g. IMS operator) and all IP-CANs potentially involved in CC interception. When the appropriate IP-CANs cannot be determined at the creation of the Warrant, the LEA is required to issue the same Warrant ID to the Service Provider and the TTP.

When the IP-CAN CCTF or TTP receives a DTC, the Warrant ID may be used along with the security credentials of the dynamic triggering command to verify that the CC Invocation request is valid and that a corresponding Warrant exists in the CCTF and/or TTP. If the electronic verification process fails, then an error message is sent to the LEMF by the Service Provider who originated the DTC.  Some nations may implement a manual verification process to ensure that a valid Warrant exists in the IP-CAN(s) domain.   The Warrant ID is not sent to the LEMF via HI2 or HI3 as an additional parameter. However, the CSP may send the Warrant ID in the LIID field of HI2 and HI3 if required by national requirements.

Unlike the DTCN, the Warrant ID is not necessarily unique per communication or per target identity (a single surveillance order may contain multiple target IDs)and when a TOF initiates Dynamic Triggering for multiple communication sessions for a single target identity, the Warrant ID alone cannot be used for correlation purposes.

EDITOR’S Note:- What minimum level of structure needs to be standardised to ensure that the Surveillance order ID is globally unique. The equivalent of the CID, DCC field plus a unique LEA / issuing authority identifier are required. The rest could be left open to national definitions.

## 6.7	Security Related Information

TBC

## 6.8	Destination IP-CAN(s) Operator Identity

For the LEMF to easily achieve correlation of CC to IRI across multiple operators, the LEMF needs to know the identity of the Destination IP-CAN(s) Operator(s). This is the identity of the destination IP-CAN Operator from the perspective of the Service Operator (e.g. IMS operator) originating the Dynamic Triggering command.

Due to Network Domain Security/Hiding, in most operators, it is likely that the Service Operator (e.g. IMS operator) can only uniquely identify the IP-CAN operator rather than the specific IP-CAN used by the communication.

## 6.9	Originating Service Operator Identity

For the LEMF to easily achieve correlation of CC to IRI across multiple operators, the LEMF needs to know the identity of the Originating Service Operator. This is the identity of the Service Operator as known by the IP-CAN operator.

## 6.10 Target Identity

The TTP and/or IP-CAN AF may wish to explicitly verify that the target ID in the service domain is authorised to be intercepted. The method by which authorisation is verified is outside the scope of the present document, As Surveillance Orders may list multiple target IDs to be intercepted. The Surveillance Order shall not be used by itself to confirm authorisation to intercept in the IP-CAN.

The Target Identity (ID) against which the dynamic triggering command has been generated shall be included in the dynamic triggering command. The Target ID shall be that used by the Service Domain IRI interception. Where the Target ID differs from the target information included in the DT command for invocation at the CC_IIF the CSP shall ensure that the binding of Target ID to target information is valid at the time of interception.

NOTE:	It is outside the scope of this TS to define the complete Surveillance Order format or distribution from LEA to CSPs.

# 7	Triggering Interface Details

7.1	Internal Triggering Command Interfaces

This section provides details of the interfaces which are internal to a single network domain and do not cross domain or network borders.

### 7.1.1. Content of Communications Triggering Interface (CCTI)

CCTI messages from the TOF to the CCTF, contain the Surveillance Order ID and the DTCN plus service level target identities as observed at the TOF.

### 7.1.2 Content of Communications Control Interface (CCCI)

CCCI messages from the CCTF to the TRF, contain the Surveillance Order ID and the DTCN plus IP-CAN level target identities or service level identities which the CC-IIF can be resolve into target level identities, in order to allow interception in the IP-CAN to be activated or deactivated or invocated or revoked.

### 7.1.3 Content of Communications Triggering Interface Gateway Initiating (CCTIGI)

### 7.1.4 Content of Communications Triggering Interface Gateway Receiving (CCTIGR)

## 7.2	External Triggering Interfaces

This section provides details of the interfaces which cross network domain or network operator borders.

### 7.2.1 Content of Communications Triggering Function Gateway Inter Domain 1 (CCTFGID1)

### 7.2.2 Content of Communications Triggering Function Gateway Inter Domain 2 (CCTFGID2)

7.2.3 Content of Communications Triggering Function Gateway Inter Domain 3 (CCTFGID1)

## 7.3	LEMF Correlation Interfaces

This section provides details of the interfaces which provide LEMF correlation information.

The TO1a, TO1b, TR1a and TR1b interfaces in figure 4.1, figure 4.2 and figure 4.3 are intended to provide the LEMF with explicit correlation information, such that the LEMF knows which operator’s HI2 and HI3 communications pipes contain IRI or CC relating to a dynamic triggering interception. Without these interfaces, the LEMF would be required to constantly inspect all LI streams received from all operators in case IRI or CC was present relating to a dynamic triggering intercept. The serving IP-CAN or IP-CANs may change periodically during the communication due to user mobility.

Support and implementation of these interfaces is mandatory in all multi operator scenarios.

In single operator scenarios where the operator has multiple IP-CANs, support and implementation of the TO1a and TO1b interfaces is mandatory.

In single operator scenarios where the operator has multiple IP-CANs but does not have a common MF for all IP-CANs and the service domain (e.g. IMS domain), support and implementation of the TR1a and TR1b interfaces is mandatory.

In single operator scenarios, these interfaces may be omitted if there is only one IP-CAN.

### 7.3.1	TO1a

TO1a messages from the CCTF to the MF contain the Warrant ID, DTCN and the Identity of Destination IP-CAN(s) Operator from the perspective of the Service Operator. The Warrant ID and DTCN are provided by the DTC.

### 7.3.2	TO1b

TO1b messages from the MF to the LEMF contain the Warrant ID, DTCN and  the Identity of Destination IP-CAN(s) Operator from the perspective of the Service Operator. The Warrant ID and DTCN are provided by the DTC.

TO1b messages shall be transported between MF and LEMF using existing HI2 interfaces.

EDITOR’S NOTE: A CR is required to 102.232 to include the necessary HI2 message extensions necessary to carry the TO1b interface messages.

### 7.3.3	TR1a

TR1a messages from the CCTF to the MF in the IP-CAN operator’s network contain the Warrant ID, DTCN and the Identity of Originating Service Operator of the Dynamic Triggering commands from the perspective of the IP-CAN Operator. It is a national option to omit the Identity of Originating Service Operator. The Warrant ID and DTCN are provided by the DTC.

### 7.3.4	TR1b

TR1b messages from the MF to the LEMF contain the Warrant ID, DTCN and the Identity of Originating Service Operator of the Dynamic Triggering commands from the perspective of the IP-CAN Operator. It is a national option to omit the Identity of Originating Service Operator. The Warrant ID and DTCN are provided by the DTC.

TR1b messages shall be transported between MF and LEMF using existing HI2 interfaces.

EDITOR’S NOTE: A CR is required to 102.232 to include the necessary HI2 message extensions necessary to carry the TO1b interface messages.

# 8 DT Commands and Messages

## 8.1 Command Types

Dynamic Triggering commands between the TOF and TRF in Figures 4.1 – 4.3 in section 4 have the following 6 command types:-

- Activation

- Invocation

- Combined Activation and Invocation

- Deactivation

- Revocation

- Keep Alive

A description of each of the commands is given in the sections 7.X.1 – 7.X.6. While the meaning of any command is the same regardless of over which interface the command is flowing, some parameters (eg target identities) may be changed as the commands flow between the TOF and TRF.

### 8.1.1 Activation

The Activation command is used to activate the surveillance order in the TRF, without applying the target identities against which the interception will be performed.

If an activation command is received at a TRF which relates to an already activated surveillance order, the TRF shall update the details of the Activation without removing any active interceptions which have previously been invoked.

## 8.1.2 Invocation

The Invocation command is used to apply target identities to a Surveillance order which has been previously activated on a TRF.

An Invocation command sent to a TRF, where a valid Surveillance Order activation has not been received shall be rejected by the TRF.

If an Invocation command is received at a TRF which relates to an already invoked dynamic triggering interception, the TRF shall update the details of the Invocation. Any active interception for which the target identities have been removed as a result of the update shall be immediately terminated.

## 8.1.3 Combined Activation and Invocation

The Combined Activation and Invocation command is simply the combination of the information carried in the Activation and Invocation in a single command, where Activation and Invocation are required to happen at the same time.

A combined Activation and Invocation command may be split into a separate Activation and Invocation command or vice versa by a CCTF.

A Combined Activation and Invocation command received at a TRF where a valid activation already exists shall be rejected.

## 8.1.4 Deactivation

The Deactivation command is used to remove an active Surveillance Order from a TRF.

The Deactivation command shall immediately terminate any invoked interception for that Surveillance Order and remove the associated target identities from the TRF. The Deactivation command will therefore have the effect of both revoking and deactivating the Surveillance Order and the invoked target identities.

## 8.1.5 Revocation

The Revocation command is used to remove the active target identities from the TRF. Any active interception for which the target identities have been removed as a result of the revocation command shall be immediately terminated.

The Revocation command does not change the Surveillance Order activation status.

## 8.1.6 Keep Alive

The Keep Alive command is an optional command sent from the Service Domain CCTF to the TRF in order to maintain the interception. This command is intended to prevent the case where the Service Domain session ends and the CCTF in the Service Domain fails to revoke or deactivate the CC LI in the IP-CAN (eg TOF fails to report end to CCTF).

## 8.2 Response Types

Dynamic Triggering commands responses between the TRF and TOF in Figures 4.1 – 4.3 in section 4 have the following 2 command response types:-

- Acknowledged

- Rejected

## 8.2.1 Acknowledged

The Acknowledge command is a simple acknowledgement command to the previous node in the dynamic triggering command flow that the next hop node has received the dynamic triggering command, verified the security credentials and accepted the command for processing.

The acknowledge command shall also be sent by the TRF to the originating CCTF to indicate that the dynamic triggering command sent by the CCTF to the TRF has been accepted successfully.

The Acknowledged and Rejected commands are the only two commands that are allowed to flow in the direction from the TRF to the TOF.

## 8.2.2 Rejected

The Rejected command is a simple rejection command to the previous node in the dynamic triggering command flow that the next hop node has received the dynamic triggering command and rejected it. The reject command shall indicate the following reject reasons depending on where in the command flow chain between the TOF and TRF the reject occurs.

- Security credentials

- Invalid Surveillance Order

- Target Identities Invalid for IP-CAN

- Keep Alive Period Exceeded

The Rejected command shall also be sent by any node receiving a reject to the originating CCTF to indicate that the dynamic triggering command sent by the CCTF to the TRF has been rejected.

The Acknowledged and Rejected commands are the only two commands that are allowed to flow in the direction from the TRF to the TOF.

## 8.3 LEMF Message Types

LEMF Correlation messages between the CCTF and MF or MF and LEMF have the following 3 message types:-

- Begin

- Update IP-CAN

- End

## 8.3.1 Begin

The Begin message is sent by the CCTF to the MF (over TO1a and/or TR1a) and in turn the MF to the LEMF (over TO1b and TR1b as HI2 messages), to indicate that a dynamic triggering interception is being invoked.

The Begin message is the result of the Invocation Command only and does not result from an Activation command.

A Begin message shall be generated by an IP-CAN CCTF (over TR1a and TR1b) where a new serving IP-CAN becomes responsible for providing HI3 to the LEMF following a change of serving IP-CAN in the service domain.

## 8.3.2 Update IP-CAN

The Update message is sent by the CCTF to the MF (over TO1a and/or TR1a) and in turn the MF to the LEMF (over TO1b and TR1b as HI2 messages), to indicate that the target identities or the IP-CAN identity being used for CC interception have changed.

## 8.3.3 End

The End message is sent by the CCTF to the MF (over TO1a and/or TR1a) and in turn the MF to the LEMF (over TO1b and TR1b as HI2 messages) to indicate that CC interception has been terminated in the IP-CAN either due to the end of the target user communication or due to user mobility resulting in interception in the old serving IP-CAN being terminated.

The End message is the result of the Revocation Command.

The End message is the result of a Deactivation command only when the Deactivation commands results in the combined Revocation and Deactivation of the CC LI in the IP-CAN (ie a Revocation command has not been sent to the TRF prior to the Deactivation command).

# 9	Security

EDITOR’S NOTE – This section should contain information relating to the security architecture and mechanisms required for Dynamic Triggering – Input from 3GPP SA3 and TISPAN WG7 required.

Dynamic Triggering as defined in the present document provides a standardised mechanism for the dynamic invocation and revocation of Content of Communication interception. It is therefore extremely important that the dynamic triggering functions are afforded an adequate level of security to protect them from misuse or eavesdropping of commands between the functions involved in dynamic triggering.

This clause defines the minimum set of security mechanisms which must be applied on dynamic triggering interfaces and the links between those interfaces for the generic case where the IRI node initiating the dynamic triggering command and the transport domain CC node receiving the command are in different security or trust domains (security domains may or may not belong to the same operator).

It is accepted that in some countries or operator scenarios where all dynamic triggering functions are contained within the same security domain and that domain provides an adequate level of security then it may be acceptable not to use one or more of the mechanisms listed in this clause. However the following principles must be observed in such cases which are the bias for the security mechanism in this clause.

- The Security mechanisms applied to the Dynamic Triggering interface must not increase the risk of exploit of the underlying network or service for which dynamic triggering is being used.

- Each node or function must be able to authenticate the identity of the source of any dynamic triggering commands received at that node.

- All dynamic triggering commands should be protected from malicious manipulation (Integrity protection mechanism should be provided).

- All dynamic triggering commands should be protected from exploit if intercepted (i.e. Confidentiality protection mechanism should be provided).

- It should not be possible for an attacker to correlate an initiation of a communication at the service level with a corresponding dynamic triggering command event (i.e. mechanism to protect against traffic analysis exploits should be provided).

- Only the minimum data set required to allow Dynamic Triggering to occur shall be exposed and transferred between functions involved in Dynamic Triggering.

- Any national requirements which should be applied in addition to the mechanisms defined in this clause.

EDITORS NOTE:- It is the current working assumption that TLS on transport level with additional integrity checks on protocol level will be used to provide Dynamic Triggering security. However IPSEC is also considered as a possible candidate security mechanism as part of any final solution.

Annex A (Normative):
TBC (If Required)

TBC

# Annex B (Informative): Cross Legal Jurisdiction Dynamic Triggering

This annex is intended to show how cross legal jurisdiction dynamic triggering could work under Mutual Legal Assistance or similar international agreements. The annex provides informative information on how the current normative standard could be extended cross legal jurisdiction.

## B.1 Cross Legal Jurisdiction Dynamic Triggering

Figure 4.3 in section 4 assumes that both the Service Domain operator and the IP-CAN operator are located in the same jurisdiction. In figure 4.1 and figure 4.2 there is an assumption that the interception can be activated within a single legal jurisdiction even if the IP-CAN or Service domains may physically span multiple legal jurisdictions due to the wide geographical implementation nature of modern telecoms networks. However, the reference models in Figures 4.1, 4.2 and 4.3 do not support cross border dynamic triggering where the CSP legal entities involved in dynamic triggering are in different legal jurisdictions.

Figure B.1, shows the extension of Figure 4.3 required to support cross legal jurisdiction dynamic triggering. The internal interfaces within the service domain and IP-CAN operators are exactly the same as Figure 4.3 but have been excluded from Figure A.1 for simplicity.

Figure B.1 Cross Legal Jurisdiction Dynamic Triggering Reference Model

In figure B.1, the service domain operator in legal jurisdiction A sends the request to the TTP A in legal jurisdiction A over the CCTFGID2 reference point. TTP A is then responsible for forwarding the command to TTP B in legal jurisdiction B over the CLJTTP reference point. TTP B then sends the command to the IP-CAN operator over the CCTFGID3 reference point.

It is a national or international legal issue outside the scope of this specification to define how the LEMF in legal jurisdiction B will provide a copy of any HI2 or HI3 information to the LEMF in legal jurisdiction A over the CLJLEMF reference point or vice versa.

Note 1:- While not specifically prohibited by this specification, it is assumed that direct communication of triggering commands across country borders over the CCTFGID1 interface in figure 4.3 is not allowed in Cross Legal Jurisdiction dynamic triggering scenarios and therefore this option is not provided in figure B.1.

Note 2:- This TS only provides the technical capabilities required to perform Cross Legal Jurisdiction Dynamic Triggering. It is a national or international legal issue to ensure that the necessary legal agreements are in place to support figure B.1. Any such agreements are outside the scope of this specification.

Annex Z (Informative):
Change Request History

| Status of the present document: TS 102 677 Dynamic Triggering of Content of Communication Interception |  |  |
| --- | --- | --- |
| Date | Version | Remarks |
| Rapporteur is Alex Leadbeater (BT Group) |  |  |

# History

| Document history |  |  |
| --- | --- | --- |
| V0.0.1 | 05/03/2009 | Initial TS Skeleton Draft |
| V0.0.2 | 20/05/2009 | Output from TC LI Rap#21 Texel. |
| V0.0.3 | 01/07/2009 | Output from TC LI#21 Sophia Antipolis. |
| V0.1.0 | 07/07/2009 | Clean version 0.0.3 to SA3-LI#34 and TC LI#22 |
| V0.1.1 | 04/09/2009 | Corruption of figure 5.1 labels and section 6.7 heading label corrected. |
| V0.1.2 | 24/09/2009 | Output from TC LI#22 Trouville including track changes. |