3GPP TSG-SA WG3-LI Meeting #37	SA3LI10_047

- New York City, USA, 4-6 May 2010

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.108 | CR | CRNum |  rev | - |  Current version: | 8.10.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | ASN.1 Description of eps-sending-of-IRI Subdomain ID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Alcatel-Lucent |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3-LI |  |  |  |  |  |  |  |  |  |
| Work item code:  | LI8 |  |  |  |  |  |  | Date:  |  | 03/05/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-8 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Correction of error in eps-sending-of-IRI Subdomain ID; Consistency of EPS ASN.1 Domain ID format with the format of other modules. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Align the Subdomain ID for eps-sending-of-IRI with the Subdomain ID in the Object Identifier Definition; Align the EPS IRI Subdomain ID format with the Subdomain ID format of the other B Annexes |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Error in the abstract syntax for EPS IRI delivery |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | B.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  |  | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
|  |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

# B.9	Intercept related information (HI2 SAE/EPS and IMS)

Declaration of ROSE operation umts-sending-of-IRI is ROSE delivery mechanism specific. When using FTP delivery mechanism, data UmtsIRIsContent must be considered.

- ASN1 description of IRI (HI2 interface)

EpsHI2Operations {itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi2eps(8) r8(8) version-4(4)}

DEFINITIONS IMPLICIT TAGS ::=

BEGIN

IMPORTS

OPERATION,

ERROR

FROM Remote-Operations-Information-Objects

{joint-iso-itu-t(2) remote-operations(4) informationObjects(5) version1(0)}

LawfulInterceptionIdentifier,

TimeStamp,

Network-Identifier,

National-Parameters,

National-HI2-ASN1parameters,

DataNodeAddress,

IPAddress,

IP-value,

X25Address

FROM HI2Operations

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2)

lawfulIntercept(2) hi2(1) version10(10)}; -- Imported from TS 101 671v3.3.1

-- Object Identifier Definitions

-- Security DomainId

lawfulInterceptDomainId OBJECT IDENTIFIER ::= {itu-t(0) identified-organization(4) etsi(0)

securityDomain(2) lawfulIntercept(2)}

-- Security Subdomains

threeGPPSUBDomainId OBJECT IDENTIFIER ::= {lawfulInterceptDomainId threeGPP(4)}

hi2epsDomainId OBJECT IDENTIFIER	::= {threeGPPSUBDomainId hi2eps(8) r8(8) version-4(4)}

eps-sending-of-IRI  OPERATION ::=

{

ARGUMENT	EpsIRIsContent

ERRORS		{ OperationErrors }

CODE		global:{threeGPPSUBDomainId hi2eps(8) opcode(1)}

}

-- Class 2 operation . The timer shall be set to a value between 3 s and 240 s.

-- The timer.default value is 60s.

-- NOTE:	The same note as for HI management operation applies.

EpsIRIsContent		::= CHOICE

{

epsiRIContent			EpsIRIContent,

epsIRISequence			EpsIRISequence

}

EpsIRISequence		::= SEQUENCE OF EpsIRIContent

-- Aggregation of EpsIRIContent is an optional feature.

-- It may be applied in cases when at a given point in time

-- several IRI records are available for delivery to the same LEA destination.

-- As a general rule, records created at any event shall be sent

-- immediately and not withheld in the DF or MF in order to

-- apply aggragation.

-- When aggregation is not to be applied,

-- UmtsIRIContent needs to be chosen.

EpsIRIContent		::= CHOICE

{

iRI-Begin-record		[1] IRI-Parameters,	-- include at least one optional parameter

iRI-End-record			[2] IRI-Parameters,

iRI-Continue-record		[3] IRI-Parameters,	-- include at least one optional parameter

iRI-Report-record		[4] IRI-Parameters	-- include at least one optional parameter

}

unknown-version 		ERROR ::= { CODE local:0}

missing-parameter		ERROR ::= { CODE local:1}

unknown-parameter-value	ERROR ::= { CODE local:2}

unknown-parameter		ERROR ::= { CODE local:3}

OperationErrors ERROR ::=

{

unknown-version |

missing-parameter |

unknown-parameter-value |

unknown-parameter

}

-- This values may be sent by the LEMF, when an operation or a parameter is misunderstood.

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules.

IRI-Parameters		::= SEQUENCE

{

hi2epsDomainId				[0]	OBJECT IDENTIFIER,  -- 3GPP HI2 EPS domain

lawfulInterceptionIdentifier	[1] LawfulInterceptionIdentifier,

-- This identifier is associated to the target.

timeStamp				[3] TimeStamp,

-- date and time of the event triggering the report.)

initiator 				[4] ENUMERATED

{

not-Available		(0),

originating-Target	(1),

-- in case of GPRS, this indicates that the PDP context activation, modification

-- or deactivation is MS requested

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is UE requested

terminating-Target	(2),

-- in case of GPRS, this indicates that the PDP context activation, modification or

-- deactivation is network initiated

-- in case of EPS, this indicated that the EPS detach, bearer activation, modification

-- or deactivation is network initiated

...

} OPTIONAL,

locationOfTheTarget		[8] Location OPTIONAL,

-- location of the target subscriber

partyInformation 		[9] SET SIZE (1..10) OF PartyInformation OPTIONAL,

-- This parameter provides the concerned party, the identiy(ies) of the party

--)and all the information provided by the party.

serviceCenterAddress	[13] PartyInformation OPTIONAL,

-- e.g. in case of SMS message this parameter provides the address of  the relevant

-- server within the calling (if server is originating) or called (if server is

-- terminating) party address parameters

sMS						[14] SMS-report OPTIONAL,

-- this parameter provides the SMS content and associated information

national-Parameters		[16] National-Parameters OPTIONAL,

ePSCorrelationNumber	[18] EPSCorrelationNumber OPTIONAL,

ePSevent 				[20] EPSEvent OPTIONAL,

-- This information is used to provide particular action of the target

-- such as attach/detach

sgsnAddress 			[21] DataNodeAddress OPTIONAL,

gPRSOperationErrorCode 	[22] GPRSOperationErrorCode OPTIONAL,

ggsnAddress 			[24] DataNodeAddress OPTIONAL,

qOS						[25] UmtsQos OPTIONAL,

networkIdentifier		[26] Network-Identifier OPTIONAL,

sMSOriginatingAddress 	[27] DataNodeAddress OPTIONAL,

sMSTerminatingAddress 	[28] DataNodeAddress OPTIONAL,

iMSevent				[29] IMSevent OPTIONAL,

sIPMessage				[30] OCTET STRING  OPTIONAL,

servingSGSN-number		[31] OCTET STRING (SIZE (1..20))	OPTIONAL,

servingSGSN-address		[32] OCTET STRING (SIZE (5..17)) 	OPTIONAL,

-- Octets are coded according to 3GPP TS 23.003 [25]

...,

-- Tag			[33] was taken into use by ETSI module in TS 101 671v2.13.1

ldiEvent	 			[34] LDIevent OPTIONAL,

correlation 			[35] CorrelationValues OPTIONAL,

ePS-GTPV2-specificParameters   [36] EPS-GTPV2-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of GTPV2 based intercepted messages

ePS-PMIP-specificParameters    [37] EPS-PMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of PMIP based intercepted messages

ePS-DSMIP-SpecificParameters   [38] EPS-DSMIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of DSMIP based intercepted messages

ePS-MIP-SpecificParameters     [39] EPS-MIP-SpecificParameters OPTIONAL,

-- contains parameters to be used in case of MIP based intercepted messages

servingNodeAddress             [40] OCTET STRING               OPTIONAL,

-- this parameter is kept for backward compatibility only and should not be used

-- as it has been superseeded by parameter visitedNetworkId

visitedNetworkId               [41] UTF8String                 OPTIONAL,

-- contains the visited network identifier inside the EPS Serving System Update for

-- non 3GPP access, coded according to [53]

national-HI2-ASN1parameters	[255]	National-HI2-ASN1parameters	OPTIONAL

}

-- Parameters having the same tag numbers must be identical in Rel-5 and onwards modules