3GPP TSG-WG3 SA3-LI Meeting #41 	SA3LI11_088r1

- Philadelphia, USA, 10-12 May 2011

| CR-Form-v9.7 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.106 | CR | CRNum |  rev | - |  Current version: | 10.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | TS 33.106 Update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | SA3-LI (TPS) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | LI11 |  |  |  |  |  |  | Date:  |  | 11/5/2011 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | 11 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change:  |  | Update to include new requirements and align with TS 33.107 and TS 33.108 |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Updated terminology thoughout document, new sections 5.2.2.4, 5.11, and new informative annex. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Misalignment and mistakes in implementation. Possible regulatory impact. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

*** FIRST CHANGE ***

# Introduction

This Technical Specification has been produced by the 3GPP TSG SA to allow for the standardisation in the area of Lawful Interception (LI) of telecommunications. This document describes in general the requirements for lawful interception.

Laws of individual nations and regional institutions (e.g. European Union), and sometimes licensing and operating conditions define a need to intercept telecommunications traffic and related information in modern telecommunications systems. It has to be noted that lawful interception shall always be done in accordance with the applicable national or regional laws and technical regulations.

*** NEXT CHANGE ***

# 1	Scope

The present document provides Stage 1 interception requirements within a 3GPP network.

The specification describes the service requirements from a Law Enforcement point of view only. The aim of this document is to define an interception system for 3GPP networks that supports a number of regional interception regulations, but these regulations are not repeated here as they vary. Regional interception requirements shall rely on this specification to derive such information as they require.

These interception requirements shall be used to derive specific network requirements.

For details see:

Stage 2:	3GPP TS 33.107 [9];

Stage 3:	3GPP TS 33.108 [10].

*** NEXT CHANGE ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	void.

[2]	void.

[3]	void.

[4]	ETSI ES 201 671: "Handover Interface for the lawful interception of telecommunications traffic".

[5]	void.

[6]	void.

[7]	void.

[8]	J-STD-025-A: "Lawfully Authorized Electronic Surveillance".

[9]	3GPP TS 33.107: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Lawful interception architecture and functions".

[10]	3GPP TS 33.108: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Handover interface for Lawful Interception".

[11]	3GPP TS 22.220: "Service Requirements for Home NodeBs and Home eNodeBs".

[12]	3GPP TS 22.182: "Customized Alerting Tones (CAT) Requirements; Stage 1".

[13]	3GPP TR 23.872: "Study on Architecture of IP Mulimedia subsystem (IMS) based Customized Alerting Tone (CAT)".

[14]	3GPP TS 24.182: "IP Multimedia Subsystem (IMS) Customized Alerting Tones (CAT); Protocol Specificiation".

[15]	3GPP TR 29.882: "Customized Alerting Tone (CAT) in 3G CS Domain".

[16]	3GPP TS 22.183: "Customized Ringing Signal (CRS) Requirements; Stage 1".

[17]	3GPP TS 24.183: "IP Multimedia Subsystem (IMS) customized Ringing Signal (CRS); Protocol Specification".

[x]	ETSI TS 101 671: "Lawful Interception (LI) Handover Interface for the lawful interception of telecommunications traffic".

NOTE: Periodically, TS 101 671 is published as ES 201 671. A reference to the latest version of the TS as above reflects the latest stable content from ETSI/TC LI.

[y]	3GPP TR 21.905:  “3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Vocabulary for 3GPP Specifications".

*** NEXT CHANGE ***

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [y] and the following apply.

Content of Communication: information exchanged between two or more users of a telecommunications service, excluding intercept related information. This includes information which may, as part of some telecommunications service, be stored by one user for subsequent retrieval by another.

Customized Alerting Tone: An indication that is customized by the called party or the calling subscriber that is played to the calling party during call establishment or during an established call session indicating that the called party is being alerted, the progress of a communication request, or any alerting event during a call session. A Customized Alerting Tone may be a piece of recorded or composed music, greeting words, voice, advertisement or video.

Customized Ringing Signal: An indication to the called party as an incoming communication indication during the establishment of a communication that is customized by the calling party or the called party. A Customized Ringing Signal (CRS) may e.g. be a picture, a piece of recorded or composed music, greeting words, voice, advertisement or video.

Intercept Related Information: information or data associated with telecommunication services involving the target identity, specifically communication associated information or data (e.g. unsuccessful communication attempts), service associated information or data, and location information.

Interception Area: is a subset of the Public Land Mobile Network (PLMN) service area comprised of a set of cells which define a geographical zone.

Location Dependent Interception: is interception within a PLMN service area that is restricted to one or several Interception Areas (IA).

Subject Based Interception: Interception that is invoked using a specific Target Identity

Target Identity: A technical identity that uniquely identifies a target of interception. One target may have one or several identities.

*** NEXT CHANGE ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [y] and the following apply:

CAT	Customized Alerting Tone

CC	Content of Communication

CRS	Customized Ringing Signal

CS	Circuit Switched

CSP	Communications Service Provider

CSG	Closed Subscriber Group

HeNB	Home eNodeB

H(e)NB	HNB and HeNB

HNB	Home NodeB

IA	Interception Area

IP	Internet Protocol

IRI	Intercept Related Information

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LI	Lawful Interception

MS	Mobile Station

PS	Packet Switched

QoS	Quality of Service

SIP	Session Initiation Protocol

UE	User Equipment

UTC	Coordinated Universal Time

WLAN	Wireless Local Area Network

*** NEXT CHANGE ***

# 4	Relationship to Regional Requirements

Interception requirements are subject to national law and international treaties and should be interpreted in accordance with applicable national policies.

Requirements universally called out in regional interception regulatory requirements are supported by the system defined in this document. Requirements unique to a specific region are not addressed (some examples are given in Section 2 as references).

The intercept system defined here provides subject based interception. Other techniques are outside the scope of this specification.

*** NEXT CHANGE ***

## 5.1	Description of requirements

This section gives the general description of lawful interception requirements.

*** NEXT CHANGE ***

### 5.1.1	General technical requirements

Figure 1 shows the general system for interception. Technical interception is implemented within a 3GPP network by special functionality on network elements shown in the figure. Specific lawful interception architecture and functions are found in TS 33.107 [9].

LEMF

3GPP

Network

Nodes

Administration

Function

Delivery Function

3GPP Network

LEA

INTERCEPT REQUEST

User Equipment

TECHNICAL INTERCEPTION HANDOVER INTERFACE

IRI

CC

CC

IRI

INTERCEPT REQUEST

Figure 1: General model for interception

*** NEXT CHANGE ***

### 5.1.2	General principles

A 3GPP network shall provide access to the intercepted Content of Communication (CC) and the Intercept Related Information (IRI) of the mobile target and services related to the target (e.g. Call Forwarding) on behalf of Law Enforcement Agencies (LEAs).

A mobile target in a given 3GPP network can be a subscriber of that 3GPP network, or a user roaming from another 3GPP network or from any other network capable of using that 3GPP network. The intercepted CC and the IRI can only be delivered for activities on that given 3GPP network.

For interception, there needs to be a means of identifying the target, correspondent and initiator of the communication.  Target identities used for interception for each domain and service are found inTS 33.107 [9] and TS 33.108 [10].

When encryption is provided and managed by the network, it shall be a national option as to whether the network provides the intercepted communication to the LEA decrypted, or encrypted with keys and additional information to make decryption possible. End-to-end encryption implemented in the user equipment based on encryption features provided by the operator is considered to be a network-managed encryption and is subject to the same requirements. See Section 5.7 for additional requirements.

Encryption not provided or managed by the network, e.g., user provided end-to-end encryption, cannot be removed by the network. In the case that the Communication Service Provider (CSP) provides encryption keys to the subscriber or customer but does not provide the encryption itself, the CSP shall provide the keys to the LEA if required by national regulations.

When compression is provided and managed by the network, it shall be a national option as to whether the network provides the intercepted communication to the LEA decompressed, or compressed with information to make decompression possible.

When encoding is provided and managed by the network, it shall be a national option as to whether the network provides the intercepted communication to the LEA decoded, or encoded with capability (e.g., codec information) to make decoding possible.

Location Dependent Interception, (LDI) allows a 3GPP network to service multiple interception jurisdictions within its service area. Multiple law agencies with their own interception areas can be served by the 3GPP network. All the information or rules given for interception within a 3GPP network apply to interception within an Interception Area (IA) when LDI is invoked. A target may be marked in one or more different IAs within the same 3GPP network. Interception is neither required nor prohibited by this standard when LDI is active and the location of the target subscriber is unknown or unavailable.

*** NEXT CHANGE ***

### 5.1.3	Applicability to telecommunication services

The requirement for lawful interception is that all telecommunications services in the 3GPP network should be capable of meeting the requirements within this document.

*** NEXT CHANGE ***

### 5.1.4	Interception within the Home and Visited Network

The visited network shall intercept only those services that the visited network provides to the target subscriber. Furthermore, the visited network shall not be required to intercept services executed by the home network.

Based upon national regulations, services executed in the home network by a target who is roaming in a different network may be intercepted in the home network.

*** NEXT CHANGE ***

### 5.2.1	Intercept administration requirements

As depicted in Figure 1, the  Law Enforcement Agency (LEA) provides the intercept request (e.g., lawful authorization or warrant) to the CSP. The intercept request identifies, at a minimum, the target, the type of intercept (i.e., IRI-only, or IRI and CC) that is authorized, the authorized period for interception, and the LEA delivery address(es) for  the intercepted information

The CSP shall securely administer the intercept (e.g., activate, deactivate, show, or list targets) within the 3GPP network as quickly as possible. The CSP’s administration function shall use appropriate authentication and audit procedures.  When LDI is used, the administration function shall allow specific IAs to be associated with target subscribers.

*** NEXT CHANGE ***

#### 5.2.1.1	Activation of LI

For the specified target and based on the warrant, the 3GPP network shall activate the delivery of either IRI, or both the IRI and the CC to the designated LEA destination addresses.

*** NEXT CHANGE ***

#### 5.2.1.2	Deactivation of LI

As a result of deactivation, the 3GPP network  shall stop all, or a part of, interception activities for the specified target.

*** NEXT CHANGE ***

#### 5.2.1.3	Security of processes

The intercept function shall only be accessible by authorised personnel.

Only authorised personnel can be aware that an intercept function has been activated on a target.  No indication shall be given to any person except authorised personnel that the intercept function has been activated on a target.

To be effective, interception must take place without the knowledge of any party to the communication.

Authentication, encryption, log files and other mechanisms may be used to maintain security in the system.

CSPs shall ensure that its equipment, facilities, or services that provide a subscriber with the ability to originate, terminate, or direct communications are capable of facilitating authorized communications interceptions and access to intercept related information unobtrusively and with a minimum of interference with any subscriber's telecommunications service and in a manner that protects:

- the privacy and security of communications (both signalling and content of communication) not authorized to be intercepted; and

- information regarding the LEA’s interception of communications.

Audit procedures, performed by the CSP, should have access to accurate logs of administration commands and accesses to functions and interception information.  Log files shall only be accessible by authorised personnel.

*** NEXT CHANGE ***

#### 5.2.2.1	Invocation events for lawful interception

In general, Lawful interception should be invoked when the transmission of information or an event takes place that involves the target. Examples of when Lawful interception could be invoked are when:

-	A circuit switched (CS) call is requested, originated from, terminated to, or redirected by the target;

-	Location information related to the target facility is modified by the subscriber attaching or detaching from the network, or if there is a change in location;

-	An SMS transfer is requested - either originated from or terminated to the target;

-	A data packet is transmitted to or from a target;

-	A Conference Call is targeted.

*** NEXT CHANGE ***

#### 5.2.2.2	Invocation and removal of interception regarding services

The invocation of lawful interception shall not alter the operation of a target's services or provide indication to any party involved in a target’s communication or to any others (e.g., non-authorized personnel). Lawful interception shall not alter the services available for the subscribers.

If lawful interception is activated during a CS service, the currently active CS service is not required to be intercepted. If lawful interception is deactivated during a CS service, all on-going intercepted activities may continue until they are completed.

If lawful interception is activated when a packet switched (PS) service is already in use, the next packets transmitted shall be intercepted. If lawful interception is deactivated during a PS service, the next packets shall not be transmitted to the LEMF.

*** NEXT CHANGE ***

#### 5.2.2.3	Correlation of information and product

When only IRI is delivered, an unambiguous correlation shall be established between associated IRI within the singled domain for the same communication or session (for example, different legs in CS).

When both IRI and CC are delivered, an unambiguous correlation shall be established between associated IRI, IRI and CC, and associated CC within the single domain (for example different legs in CS or different packets in PS).

*** NEXT CHANGE – NEW SECTION***

#### 5.2.2.4	Timing

The IRI and CC shall be delivered in as near real time as possible.

NOTE:  There may be regional or national requirements on the timing requirements for delivery of IRI and CC.    This includes the requirement for the CSP to timestamp IRI and CC delivery with a time zone indication (e.g., UTC offset) as part of the timestamp.

*** NEXT CHANGE ***

## 5.3	Exceptional procedures

A failure with the establishment of the connection towards the LEA shall not result in any interruption of the target’s on-going telecommunications service.

It is a national option to have buffering of IRI in the 3GPP network to cope with interruptions in the connection of the LI delivery.

*** NEXT CHANGE ***

## 5.4	Interworking considerations

The 3GPP network, home or visited, shall not be responsible to interpret the protocol used by the target, or to remove user level compression or encryption.

If the target accesses the 3GPP network via another access network (e.g., an Interworking Wireless Local Area Network (I-WLAN)), the 3GPP network shall provide the LEA with the identity of the access network (as known by the 3GPP operator).  When the target’s communications or signalling information is no longer available to the 3GPP network due to redirection or handover to another network operator, it is a national option that the 3GPP network shall provide, when available, the LEA with the identity of the network operator that has access to the target’s communications or signalling information.

*** NEXT CHANGE ***

## 5.5	Charging aspects

The 3GPP network shall be capable of producing charging data related to  intercept, including the following mechanisms.

-	Use of network resources;

-	Activation and deactivation of the target;

-	Every intercept invocation;

-	Flat rate charging.

It shall be possible to produce this data in such a way that access by non-authorised personnel or the target is precluded.

*** NEXT CHANGE ***

## 5.6	Minimum service requirements

Quality of service (QoS), capacity, integrity and reliability of the delivered IRI and CC are the subject of bilateral agreement between the relevant authorities and the CSP. Security is an attribute of the negotiated delivery mechanism between the CSP and the LEA. The QoS towards the delivery function provided by the network must be, at a minimum, the same QoS as what  the network provides to the target.

*** NEXT CHANGE ***

## 5.7	LI Requirements for Encrypted Services

Clause 5.1.2 provides a general description of requirements relating to network applied encryption. The additional requirements in this section do not apply where encryption is provided by the network between any network nodes or user equipment (UE) (e.g., hop by hop IMS signaling security or End to Access Edge radio bearer encryption), where this encryption does not affect the ability of the core network to perform interception according to the requirements provided by this specification.  In addition to the general requirments, the following additonal LI requirements shall apply to network provided and/or network administered end to end or end to middle encryption, where this encryption prevents en-clair capture of communications required to be intercepted.

1.	When an encryption service is provided by the PLMN, lawful interception shall take place as for an unencrypted communications.

a.	In addition, encrypted communications shall be decrypted, or the decryption keys and any required associated information (e.g.  roll over counters) shall be provided to the LEMF.

b.	For the specific case where a key server based solution is used, it is a national option for the operator to make keys and any associated information (e.g.  roll over counters) directly available to the LEMF for the decryption of communications.

2.	Interception shall be performed in such a manner as to avoid detectability by the Target or others. In particular:

a.	There shall be no significant difference in latency during call setup or during communications compared to a non-intercepted communications.

b.	Interception of a Target shall not prevent the use of key exchange applications which provide a user key confirmation mechanism.

NOTE:	Key confirmation mechanisms such as an authentication string to be exchanged verbally are commonly used to provide additional assurance of authentication.

c.	Should interception fail during a call (or during call setup), the call shall be unaffected.

3.	Where the CSP provides decryption of the communication, it is the operator’s choice where in the network this decryption is performed. However, following decryption, all IRI and CC shall be provided to the LEMF using handover mechanisms as per an unencrypted communication.

4.	An encryption solution shall not prohibit commencement of Interception and decryption of an existing communication.

5.	If key material and any associated information are available, it shall be possible to retrospectively decrypt encrypted communications.

NOTE:	Unless national regulations require otherwise the operator is not required to retain key material or any communications after the end of a communication.

For requirements in the present clause and clause 5.1.2, the CSP is not obligated to comply with the requirements for any encryption which a Target may use which is outside the control of the CSP (e.g., 3rd party end to end VOIP software).

*** NEXT CHANGE ***

## 5.8	Lawful Interception for Customized Alerting Tone (CAT)

CAT is a service defined in TS 22.182 [12], TR 23.872 [13], TS 24.182 [14], and TR 29.882 [15]. The target may be either the calling or the called party. The CSP providing the CAT service, and doing the interception, should report the following:

-	The CAT sent to the calling party

-	When the target activates, modifies (e.g., changes to content, content descriptors, and timing descriptors), and deactivates their CAT

-	When the target copies another subscriber’s CAT

-	When another subscriber copies the target’s CAT

-	When the target up loads or down loads CAT, the CAT should be delivered

-	When available, the access method (e.g., via MS/UE or web) the target used to activate, modify, and deactivate their CAT settings

-	The identity whose CAT is played.

Intercepted CAT may, depending on national regulations, be:

-	"played" as part of the CC, during a call set up or,

-	Delivered as a file in the IRI Record.

NOTE:	Depending on national regulations, intercepted CAT media may be considered content or a signalling.

*** NEXT CHANGE ***

## 5.9	Lawful Interception for Customized Ringing Signal (CRS)

CRS is a service defined inTS 22.183 [16] and TS 24.183 [17].  The target may be either the calling or the called party.  The CSP providing the CRS service, and doing the interception, should report the following:

-	The CRS sent to the called party

-	When the target activates, modifies (e.g., changes to content, content descriptors, and timing descriptors), and deactivates their CRS

-	When the target copies another subscriber’s CRS

-	When another subscriber copies the target’s CRS

-	When the target up loads or down loads CRS, the CRS should be delivered

-	When available, the access method (e.g., via MS/UE or web) the target used to activate, modify, and deactivate their CRS settings

-	The identity whose CRS is played.

Intercepted CRS may, depending on national regulations, be:

-	"played" as part of the CC, during a call set up or,

-	Delivered as a file in the IRI Record.

NOTE:	Depending on national regulations, intercepted CRS media may be considered content or a signalling.

*** NEXT CHANGE ***

## 5.10	Lawful Interception for Home Node B and Home enhanced Node B (H(e)NB)

HNB and HeNB are jointly referred to as H(e)NB, as defined in TS 22.220 [11]. The location of the H(e)NB is the location information used by the operator to verify the location for H(e)NB activation.

For the purpose of LI, a target may be a subscriber attached to a H(e)NB, a Closed Subscriber Group (CSG), or it is a national option to allow targeting a H(e)NB itself.

The LI requirements for H(e)NB local routing, selected IP traffic offload (SIPTO) or local IP access (LIPA) are FFS.

Interception should be done in such a manner to avoid detectability by the target or others.

When a target receives service from the PLMN via a H(e)NB, the following applies:

-	the interception capabilities shall take place as for normal PLMN use

-	H(e)NB information (e.g., location and identification) shall also be provided to the LEMF

-	If available, the location reported for the target is the H(e)NB location where the target’s MS is attached

-	Target attachment to the H(e)NB and handovers to/from the H(e)NB shall be reported to the LEMF

-	There may be national requirements to identify specific information that is required to be reported

When the target is the CSG, the CSP shall report the following:

-	modifications (e.g., additions, deletions, changes in time limits for temporary CSG Members) of the CSG list for the H(e)NB

-	When available, the access method (e.g., via MS/UE or web) the H(e)NB Hosting Party used to modify the CSG list, if multiple access methods are allowed

-	CSG member’s handovers to/from the H(e)NB

-	CSG members attachments to the H(e)NB

-	CSG members communications via the H(e)NB

-	It is a national option whether interception on CSG members’ communications continues after handover occurs from the H(e)NB

NOTE:	The requirements for the CSG are FFS.

When the target is the H(e)NB, then the CSP shall report the following:

-	activation and deactivation of the targeted H(e)NB

-	IP address information regarding the secure tunnel endpoints between the H(e)NB and the Femto Security Gateway in the home network

-	modifications (e.g., additions, deletions, changes in time limits for temporary CSG Members) of the CSG list for the H(e)NB

-	When available, the access method (e.g., via MS/UE or web) the target used for the modification of the CSG list, if multiple access methods are allowed

-	handovers to/from the H(e)NB.

-	MS/UE registrations on the H(e)NB

-	communications via the H(e)NB

-	It is a national option whether interception on H(e)NB communications continues after handover occurs from the H(e)NB

NOTE:	The requirements for the CSG are FFS.

*** NEXT CHANGE –NEW SECTION***

## 5.11	Location Information

Depending on national requirements, the CSP may be required to report the location of the Target at the beginning and end of CS calls and PS and IMS sessions on a per warrant or per intercept basis.  It may also be a national requirement for the CSP to report the location of the Target during on-going communications.

Note:  Currently, in some cases, the location of the Target might not be available.

The location information reported to the LEA shall be trusted location information (i.e., the location information is either network derived or network verified).

Note:  IMS trusted location information is under study in SA2 and reporting for LI is FFS.

*** NEXT CHANGE ***

# 6	Handover Interface Requirements

- Handover interface requirements are defined in TS 33.108 [10].

- There may be national or regional specifications (e.g., ETSI ES 201 671 [4], ETSI TS 101 671 [x], and J-STD-025-A [8]).

*** NEXT CHANGE – NEW INFORMATIVE ANNEX***

Annex TBN (informative):
Bibliography

The documents listed below are not explicitly cited in this specification but are provided for background and for historical information.

3GPP TR 41.033: "Lawful Interception requirements for GSM".

3GPP TS 42.033: "Lawful Interception - stage 1".

3GPP TS 43.033: "Lawful Interception; stage 2".

European Union Council Resolution on the Lawful Interception of Telecommunications (17 January 1995).

ETSI TS 101 331: "Lawful Interception (LI); Requirements of Law Enforcement Agencies".

ETSI ES 201 158: "Lawful Interception; Requirements for network functions".