3GPP TSG-SA3 LI, Meeting #47	SA3LI12_114

16-18 October 2012, Tucson, AZ, USA

Source:	Sprint

Title:	Limitations on specifying HeNB & CSG as the target of LI

Document for:	Discussion

Agenda Item:	7.3

Work Item / Release:	LI12

## Introduction

This discussion paper presents the significant issues surrounding the requirements in the Release 12 stage 1 LI specification (TS 33.106) on what kinds of LI targets must be supported by operator LI services.  In addition to the “traditional” targeting of an identified subscriber or user, TS 33.106 Release 12 adds the requirement for the LI service to allow the H(e)NB itself and a CSG to be specified as the target of LI.

## Discussion

This discussion is divided into two parts, the first containing issues of LI targeting the H(e)NB itself and the second containing the issues of targeting a CSG.

## LI targeting the H(e)NB itself

- H(e)NB in a RAN sharing scenario.

In a RAN sharing deployment (MOCN), a H(e)NB may be attached to two or more core networks (MMEs in LTE) and broadcast multiple PLMNID SIBs.  The UE would select a PLMNID in the ATTACH, and the H(e)NB would route the traffic to the MME in the network corresponding to the selected PLMNID.  The effect is that if an IAP is in the MME and/or P-GW (or other core element) and the H(e)NB was the target of LI, then only the H(e)NB traffic with that PLMNID would be reported, not the traffic with a PLMNID of any other core networks.

- LI transition states out of H(e)NB are not defined

The currently a requirement exists for continuing the LI on a communication after handover from H(e)NB to (e)NodeB.  However the scope or definition of this communication is undefined.  The LI service, because if it is anchored by a eNodeB identity rather than a subscriber identity can’t use the same definition of start and end of communication as for a subscriber (the binding is different and is dependent on mobility events). Because of this un-derivable requirement, the scope, impact, and possible solutions can’t be identified.  For example, if a UE has attached to the network, hands over to a HeNB (which is the LI target), initiates another handover to a eNodeB, then goes into idle mode.  At this point, it is likely that the LI will stop since there is no active bearer for the user.  The UE could re-attach and continue the IP session that was ongoing.  To change from an (e)NodeB anchored LI trigger to a subscriber anchored LI trigger in this case to continue the LI is not covered by the requirement, and appears to be a significant LI architecture change.

- Multi-mode communication is undefined.

With the emergence of multi-mode UEs and aggressive industry pursuit of Wifi offload capabilities, it is likely that smart phones especially, will have only part of their bearers operating through the H(e)NB tunnelling (such as S1 for LTE). High bandwidth bearers (such as video and audio) will be pushed to Wifi offload, possibly through the same backhaul as the H(e)NB operates.  However a H(e)NB based trigger (where target of LI is the H(e)NB itself) will only detect and capture the S1 tunnelled (likely to be only NAS).  Hence the LI interception would only be reporting a partial version of the communication.  The considerations for this issue as the same as the previous LI transition states out of H(e)NB are not defined.  Furthermore, since the UE can’t necessarily detect that is attached to a H(e)NB instead of a (e)NodeB, it is not currently possible to push any policy to the UE (such as ANDSF) to only use H(e)NB bearers when attached to a H(e)NB that is itself the target of LI.  Stage 2 and stage 3 changes to the core network and to UE network selection would be required to support this capability.

- Higher probability of mid-call triggering required

In subscriber anchored LI service, mid-call triggering only applies to the scenario where the interception is activated while the target subscriber is currently involved in a communication. It is a relatively infrequent occurrence. However, if the H(e)NB itself is the specified target of LI, then the mid-call triggering will occur much more frequently – every time the UE performs an active mode handover to the H(e)NB.  Given that there are still significant issues with mid-call triggering itself as a reliable LI feature, mid-call triggering should be addressed before any consideration of supporting specifying the H(e)NB itself as a LI target.  Also the work on mid-call triggering is significant enough by itself, it must be explicitly addressed by SA 3 LI rather than remaining a requirement implicitly derived from any H(e)NB LI requirements.

- Service level (IMS) is not notified when a UE transitions in or out of a H(e)NB

Since a basic tenet of IMS is its access independence, access and mobility specific events as well as core network user identities are not reported to IMS. Likewise, for the access and core network, a significant portion of IMS events, signalling, and identities is not known. So within the current 3GPP core, access and IMS architectures there are no defined mechanism for access triggers (in this case attaching to a H(e)NB which is the target of LI) of interest to LI to trigger an IMS LI event (start, stop LI of IMS communication).  There are no functions, reference points nor protocols to provide support for this cross layer triggering.  Given the potential 3GPP architectural impacts, a separate work item (at least a building block) to accomplish this needs to be defined to allow assessing the resulting scope of the work.

,

- Privacy concerns

Because several operating modes of (H(e)NB (such as open access) operate indistinguishably from (e)NodeB signalling (such as S1), there is no standards based approach to allow an operator to limit LI anchored on H(e)NodeBs only.  By providing a capability which can be applied to eNodeBs as well, a number of areas of concern need to be explored and addressed before proceeding with this capability: Capacity of LI infrastructure in the presence of very large simultaneous communication sessions being reported; dramatic increase of probability of capturing and reporting on communication that is outside the scope and intent of a particular warrant.

## LI targeting a CSGID

- LI triggered by CSGID matching armed in the entire network

Since the same CSGID can be used in multiple H(e)NBs, the LI trigger for the CSGID must be active in all IAPs (such as MME) in a network.  In the case where a CSGID has wide deployment (say in multiple locations of an enterprise) all the communication within the network matching the CSGID will be reported. This can impact the capacity of the LI infrastructure in the presence of very large simultaneous communication session being reported as well as a dramatic increase of probability of capturing and reporting on communication that is outside the scope and intent of a particular warrant. Finally the performance impact of potentially having multiple CSG targeted LI triggers in the entire network (e.g. all MME IAPs) needs to be analysed.

- No guarantee of detecting all communication associated with a CSGID

3GPP supports multiple CSGID both in the H(e)NB and UE.  If a UE is allowed in two or more of the CSGID configured in a H(e)NB, the UE only reports one of several possible CSGIDs that it matches which is verified by the MME based on HSS information. If the LI trigger based on a CSGID is one of the set that is matched but not reported to the MME, then the LI trigger will not fire and the communication not reported or delivered to the LEA.

- Master CSG list management is not standardized

Within 3GPP, CSG membership is maintained on a per subscriber basis and is stored both in the USIM and in the HSS. In 3GPP there is no definition or concept of a master list of all subscribers who have a particular CSGID in their UICC and HSS subscriber record.  It may be possible for an operator to implement or deploy such a master CSG list, but it is not standardized and its presence in a network deployment can’t be assumed. Because of this, either CSG triggers for changes to CSG membership has to be active against every single subscriber to filter out CSGID changes by CSGID. For this solution, concerns about scalability and performance must be addressed. Another alternative to address this is to develop standardized CSG master list requirements, architecture and protocol will need to be defined to represent standardized CSG master list creation, change and deletion procedures.

- H(e)NB itself as LI target issue applying to CSG as LI target

The following issues described about also apply to using a CSG as LI target:

  - CSG on a H(e)NB in a RAN sharing scenario

  - LI transition states out of a CSG on a H(e)NB to a different CSG or (e)NodeB are not defined

  - Multi-mode communication is undefined

  - Higher probability of mid-call triggering required

  - Service level (IMS) is not notified when a UE transitions in or out of a CSG on a H(e)NB.

## Conclusion and recommendation

The number of technical issues alone spanning stage 1, 2 and 3 topics along with the significant network development  and deployment impact of meeting the current requirements provides sufficient justification for removing the current requirements from TS 33.106 regarding supporting targeting H(e)NB itself and CSG in the LI service.

Discuss the technical issues in this paper and agree on the related CR in SA3LI12_115.