3GPP TSG-SA3 LI Meeting #51	SA3LI13_095

# Burlington, VT, USA; October 2013

| CR-Form-v10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev | - | Current version: | 12.4.0 |  |
| For HELP on using this form look at the pop-up text over the symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of informative Annex to address Call Scenarios pertaining to VoIP LI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Nokia Solutions and Networks) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI12 |  |  |  |  |  |  | Date: |  | 2013-10-11 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) |  |
| Reason for change: |  | To add an informative Annex to illustate various LI configurations for VoIP using the call scenarios. For VoIP, the CC interception may happen at the PDN-GW, IMS-AGW, TrGW or a IM-MGW. For a given call scenario, only one of the four is involved in providing the CC interception. The informative Annex provides the call scenarios and explains in the form of diagrams the use-cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new informative Annex is added to 3GPP TS 33.107. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This is part of defining the LI architecture for CC interception for VoIP. Without this, the call scenarios that define which network node is used to provide the CC interception for VoIP call remains unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

*** START OF FIRST MODIFICATION ***

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [13] and the following apply:

3GMS	3rd Generation Mobile Communications System

3G GGSN	3rd Generation Gateway GPRS Support Node

3G GSN		3rd Generation GPRS Support Node (GGSN/SGSN)

3G MSC		3rd Generation Mobile Switching Center

3G SGSN	3rd Generation Serving GPRS Support Node

3G UMSC	3rd Generation Unified Mobile Switching Centre

AAA	Authentication, Authorization, and Accounting

ADMF	Administration Function

AGW	Access Gateway

AN	Access Network

AP	Access Provider

AS	Application Server

BM-SC	Broadcast-Multicast Service Centre

BSF	Bootstrapping Serving Function

B-TID	Bootstrapping Transaction Identifier

CC	Content of Communication

CS	Circuit Switched

CSCF	Call Session Control Function

CSG	Closed Subscriber Group

DF	Delivery Function

DSMIP		Dual Stack Mobile IP

ECT	Explicit Call Transfer

EPC	Evolved Packet Core

ePDG	Evolved PDG

EPS	Evolved Packet System

E-UTRAN	Evolved UTRAN

FTP	File Transfer Protocol

GBA	Generic Bootstrapping Architecture

GGSN	Gateway GPRS Support Node

GPRS	General Packet Radio Service

GSM	Global System for Mobile communications

GSN	GPRS Support Node (SGSN or GGSN)

HA	Home Agent

HeMS	HeNB Management System

HeNB	Home enhanced NodeB

HeNB GW	HeNB Gateway

H(e)NB	Home and Home enhanced NodeB

HI	Handover Interface

HMS	HNB Management System

HNB	Home NodeB

HNB GW	HNB Gateway

HRPD	High Rate Packet Data

HSS	Home Subscriber Server

IA	Interception Area

IBCF	Interconnecting Border Control Function

ICEs	Intercepting Control Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, HLR, AAA Server, PDG, MME, S-GW, PDN-GW, HSS)

IETF	Internet Engineering Task Force

IM-MGW	IMS Media Gateway

IMEI	International Mobile station Equipment Identity

IMS	IP Multimedia Core Network Subsystem

IMS-AGW	IMS Access Gateway

IMSI	International Mobile Subscriber Identity

INEs	Intercepting Network Elements (3G MSC Server, 3G GMSC Server, P-CSCF, S-CSCF, SGSN, GGSN, MGW, HLR, AAA Server, PDG)

IP	Internet Protocol

IRI	Intercept Related Information

I-WLAN	Interworking WLAN (3GPP WLAN interworking subnetwork)

LAN	Local Area Network

LDI	Location Dependent Interception

LEA	Law Enforcement Agency

LEMF	Law Enforcement Monitoring Facility

LIPA	Local IP Access

LTE	Long Term Evolution

MBMS	Multimedia Broadcast/Multicast Service

MGCF	Media Gateway Control Function

MGW	Media Gateway

ME	Mobile Entity

MIP	Mobile IP

MME	Mobility Management Entity

MN	Mobile Node

MSISDN	Mobile Subscriber ISDN Number

NAF	Network Application Function

NAI	Network Access Identifier

NO	Network Operator

PCRF	Policy and Charging Rules Function

P-CSCF	Proxy CSCF

PDG	Packet Data Gateway

PDN 	Packet Data Network

PDN-GW	PDN Gateway

PMIP	Proxy Mobile IP

PoC	Push to talk over Cellular

PS	Packet Switched

RA	Routing Area

RAI	Routing Area Identity

SAI	Service Area Identity

S-CSCF	Serving CSCF

SeGW	Security Gateway

SGSN	Serving GPRS Support Node

SIP	Session Initiation Protocol

SMS	Short Message Service

S-GW	Serving Gateway

TEL URL	"tel" URL, as defined in RFC 2806 [9]

TLS	Transport Layer Security

TrGW	Transit Gateway

UE	User Equipment

UMTS	Universal Mobile Telecommunication System

URI	Universal Resource Identifier

URL	Universal Resource Locator

VoIP	Voice over IP

WLAN	Wireless LAN

*** END OF FIRST MODIFICATION ***

*** SECOND MODIFICATION *** NEW CLAUSE ***

Annex X

Informative Annex

VoIP Lawful Inteception Call Scenarios

The interception of CC for VoIP is done at one of the following access nodes:

- PDN-GW

- IMS-AGW

- TrGW

- IM-MGW.

The trigger for the CC interception at those access nodes is generated as follows:

- P-CSCF for PDN-GW and IMS-AGW

- IBCF for TrGW.

- MGCF for IM-MGW.

At any given time and for any given call, only one access node is required to provide the CC interception. The node that provides the CC interception can be different, primarily, based on the call scenario.

This informative Annex provides the call scenarios.

X.1 Originating Call from the Target with CC Interception at the PDN-GW

Figure X.1 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target originates a call with PDN-GW providing the CC interception.

Figure X.1: VoIP lawful interception for an originating call with CC interception at the PDN-GW

The cloud shown with the label “voice services” is to indicate that the target is making a voice call and the called party can be within the same CSP’s network or in another CSP’s network.

Figure X.1 shows that the IRI interception is done S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure X.1). The CC interception is done at the PDN-GW. The P-CSCF sends the CC intercept trigger to the PDN-GW over the Rx and Gx reference points.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

X.2 Originating Call from the Target with CC Interception at the IMS-AGW

Figure X.2 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target originates a call with IMS-AGW providing the CC interception.

Figure X.2: VoIP lawful interception for an originating call with CC interception at the IMS-AGW

The cloud shown with the label “voice services” is to indicate that the target is making a voice call and the called party can be within the same CSP’s network or in another CSP’s network.

Figure X.2 shows that the IRI interception is done S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure X.2). The CC interception is done at the IMS-AGW. The P-CSCF sends the CC intercept trigger to the IMS-AGW over the Iq reference point.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

X.3 Terminating Call to the Target with CC Interception at the PDN-GW

Figure X.3 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with PDN-GW providing the CC interception.

Figure X.3: VoIP lawful interception for a terminating call with CC interception at the PDN-GW

The cloud shown with the label “voice services” is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network.

Figure X.3 shows that the IRI interception is done S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure X.3). The CC interception is done at the PDN-GW. The P-CSCF sends the CC intercept trigger to the PDN-GW over the Rx and Gx reference points.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

X.4 Terminating Call to the Target with CC Interception at the IMS-AGW

Figure X.4 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with IMS-AGW providing the CC interception.

Figure X.4 VoIP lawful interception for a terminating call with CC interception at the IMS-AGW

The cloud shown with the label “voice services” is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network.

Figure X.4 shows that the IRI interception is done S-CSCF or AS. However, as specified in 7A, the IRI interception can also be done at the P-CSCF (not shown in figure X.4). The CC interception is done at the IMS-AGW. The P-CSCF sends the CC intercept trigger to the IMS-AGW over the Iq reference point.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

X.5 Intra-CSP Forwarded Call with CC Interception at the PDN-GW

Figure X.5 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with PDN-GW providing the CC interception and the call is forwarded to another IMS subscriber within the CSP’s network.

Figure X.5: VoIP lawful interception for an intra-CSP forwarded call with CC interception at the PDN-GW

The cloud shown with the label “voice services” is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network.

Figure X.5 shows that the IRI interception is done S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the PDN-GW. The P-CSCF (that provides the proxy functions to the forwarded to party) sends the CC intercept trigger to the PDN-GW over the Rx and Gx reference points.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

X.6 Intra-CSP Forwarded Call with CC Interception at the IMS-AGW

Figure X.6 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call with IMS-AGW providing the CC interception and the call is forwarded to another IMS subscriber within the CSP’s network.

Figure X.6: VoIP lawful interception for an intra-CSP forwarded call with CC interception at the IMS-AGW

The cloud shown with the label “voice services” is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network.

Figure X.6 shows that the IRI interception is done S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the IMS-AGW. The P-CSCF (that provides the proxy functions to the forwarded to party) sends the CC intercept trigger to the IMS-AGW over the Iq reference point.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

X.7 Inter-CSP Forwarded Call to a CS Domain

Figure X.7 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call and the call is forwarded to a subscriber on the CS domain of another CSP’s network.

Figure X.7: VoIP lawful interception for an inter-CSP forwarded call to a CS Domain

The cloud shown with the label “voice services” is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network.

Figure X.7 shows that the IRI interception is done S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the IM-MGW. The MGCF sends the CC intercept trigger to the IM-MGW over the Mn reference point.

X.8 Inter-CSP Forwarded Call to an IMS Domain

Figure X.8 provides the lawful interception architecture to illustrate the interception and delivery of IRI and CC, when a target receives an incoming call and the call is forwarded to a subscriber on the IMS domain of another CSP’s network.

Figure X.8: VoIP lawful interception for an inter-CSP forwarded call to an IMS Domain

The cloud shown with the label “voice services” is to indicate that the target is receiving an incoming voice call and the calling party can be within the same CSP’s network or in another CSP’s network.

Figure X.8 shows that the IRI interception is done S-CSCF or AS. In this scenario, the IRI interception cannot occur in the P-CSCF since the P-CSCF that provides the proxy functions to the target is not on the signalling path.  The CC interception is done at the TrGW. IBCF sends the CC intercept trigger to the TrGW over the Ix reference point.

The activation of intercept is done by the ADMF for IRI interception and CC interception using the same target identity (SIP URI or TEL URI).

*** END OF SECOND MODIFICATION *** NEW CLAUSE ***