3GPP TSG-SA3LI Meeting #52 	SA3LI14_014r1

Sophia Antipolis, France, 4-6 February 2014

| CR-Form-v11 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |
|  | 33.107 | CR | CRNum | rev |  | Current version: | 12.5.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Basic architecture and conditions to deliver location information based Civic Addresses |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (Ministere du Redressement Productif – France) |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LI-7A |  |  |  |  |  |  | Date: |  | 2014-01-28 |  |  |  |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-12 |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) |  |  |  |  |
| Reason for change: |  | Add civic address information on top of existing location information such as Global Cell ID or an IP address. |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add civic address information on top of existing location information such as Global Cell ID or an IP address. The content of this CR complement the CR SA3 LI14 013 and the CR SA3 LI14 015 that may be approved during this meeting. |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficulties from operators to provide geographical coordinates to the LEMF in some countries or jurisdiction that required civic address location information, on top of other location information, especially when reporting location is based on small cell or some IWLAN hotspots. The alternative may be the development of solution based on “national parameters” in HI2. |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | §2 references; §4 Functional architecture |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |

*** FIRST MODIFICATION ***

- 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	ETSI ES 201 158 (V1.2.1 April 2002): "Lawful Interception; Requirements for network functions".

[3]	ETSI ES 201 671 (V3.1.1 May 2007): "Handover Interface for the lawful interception of telecommunications traffic".

[4]	Void

[5]	Void

[6]	Void

[7]	3GPP TS 33.106: "3G Security; Lawful Interception Requirements".

[8]	ANSI J-STD-025-A (April 2003): "Lawfully Authorised Electronic Surveillance".

[9]	 VOID

[10]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description".

[11]	3GPP TS 33.108: "3G Security; Handover interface for Lawful Interception".

[12]	 Void

[13]	3GPP TS 21.905: "Vocabulary for 3GPP Specifications".

[14]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) Interworking; System Description".

[15]	3GPP TS 23.008: "Organization of subscriber data".

[16]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[17]	3GPP TS 24.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; User Equipment (UE) to network protocols; Stage 3".

[18]	IETF RFC 1122 (October 1989): "Requirements for Internet Hosts -- Communication Layers".

[19]	IETF RFC 1123 (October 1989): "Requirements for Internet Hosts -- Application and Support".

[20]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and Functional Description".

[21]	3GPP TS 24.147: "Conferencing Using the IP Multimedia (IM) Core Network (CN) subsystem 3GPP Stage 3".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 29.273: "Evolved Packet System (EPS); 3GPP EPS AAA interfaces".

[25]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[26]	3GPP TS 29.212: "Policy and Charging Control (PCC); Reference points".

[27]	Void

[28]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[29]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[30]	3GPP TS 23.272: " Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2"

[31]	3GPP TS 22.220: " Service Requirements for Home NodeBs and Home eNodeBs".

[32]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network  (E-UTRAN); Overall description; Stage 2".

[33]	3GPP TS 25.467: "UTRAN architecture for 3G Home Node B (HNB); Stage 2"

[34]	3GPP TS 33.320: "Security of Home Node B (HNB) / Home evolved Node B (HeNB) ".

[35]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[36]	IETF RFC 3966 (December 2004): "The Tel URILs for Telephone Numbers ".

[37]	3GPP TS 29.060: "General Packet Radio Service (GPRS); GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[38]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[39]	IETF RFC 791: "Internet Protocol".

[40]	IETF RFC 2460: "Internet Protocol, Version 6 (IPv6) Specification".

[41]	IEFT RFC 3697: "IPv6 Flow Label Specification"

[42]	IETF RFC 4776 “Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information"

[43]	IETF RFC 5139 “Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)“

[44]	ISO.3166-2 International Organization for Standardization, "Codes for the representation of names of countries and their subdivisions - Part 2: Country subdivision code".

[45]	UPS SB42-4 Universal Postal Union (UPU), "International Postal Address Components and Templates".

[46]		      ISO 639-1:2002, “Codes for the representation of names of languages -- Part 1: 		      Alpha-2 code”

*** END OF FIRST MODIFICATION ***

*** SECOND MODIFICATION ***

- 4	Functional architecture

The following figures contain the reference configuration for the lawful interception. The circuit-switched configuration is shown in figure 1a. The packet-switched configuration is shown in figure 1b. Intercept configurations for HLR and IMS are shown in figures 1c and 1d. The WLAN interworking configuration is shown in figure 1e. The various entities and interfaces are described in more detail in the succeeding clauses. The additional intercept configurations for Evolved 3GPP Packet Switching Domain are described in clause 12.

PS domain of the UMTS system (GSN and Multimedia Packet Data services), 3GPP-WLAN interworking network and Evolved Packet Switching Domain provide UMTS/GSM/EPS customer's mobile equipment (UE) with connectivity service to another end of the communication. Another end of the communication may be a network element (server) or another UE. Therefore, UMTS/EPS system provides IP layer TS 23.008 [15] services. Hence, UMTS/EPS NO/AP is responsible only for IP layer interception of CC data. In addition to CC data, the LI solution for UMTS/EPS offers generation of IRI records from respective control plane (signalling) messages. The IP layer connectivity service is needed to support application layer TS 29.234 [16] service provision to UMTS/GSM/EPS customers. For instance, the following are examples of application layer services: email service; web browsing service; FTP service; audio services  (e.g. VoIP, PoC); other multimedia services (MBMS, video telephony); The majority of the application layer services require addition of respective server functionality to the network. Note that it is not necessary that such application layer SP should be the same commercial entity as the UMTS/EPS AP/NO in question.

NOTE 1:	Location information of the target, delivered by ICE, when it is available, may be completed by a request by the HI2 mediation function,(MF) every time the location information of the target (the existing cell ID or IP address of the e Node B / E (H) node B or hotspot ) go through the MF to a database that map cell ID or IP address by cell or WLAN coverage description based, for example, geographical or civic address information. Such data base have to be maintained and updated regularly.

This solution works only with specific Wi-Fi architectures as IAP and mediation will get and enrich HI2 from the IP address of the NAT access point to which is connected the WLAN or HnodeB, Such specific access point may be protected by a NAT, that is not the NAT protecting the GGSN/PGWto which is connected the hotspot or the small cell. It is recommended shall not to use such solution as the civic address may indicate wrong  locationinformation to the LEMF be used if the when the WLANi-Fi architectures is meshe d or complex WLAN architecture distributed with topology hidding or protection bring with NAT that will be shared by many hotspot or E Node B or e H Node B. The IP address of this NAT will give wrong information. National regulation will clarify if such usages is allowed with such architecture.

NOTE 2: For instance in MBMS a BM-SC and especially content providing server may be operated by different commercial entity than UMTS network.

Figure 1a: Circuit switched intercept configuration

Figure 1b: Packet Switched Intercept configuration

Figure 1c: HLR Intercept configuration

Figure 1d: IMS-CSCF Intercept configuration

Figure 1e: WLAN Interworking Intercept configuration

Figure 1f: IMS Conferencing Intercept configuration

The reference configuration is only a logical representation of the entities involved in lawful interception and does not mandate separate physical entities.

Regional Mediation Functions, which may be transparent or part of the administration and delivery functions, are used to convert information on the HI1, HI2 and HI3 interfaces in the format described in various national or regional specifications. For example, if ETSI ES 201 671 [3] or ANSI J-STD-025 [8] is used, then the adaptation to HI1, HI2 and HI3 will be as defined in those specifications.

There is one Administration Function (ADMF) in the network. Together with the delivery functions it is used to hide from the 3G ICEs that there might be multiple activations by different Law Enforcement Agencies (LEAs) on the same target. The administration function may be partitioned to ensure separation of the provisioning data from different agencies.

See the remaining clauses of this document for definitions of the X1_1, X1_2, X1_3, X2 and X3 interfaces.

Interception at the Gateways is a national option. However, if 3G direct tunnel functionality with the GGSN, as defined in TS 23.060 [10], is used in the network, then the GGSN shall perform the interception of IRI and the content of communications.

In figure 1a DF3 is responsible for two primary functions:

-	Call Control (Signalling) for the Content of Communication (CC); and

-	Bearer Transport for the CC.

HI3 is the interface towards the LEMF. It must be able to handle the signalling and the bearer transport for CC.

In figures 1a, 1b, 1e and 1f, the HI2 and HI3-interfaces represent the interfaces between the LEA and two delivery functions. The delivery functions are used:

-	to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2 (based on IAs, if defined);

-	to distribute the Content of Communication (CC) to the relevant LEA(s) via HI3 (based on IAs, if defined).

In figures 1c and 1d the HI2 interface represents the interface between the LEA and the delivery function. The delivery function is used to distribute the Intercept Related Information (IRI) to the relevant LEA(s) via HI2.

NOTE 3:	With reference to figure 1c, CC interception does not apply to HLR.

NOTE 4:	For IMS, figure 1d relates to the provision of IRI for SIP messages handled by the CSCF. Interception of CC for this case can be done at the GSN under a separate activation and invocation, according to the architecture in Figure 1b (see also clause 7.A.1).

*** END OF SECOND MODIFICATION ***

*** END OF MODIFICATIONS ***