SA3$#99-LI	 S3i250662

July 15-18, 2025, Overland Park (Kansas City)

Source:	Nokia

Title:	TS 33.128: Introduction to the CRs

Document for:	Agreement

Agenda Item:	10

Work Item / Release:	LI19-GUIDE

Abstract of the contribution: This document is information and gives an overview of the rationale for changing the text as proposed in the CRs submitted as input to TS 33.128.

BACKGROUND

While generating the pCRs to TR 33.929-9, the TS 33.127 and TS 33.128 were studied to align the TR 33.929-9 with the normative specifications: TS 33.127 and TS 33.128. The ATIS published ATIS-0700016 and the OMA.

In that effort, certain text in TS 33.128 which are probably errors were discovered. I had time to discuss only the change #1 with OTD to receive consensus.

DISCUSSION

Target non-local ID

The TS 33.127 says that the concept of target non-local ID does not apply to PTC.

### 7.6.8 Network topologies The PTC server resides in the home network. The PTC Server shall provide IRI-POI and CC-POI functionality as illustrated in figure 7.6.1.2-1. The concept of target non-local ID does not apply to PTC.

No effort is made to change the above text.

However, it would be worthwhile to discuss and agree on the above text is in TS 33.127 as intended.

In other words, when PTC client initiates a session to another PTC client (or ad-hoc group) to multiple destinations, the destination PTC Client ID when in the IMPU format cannot be a target non-local ID in the originating domain. Likewise, the Originating PTC client's ID when in the IMPU format, cannot be a target non-local in any of the destination domains.

Furthermore, when a PTC client joins a chat group PTC session, that PTC client's ID when in the IMPU format cannot be a target non-local ID in the chatgroup provider's domain. Likewise, the chatgroup ID cannot be a target non-local ID in the domains of any PTC clients joining the chat-group session.

CRs – corrections to the TS 33.128

# 1 Chat-group ID as the target ID not applicable to certain xIRIs:

Certain xIRIs defined in TS 33.128 are applicable only when a PTC Client is a target. The Chat Group ID which can be a target is not associated with any PTC Client. Therefore, it is necessary to remove the Chat Group ID from the list of target identities in the following xIRIs:

- PTC Registration.

- PTC pre-established session.

- PTC instant personal alert.

- PTC target presence.

- PTC participant presence.

- PTC list management.

- PTC access policy.

The CRs (co-signed by OTD) S3i250623 to S3i250626 remove the Chatgroup ID as one of the target ID possibilities from the above xIRIs. A change to the table that describe the xIRI parameters.

#2 TBCP: Release is from Client to Server:

The triggers listed for the Floor Control message has an error where the trigger says TBCP: Release is sent to the PTC Client whereas it is actually sent from the PTC Client to the PTC Server.

The CRs S3i250658 to S3i250661 fixes the wording.

#3 Party Join and Party Join when the PTC Client is a target:

The TS 33.128 o assumes that when a party joins an ongoing chat-group session, the participating PTC servers of PTC Clients previously present on the chat-group session would receive a SIP: NOTIFY message.

The following is assumed in TS 33.128:

According to OMA architecture diagram that is not the case. The following reflects the flow according to OMA spec:

Without such a NOTIFY, the Party Join and Party Drop xIRIs can only be generated from the IRI-POI present in the Chatgroup Server. That means those xIRIs can only be generated when the Chat-group ID is the target.

The CRs S3i250658 to S3i250661 fixes the wording.

PROPOSAL

If there is a consensus on the above points, then changes proposed in the CRs can be implemented.