3GPP TSG-SA3 Meeting #100-LI	s3i260051

# Sophia-Antipolis, France, 27th Jan 2026 - 30th Jan 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0309 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI for IMS DC Enhancements Stage 2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2026-01-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LI for application management for IMS Data Channel is missing from the specification. Additionally, corrections to the figure in 7.4.1 are made to separate MF and DC-AS and changes ‘DCSF’ to ‘DCSF/MMTel Enabler Server’ in the figure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add Stage 2 for LI for application bootstrap for IMS Data Channel to include new POI at the MMTel Enabler Server or additional xIRI reporting from the DCSF if it containes MMTel Enabler Server functionality. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remains incomplete. Solution for IMS Data Channel LI remains incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.4.1, 7.4.2.1, 7.4.3.1, 7.4.3.2, 7.4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Has content (figure 7.4.1-1 changes) merged from s3i260041. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Was s3i260012 |  |  |  |  |  |  |  |  |

**START OF CHANGES**

**START OF FIRST CHANGE**

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System".

[3]	3GPP TS 33.126: "Lawful interception requirements".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[6]	OMA-TS-MLP-V3_5-20181211-C: "Open Mobile Alliance; Mobile Location Protocol, Candidate Version 3.5", https://www.openmobilealliance.org/release/MLS/V1_4-20181211-C/OMA-TS-MLP-V3_5-20181211-C.pdf.

[7]	ETSI TS 103 120: "Lawful Interception (LI); Interface for warrant information".

[8]	ETSI TS 103 221-1: "Lawful Interception (LI); Internal Network Interfaces; Part 1: X1".

[9]	3GPP TS 33.501: "Security Architecture and Procedures for the 5G System".

[10]	ETSI GR NFV-SEC 011: "Network Functions Virtualisation (NFV); Security; Report on NFV LI Architecture".

[11]	3GPP TS 33.107: "3G Security; Lawful interception architecture and functions".

[12]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[13]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[14]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[15]	3GPP TS 33.128: "Protocol and Procedures for Lawful Interception; Stage 3".

[16]	ETSI TS 103 221-2: "Lawful Interception (LI); Internal Network Interfaces; Part 2: X2/X3".

[17]	MMS Architecture OMA-AD-MMS-V1_3-20110913-A.

[18]	Multimedia Messaging Service Encapsulation Protocol OMA-TS-MMS_ENC-V1_3-20110913-A.

[19]	3GPP TS 22.140: "Multimedia Messaging Service (MMS); Stage 1".

[20]	ETSI GS NFV-IFA 026: "Network Functions Virtualisation (NFV) Release 3; Management and Orchestration; Architecture enhancement for Security Management Specification".

[21]	3GPP TS 33.108: "Handover Interface for Lawful Interception (LI)".

[22]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for 
Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[23]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[24]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[25]	OMA-AD-PoC-V2_1-20110802-A: "Push to talk over Cellular (PoC) Architecture".

[26]	GSMA IR.92: "IMS Profile for Voice and SMS".

[27]	GSMA NG.114: "IMS Profile for Voice, Video and Messaging over 5GS".

[28]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[29]	ETSI GS NFV-SEC 012: "Network Functions Virtualisation (NFV) Release 3; Security; System architecture specification for execution of sensitive NFV components".

[30]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[31]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage3".

[32]	3GPP TS 29.122: "T8 reference point for Northbound APIs".

[33]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[34]	OMA-AD-CPM-V2_2-20170926-C: "Open Mobile Alliance, OMA Converged IP Messaging System Description", http://www.openmobilealliance.org/release/CPM/V2_2-20200907-C/OMA-AD-CPM-V2_2-20170926-C.pdf.

[35]	GSMA RCC.07: "Rich Communication Suite – Advanced Communications Services and Client Specification".

[36]	IETF RFC 4975: "The Message Session Relay Protocol (MSRP)".

[37]	IETF RFC 6714: "Connection Establishment for Media Anchoring (CEMA) for the Message Session Relay Protocol (MSRP)".

[38]	IETF RFC 3862: "Common Presence and Instant Messaging (CPIM): Message Format".

[39]	3GPP TS 24.229: "IP Multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[40]	IETF RFC 8224: "Authenticated Identity Management in the Session Initiation Protocol (SIP)".

[41]	IETF RFC 8946: "Personal Assertion Token (PASSporT) Extension for Diverted Calls".

[42]	IETF RFC 9795: "Personal Assertion Token (PASSporT) Extension for Rich Call Data".

[43]	IETF RFC 7095: "jCard: The JSON Format for vCard".

[44]	3GPP TS 24.196: "Enhanced Calling Name (eCNAM)".

[45]	IETF RFC 8816: "Secure Telephone Identity Revisited (STIR) Out-of-Band Architecture and Use Cases".

[46]	IETF RFC 9475: "Messaging Use Cases and Extensions for Secure Telephone Identity Revisited (STIR)".

[47]	3GPP TS 33.535: "Authentication and Key Management for Applications (AKMA) based on 3GPP credentials in the 5G System (5GS)".

[48]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[49]	3GPP TS 33.222: "Generic Authentication Architecture (GAA); Access to network application functions using Hypertext Transfer Protocol over Transport Layer Security (HTTPS)".

[50]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[51]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[52]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[53]	3GPP TS 26.501: "5G Media Streaming (5GMS); General description and architecture".

[54]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[55]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[56]	3GPP TS 38.455: "NR Positioning Protocol A (NRPPa)".

[57]	3GPP TS 36.455: "LTE Positioning Protocol A (LPPa)".

[58]	3GPP TS 23.140: "Multimedia Messaging Protocol. Functional Description. Stage 2".

[59]	3GPP TS 32.303: "Proximity-based Services (ProSe); Stage 2".

[60]	3GPP TS 23.304: "Proximity based Services (ProSe) in the 5G System (5GS)".

[61]	3GPP TS 32.277: "Proximity-based Services (ProSe) charging".

[62]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[63]	3GPP TS 32.240: "Telecommunications management;Charging management; Charging architecture and principles".

[64]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[65]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[66]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[67]	ETSI TS 104 007: "Lawful Interception (LI); Lawful Interception Architecture".

[68]	ETSI TS 104 000: "Lawful Interception (LI); Internal Network Interface X0".

[69]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[XX]	3GPP TS 23.392: "Application enablement aspects for MMTel".

**END OF FIRST CHANGE**

**START OF SECOND CHANGE**

### 7.4.1	General

Figure 7.4.1-1 depicts the EPS/5GS-Anchored IMS High Level LI Architecture.

Figure 7.4.1-1: EPS/5GS-Anchored IMS High Level LI Architecture

**END OF SECOND CHANGE**

**START OF THIRD CHANGE**

#### 7.4.2.1	Overview

The capabilities defined in this clause apply to the interception of IMS-based services. The target of interception can be a subscriber of the CSP, an inbound roamer or a non-local ID.

The network function involved in providing the interception of IMS-based services are determined based on the deployment option, the network configuration, LI service scope and the IMS session including the roaming scenarios. The IRI-POI functions are provided by the network functions that handle the IMS signalling messages (those network functions are referred to as IMS Signalling Functions) and the triggered CC-POI functions are provided by the network functions that handle the media (these network functions are referred to as IMS Media Functions). The CC-TF functions are also provided by the network functions that handle the IMS signalling messages (referred to as IMS Signalling Functions) and manage the IMS Media Functions. The network functions that provide the CC-TF functions can be different from the network functions that provide the IRI-POI functions.

NOTE 1:	The concepts presented above also apply to IMS Data Channel sessions to include bootstrap and application management procedures.

NOTE 2: 	The term "IMS signalling messages" usually refers to SIP messages. However, when DCSF provides the IRI-POI/CC-TF functions (see figure 7.4-1), the term refers to HTTP messages. As a result, parameter used for a specific LI function may not always be a SIP parameter.

NOTE 3:	The term "IMS Signalling Function" may include DCSF as well in some instances.

An architecture depicting the LI for IMS is depicted in figure 7.4.2.1-1 below.

Figure 7.4.2.1-1: IMS LI architecture

The LICF present in the ADMF receives the warrant from an LEA, derives the intercept information from the warrant and provides it to the LIPF. The LIPF present in the ADMF provisions IRI-POI, CC-TF, MDF2 and MDF3 over the LI_X1 interfaces.

The CC-TF sends the CC intercept trigger to the CC-POI over LI_T3 interface. The IRI-POI generates the xIRI and delivers the same to the MDF2 over LI_X2 interface. The CC-POI generates the xCC and delivers the same to the MDF3 over LI_X3 interface.

The MDF2 generates IRI messages from the received xIRI and delivers those IRI messages to the LEMF over LI_HI2 interface. The MDF3 generates the CC from the received xCC and delivers that CC to the LEMF over LI_HI3 interface.

The network configuration and IMS service scenarios including the roaming scenarios determine the network functions that provide the IRI-POI, CC-TF and CC-POI functions. The network function that provides the IRI-POI or CC-TF is referred to as IMS Signalling Function in figure 7.4.2.1-1 and the network function that provides the CC-POI functions is referred to as IMS Media Function in figure 7.4.2.1-1.

NOTE 4:	The details of correlation between the xIRI and the xCC when IRI-POI and CC-TF are not co-located is not defined in the present document. The IRI-POI and CC-TF are logical functions and they may be handled by the same process when they are co-located in the same IMS Signalling Function.

**END OF THIRD CHANGE**

**START OF FOURTH CHANGE**

#### 7.4.3.1	General

The IRI-POI detects the IMS Signalling messages that are related to a target subscriber and then generates and delivers the related xIRI to the MDF2 over LI_X2.

The following IMS Network Functions (i.e. IMS Signalling Functions) that handle SIP signalling for IMS sessions may provide the IRI-POI functions:

-	S-CSCF.

-	E-CSCF.

-	P-CSCF.

-	IBCF.

-	MGCF.

-	Conference AS/MRFC.

-	Telephony AS.

The following IMS Network Function that handles the HTTP messages for IMS Data Channel may provide the IRI-POI functions:

-	DCSF.

The following IMS Network Function handles application management and bootstrap for IMS Data Channel and may provide additional IRI-POI function:

- MMTel Enabler Server.

The DSCF may have the functionality of the MMTel Enabler Server built in. In such a case, the DCSF provides both the bootstrap and application management IRI-POI functions as well as the data channel HTTP messages IRI-POI.

Clause 7.4.6 gives more information from network topology/session perspective how different IMS Network Functions are to be used in providing the IRI-POI functions. The Telephony AS is one of the IMS Network Functions that provides the IRI-POI for STIR/SHAKEN and RCD/eCNAM (see clause 7.14.2).

#### 7.4.3.2	IRI events

The IRI-POI present in the IMS Signalling Function generates the following xIRI:

-	Encapsulated SIP message.

-	CC unavailable in serving PLMN.

-	Start of interception with an established IMS session.

-	Data channel setup message.

-	Data channel termination message.

-	Start of interception with an established IMS data channel.

-	IRI Only RTP Packet.

-	Data channel application response message.

The encapsulated SIP message xIRI is generated and delivered to the MDF2 when the IRI-POI in the IMS Signalling Function detects that a SIP message is received from, or sent to, a target or processed on behalf of a target at the IMS Signalling Function.

The CC unavailable in PLMN xIRI is generated and delivered to the MDF2 for the session scenarios where access to the target media is not available to the CSP (see clause 7.4.7.1).

The start of interception with an established IMS session xIRI is generated when an interception is activated on an established IMS session. To support the possibility of generating such an xIRI, the IMS Signalling Function shall store and maintain the session related information including the media information for the life of all IMS sessions.

The data channel setup message xIRI is generated and delivered to the MDF2 when the IRI-POI in the DCSF detects that the DCSF has received a DC call event from the IMS AS in the form of a DC control request and has responded with a media instruction set (see TS 23.228 [13], clause AC.7.1).

NOTE 1:	Data channel setup invokes procedures within the HSS. The reporting of these events is defined in clause 7.2.3 of the present document.

The data channel termination message xIRI is generated and delivered to the MDF2 when the IRI-POI in the DCSF detects that either UE has triggered a SDP renegotiation to release the application data channel or the application has been closed (see TS 23.228 [13], clause AC.7.6).

NOTE 2:	LI for IMS DC across international NNI is not considered in the present document.

NOTE 3:	Roaming cases for IMS data channel are not considered in the present document.

The start of interception with established IMS data channel xIRI is generated and delivered to the MDF2 when the IRI-POI in the DCSF detects that an interception has been activated for a target with an already established IMS data channel session.

The IRI Only RTP Packet xIRI is generated and delivered when authorized IRI is present in the RTP Stream (e.g. when the delivery of Post Dialled Digits is required).

The data channel application response message xIRI is generated and delivered to the MDF2 when the IRI-POI in the MMTel Enabler Server detects that the MMTel Enabler Server has responded to a Get Root application request message, sent from the MMTel Enabler Client (see TS 23.392 [XX], clause 8.3.2.2) or when the IRI-POI in the MMTel Enabler Server detects that the MMTel Enabler Server has responded to an DC Application Profile request (see TS 23.392 [XX], clause 8.3.3.2).

**END OF FOURTH CHANGE**

**START OF FIFTH CHANGE**

#### 7.4.6.2	IMS Network Functions providing the IRI-POI

The IMS Network Functions that handle the target side of the session provide the IRI-POI functions except when the alternate option is used for the non-local ID target. When the alternate option is used for the non-local ID target, the IMS network function that handles the session-leg of the local served user connected directly to the non-local ID target.

Table 7.4.6.2-1 below identifies the IMS Network Functions in providing the IRI-POI functions in a non-roaming case for various session scenarios.

Table 7.4.6.2-1: IMS Network Functions providing the IRI-POI functions (non-roaming case)

| Session type/target type | Default | Alternative option |
| --- | --- | --- |
| Normal sessions | S-CSCF | P-CSCF |
| SMS over IMS | S-CSCF | P-CSCF |
| Emergency sessions | E-CSCF | P-CSCF (NOTE 1) |
| SMS over IMS to emergency services | E-CSCF | P-CSCF (NOTE 1) |
| Redirected sessions: intra-PLMN | S-CSCF | P-CSCF |
| Redirected sessions: inter-PLMN (CS domain) | S-CSCF | MGCF |
| Redirected sessions: inter-PLMN (IMS domain) | S-CSCF | IBCF |
| Conference (NOTE 2) | Conf-AS/MRFC | - |
| Non-local ID in CS domain (NOTE 3, NOTE 3A) | MGCF | S-CSCF |
| Non-local ID in IMS domain (NOTE 3, NOTE 3A) | IBCF | S-CSCF |
| Non-local ID for SMS over IMS (NOTE 3) | S-CSCF | P-CSCF (NOTE 3A) |
| Data Channel Sessions | DCSF, MMTel Enabler Server | - |

NOTE 1:	For originated emergency sessions (or SMS over IMS to emergency services centre) handled in the fixed networks, where S-CSCF is also part of an emergency session, the S-CSCF based IRI-POI as a deployment option may also be considered.

NOTE 2:	A conference ID can also be a target. Conf-AS stands for conference AS (see NOTE 2 in clause 7.4.4.1). When a normal session is extended to a conference session, the IMS signalling functions that provide the IRI-POI functions prior to the conference may continue to provide the IRI-POI functions in addition to the conference AS/MRFC.

NOTE 3:	Non-roaming means that the local served user is non-roaming.

NOTE 3A:	The default/alternate option used when the target is non-local ID is mutually independent of default/alternate option used when the target is local served user.

Table 7.4.6.2-2 below identifies the IMS Network Functions in providing the IRI-POI functions in a roaming case for various session scenarios.

Table 7.4.6.2-2: IMS Network Functions providing the IRI-POI functions (roaming case)

| Session type/target type | Local Breakout (LBO) |  |  |  | Home Routed (HR) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HPLMN |  | VPLMN |  | HPLMN |  | VPLMN |  |
|  | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option | Default | Alternate Option |
| Normal sessions | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| SMS over IMS | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| Emergency sessions/SMS over IMS | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| SMS over IMS to emergency services | - | - | E-CSCF | P-CSCF | - | - | E-CSCF | P-CSCF |
| Redirected sessions | S-CSCF | See table 7.4.6.2-3 | - | - | S-CSCF | See table 7.4.6.2-3 | - | - |
| Conference (NOTE 2) | Conf-AS/MRFC | - | - | - | Conf-AS/MRFC | - | - | - |
| Non-local ID (E.164) in CS domain (NOTE 3A, NOTE 4, NOTE 4A) | MGCF | S-CSCF | P-CSCF | IBCF (NOTE 4B) | MGCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID in SIP/IMS domain (NOTE 3A, NOTE 4, NOTE 4A) | IBCF | S-CSCF | P-CSCF | IBCF (NOTE 4B) | IBCF | S-CSCF (NOTE 3A) | N9HR/S8HR | - |
| Non-local ID for SMS over IMS (NOTE 4) | S-CSCF | IBCF | P-CSCF | - | S-CSCF | P-CSCF | N9HR/S8HR | - |
| Data Channel Sessions | DCSF, MMTel Enabler Server | - | n/a | n/a | DCSF, MMTel Enabler Server | - | n/a | n/a |

NOTE 4:	For roaming, this means the local served user is roaming. Also, see NOTE 3.

NOTE 4A:	The default/alternate options used in the HPLMN and default/alternate options used in the VPLMN are mutually independent.

NOTE 4B:	This alternate option may be used only in the VPLMN for IMS sessions with home-routed media.

The interception capabilities for normal sessions as defined in tables 7.4.6.2-1 (non-roaming) and 7.4.6.2-2 (roaming) shall be used for the cases where the Conf-AS and the PTSC-Server are not under the control of CSP serving the warrant.

Table 7.4.6.2-3: Extension of table 7.4.6.2-2

| Session type/target type |  | Local Breakout (LBO) | Home Routed (HR) |
| --- | --- | --- | --- |
| Redirected sessions: Intra-PLMN | Redirected-to party non-roaming | P-CSCF | P-CSCF |
|  | Redirected-to party is roaming | IBCF | P-CSCF |
| Redirected sessions Inter-PLMN | Redirected-to party in CS domain | MGCF | MGCF |
|  | Redirected-to party in IMS domain | IBCF | IBCF |

Table 7.4.6.2-3 shows the IMS Network Functions that provide the IRI-POI functions in the HPLMN for redirected sessions in a roaming case when the alternate option is used to provide the IRI-POI functions for the normal case.

NOTE 5:	For the redirected do not answer related sessions, the IMS Network Functions that provide the IRI-POI functions prior to the redirection are as illustrated in table 7.4.6.2-2 (normal case) and after the redirection are as illustrated in table 7.4.6.2-3.

The IMS Network Functions that provide the IRI-POI for STIR/SHAKEN and RCD/eCNAM are listed in clause 7.14.2.

**END OF FIFTH CHANGE**

**END OF ALL CHANGES**