3GPP TSG-SA3 Meeting #100-LI	s3i260054

# Sophia-Antipolis, France, 27th Jan 2026 - 30th Jan 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.128 | CR | 0797 | rev | 1 | Current version: | 18.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to User Identifiers Structures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3-LI (OTD_US) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI18 |  |  |  |  |  | Date: |  |  | 2026-01-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The ASN.1 structures for groups of target identifiers differ resulting in incorrect reporting in some messages. Across the various identifier sets, the ASN.1 structure defines a way to send one or more sequences of identifiers, a single sequence of identifiers or a single identifier. This results in the inability to send multiple identifiers when required. The tables describing the identifier structures also do not match the ASN.1 adding to the potential confusion. This contribution proposes a backwards compatible solution that minimizes the impacts to existing implementations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrects ASN.1 and tables for identifier groups. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It will not be possible to report all required identifiers in some messages. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.3.3, 7.13.3.1.2.2, 8.3.2.1, 8.3.2.2, New 8.3.2.X1, New 8.3.2.X2, Attachment TS33128Payloads.asn |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Schema changes for this CR can be found on the Forge: Merge Request: !353 Commit Hash: 53da2d3575edea6aa563683830473fdfee735edb TS 33.128 CR 0798 is the release 19 mirror of this CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | s3i260014 |  |  |  |  |  |  |  |  |

## **** START OF FIRST CHANGE (MAIN DOCUMENT) ****

##### 7.2.3.3.3	Start of Interception with target registered at the HSS

The IRI-POI in the HSS shall generate an xIRI containing the HSSStartOfInterceptionWithRegisteredTarget record when the IRI-POI present in the HSS detects that interception is activated for a UE that has already been registered at the HSS.

The HSS may have stored target subscription data for both EPC and IMS. In such a case, a single HSS Start of Interception with Registered Target xIRI shall be generated containing the target context.

Table 7.2.3.3.3-1: Payload for HSSStartOfInterceptionWithRegisteredTarget record

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| hSSIdentities | HSSIdentities | 1 | Indicates the identifiers for which the subscription data sets apply. Shall include one or more subscriber identifier. See TS 29.562 [101] clause 6.2.3.1. | M |
| subscriptionDataSets | SubscriptionDataSets | 1 | Includes current subscription information for the target UE stored at the HSS. Encoded according to TS 29.562 [101] clause 6.2.6.2.4. The SBIReference for this parameter shall be populated with 'TS29562_Nhss_imsSDM.yaml#/components/schemas/ImsProfileData'. | M |
| pSUserState | SBIType | 1 | Indicates the user state in the PS domain as known at the HSS. Encoded according to TS 29.562 [101] clause 6.2.6.3.15. The SBIReference for this parameter shall be populated with 'TS29562_Nhss_imsSDM.yaml #/components/schemas/PsUserState'. | M |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

###### 7.13.3.1.2.2	Type: RCSIdentity

Table 7.13.3.1.2.2-1: Choices for RCSIdentity parameter

| Choice name | Type | Description |
| --- | --- | --- |
| fiveGSIdentities | FiveGSSubscriberIDs | Shall be chosen if the identities being reported are 5GS Identities. |
| ePSIdentities | EPSSubscriberIDs | Shall be chosen if the identities being reported are EPS Identities. |
| iMSIdentities | IMSSubscriberIDs | Shall be chosen if the identities being reported are IMS Identities. |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 8.3.2.1	Type: UserIdentifiers

As there are often multiple identifiers that may be known at an NF or by the MDF, a single type capable of reporting multiple User Identifiers was defined.

Table 8.3.2.1-1: Structure of the UserIdentifiers type

| Field name | Type | Cardinality | Description | M/C/O |
| --- | --- | --- | --- | --- |
| fiveGSSubscriberIDs | FiveGSSubscriberIDs | 0..1 | Contains the list of 5GS identifiers for a user. Shall be present when any 5GS identifiers are known at the NF where the POI is located or at the MDF. | C |
| ePSSubscriberIDs | EPSSubscriberIDs | 0..1 | Contains the list of EPS identifiers for a user. Shall be present when any EPS identifiers are known at the NF where the POI is located or at the MDF. | C |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 8.3.2.2	Sequence Of user identifier types

Table 8.3.2.2-1 contains the details for types that consist only of a SEQUENCE OF or SET OF.

Table 8.3.2.2-1: Details of SEQUENCE OF Types

| Type name | Definition | Cardinality | Description |
| --- | --- | --- | --- |
| FiveGSSubscriberIDs | SEQUENCE OF FiveGSSubscriberID | 1..MAX | Contains the list of 5GS identifiers for a user. Shall be populated with any 5GS identifiers are known at the NF where the POI is located or at the MDF. |

## **** START OF NEXT CHANGE (MAIN DOCUMENT) ****

#### 8.3.2.X1	Type: IMSSubscriberIDs

Table 8.3.2.X1-1: Definition of Choices for IMSSubscriberIDs

| CHOICE | Type | Description |
| --- | --- | --- |
| deprecatedIMPI | IMPI | No longer used in this version of the specification. |
| deprecatedIMPU | IMPU | No longer used in this version of the specification. |
| iMSSubscriberIDs | SEQUENCE (SIZE (1..MAX)) OF IMSSubscriberIdentity | Contains the list of IMS identifiers for a user. Shall be populated with any IMS identifiers are known at the NF where the POI is located or at the MDF. |

#### 8.3.2.X2	Type: IMSSubscriberIdentity

Table 8.3.2.X2-1: Definition of Choices for IMSSubscriberIdentity

| CHOICE | Type | Description |
| --- | --- | --- |
| iMPI | IMPI | Chosen when the identifier being reported is an IMPI. |
| iMPU | IMPU | Chosen when the identifier being reported is an IMPU. |
| pEI | PEI | Chosen when the identifier being reported is a PEI. |
| iMEI | IMEI | Chosen when the identifier being reported is an IMEI. |

## **** END OF MAIN DOCUMENT CHANGES ****

## **** START OF FIRST CHANGE (ATTACHMENTS) ****

---a/33128/r18/TS33128Payloads.asn
+++b/33128/r18/TS33128Payloads.asn

@@ -6613,8 +6613,17 @@ IMSIUnauthenticatedIndication ::= BOOLEAN

6613	6613

6614	6614		IMSSubscriberIDs ::= CHOICE

6615	6615		{

6616		-	    iMPI        [1] IMPI,

6617		-	    iMPU        [2] IMPU

6616	+	    deprecatedIMPI        [1] IMPI,

6617	+	    deprecatedIMPU        [2] IMPU,

6618	+	    iMSSubscriberIDs      [3] SEQUENCE (SIZE (1..MAX)) OF IMSSubscriberIdentity

6619	+	}

6620	+

6621	+	IMSSubscriberIdentity ::= CHOICE

6622	+	{

6623	+	    iMPI [1] IMPI,

6624	+	    iMPU [2] IMPU,

6625	+	    pEI  [3] PEI,

6626	+	    iMEI [4] IMEI

6618	6627		}

6619	6628

6620	6629		Initiator ::= ENUMERATED

## **** END OF ALL DOCUMENT CHANGES ****