3GPP TSG-SA3 Meeting #100-LI	s3i260058

# Sophia-Antipolis, France, 27th Jan 2026 - 30th Jan 2026

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.127 | CR | 0314 | rev | - | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | LI on Indirect Network Sharing (INS) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | SA3LI (Ministère Economie et Finances) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code: | LI19 |  |  |  |  |  | Date: |  |  | 2026-01-28 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Absence of LI for Indirect Network Sharing in Stage 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add LI for Indirect Network Sharing in Stage 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The LI for Indirect Network Sharing would still be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.7, 6.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF FIRST CHANGE ***

#### 6.2.2.7	Network topologies

The AMF shall provide the IRI-POI functions in the following network topology cases:

-	Non-roaming case.

-	Roaming case, in VPLMN.

-	Roaming case, in HPLMN for non-3GPP access.

In a roaming case, it is possible that the target UE may use non-3GPP access with the N3A Entity present in the HPLMN.

The non-roaming case includes the AMF operating in the context of a Hosting Operator for a subscription held by a Participating Operator in an Indirect Network Sharing deployment as described in TS 23.501 [2] clause 5.18.

*** END OF FIRST CHANGE ***

*** START OF SECOND CHANGE ***

#### 6.2.3.6	Network topologies

The SMF shall provide the IRI-POI functions in the following network topology cases:

-	Non-roaming case.

-	Roaming case, in VPLMN.

-	Roaming case, in HPLMN.

-	Non-3GPP access case, in the PLMN where N3A Entity resides.

The non-roaming case includes the SMF operating in the context of either a Hosting or Participating Operator in an Indirect Network Sharing deployment as described in TS 23.501 [2] clause 5.18.

When the target UE has multiple PDU sessions active, the generation and delivery of xCC for each PDU session shall be done independently, each with separate correlation information.

When a target UE's PDU session involves multiple Data Network (DN) connections (i.e. multiple connections to the same DN as described in clause A.3 of the present document), the generation and delivery of xCC shall be done in such a way that:

-	All applicable user plane packets are captured and delivered.

-	Duplicate delivery of CC is suppressed to the extent possible.

-	Each user plane packet is delivered with the associated DN Access Identifier (DNAI).

A PDU session may involve more than one UPFs. In that case, the CC-TF present in the SMF shall determine which UPF(s) is (are) more suitable to provide the CC-POI functions adhering to the above requirements. Furthermore, independent of which UPF is used to generate the xCC, the CC delivered from the MDF3 shall be correlated to the IRI messages related to the PDU session.

Similarly, the CC-POI in the STF is responsible for providing the xCC for the target's communication traffic in a decrypted form.

The remainder of the present clause provides details of IRI-intercept and, as applicable, CC-intercept of specific services encrypted by CSP-provided keys.

*** END OF SECOND CHANGE ***