| 3GPP TS 33.129-1 V0.0.10 (2026-01) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security; Security assurance specification for 3GPP Lawful Interception system; Common; (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. ©2026, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Adaptations of TS 33.117 catalogue of security requirements and related use cases	9

4.1	Introduction	9

4.1.1	General	9

4.1.2	Use of tools in testing	10

4.1.3	Documentation requirements	10

4.1.4	Requirements list	10

4.1.5	Document Part Organization	10

4.2	Security functional requirements and related test cases specific to LI functions	11

4.2.1	Introduction	11

4.2.2	Security functional requirements on LI functions deriving from 3GPP specifications and related test cases	11

4.2.2.1	General	11

4.2.2.2	< template test for security functional test >	12

4.2.2.3	Security functional requirements on LI functions deriving from 3GPP specifications and related test cases – general interface aspects	12

4.2.2.3.1	Introduction	12

4.2.2.3.2	Protection at the transport layer	12

4.2.2.3.3	Authorization of NF service access	12

4.2.2.4	Credential affinity	13

4.2.2.5	Credential expiry	13

4.2.2.6	Credential chain of trust	13

4.2.2.7	Credential binding	13

4.2.2.8	Element of LI binding	13

4.2.3	Technical baseline	13

4.2.3.1	Introduction	13

4.2.3.2	Protecting data and information	13

4.2.3.2.1	Protecting data and information – general	13

4.2.3.2.1.1	Introduction	13

4.2.3.2.1.2	A crypto inventory exists	13

4.2.3.2.1.3	Replaceable crypto modules	14

4.2.3.2.1.4	Replaceable crypto methods	15

4.2.3.2.2	Protecting data and information – unauthorized viewing	16

4.2.3.2.2.1	Introduction	16

4.2.3.2.2.2	General-purpose logs must not contain LI data	16

4.2.3.2.2.3	General-purpose logs must not contain LI data (comms?)	17

4.2.3.2.2.4	LI logging is segregated from non-LI logging	18

4.2.3.2.2.5	LI target leak, log leak	19

4.2.3.2.3	Protecting data and information in storage	19

4.2.3.2.3.1	Introduction	19

4.2.3.2.3.2	No unencrypted LI data	19

4.2.3.2.4	Protecting data and information in transfer	19

4.2.3.2.4.1	Introduction	19

4.2.3.2.4.2	No unencrypted internal comms	20

4.2.3.2.4.3	Network observation by VI admin	20

4.2.3.2.4.4	Firewalls between virtual entities	20

4.2.3.2.5	Logging access to personal data	20

4.2.3.3	Protecting availability and integrity	20

4.2.3.3.1	Introduction	20

4.2.3.3.2	Vulnerability patching – original code	20

4.2.3.3.3	Vulnerability patching – open source	20

4.2.3.4	Authentication and authorization	20

4.2.3.4.1	Introduction	20

4.2.3.4.2	Logging of failed authentication attempts	20

4.2.3.4.3	No non-LI accounts	20

4.2.3.4.4	Unique credentials	20

4.2.3.4.5	System hardening of VM accounts	20

4.2.3.5	Protecting sessions	20

4.2.3.6	Logging	20

4.2.3.6.1	Introduction	20

4.2.3.6.2	Mandatory log-disabling alarms	20

4.2.3.6.3	Loud logs	20

4.2.4	Operating Systems	20

4.2.5	Web Servers	21

4.2.6	Network Devices	21

4.3	Adaptations of hardening requirements and related test cases specific to LI functions	21

4.3.1	Introduction	21

4.3.2	Technical baseline	21

4.3.2.1	Introduction	21

4.3.2.2	Third-party components are up-to-date	21

4.3.3	Operating systems	21

4.3.4	Web servers	21

4.3.4.1	Introduction	21

4.3.4.2	Only POST allowed (for Xn)	21

4.3.4.3	Third-party components are up-to-date	21

4.3.5	Network devices	21

4.3.6	Network functions in service-based architecture	21

4.4	Adaptations of basic vulnerability testing requirements and related test cases specific to LI functions	21

4.4.1	Introduction	21

4.4.2	Port Scanning	22

4.4.3	Vulnerability scanning	22

4.4.4	Robustness and fuzz testing	22

5	Security Elements	22

5.1	Assets	22

5.1.1	Assets Overview	22

5.1.2	Detailed Asset Descriptions	22

5.1.2.1	Target Identifiers	22

5.1.2.2	LI Access Credentials	23

5.1.2.3	 Network Function	23

5.1.2.4	 Network Service	23

5.1.2.5	 LI Function	23

5.1.2.6	LI Product Delivery	23

5.1.2.7	LEMF Attributes	23

5.1.2.8	Network User Behaviour	24

5.1.2.9	Crypto Store	24

5.1.2.10	API or Configuration State	24

5.1.2.11	LI Product	24

5.2	Attackers	25

5.2.1	Attackers Overview	25

5.2.2	Detailed Attacker Descriptions	25

5.2.2.1	 Internal Administrator	25

5.2.2.2	Internal Administrator Granular Breakdown	26

5.2.2.2.1	Internal Log Auditor	26

5.2.2.2.2	Internal Configuration Manager	26

5.2.2.2.3	Internal Infrastructure/Platform Administrator	26

5.2.2.2.4	Internal Analytics or Monitoring Observer	26

5.2.2.2.5	Internal Facilities or Data Center Operator	27

5.2.2.3	 External Agent	27

5.2.2.4	 Automated or AI Agent	27

5.2.2.5	 Passive Observer	27

5.2.2.6	Compromised Network Function	28

5.3	Threats	28

5.3.1	Threats Overview	28

5.3.2	Detailed Threat Descriptions	28

5.3.2.1	Log Leak	28

5.3.2.2	Config Leak	29

5.3.2.3	LI Leak via CPU Side Channel 1	29

5.3.2.4	LI Leak via CPU Side Channel 2	29

5.3.2.5	LI Leak via Network Flow Side Channel	29

5.3.2.6	LI Detection via Timing Anomalies	29

5.3.2.7	MDF–LEMF LI Product Flow Detection	30

5.3.2.8	POI–MDF Link Failure	30

5.3.2.9	Session Count	30

5.3.2.10	Interface Security	30

5.3.2.11	Unauthorized Use of Unnecessary Features	30

5.3.2.12	Unauthorized Use of Unnecessary Services	31

6	Common Tests	31

6.1	Supply Chain	31

6.1.1	 Vendor	31

6.1.1.1	 Standardized Cryptographic Algorithms and Primitives	31

6.1.1.2	Replaceable Cryptographic Modules	31

6.1.1.3	Configurable and Exchangeable Cryptographic Methods	31

6.1.1.4	Log Leak of LI Identifiers During LI Provisioning	31

6.1.1.5	Log Leak of LI Identifiers During Communication Time	31

6.1.1.6	Log Segregation	31

Annex <A> (normative):	<Normative annex for a Technical Specification>	31

Annex <B> (informative):	Vulnerability/Compromise Spectrum	32

B.1	Vulnerability/Compromise Spectrum	32

EDITORS NOTES:	34

Parking Lot	34

Annex <C>: <Informative annex title for a Technical Report>	35

Annex <D> (informative):	Bibliography	36

Annex <E> (informative):	Index	37

Annex <F> (informative):	Change history	38

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it shall be the second unnumbered clause.

# 1	Scope

The present document contains objectives, requirements and test cases that are specific to the Lawful Interception (LI) elements of a 3GPP network.

The present document refers to TS 33.117 [2] and formulates specific adaptions of the requirements and test cases given there, as well as specifying additional requirements and test cases unique to the Lawful Interception system. This part of the present document covers the general framework for LI SCAS, terminology, objectives, baseline requirements and test cases common, possibly after adaptation, to all elements of the 3GPP LI system. The remaining parts of this document cover additional requirements and test cases specific to elements that fulfil specific roles within the LI system. The specific requirements and tests applicable to a specific function or network element shall be the union of those listed in TS 33.117 [2], the common part and those listed in the parts applicable to any role that function is responsible for. It is not an intention of the present document to provide an exhaustive set of test cases that would be sufficient to demonstrate conformance with all security procedures or requirements.

Editor's note (April 7th, 2025):

Also, consider breaking down along this line: elements that have provisioning ability and elements that don't. The idea is that compromising an element that does have provisioning ability into other elements can be more damaging, while taking control of an element that does not have provisioning ability keeps the damage localized.

# 2	References

Editor's Note: 1) English vs American spelling (e.g., "artefacts" vs "artifacts"), 2) full stops in lists.

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.916: "Security Assurance Methodology (SECAM) for 3GPP network products".

[4]	3GPP TS 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[5]	3GPP TS 33.126: "Lawful Interception Requirements".

[6]	3GPP TS 33.127: "Lawful Interception (LI) architecture and functions".

[7]	3GPP TS 33.128: "Protocol and procedures for Lawful Interception (LI); Stage 3".

[8]	GSMA FS.50: "Network Equipment Security Assurance Scheme - Requirements for Security Assurance Specification Development".

[9]	GSMA FS.62: "Network Security Assurance Scheme - Adoption Procedure for Security Assurance Specifications".

[10]	3GPP TS 33.129-2: "Security assurance specification for 3GPP Lawful Interception system; NF-Embedded LI functions"

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

It is preferred that the reference to TR 21.905 be the first in the list.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Adaptations of TS 33.117 catalogue of security requirements and related use cases

Editor's Note: It may be worth updating the structure of this to mirror TS 33.117 (as done in TS 33.129-2)

## 4.1	Introduction

### 4.1.1	General

The present document was developed based on the guidance and methodology described in 3GPP TR 33.916 [3]. 3GPP has organized SCAS documents by publishing one Technical Specification (TS) per architectural network element. In contrast, this document takes a different approach and collects all LI SCAS in the present document.

Within each clause, TS 33.126 [5] is used to derive LI element specific requirements and tests. Requirement numbering from TS 33.126 [5] is referenced herein to enable compliance tracking.

Security requirements for LI functions include requirements derived from LI-specific security functional requirements in relevant specifications (e.g. TS 33.126 [5], TS 33.127 [6] and TS 33.128 [7]), the requirements listed in TS 33.117 [2] as well as the security requirements described as common to the LI system as described in the present document. Unless explicitly stated, tests in the present document will be applied to both the enclosing Network Function as well as to the enclosed LI function.

Unless otherwise specified, the prerequisites for testing listed in TS 33.117 [2] apply to the tests in this document. Additionally, unless otherwise specified, the prerequisites in this clause apply to all tests in any part of this document.

There are multiple privilege levels in the network. At a minimum, there are at least two:

-	Lawful Interception privileged.

-	Non-LI privileged administrator.

-	Non-administrator.

Deployments in the field may split access more granularly, but this is the minimum required to perform tests in the present specification.

### 4.1.2	Use of tools in testing

### 4.1.3	Documentation requirements

### 4.1.4	Requirements list

The following TS 33.126 [5] requirements are tested in the present document:

TS 33.126 R6.6-30: The CSP shall ensure that non-authorized personnel or processes (including automated or Artificial Intelligence based systems) that are part of the service cannot detect that interception is taking place.

### 4.1.5	Document Part Organization

Editor's Note: The part structure of 33.129 should be re-visited when the maturity of the project warrants it, and likely not settle to its final form until the project is ~80%+ along.

-	Part 1 - Common

-	Part 1 of TS 33.129 contains the general framework of LI SCAS. This part contains descriptions of threats and attacks.

-	Part 2 - NF-Embedded LI functions

-	Part 2 of TS 33.129 contains SCAS tests for LI functions that are embedded in network functions - i.e., their lifecycle is inexorably tied to a network function. Inevitably, a tension exists because access control to the network function and access control to the embedded LI function have to be separated. Most tests in this part exercise the implementation of this separation.

-	Part 3 - Non-NF-Embedded LI functions

-	Part 3 of TS 33.129 contains SCAS tests for LI functions that are stand-alone - i.e., not associated with any network functions. This separation tends to reduce security considerations when compared to embedded functions. The figure below reflects examples of stand-alone LI functions.

-	Part 4 - Mediation and Delivery Function (MDF)

-	Part 4 of TS 33.129 contains SCAS tests that apply to the Mediation and Delivery Function (MDF).

-	Part 5 - Administration Function (ADMF)

-	Part 5 of TS 33.129 contains SCAS tests that apply to the LI Administration Function (ADMF).

Figure 4.1.5-1: TS 33.129 parts coverage

The LEMF is outside the scope of the present document (multipart specification), but attributes of the LEMF (such as delivery paths in the MDF) are within the scope of the present document (multipart specification).

## 4.2	Security functional requirements and related test cases specific to LI functions

### 4.2.1	Introduction

The present clause describes the security functional requirements and the corresponding test cases for LI functions. The proposed security requirements are classified in two groups:

-	Security functional requirements derived from TS 33.501 [2] and detailed in clause 4.2.2.

-	General security functional requirements which include requirements not already addressed in TS 33.501 [2] but whose support is also necessary to ensure that the LI function conforms to a common security baseline detailed in clause 4.2.3.

### 4.2.2	Security functional requirements on LI functions deriving from 3GPP specifications and related test cases

#### 4.2.2.1	General

The general approach in TS 33.117 [2] clause 4.2.2.2 applies to LI functions. The relevant requirements and test cases for the product class of the NF in which the LI function is embedded also apply. Unless otherwise specified, the requirements and test cases in TS 33.117 [2] clause 4.2.2.3 related to SBA / SBI do not apply to the LI function but apply to the NF in which the LI function is embedded, where relevant.

#### 4.2.2.2	< template test for security functional test >

Requirement Name: e.g. Confidentiality protection of user data transported over N3 interface.

Requirement Reference: e.g. TS 33.501 [2], Clause x.y, TS 33.126 [5] clause x.y

Requirement Description: e.g. The transported user data between gNB and UPF is confidentiality protected as specified in TS 33.501 [2], clause 9.3.

Threat Reference: e.g. TR 33.926 [7], Clause L.2.2, "No protection or weak protection for user plane data ".

TEST CASE:

Test Name: TC_LI_XYZ

Purpose:

e.g. Verify that the transported user data between gNB and UPF are confidentiality protected over N3 interface.

Procedure and execution steps:

Pre-Condition:

-	e.g. UPF network product is connected in simulated/real network environment.

-	e.g. The tunnel mode IPsec ESP and IKE certificate authentication is implemented.

-	e.g. Tester shall have knowledge of the security parameters of tunnel for decrypting the ESP packets.

-	e.g. Tester shall have access to the N3 interface between gNB and UPF.

-	e.g. Tester shall have knowledge of the confidentiality algorithm and confidentiality protection keys used for encrypting the encapsulated payload.

Execution Steps:

e.g. The requirement mentioned in this clause is tested in accordance with the procedure mentioned in clause 4.2.3.2.4 of TS 33.117 [2].

Expected Results:

e.g. The user data transported between gNB and UPF is confidentiality protected.

Expected format of evidence:

e.g. Evidence suitable for the interface, e.g., evidence can be presented in the form of screenshot/screen-capture.

#### 4.2.2.3	Security functional requirements on LI functions deriving from 3GPP specifications and related test cases – general interface aspects

##### 4.2.2.3.1	Introduction

The purpose of the sub-clauses of clause 4.2.2.3 is to identify and describe the general baseline requirements from the interfaces in the LI architecture and the corresponding test cases. The general baseline requirements are applicable to all LI functions within the LI system independent of a specific network product class.

##### 4.2.2.3.2	Protection at the transport layer

Editor’s Note: Rewrite test from TS 33.129-2 4.2.2.3, 4.2.2.4, and 4.2.2.5 to apply to non-X1.

##### 4.2.2.3.3	Authorization of NF service access

Editor’s Note: Move TS 33.129-2 4.2.2-4.2.9 here?

#### 4.2.2.4	Credential affinity

#### 4.2.2.5	Credential expiry

#### 4.2.2.6	Credential chain of trust

#### 4.2.2.7	Credential binding

#### 4.2.2.8	Element of LI binding

### 4.2.3	Technical baseline

#### 4.2.3.1	Introduction

The present clause provides baseline technical requirements.

#### 4.2.3.2	Protecting data and information

##### 4.2.3.2.1	Protecting data and information – general

###### 4.2.3.2.1.1	Introduction

The tests listed in clause 4.2.3.2.1 of TS 33.117 [2] shall be applied with the additions and clarifications included in the following sub-clauses.

###### 4.2.3.2.1.2	A crypto inventory exists

Requirement Name: Standard Cryptographic Primitives

Requirement Reference: TBD

Requirement Description: Only standardized cryptographic algorithms and primitives published by accredited organizations shall be used.

References:

Asset reference: 		AS-LI-FUNCTION

Attacker reference: 	TBD

Threat reference: 		TBD

Test case:

Test Name / ID: TC_LI_COMMON_STANDARDIZED_CRYPTO_ALGORITHMS

Purpose:

Editor's Note: re-write to reduce scope to mere inventory to allow discovery, making sure we include hardware accelerators. Also, ensure sufficient crypto agility to remove newly broken algorithms, including to cover quantum future-proofing.

All cryptographic algorithms, primitives, protocols, and parameters used in the PUT shall conform to standards published by accredited bodies (e.g., ETSI, SOGIS, NIST, ISO/IEC, IETF, BSI). Any deviation shall be itemized and justified.

Pre-Conditions:

1.	Approved cryptographic policy/whitelist (algorithms, modes, curves, key sizes, digests, KDFs/MACs from e.g., ETSI, SOGIS, NIST, ISO/IEC, IETF, BSI) is available.

2.	Inventory of crypto dependencies (libs, HSMs, protocol stacks) is available.

3.	Access to code/configs/binaries and running services exists.

Execution Steps:

1.	Extract crypto usages from code/configs/binaries (static analysis, grep, library settings).

2.	Enumerate supported/negotiated ciphers on network services (e.g., testssl.sh, nmap --script ssl-enum-ciphers).

3.	Compare against the whitelist and accredited standards.

4.	Negative: attempt to enable a disallowed algorithm/mode (e.g., RC4, MD5, SHA-1 signatures, 3DES, RSA-1024) and confirm the pipeline or config hardening blocks it.

Expected Results:

1.	Only approved algorithms are present/negotiable; deprecated/non-standard options are disabled.

2.	Attempts to enable disallowed crypto are rejected by Continuous Integration/policy or runtime controls.

Expected Evidence:

1.	List of accreditation bodies and standards used

2.	Whitelist policy + mapping table.

3.	Scanner outputs and config snippets showing allowed sets.

4.	Logs showing rejection of disallowed algorithms.

###### 4.2.3.2.1.3	Replaceable crypto modules

Editor’s Note: candidate for tweaking. Perhaps too strict? Perhaps TS 102 232 and TS 33.128 should be enhanced to require this modularity.

Requirement Name: Modular Cryptographic Implementations

Requirement Reference: TBD

Requirement Description: Cryptographic methods must be implemented in replaceable modules. Static implementations are prohibited, as they hinder corrections, replacements, and upgrades in the event of security incidents, evolving threats (e.g., quantum computing), or changing performance requirements. Implementations must allow seamless substitution of algorithms and provide sufficient hardware resources to support stronger cryptographic methods.

References:

Asset reference: 		AS-LI-FUNCTION

Attacker reference: 	TBD

Threat reference: 		TBD

Test Case:

Test Name / ID: TC_LI_COMMON_REPLACEABLE_CRYPTO_MODULES

Purpose:

Verify that cryptographic methods are implemented in modular, replaceable components such that algorithms, key lengths, or libraries can be substituted without requiring major redesign or system downtime.

Pre-Conditions:

1.	System architecture documentation describing cryptographic module boundaries is available.

2.	Inventory of cryptographic libraries and their integration points is available.

3.	Test environment with ability to swap crypto libraries/configurations is set up.

Execution Steps:

1.	Inspect software design and code for modular integration of cryptographic functions (e.g., plugin, API, or dynamic linking approach).

2.	Attempt replacement of an existing crypto library with an alternative approved one (e.g., replace OpenSSL with GnuTLS, or update a library version).

3.	Validate system operation with substituted algorithm or key length.

4.	Negative: attempt to hard-code or statically link crypto algorithm implementations and verify that this is detected and flagged.

Expected Results:

1.	Cryptographic functions are implemented as replaceable modules (dynamic, configurable, or API-based).

2.	System continues to operate correctly after replacing an algorithm or library.

3.	Statically embedded or hard-coded cryptographic implementations are absent or flagged as non-compliant.

4.	Hardware resources (e.g., CPU, HSM, accelerator capacity) are sufficient to handle stronger crypto algorithms when required.

Expected Evidence:

1.	Architecture documentation showing modular crypto design.

2.	Test logs or CI outputs demonstrating successful swap of crypto libraries/algorithms.

3.	Configuration or build scripts showing dynamic linking or pluggable crypto modules.

4.	Negative test evidence showing static or hard-coded crypto is rejected.

###### 4.2.3.2.1.4	Replaceable crypto methods

Editor’s Note: candidate for tweaking. Perhaps too strict? Perhaps TS 102 232 and TS 33.128 should be enhanced to require this modularity.

Requirement Name: Configurable Cryptographic Methods

Requirement Reference: TBD

Requirement Description: Applications must support configuration of cryptographic methods and provide functions to exchange encryption algorithms. Deactivation and modification capabilities (e.g., cipher suite adjustments) must be built during development. Functions to exchange encryption algorithms (re-encryption) apply only to persistent data storage, not to transport encryption. This enables substitution of broken schemes due to new attacks or future computing architectures.

References:

Asset reference: 		AS-LI-FUNCTION

Attacker reference: 	TBD

Threat reference: 		TBD

Test Case:

Test Name / ID: TC_LI_COMMON_CONFIGURABLE_CRYPTO_METHODS

Purpose:

Verify that cryptographic methods are configurable and that applications can exchange encryption algorithms for persistent data storage. Ensure that cipher suite modification and algorithm substitution functions are implemented and usable.

Pre-Conditions:

1.	Documentation of configurable crypto parameters and supported cipher suites is available.

2.	Test data sets in persistent storage encrypted with an initial algorithm (e.g., AES-128-CBC) are prepared.

3.	Administrative access to application configuration and key management is available.

Execution Steps:

1.	Inspect code/configuration interfaces for crypto parameterization (cipher suites, key lengths, modes).

2.	Modify crypto configuration to deactivate or change cipher suites (e.g., remove AES-128-CBC, enable AES-256-GCM).

3.	Perform re-encryption of stored data from one algorithm to another.

4.	Verify that transport encryption (e.g., TLS sessions) does not expose runtime re-encryption functions.

5.	Negative: attempt re-encryption of transport sessions and confirm it is unsupported.

Expected Results:

1.	Application supports enabling/disabling and modification of cryptographic methods at configuration level.

2.	Re-encryption of persistent data from one algorithm to another is successful.

3.	Transport encryption is not subject to runtime re-encryption functions.

4.	Attempts to re-encrypt transport sessions are rejected.

Expected Evidence:

1.	Configuration documentation and sample settings for cryptographic method changes.

2.	Test logs showing cipher suite modification and successful re-encryption of persistent data.

3.	Screenshots/configuration outputs verifying AES-128-CBC replaced with AES-256-GCM (or equivalent).

4.	Negative test results confirming re-encryption of transport encryption is blocked.

##### 4.2.3.2.2	Protecting data and information – unauthorized viewing

###### 4.2.3.2.2.1	Introduction

The tests listed in clause 4.2.3.2.2 of TS 33.117 [2] shall be applied with the additions and clarifications included in the following sub-clauses.

Editor’s note: It is unclear if these belong here as these seem to be more applicable to NE-embedded.

###### 4.2.3.2.2.2	General-purpose logs must not contain LI data

Requirement Name: Confidentiality of LI Identifiers

Requirement Reference: TS 33.126: R6.6 - 30	Undetectability by Non-Authorized Parties

Requirement Description: General-purpose logs must not contain Lawful Interception (LI) identifiers or other sensitive LI information. Only LI-protected audit logs, only accessible to LI-cleared personnel, may contain such details.

References:

Asset reference: 		AS-LI-FUNCTION

Attacker reference: 	AT-INTERNAL

Threat reference: 		T-LOG

Test Case:

Test Name / ID: TC_LI_LOG_IDENTIFIER_LEAK_PROVISIONING

Purpose:

Validate that LI identifiers and LI-specific events are not written during provisioning activities to general-purpose system or application logs accessible by non-LI-authorized administrators.

Pre-Conditions:

1.	LI functionality is activated and operational in the system under test.

2.	A general-purpose logging subsystem (e.g., syslog, journald, or vendor-specific log service) is enabled.

3.	A test operator has LI clearance and can initiate LI operations.

4.	A test operator has administrator-level access to general-purpose logs but no LI clearance (to simulate risk exposure).

Execution Steps:

1.	Initiate an LI session (e.g., activate an intercept, provision target identifiers).

2.	Trigger LI-related events likely to produce logs (e.g., provisioning errors, interface restarts).

3.	Collect all general-purpose logs from the system during and after LI operations.

4.	Inspect logs for presence of LI identifiers (e.g., IMSI, MSISDN, warrant ID, intercept ID).

5.	Repeat under different conditions (normal activation, error cases, reconfiguration).

Expected Results:

1.	General-purpose logs contain no LI identifiers or warrant information.

2.	LI events in general-purpose logs, if present, are limited to non-sensitive metadata (e.g., “LI subsystem error” without identifiers).

3.	Detailed LI logs with identifiers are only recorded in LI-protected audit logs, accessible to LI-cleared personnel.

Expected Evidence:

1.	Copy of collected general-purpose logs during LI activity.

2.	Annotated inspection showing absence of LI identifiers.

3.	Copy of LI-protected logs confirming that identifiers are only stored in the restricted audit facility.

4.	Plain-language conclusion affirming compliance.

###### 4.2.3.2.2.3	General-purpose logs must not contain LI data (comms?)

Requirement Name: Confidentiality of LI Identifiers

Requirement Reference: TS 33.126: R6.6 - 30	Undetectability by Non-Authorized Parties

Requirement Description: General-purpose logs must not contain sensitive LI information. Only LI-protected audit logs, only accessible to LI-cleared personnel, may contain such details. When identifiers of intercepted targets appear in general-purpose logs (e.g., as part of routine signaling or service activity), no additional information shall indicate that the subscriber is subject to Lawful Interception.

References:

Asset reference: 		AS-LI-FUNCTION

Attacker reference: 	AT-INTERNAL

Threat reference: 		T-LOG

Test Case:

Test Name / ID: TC_LI_LOG_IDENTIFIER_LEAK_RUN_TIME

Purpose:

Validate that general-purpose logs, while possibly containing subscriber identifiers as part of normal service records, do not contain information or markers that reveal LI surveillance activity.

Pre-Conditions:

1.	LI provisioning is active for a target identifier in the system under test.

2.	General-purpose logging (e.g., network event logs, call detail records, session initiation logs) is enabled and accessible to administrators without LI clearance.

3.	The target subscriber initiates communication (e.g., call setup, session initiation, messaging).

Execution Steps:

1.	Provision a target identifier for interception in the LI system.

2.	Have the target subscriber initiate multiple communications (e.g., calls, sessions, messages).

3.	Collect general-purpose logs generated during these communications.

4.	Inspect logs for signs of surveillance context, such as:

a)	Explicit LI-related flags, tags, or annotations.

b)	Error messages referencing LI functions.

c)	Duplicated or anomalous log entries indicating redirection or duplication of traffic.

5.	Compare log entries of intercepted subscriber activity with log entries of non-intercepted subscribers for consistency.

Expected Results:

1.	General-purpose logs contain only routine identifiers and service-related information (e.g., IMSI, MSISDN, session IDs).

2.	No indication is present that the subscriber is under interception (no LI flags, tags, error codes, or, particularly, duplicate entries).

3.	Log patterns for intercepted subscribers are indistinguishable from non-intercepted subscribers.

4.	Surveillance-related records are only present in LI-protected audit logs.

Expected Evidence:

1.	Copies of general-purpose logs showing intercepted and non-intercepted subscriber activities.

2.	Annotated analysis demonstrating that intercepted subscriber logs do not reveal surveillance status.

3.	Copy of LI-protected logs showing that surveillance records are confined to the restricted LI audit domain.

4.	Plain-language conclusion confirming that LI activity remains undetectable in general-purpose logs.

###### 4.2.3.2.2.4	LI logging is segregated from non-LI logging

Requirement Name: Segregation of LI Logging

Requirement Reference: TS 33.126: R6.6 - 30	Undetectability by Non-Authorized Parties

Requirement Description: LI audit and operational logs must be segregated from all non-LI logging subsystems. Access to LI logs must be restricted to LI-cleared personnel only.

References:

Asset reference: 		AS-LI-FUNCTION

Attacker reference: 	AT-INTERNAL

Threat reference: 		TBD

Test Case:

Test Name / ID: TC_LI_LOG_SEGREGATION

Purpose:

Validate that the LI logging subsystem is logically and physically segregated from non-LI (general purpose) logs, ensuring that LI log data cannot be accessed through non-LI interfaces or roles.

Pre-Conditions:

1.	LI subsystem is deployed and active.

2.	General-purpose system logging (e.g., syslog, journald, platform management logs) is operational.

3.	LI-specific logging facility is configured and accessible only to LI-cleared roles.

4.	Test operators possess two distinct roles: one LI-cleared and one non-cleared administrative account.

Execution Steps:

1.	Generate LI-related events (e.g., provisioning, activation, LI delivery errors).

2.	Generate non-LI system events (e.g., user login, system health checks, configuration updates).

3.	Collect logs from both the LI logging subsystem and the general logging subsystem.

4.	Attempt to access LI logs using the non-cleared administrator role.

5.	Verify log storage segregation (e.g., separate file paths, databases, log streams).

6.	Inspect general logs for any LI log entries, references, or cross-linkages.

Expected Results:

1.	LI log data is stored in a distinct, access-controlled subsystem.

2.	LI logs are not visible through general log collectors, APIs, or management tools.

3.	Non-LI logs do not contain pointers, cross-references, or metadata revealing LI activity.

4.	Access attempts to LI logs by non-cleared roles are denied and auditable.

Expected Evidence:

1.	Copies of LI and non-LI log directories or service configuration showing segregation.

2.	Access-control matrix demonstrating that only LI-cleared roles can access LI logs.

3.	Screen captures or outputs of denied access attempts from non-cleared accounts.

4.	Plain-language conclusion confirming effective segregation of LI and non-LI logging domains.

###### 4.2.3.2.2.5	LI target leak, log leak

Editor’s Note: This test seems equivalent to 4.2.3.2.2.2 and 4.2.3.2.2.3 above. Not sure whether to use this for something else or delete it). (Network logs perhaps?)

##### 4.2.3.2.3	Protecting data and information in storage

###### 4.2.3.2.3.1	Introduction

The following addition and clarifications to clause 4.2.3.2.3 of TS 33.117 [2] apply for LI functions.

###### 4.2.3.2.3.2	No unencrypted LI data

##### 4.2.3.2.4	Protecting data and information in transfer

###### 4.2.3.2.4.1	Introduction

The following addition and clarifications to clause 4.2.3.2.4 of TS 33.117 [2] apply for LI functions.

###### 4.2.3.2.4.2	No unencrypted internal comms

###### 4.2.3.2.4.3	Network observation by VI admin

###### 4.2.3.2.4.4	Firewalls between virtual entities

##### 4.2.3.2.5	Logging access to personal data

There are no LI-specific additions to clause 4.2.3.2.5 of TS 33.117 [2].

#### 4.2.3.3	Protecting availability and integrity

##### 4.2.3.3.1	Introduction

The tests listed in TS 33.117 [2] clause 4.2.3.3 shall be applied with the additions and clarifications included in the following sub-clauses.

##### 4.2.3.3.2	Vulnerability patching – original code

TBD – not sure is this is within scope of SCAS

##### 4.2.3.3.3	Vulnerability patching – open source

TBD – not sure is this is within scope of SCAS

#### 4.2.3.4	Authentication and authorization

##### 4.2.3.4.1	Introduction

The tests listed in TS 33.117 [2] clause 4.2.3.4 shall be applied with the additions and clarifications included in the following sub-clauses.

##### 4.2.3.4.2	Logging of failed authentication attempts

##### 4.2.3.4.3	No non-LI accounts

##### 4.2.3.4.4	Unique credentials

##### 4.2.3.4.5	System hardening of VM accounts

#### 4.2.3.5	Protecting sessions

There are no LI-specific additions to clause 4.2.3.5 of TS 33.117 [2].

#### 4.2.3.6	Logging

##### 4.2.3.6.1	Introduction

The tests listed in TS 33.117 [2] clause 4.2.3.6 shall be applied with the additions and clarifications included in the following sub-clauses.

##### 4.2.3.6.2	Mandatory log-disabling alarms

##### 4.2.3.6.3	Loud logs

### 4.2.4	Operating Systems

There are no LI-specific additions to clause 4.2.4 of TS 33.117 [2].

### 4.2.5	Web Servers

There are no LI-specific additions to clause 4.2.5 of TS 33.117 [2].

### 4.2.6	Network Devices

There are no LI-specific additions to clause 4.2.6 of TS 33.117 [2].

## 4.3	Adaptations of hardening requirements and related test cases specific to LI functions

### 4.3.1	Introduction

Clause 4.3 specifies the LI function-specific adaptations of hardening requirements and related test cases.

### 4.3.2	Technical baseline

#### 4.3.2.1	Introduction

The following additions to clause 4.3.2 in TS 33.117 [2] apply for LI functions.

#### 4.3.2.2	Third-party components are up-to-date

### 4.3.3	Operating systems

There are no LI-specific additions to clause 4.3.3 in TS 33.117 [2].

### 4.3.4	Web servers

#### 4.3.4.1	Introduction

The following additions to clause 4.3.4 in TS 33.117 [2] apply for LI functions.

#### 4.3.4.2	Only POST allowed (for Xn)

#### 4.3.4.3	Third-party components are up-to-date

### 4.3.5	Network devices

There are no LI-specific additions to clause 4.3.5 in TS 33.117 [2].

### 4.3.6	Network functions in service-based architecture

There are no LI-specific additions to clause 4.3.6 in TS 33.117 [2].

## 4.4	Adaptations of basic vulnerability testing requirements and related test cases specific to LI functions

### 4.4.1	Introduction

There are no LI specific additions to clause 4.4.1 of TS 33.117 [2].

### 4.4.2	Port Scanning

There are no LI specific additions to clause 4.4.2 of TS 33.117 [2].

### 4.4.3	Vulnerability scanning

There are no LI specific additions to clause 4.4.3 of TS 33.117 [2].

### 4.4.4	Robustness and fuzz testing

The test cases under clause 4.4.4 of TS 33.117 [2] are applicable to LI functions.

# 5	Security Elements

## 5.1	Assets

### 5.1.1	Assets Overview

Assets are categorized more granularly than functional blocks. These are the specific elements that, if exposed or influenced, could compromise LI.

Table 5.1.1-1: Asset definitions

|  | Name | Description |
| --- | --- | --- |
| AS-TARGET | Target Identities(s) | A network or service identity that uniquely identifies a target for interception from all other non-targets within one or more CSP services. |
| AS-LI-CREDS | LI Access Credentials | Tokens, passwords, certificates used to authenticate LI operations. |
| AS-NF | Network Function | Over and above SA3 general assumptions, e.g., log states, memory artifacts, ephemeral labels. |
| AS-NETWORK-SERVICE | Network Service | Normal functioning of a network service. |
| AS-LI-FUNCTION | LI Function | LI function internal state and artifacts - e.g., job queues, service status, inter-process calls. |
| AS-LI-PRODUCT | LI Product Delivery Link | Continuity (interruptions), quality (jitter, timing anomalies) of the delivery of LI product, between POIs and MDF, or MDF and LEMF. |
| AS-LEMF | LEMF Attributes | LEMF attributes, such as IP address, domain names, paths, etc. |
| AS-USER-PATTERN | Network User Behavior Attributes | Application or flow-level characteristics that may affect network user attributes (e.g., privacy). |
| AS-CRYPTO-STORE | Crypto Store | System-trusted certificate lists, keystores, or chains. |
| AS-API-CONF | API or Configuration State | Network API configuration artifacts. |
| AS-LI-PRODUCT | LI Product | LI content or metadata. |

### 5.1.2	Detailed Asset Descriptions

Editor's Note: Review all of these and compare them to TR 33.926 to remove overlap and deconflict. Align the format, naming, etc. with the Annexes in 33.926?

#### 5.1.2.1	Target Identifiers

Asset Code: AS-TARGET

Asset Name: Target Identities

Asset Description:

A network or service identity that uniquely identifies a target for interception from all other non-targets within one or more CSP services. One target may have one or several target identities. The Target Identity can be a long-term subscription based identity, a short term network identity, a public identity or an internal (private) identity.

#### 5.1.2.2	LI Access Credentials

Asset Code: AS-LI-CREDS

Asset Name: LI Access Credentials

Asset Description:

These are the authentication elements used to authorize and secure LI operations. They may include passwords, API tokens, client certificates, SSH keys, or internal authorization grants.

#### 5.1.2.3		Network Function

Editor's Note: Reframe this in terms of TS 33.926 GNP and provide reference for the assumptions mentioned below.

Asset Code: AS-NF

Asset Name: Network Function

Asset Description:

A network function (NF) encompasses any virtualized or physical function that plays a role in packet handling, control signaling, or service delivery. Beyond SA3’s baseline assumptions, this asset includes ephemeral internal states, such as memory artifacts, temporary routing rules, and runtime labels, that may be visible to co-located workloads or side-channel analysis.

#### 5.1.2.4		Network Service

Asset Code: AS-NETWORK-SERVICE

Asset Name: Network Service

Asset Description:

This can be thought of simplistically as a chain of network functions. However, this category includes the end-to-end behavior of services delivered by the network, including latency, throughput, jitter, and session continuity.

#### 5.1.2.5		LI Function

Asset Code: AS-LI-FUNCTION

Asset Name: LI Function

Asset Description:

This asset represents the internal mechanisms and runtime state of the LI function itself. It includes service status flags, job queues, inter-process communication, service mesh labels, and other operational artifacts.

#### 5.1.2.6	LI Product Delivery

Asset Code: AS-LI-PRODUCT

Asset Name: LI Product Delivery

Asset Description:

This refers to the delivery chain that carries LI content and Intercept Related Information (IRI) from network function Points of Interception (POIs) to the mediation function (MDF), and from there to the Law Enforcement Monitoring Facility (LEMF).

#### 5.1.2.7	LEMF Attributes

Asset Code: AS-LEMF

Asset Name: LEMF Attributes

Asset Description:

This includes the IP address, domain name, or network path of the LEMF endpoint. If the LEMF is statically assigned or its routing patterns are predictable, adversaries may correlate traffic to or from the LEMF to active surveillance.

#### 5.1.2.8	Network User Behaviour

Asset Code: AS-USER-PATTERN

Asset Name: Network User Behaviour Attributes

Asset Description:

These are behavioural signatures or application-layer patterns associated with end users. They may include traffic types, flow durations, usage spikes, or session frequency.

#### 5.1.2.9	Crypto Store

Asset Code: AS-CRYPTO-STORE

Asset Name: Crypto Store

Asset Description:

This includes keystores, certificate chains, and lists of trusted Certificate Authorities (CAs) used within LI systems or the broader network.

#### 5.1.2.10	API or Configuration State

Asset Code: AS-API-CONF

Asset Name: API or Configuration State

Asset Description:

This encompasses the current state and metadata of network APIs, configuration templates, or orchestrator settings used in service delivery.

#### 5.1.2.11	LI Product

Asset Code: AS-LI-PRODUCT

Asset Name: LI Product

Asset Description:

The LI product comprises content (e.g. voice, SMS, data) and metadata (e.g., call records, IP flow info) delivered as IRI to the LEMF. This is among the highest-value assets in the system and the direct object of the lawful interception operation.

## 5.2	Attackers

### 5.2.1	Attackers Overview

Attackers can be human or automated. The following table lists the attackers considered in the present multi-part document.

Table 5.2-1: Attacker definitions

| Code | Name | Description |
| --- | --- | --- |
| AT-INTERNAL | Internal administrator | Has privileged access to network functions or management interfaces but no LI authorization. May be, for example:  A log auditor  A config manager  A general platform admin  A network analytics observer  A data center operator |
| AT-INTERNAL-LOG | Internal Log Auditor | Has privileged access to system and application logs but no LI authorization. |
| AT-INTERNAL-CONFIG | Internal Configuration Manager | This attacker has legitimate access to system and application logs for operational, compliance, or troubleshooting purposes but no LI authorization. |
| AT-INTERNAL-INFRA | Internal Infrastructure/Platform Administrator | An infrastructure administrator manages underlying compute, storage, and networking resources but no LI authorization. |
| AT-INTERNAL-OBSERVE | Internal Analytics or Monitoring Observer | This attacker uses monitoring dashboards, telemetry pipelines, or data visualization tools to analyze network performance and user behavior. |
| AT-INTERNAL-DATACENTER | Internal Facilities or Data Center Operator | This role includes physical or environmental access, such as rack technicians, cabling engineers, or maintenance personnel. |
| AT-EXTERNAL | External agent | Has no legitimate access to any internal network interface (although may have access to an external interface such as an end user). May exploit e.g., exposed APIs, timing, network resources. |
| AT-AUTO-AI | Automated or AI Agent | Operates at higher scales, capable of fast deep analysis and pattern detection or timing inference. |
| AT-PASSIVE | Passive Observer | Non-real-time observer with access to historical data, can compare pre and post provisioning states, correlate metrics across tenants/users. |
| AT-COMPROMISED-NF | Compromised NF | An attacker has taken control of an NF and uses it as a pivot point to attack other NFs in the same "perimeter". |

Editor's Note: Be mindful to not limit attacker to "casual" level - include well-resourced attackers in every test analysis

### 5.2.2	Detailed Attacker Descriptions

#### 5.2.2.1		Internal Administrator

Attacker Code: AT-INTERNAL

Attacker Name: Internal Administrator

Attacker Description:

An internal administrator refers to a human actor with privileged access to core network systems, but without explicit authorization to access or manage Lawful Interception (LI) functions. This attacker may hold legitimate operational roles, such as log auditor, configuration manager, infrastructure administrator, or analytics engineer, and typically has routine access to sensitive systems, including logging platforms, telemetry pipelines, or network orchestration tools. Due to proximity and privilege level, this attacker is capable of indirect inference or misuse of general-purpose tools to extract sensitive LI-related insights. These threats are particularly insidious because they originate from trusted roles within the perimeter.

#### 5.2.2.2	Internal Administrator Granular Breakdown

##### 5.2.2.2.1	Internal Log Auditor

Attacker Code: AT-INTERNAL-LOG

Attacker Name: Internal Log Auditor

Attacker Description:

This attacker has legitimate access to system and application logs for operational, compliance, or troubleshooting purposes. While not authorized for LI access, they may infer surveillance activity by observing timing correlations, unusual service requests, or log entries tied to specific identifiers. Because log data often spans multiple services and timestamps, the log auditor can piece together patterns across different layers, particularly when logs are insufficiently redacted or when metadata about LI functions is leaked due to poor segregation. The risk is elevated in environments where centralized observability tools are broadly accessible. Importantly, such individuals may not always be direct employees of the CSP; they may include contractors, outsourced service providers, or regulatory agency auditors. The presence of third parties with log visibility adds further complexity to managing access boundaries and ensuring strict compartmentalization of LI-related activity.

##### 5.2.2.2.2	Internal Configuration Manager

Attacker Code: AT-INTERNAL-CONFIG

Attacker Name: Internal Configuration Manager

Attacker Description:

This internal actor has control over or visibility into network and service configuration interfaces, such as provisioning databases, orchestration tools, or infrastructure-as-code systems. While they may not intentionally target LI systems, their access could expose sensitive artifacts like target identifiers, service activation records, or routing behaviors that indirectly reveal LI activity. A misconfiguration, audit bypass, or direct misuse of privilege could lead to the exposure or tampering of LI-related parameters. This attacker is especially dangerous when configuration changes are not subject to strict role-based access control and change logging.

##### 5.2.2.2.3	Internal Infrastructure/Platform Administrator

Attacker Code: AT-INTERNAL-INFRA

Attacker Name: Internal Infrastructure/Platform Administrator

Attacker Description:

An infrastructure administrator manages underlying compute, storage, and networking resources. They typically have root-level access to virtual machines, containers, or Kubernetes clusters where LI functions may reside. While this actor may not understand the semantics of LI operations, their access allows for memory inspection, storage snapshotting, or inter-process observation that could compromise confidentiality. Their role sits beneath LI in the stack, creating a high-privilege threat vector. This risk is heightened in cloud-native environments with shared platforms or insufficient tenant isolation.

##### 5.2.2.2.4	Internal Analytics or Monitoring Observer

Attacker Code: AT-INTERNAL-OBSERVE

Attacker Name: Internal Analytics or Monitoring Observer

Attacker Description:

This attacker uses monitoring dashboards, telemetry pipelines, or data visualization tools to analyze network performance and user behavior. Although not involved in LI, they may observe outliers, such as sudden QoS changes, endpoint redirection, or anomalous traffic patterns, that align with surveillance activities. Their access is typically read-only, but because they work across many users and services, they can form inferences at scale. If monitoring tools lack data masking or correlation protections, this actor could indirectly deanonymize LI operations.

##### 5.2.2.2.5	Internal Facilities or Data Center Operator

Attacker Code: AT-INTERNAL-DATACENTER

Attacker Name: Internal Facilities or Data Center Operator

Attacker Description:

This role includes physical or environmental access, such as rack technicians, cabling engineers, or maintenance personnel. Though easily overlooked, data center operators may intercept or observe physical indications of LI provisioning (e.g., device identifiers, port activation, or localized power usage). If surveillance workloads are not obfuscated within broader platform operations, these actors could observe installation patterns or hardware identifiers tied to LI. This threat is particularly relevant in private or hybrid data centers where physical separation is minimal and operator duties cross tenant boundaries.

#### 5.2.2.3		External Agent

Attacker Code: AT-EXTERNAL

Attacker Name: External Agent

Attacker Description:

An external agent is a threat actor entirely outside the administrative domain of the Communication Service Provider (CSP). This entity lacks any legitimate access to internal network functions or LI systems, but may attempt to exploit publicly exposed services, such as user-facing APIs, mobile access interfaces, or Internet-exposed network resources. Attacks may include techniques such as timing analysis, metadata harvesting, endpoint compromise, or denial-of-service strategies. While the external agent has the least direct access, the increasing attack surface from 5G cloud-native and multi-access edge architectures can make this attacker class surprisingly effective, especially when combined with reconnaissance automation.

#### 5.2.2.4		Automated or AI Agent

Attacker Code: AT-AUTO-AI

Attacker Name: Automated or AI Agent

Attacker Description:

An automated or AI-driven attacker is a non-human agent capable of processing vast quantities of data in real time or near-real time. These agents leverage machine learning, pattern recognition, or inference engines to discover anomalies, detect provisioning patterns, or identify LI-related events across large data streams. Whether deployed internally or externally, this attacker type can operate continuously, scaling beyond human capacity, and is particularly effective in identifying subtle timing correlations, job queue patterns, or service quality variations. Such agents may be adversarially trained and can pose a significant risk in hybrid environments where observability is distributed.

#### 5.2.2.5		Passive Observer

Attacker Code: AT-PASSIVE

Attacker Name: Passive Observer

Attacker Description:

A passive observer is an attacker who does not interfere with live operations but gains access to historical data, telemetry, or pre/post-state snapshots. This actor may compare logs, traces, or service states before and after suspected LI provisioning to infer the presence of surveillance activities. Passive observers may exist within the CSP’s environment or in adjacent systems such as logging infrastructure or analytics data lakes. While they do not actively engage in attacks, their ability to correlate changes over time, particularly across multi-tenant environments, makes them a notable threat, especially when data retention and separation controls are weak.

#### 5.2.2.6	Compromised Network Function

Attacker Code: AT-COMPROMISED-NF

Attacker Name: Compromised Network Function (NF)

Attacker Description:

This attacker class represents a network function that has been overtaken, either fully or partially, by an adversary. The compromised NF operates under the control of the attacker and can serve as a pivot point to reach LI systems, intercept provisioning messages, or disrupt internal communications. It may impersonate trusted roles, inject misleading telemetry, or undermine confidentiality by leaking sensitive artifacts. Compromised NFs are especially dangerous in service-mesh architectures where trust boundaries are often implicit. Lateral movement across internal APIs or through shared memory spaces between co-located functions increases the threat radius substantially.

## 5.3	Threats

### 5.3.1	Threats Overview

The following table contains named threat types. Each SCAS test can point to one or more.

Table 5.3.1-1: Threat definitions

| Code | Name | Description |
| --- | --- | --- |
| T-LOG | Log leak | LI activity generates logging artifacts observable by non-LI authorized parties. |
| T-CONFIG | Config leak | LI activity is inferred by analysing config files extracted before and after LI provisioning. |
| T-RES-CPU | LI leak via CPU side channel 1 | LI activity is inferred by analysing CPU utilization levels, method 1. |
| T-RES-CPUT-RES-CPU | LI leak via CPU side channel 2 | LI activity is inferred by analysing CPU utilization levels, method 2. |
| T-RES-NETT-RES-NET | LI leak via network flow side channel | LI activity is inferred by analysing network bandwidth utilization levels. |
| T-TIMINGT-TIMING | LI detection via timing anomalies. | Detection of LI via timing anomalies. |
| T-TIMING | MDF-LEMF LI Product Flow Detection | Detection of MDF to LEMF LI product flow via timing anomalies. |
| T-INTERRUPTION | POI-MDF Link Failure | Link-level interruption between POI and MDF used to infer presence or behaviour of LI functions. |
| T-SESSION-COUNT | Session count | LI detection by session count analysis. |
| T-INTERFACE-SEC | Interface security | Allowance of insecure (non-TLS) connections. |
| T-FEATURE | Unnecessary features | Unnecessary features increase the attack surface. |
| T-SERVICE | Unnecessary services | Unnecessary services increased the attack surface. |
|  | "Simple MISconfiguration" |  |

### 5.3.2	Detailed Threat Descriptions

#### 5.3.2.1	Log Leak

Threat Code: T-LOG

Threat Name: Log Leak

Threat Category: Information Disclosure

Threat Description: Certain system or network logs may unintentionally record details of Lawful Interception (LI) operations, such as interface initialization, provisioning events, or errors specific to LI services. If such logs are accessible to non-LI-authorized users or monitoring systems, this could result in the detection of LI activity, violating confidentiality and undetectability requirements.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.2	Config Leak

Threat Code: T-CONFIG

Threat Name: Config Leak

Threat Category: Information Disclosure

Threat Description: Configuration files or system state snapshots taken before and after LI provisioning may reveal changes indicative of interception functionality, such as new interfaces, active LI modules, or altered routing behaviors. If such configurations are accessible to unauthorized administrators or third-party tools, they may be used to infer the existence or target of an LI operation.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.3	LI Leak via CPU Side Channel 1

Threat Code: T-RES-CPU

Threat Name: LI Leak via CPU Side Channel (Pattern A)

Threat Category: Information Disclosure

Threat Description: LI operations may cause observable variations in CPU utilization due to additional processing, packet duplication, or encryption tasks. If an attacker monitors CPU resource usage patterns over time and correlates spikes or periodicity with specific events, it may be possible to infer the presence of active LI functions or associated targets.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.4	LI Leak via CPU Side Channel 2

Threat Code: T-RES-CPU

Threat Name: LI Leak via CPU Side Channel (Pattern B)

Threat Category: Information Disclosure

Threat Description: A refined side-channel analysis leveraging advanced statistical or machine learning methods may identify LI-related process signatures at a lower resolution or across virtualized environments. Unlike direct CPU utilization spikes, this method relies on deeper behavioural fingerprinting (e.g., cache usage, execution timing patterns), which may allow even more covert inference of LI presence.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.5	LI Leak via Network Flow Side Channel

Threat Code: T-RES-NET

Threat Name: LI Leak via Network Flow Side Channel

Threat Category: Information Disclosure

Threat Description: LI systems typically duplicate or redirect traffic to an MDF or mediation device. This may introduce subtle, but detectable changes in bandwidth utilization, jitter, or packet duplication observable from within the network or via flow records (e.g., NetFlow, sFlow). An attacker monitoring network flow metadata may detect patterns associated with interception activity.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.6	LI Detection via Timing Anomalies

Threat Code: T-TIMING

Threat Name: LI Detection via Timing Anomalies

Threat Category: Information Disclosure

Threat Description: The addition of LI processing steps (e.g., duplication, filtering, routing to MDF) can result in increased latency or jitter during communication sessions. An attacker may detect the presence of LI by statistically analyzing round-trip times or response behaviors that differ from baseline behaviour when LI is not active.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.7	MDF–LEMF LI Product Flow Detection

Threat Code: T-TIMING

Threat Name: MDF–LEMF LI Product Flow Detection via Timing

Threat Category: Information Disclosure

Threat Description: The flow of intercepted traffic (IRI or CC) from the MDF to the LEMF may generate detectable timing patterns, especially if LI sessions are rare or traffic volume fluctuates. Observers with visibility into network patterns or inter-domain timestamps could detect and potentially correlate this flow with LI activities, violating the principle of transparency.

Threatened Asset: AS-TARGET Target Identifiers; AS-LI-PRODUCT LI Product

#### 5.3.2.8	POI–MDF Link Failure

Threat Code: T-INTERRUPTION

Threat Name: POI–MDF Link Failure

Threat Category: Denial of Service

Threat Description: A network or logical interruption between the Point of Interception (POI) and the MDF may unintentionally reveal the existence of LI. If LI sessions degrade differently from normal user sessions under such link failure conditions, an attacker could induce or observe faults to probe for LI presence or behavior.

Threatened Asset: AS-TARGET Target Identifiers; AS-LI-PRODUCT LI Product

#### 5.3.2.9	Session Count

Threat Code: T-SESSION-COUNT-09

Threat Name: LI Detection via Session Count

Threat Category: Information Disclosure

Threat Description: Provisioning of LI may introduce additional sessions (e.g., replicated flows, internal signalling) that appear in session monitoring tools or access logs. If the session count increases in a predictable manner during LI activation, an unauthorized observer could infer LI activity simply by counting active or historical sessions.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.10	Interface Security

Threat Code: T-INTERFACE-SEC

Threat Name: Interface Security Misconfiguration

Threat Category: Tampering / Information Disclosure

Threat Description: If LI-related interfaces (e.g., between POI, MDF and LEMF) are not properly secured using cryptographic protocols such as TLS, an attacker may eavesdrop on or manipulate LI product traffic. This threatens both confidentiality and integrity of intercepted data and may result in exposure of sensitive law enforcement operations.

Threatened Asset: AS-TARGET Target Identifiers

#### 5.3.2.11	Unauthorized Use of Unnecessary Features

Threat Code: T-FEATURE

Threat Name: Unauthorized Use of Unnecessary Features

Threat Category: Information Disclosure / Misuse of Resources

Threat Description: Default-enabled but unnecessary software or hardware features may remain active after installation or commissioning. Attackers may exploit these dormant features (e.g., debugging functions, unused interfaces, auxiliary services) to gain unauthorized access, escalate privileges, or collect sensitive information. Such features increase the system’s attack surface and may be leveraged for reconnaissance or direct compromise.

Threatened Asset: AS-LI-FUNCTION, AS-API-CONF

#### 5.3.2.12	Unauthorized Use of Unnecessary Services

Threat Code: T-SERVICE

Threat Name: Unauthorized Use of Unnecessary Services

Threat Category: Information Disclosure / Misuse of Resources

Threat Description: After installation, supplier-preset, local, or network-accessible services may remain active that are not required for the LI system’s intended operation. Attackers can exploit these services to gain unauthorized access, consume resources, or leverage unpatched configurations. Unnecessary services expand the system’s attack surface and may provide footholds for compromise, particularly if they lack hardened security configurations.

Threatened Asset: AS-LI-FUNCTION, AS-API-CONF

# 6	Common Tests

## 6.1	Supply Chain

### 6.1.1		Vendor

#### 6.1.1.1		Standardized Cryptographic Algorithms and Primitives

Moved to 4.2.3.2.1.2

#### 6.1.1.2	Replaceable Cryptographic Modules

Moved to 4.2.3.2.1.3

#### 6.1.1.3	Configurable and Exchangeable Cryptographic Methods

Moved to 4.2.3.2.1.4

#### 6.1.1.4	Log Leak of LI Identifiers During LI Provisioning

Moved to 4.2.3.2.2.2

#### 6.1.1.5	Log Leak of LI Identifiers During Communication Time

Moved to 4.2.3.2.2.3

#### 6.1.1.6	Log Segregation

Moved to 4.2.3.2.2.4

Annex <A> (normative):
<Normative annex for a Technical Specification>

Start each annex on a new page.

Annexes are labelled A, B, C, etc. and designated either "normative" or "informative" depending on their content.

Normative annexes only to appear in Technical Specifications. Use style "Heading 8".

Annex <B> (informative):
Vulnerability/Compromise Spectrum

# B.1	Vulnerability/Compromise Spectrum

Editor's Note: Review this table for completeness as the rest of the document is finalized.

Editor's Note: The "Notes" and "Description" fields may not be necessary if this points to the tests above.

Table B.1-1: Vulnerability/Compromise Spectrum

| Code | Name | Description | Notes | Test IDs Failed | Criticality (None,. Low,. Medium,. High,. Critical). |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | LI capability exists | The presence of an installed LI capability was detected. | e.g., on disk, but not running |  |  |  |
| 2 | LI capability characterization | Capabilities of the installed LI function were extracted. | e.g., capacity numbers, in terms of targets provisionable or bandwidth available for LI |  |  |  |
| 3 | LI running state detection | Detected that LI is currently running on the Target of Evaluation (TOE). |  |  |  |  |
| 4 | LI running instance characterization | Capabilities of the running instance of LI were extracted, such as number of targets provisionable, etc. | e.g., number of current taps |  |  |  |
| 5 | LI running state manipulation | LI was either started or stopped against normal procedures. |  |  |  |  |
| 6 | LI topology | Extracted network/physical location of other functions that may (do?) contain LI capability. | e.g., information about other POIs, MDFs, ADMFs, or even LEMFs |  |  |  |
| 7 | Target ID extracted | A (any) target ID was extracted. | without a priori choice of target |  |  |  |
| 8 | Specific target ID detected | A specific target ID was detected on the TOE. | with a priori choice of target |  |  |  |
| 9 | IRI extracted | IRI was extracted pertaining to any current target ID on the TOE. | without a priori choice of target |  |  |  |
| 10 | Content extracted | Content was extracted pertaining to any current target ID on the TOE. | without a priori choice of target |  |  |  |
| 11 | Specific target IRI extracted | IRI was extracted pertaining to a specific target ID on the TOE. | with a priori choice of target |  |  |  |
| 12 | Specific target content extracted | Content was extracted pertaining to a specific target ID on the TOE. | with a priori choice of target |  |  |  |
| 13 | One new target was inserted | A new target inserted outside normal procedures. |  |  |  |  |
| 14 | Multiple targets inserted | More than one target inserted outside normal procedures. | pathway to LI DOS |  |  |  |
| 15 | Full target list extracted | A complete list of target IDs on the TOE was extracted. |  |  |  |  |
| 16 | IRI of all targets extracted | IRI pertaining to all targets was extracted. |  |  |  |  |
| 17 | Content of all targets extracted | Content pertaining to all targets was extracted. |  |  |  |  |
| 18 | Control of non-LI functionality achieved through LI entry | Control of TOE functionality outside LI was achieved. |  |  |  |  |

Editor's notes (Apr 7, 2025 call):

* Add DELETION (targets, IRI, content),

* modify (INSERT/DELETE) delivery endpoints

* insert before 7: access/token compromise/privilege escalation

# EDITORS NOTES:

## Parking Lot

0. Find a way to publish an implementation/configuration guide to state "the obvious", e.g. 33.128 Annex, or its own document, etc.

1. Think about a test for trust domain crossing. If a (non-LI) network function is required to take some action on behalf of LI and does not have a POI, this creates a security concern.

2. Test that POI code and data are not accessible from outside the enclave.

3. Inter-LEA confidentiality. Is there a way to test it explicitly, or do we rely on inference from a subset of tests.

Annex <C>:
<Informative annex title for a Technical Report>

Informative annexes in Technical Reports do not use "(informative") in the title, since all annexes in TRs are informative. Use style "Heading 9" in TRs.

Annex <D> (informative):
Bibliography

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Bibliography is optional. If it exists, it shall follow the last technical annex in the document.

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

Bibliography format

<Publication>: "<Title>".

Annex <E> (informative):
Index

Use style "Heading 8" in TSs and "Heading 9" in TRs. Do not use "informative" in the title in TRs.

The Index is optional. If it exists, it shall immediately precede the Changes history annex.

Generate the index using MS Word's index field feature.

Annex <F> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-07 | SA3#98-LI | s3i250357 |  |  |  | First introduction of the baseline to the group. | 0.0.8 |
| 2026-01 | SA3#100-LI | S3i260060 |  |  |  | New agreed draft | 0.0.10 |