Source:	Bytedance

Title:	Recordings for XR HMD target device

Document for:	Discussion & Agreement

Agenda Item:	1.7 – DaCAS

# 1. Introduction

For obtaining characterization and development recordings for the DaCAS target devices, necessary recording scenarios are needed. In this contribution we provide the recordings two recording scenarios documented in S4-250654 [1], using the target device definition documented in S4-251009 [2].

# 2. Recordings

Following recording of scenarios 1 and 2 described in [1] are proposed to be included to the common database of DaCAS standardization, which can be used for developing and testing example solution for the target device documented in S4-251009 [2].

## 2.1 Recording scenario 1

Following recording scenario 1 definition in S4-250654 [1], we recorded signal with following properties.

| Sound source: | High quality loudspeaker |
| --- | --- |
| Source signal: | 1 kHz sine |
| Signal characteristics: | Frequency: 1 kHz Length: 10 s, with 1s of silence before and after the sine tone, results in total 12s Level: -0.1 dBFS (as long as calibrated SPL level is met) Sample Rate: 48 kHz Bit depth: 16-bit |
| Sound source calibration: | Source signal calibrated to 94 dB SPL at the measured microphone inlet position |
| Acoustic environment: | BEM simulation in free-field condition, with noise floor equals 10dBA (complying with NR15). |
| Detailed Positioning: | Distance: >0.5 m Source direction: Main diaphragm towards the measured microphone inlet Source angles (azi, ele): (0°,0°) relative to the measured microphone inlet |
| Reference recording: | Record 30 seconds of silence with the device under test to obtain the noise floor of the integrated microphones |
| Description/additional info: | SNR= 71.99 dB Microphone sensitivity S=-38 dBFS @ 94 dBA SPL |

Table 1. Recording for scenario 1

We stored the recorded signal to scenario_n1/scenario_1.wav, and the reference signal to scenario_n1/scenario_1_ref.wav.

## 2.2 Recording scenario 2

Following recording scenario 2 definition in S4-250654 [1], we recorded signal with following properties.

| Sound source: | High quality loudspeaker |
| --- | --- |
| Source signal: | Exponential sweep |
| Signal characteristics: | Range: 20 Hz – 20 kHz Length: 5 s Level: -3 dBFS Sample Rate: 48 kHz Bit depth: 16-bit |
| Sound source calibration: | Loudspeaker is calibrated with -3 dBFS pink noise to produce 80 dBA at the UE position. Same playback settings are then used with the defined test signal |
| Acoustic environment: | BEM simulation in free-field condition, with noise floor equals 10dBA (complying with NR15). |
| Detailed Positioning: | Distance: dUE=1.5m Source direction: Main diaphragm towards the UE Source angles (azi, ele): Required sound source directions: - Every azimuth f with maximum step of 10°: f = [0°, 10°, …,350°] - All azimuth directions at the elevation = 0° shall be always recorded - For every 3rd azimuth step (f = [0°, 30°, …, 330°]), elevations q with maximum step of 30°: q = [-60°, -30°, 0°, 30°, 60°] - Poles: = [-90°, 90°] - In practice, actual elevation direction may differ slightly (e.g., due to recording facility limitations) - Applied source directions shall be clearly documented Space between the walls and sound source/UE should be at least 0.5 m |
| Reference recording: | Free-field measurement microphone towards the loudspeaker at the UE position |
| Description/additional info: | Silence before and after the measured direction shall be at least 1 s |

Table 2. Recording for scenario 2

We stored the recorded signal to scenario_n2/ scenario_2_azi=xxx_elev=yyy.wav (xxx and yyy are defined source angles in scenario 2), and the reference signal to scenario_n2/scenario_2_ref.wav.

# 3. BEM and simulation of mic non-ideality

As previously agreed in S4-250654 [1], FEM/BEM is allowed to produce free-field recordings as long as the non-ideality of microphone is considered. We used the same method of non-ideality simulation used in [3], with parameters aligned to the mic spec of our target device.

# 4. Proposal

Based on the presented discussion, we propose to include recordings of scenarios 1 and recording 2 to the common database of DaCAS standardization, for the target device definition documented in S4-251009 [2].

# References

[1]	S4-250654: “On DaCAS recording scenarios”, Nokia.

[2]	S4-251009: “Propose a XR HMD target device”, Bytedance.

[3]	S4-250821: “[DaCAS] Recordings of smartphone-like target device”