3GPP TSG-TSGSA4 Meeting Post #111e 	 S4aI201102

# Home, ,  30.November – 31.December 2020

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SA4 baseline architecture of SA2’s 5MBS architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson LM, Enensys, BBC, Telus, Dolby Laboratiries, Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMSA-Multicast |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.757 is still under development and terminology is still not stable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | An SA4 version of the SA2 architecture is proposed, so that SA4 can work against an assumed stable reference model. The terminology will be aligned, once normative 5MBS terminology is clear. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.4.1	Baseline Network Reference Architectures

### 4.4.1.1	General

The following two network reference architecture diagrams illustrate SA4 assumptions about the SA2 5MBS Network Reference Architecture drawing in TR 23.757 [X] (Figure A.3.2-1). TS 23.757 is not yet complete and there are multiple Editor’s Notes on function names and reference point names. The present report will align with the TR 23.757 names and terms once TR 23.757 is in stable state. SA4 may still provide input to SA2 on some architectural findings.

This clause presents a variant of the network reference architecture in TR 23.757 Clause A.3 with the following changes:

-	The function MBSF-C is named MBSF and the function MBSF-U is named MBSU.

-	Reference point “xMB” only refers to an interface which is provided by the BM-SC. For the 5MBS media delivery functions, the MBSU exposes an MB-M2 interface, which is xMB-U based.

-	The MBSU is controlled by an MB-M3 interface (instead of Nmbsu). The functional scope of MB-M3 is identical to Nmbsu, however, the the reference point name is aligned with 5GMSA interface namings.

-	The MBSF is integrated in a 5GMS AF function that may expose an internal API resembling xMB-C. Support for standalone MBSF is for study.

-	A standalone MBSF may be needed for different interworking scenarios. Interworking with legacy systems is for further study.

Legend for Figure 4.4.1.2-1 and Figure 4.4.1.3-1:

-	Blue boxes: control plane functions as shown in TR 23.757 Figure A.3.2-1.

-	Yellowy/orange boxes: user plane functions as shown in TR 23.757 Figure A.3.2-1.

-	White boxes: 5GMS functions.

-	Blue lines: control plane interfaces.

-	Red lines: user plane interfaces.

-	Black labelled interfaces: existing reference points from Release 16.

-	Coloured labelled interfaces: newly coined reference points for Release 17.

### 4.4.1.2	5GMSA functions in the Trusted DN

Thie following diagram illustrates a network reference architecture with all 5GMS and 5MBS functions within the Trusted DN. A 5GMS Application Provider (typically) in an External DN configures the 5GMS features via a Release 17 version of M1d interface. Three different models are considerd:

1:	The usage of 5MBS for media distribution is completely hidden from the 5GMS Application Provider. The 5GMS System selects usage of 5MBS based on internal criteria.

2:	By means of 5GMS provisioning procedures at (extended) M1d, the 5GMS Application Provider explicitly controls the potential usage of 5MBS in certain areas and for certain content. For example, some content might not be authorized for 5MBS distribution by content rights owners. Or, some content might only be authorized for 5MBS distribution.

Figure 4.4.1.2-1: 5MBS architecture combined with 5GMS hosted in Trusted DN

### 4.4.1.3	5GMSA functions in an External DN

Thie following diagram illustrates a network reference architecture with all 5GMS within an external DN. Only the MBSU resides insode a trusted DN. An 5GMS Application Provider (typically) in an external DN configures the 5GMS features via a Release 17 version of M1d interface.

Figure 4.4.1.3-1: 5MBS architecture combined with 5GMS hosted in External DN