3GPP TSG-S4 Meeting #112e	S4aI201115

# Online, Still safe at home,  21st Janruary 2021 - <End_Date>

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.802 | CR | <CR#> | rev | <Rev#> | Current version: | 0.1.8 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Review of xMB-C wrt User Plane properties |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | S4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Ericsson LM |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GMSA-Multicast |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The BM-SC is split into a control and a user-plane function and a new API between the two new functions should be defined. Accordingly, the control and the user plane aspects of the xMB API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A simplified functional model of the BM-SC Download Delivery function is drawn and the user plane related xMB-C propoerties is accrodingly identified. Similar models can be drawn for other delivery methods. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New Clause 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 4.5	Review of existing technology

### 4.5.1	General

### 4.5.2	Review of xMB

#### 4.5.2.1	Model of a BM-SC User-Plane Function

The model below assumes that a FLUTE function according to MBMS Download Delivery (Clause 7 in TS 26.346) is mapped into an MBSU. Similar models can be created for RTP streaming and transparent delivery, however, likely not needed.

The idea of this simplified model is helping to identify the xMB-C parameters (xMB Service and Session Parameters), needed to configure an MBSU.

Figure 4.5.2.1-1: Simplified User Plane model for FLUTE (as a MBSU function)

The model depicts some key functions, from an xMB-U ingest to an MB-UPF ingest (N6). In case of  MBMS Download (e.g. used for DASH / HLS over MBMS or generic file delivery), the xMB-U ingest is an HTTP ingest, either using HTTP Pull (the MBSU pulls files / segments from an HTTP server) or HTTP Push based (the MBSU receives files, using HTTP PUT). xMB-C is used to: In case of Pull, to provide individual file URLs to get pulled and transmitted or in case of Push, provide an base URL for ingesting data to the API invoker.

The MBSU may use some file caches to store partial or complete files. Optimized implementations may pipe the a file through with only minor buffering / caching.

The HTTP meta data like Content Location (file URL), Content Length (file size), MIME type is provided by the HTTP File Receiver to the FDT Instance creation function. This acts as input (with other xMB-C parameters) to form the FDT Instance xml file.

The file partitioning function is putting a file into one or more packet payloads. In case of Raptor FEC (RFC 5053) or No Code FEC (RFCxyz), there are recommended file partitioning schemes and parameters to partition a file (or blob of data) into a sequence of packet paylods (called encoding symbols). An FDT Instance is also partitioned into one or more packet payload.

The FLUTE header creation functions inserts FLUTE header specific parameters like the TSI, sequence number (FEC Symbol ID), etc. As result, a complete UDP packet payload is created, which can be written into a UDP socket at time of transmission. The Streamer &  Pacer function sends the packets according to the defined bitrate to the configured MP-UPF ingest point, which can be an MB2-U tunnel, some direct multicast, or similar.

#### 4.5.2.2	Review of xMB

This section contains a copy of the xMB service (Clause 5.3.7) and Session (Clause 5.4.6) properties. The column “related to User Plane” indicates, whether the property is related to the user-plane handling, e.g. defining the xMB-U ingest, etc. In this case, the MBSU need to be provisioned with the property value. Likely, the property is exposed via MB-M3 (Nmbsu).

Table 4.5.2.2-1: List of xMB Service Properties

| Property Name | Related to User Plane (i.e. forwarded to MBSU) | Note |
| --- | --- | --- |
| Id | No |  |
| ServiceID | No |  |
| Service Class | No |  |
| Service Languages | No |  |
| Service Names | No |  |
| Receive Only Mode | For Study | This flag is for ROM services |
| Service Announcement Mode | No |  |
| Consumption Reporting Configuration | For Study |  |
| Push Notification URL | Yes |  |
| Push Notification Configuration | Yes |  |

Table 4.5.2.2-2: List of xMB Session Properties

| Property Name | Related to User Plane (i.e. forwarded to MBSU) | Note |
| --- | --- | --- |
| id |  |  |
| Session start | Yes | The MBSU need to know, when to start generating user plane packets |
| Session stop | Yes | The MBSU need to know, when to stop generating user plane packets |
| Max Bitrate | Yes |  |
| Max Delay | Yes |  |
| Session State | Partially | A session state is needed, but without the state “Session Announced” |
| Service Announcement start time | No |  |
| Geographical Area | ffs |  |
| QoE Reporting | No |  |
| QoE Report URL | No |  |
| Session Type | yes |  |
| Header Compression | Ffs | Unclear, whether RoHC header compression is in RAN |
| FEC | yes |  |
| Transport Mode |  |  |
| Session Description Parameters for User Plane | yes |  |
| Delivery Mode Configuration for user plane | yes |  |
| Delivery Session Description Parameters | yes |  |
| Streaming |  |  |
| SDP URL | yes |  |
| TimeShifting |  |  |
| Application (incl. DASH) |  |  |
| Application Service Description |  |  |
| Ingest Mode | yes |  |
| Application Entry Point URL |  |  |
| Push URL | yes |  |
| Unicast Delivery |  |  |
| Components |  |  |
| Files |  |  |
| Ingest Mode | yes |  |
| File List | yes | Except Unicast availability Target Reception Completion time is ffs, since unicast File Repair is included. |
| Carousel Mode |  |  |
| Carousel Scheduled Interval | yes |  |
| File delivery manifest URL | yes |  |
| Push URL | yes |  |
| Display Base URL | yes |  |
| SA file URL | no | An SA-file like concept is needed, but the MBSU is not handling it. |
| Mission Critical |  |  |
| MC-Extension |  |  |
| TMGI | no | The MBSU only need the MB-N6 tunnel information to ingest the data into the MB-UPF. The MBSF handles the TMGI. |
| QoS-Information | no | The MBSU is not responsible for control plane interactions with the MB-SMF |