3GPP SA4-MBS SWG post 133-e AH	S4aI250179

# Online, 23rd October 2025

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0006 | rev | 0 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI6 Client-driven management of media delivery service energy optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.11 adding a a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[87]	Accubattery: https://play.google.com/store/apps/details?id=com.digibites.accubattery

[88]	CTA Specification, Common Media Server Data, CTA-5006

2nd Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

3rd change
(All new text)

## 7.11	Solution #10: Client-driven selection of stream variants or delivery paths based on energy characteristics

### 7.11.1	Key Issue mapping

This Candidate Solution addresses Key Issue #6.

### 7.11.2	Functional description

#### 7.11.2.1	Introduction

While Quality of Service (QoS) information sharing mechanisms already exist, such as CMSD (Common Media Server Data) defined by CTA Wave [88], these solutions do not take into account the energy and environmental impact of content delivery.

Currently, video players lack the ability to select stream variants or delivery paths based on their energy impact in addition of QoS. This gap prevents optimization of energy efficiency during media playback while maintaining acceptable quality of experience.

The solution introduces a mechanism for sharing energy-related characteristics across the technical delivery chain to improve energy efficiency during media playback, and enabling client devices to make informed decisions about stream selection and delivery paths based on their environmental impact while maintaining acceptable quality of experience.

#### 7.11.2.2	Reference architecture for client-driven selection of stream variants or delivery paths based on energy characteristics in the generalised Media Delivery System

Figure 7.11.2.2-1 depicts a reference architecture that realises this candidate solution in the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59].

Figure 7.11.2.2-1: Reference architecture for client-driven selection of stream variants or delivery paths based on energy characteristics in the generalised Media Delivery System

The core principle of this architecture is to leverage the existing method for collecting and exposing energy-related characteristics, as already outlined in Solution #5 of Clause 7.6. Additionally, it extends this approach by enabling the Media Client to select stream variants or delivery paths based on this information.

The solution is reusing functions already defined for collection and exposure of energy-related information:

-	The Energy Information AF

-	The Energy Information Collector

In the generalised architecture, the Energy Information Collector is integrated in the Media Client, more specifically in the Media Session Handler which communicates with the Media Access Function also part of the Media Client:

-	Media Client: A UE internal function dedicated to Media Delivery comprising:

-	Media Session Handler: An entity on the UE that communicates with the Media AF in order to establish, control and support the delivery of a media session.

-	Media Access Function: An entity on the UE that communicates with the Media AS in order to access and deliver media content. The media access function for example may be further sub-divided into content delivery protocols, codecs, media types and metadata representation.

The solution is also reusing reference points M1, M3, E3, M4, M5, E5, M6, M8, M11 and E12 already defined for collection and exposure of energy-related information in the instantiation in generalised Media Delivery architecture of solution #5 in clause 7.6.

### 7.11.3	Procedures

Figure 7.11.3-1 below details the different steps for client-driven selection of stream variants or delivery paths based on energy characteristics outlined in clause 7.11.2.2.

Figure 7.11.3-1: Procedures for client-driven selection of stream variants or delivery paths based on energy characteristics in the generalised Media Delivery System

Steps 1 to 12 are taken from the solution#5 in clause 7.6.3.2. The only difference is the configuration of several AS as in this solution, media can be delivered via several AS.

In this solution, Media Playback needs to be initiated. This is done with steps 13 and 14:

13.	The Media Session Handler provides the Media Player Entries to the Media-Aware Application.

14. The Media Player is invoked with the selected Media Player Entry to start media access and playback.

Steps 15 to 22 are identical with clause 7.6.3.2. After those steps, the Media Session Handler is able to communicate stream variants or delivery paths based on energy characteristics to the Media Stream Handler which then request Media to the AS(s):

23.	The Media Session Handler selects stream variants or delivery paths to be used by the Media Stream Handler.

24. The Media Session Handler communicates this information to the Media Stream Handler.

25. Based on the Energy Information received in step 21, the Media Session Handler may instantiate a Dynamic Policy with a different energy usage profile.

NOTE:	Details of Dynamic Policy changes in response to energy-related information are for further study.

26.	The Media Player establishes the transport session for acquiring the Media Player Entry from AS1.

27.	The Media Player requests the Media Player Entry to AS1.

28.	The Media Player receives the Media Player Entry from AS1.

29.	The Media Player establishes the transport session for acquiring the Media Player Entry from AS2.

30.	The Media Player requests the Media Player Entry to AS2.

31.	The Media Player receives the Media Player Entry from AS2.

32.	The Media Session Handler exposes energy-related information to the subscribed Media-Aware Application via reference point M6. Based on the UE Energy Information collection configuration obtained in step 6, the energy-related information may include energy consumption information of different granularities, e.g. PDU Session and/or QoS flow, as described in clause 5.51.2.3 of TS 23.501 [72] and/or for individual media delivery sessions.

33.	The Media-Aware Application may expose the received energy-related information to the Application Service Provider via reference point M8 using methods beyond the scope of 3GPP standardisation.

### 7.11.4	Summary

The proposed solution enables the integration of energy-related characteristics into multimedia content delivery systems. It offers Media Clients the ability to select stream variants or delivery paths based on their environmental impact, while maintaining acceptable quality of experience.

This approach is part of a sustainable development approach to multimedia streaming, enabling end-to-end energy optimization, from content production to consumption.

This solution is based on Network Energy Information available from the Energy Information Function as well as the two new entities defined in the solution #5 in clause 7.6, with their related reference points, the Energy Information AF and the Energy Information Collector. The Media client uses energy-related characteristics delivered thanks to those components and existing mechanisms already available in 5G system, like in 5GMS, which enable to optimization of energy efficiency during media playback while maintaining acceptable quality of experience.

This solution offers several advantages:

1.	Reduced Environmental Impact: Minimizes energy consumption and CO₂ emissions associated with multimedia content delivery.

2.	User Transparency: Provides visibility into the environmental impact of content consumption.

3.	Flexibility: Compatible with existing 5G system optimizations and existing adaptive streaming protocols (DASH, HLS).

4.	Customization: Allows users to choose their level of environmental commitment. This could be done with several energy optimization modes offered to users in the Media-Aware application:

-	Eco Mode: Prioritizes energy efficiency, even at the slight expense of quality of experience.

-	Standard Mode: Balance between energy efficiency and quality of experience.

-	Performance Mode: Prioritizes quality of experience without energy constraints.

5.	Global Optimization: Takes into account the entire delivery chain, from production to consumption.

End of changes