3GPP TSG-SA4 Meeting #-e (AH) MBS SWG post 134	S4aI250204

# Online, , 18th Dec 2025 - 18th Dec 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0018 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED]: Solution for KI1 and KI4 for collecting and exposing Energy-Related information to authorized 3rd parties instantiating Media Application Service Energy Metrics configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital France R&D, SAS |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-12-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Addition of solutions to Key Issue 1 and Key Issue 4 on Energy-related Collection and Reporting to the 5G media |  |  |  |  |  |  |  |  |
| Summary of change: |  | - From Dallas meeting, proposal to split the initial contribution SA-252103 as listed hereafter: - In the SA-252103 contribution, keep the solution #10 named “the Media Application Service Energy Metrics Reporting Configuration” - In current contribution, propose the solution #12 named “The Media Application Service Energy metrics collection and reporting using new reference points” - In clauses 7.13.5/6/7/8, addition of report baseline parameters |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No collection and reporting mechanisms for Energy-related information |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

(All new text)

## 7.13	Solution #12: Media Application Service Energy metrics collection and reporting using new reference points

### 7.13.1	Key Issue mapping

This Candidate Solution addresses Key Issue #1 described in clause 6.2 and Key Issue #4.

.

### 7.13.2	Functional description

#### 7.13.2.1	Introduction

This Solution#12 provides the procedures for handling Media Application Service Energy  Metrics Reporting Configuration using reference points E3 and E5 of solution#5 architecture

Figure 7.13.2.1-1 illustrates how the approach could be instantiated.

Figure 7.13.2.1-1: Metrics reporting feature with the overall generic reference architecture for collection and exposure of energy-related information (clause 7.6.2.2)

The following reference points are involved in the proposed  to reporting Media Application Service Energy metrics:

M1	Network API used by the Media Application Provider to provision the Media Application Service Energy Metrics Reporting Configuration to the Media AF. This determines whether and which energy-related information submitted by the Energy information Collector for the UE, the Media AS and the EIF for the Core Network. The Energy Information AF instantiated in the Media AF collects the Energy-related information and creates Energy related information reports as it is in the Release 19 architecture.

E12	This reference point is used by the EIF to provide Energy-related information reports to the Energy Information AF instantiated in the Media AF. The Energy Information AF receives the information and creates the report aligned with the Media Application Service Energy metrics scheme of the configuration.

E3	After provisioning of Metrics Reporting in the Media AF and the configuration of Content Hosting in the Media AS, the Media Application Service Energy Metrics Reporting configuration is obtained by the Media AS from the Energy information AF instantiated in the Media AF. Based on this configuration, the Media AS collects Media Application Service Energy metrics, creates metrics reports aligned with the Media Application Service Energy metrics scheme of the configuration and uses this reference point to transmit the report to the Energy Information AF instantiated in the Media AF.

E5	Network API used by the UE Energy Information Collector instantiated in the Media Session Handler to obtain a Media Application Service Energy Metrics Reporting configuration from the Energy Information AF. The Energy Information Collector fetches the collected Media Application Service Energy metrics, creates the metrics report aligned with the Media Application Service Energy metrics scheme of the configuration. This API also allows the UE Energy Information Collector to receive the aggregated reports aligned with the Media Application Service Energy metrics scheme of the configuration from the Energy Information AF and propagate to the Media-aware application (via reference point E6) and optionally, to the Media Application Provider (via reference point M8).

E6	Client API used by the Media-aware Application to subscribe to energy-related information notifications from the Energy Information Collector.

M8	Network API used by the Media Application Provider to receive energy-related information from the Media-aware Application. This reference point is beyond the scope of 3GPP standardisation.

### 7.13.3	Procedures

Figure 7.13.3-1 below details the different steps for Media Application Service Energy Metrics Reporting for the Approach  in the system outlined in clause 7.13.2.1 (based on figure 5.5.3-1 of TS 26.501 [26501] but updated to generalise).

Figure 7.13.3-1: Procedures for Energy-of Service collection and reporting

The steps of the call flow are as follows:

1:	The Media AF is provisioned with the Media Application Service Energy Metrics Reporting configuration  by the Media Application Provider, each pertaining to metrics collection by the Media Player and the EIF and the Media AS functions. The configuration is transmitted to the Energy Information AF to collect and generate the metrics reports regarding required post-processing functionality and deliver to the Media Application Provider. The Energy Information AF subscribe to the EIF network function to get the Energy-related information from the Core Network.

NOTE:	This step is based on Candidate Solution #10

2:	The Media-Aware Application triggers the Service Announcement and Content Discovery procedure. The Service Announcement includes the whole Service Access Information that contains metric configuration information for Media Application Service Energy metrics scheme

3:	Time passes until the UE initiates session establishment and media playback.

4:	The Media-Aware Application informs the Media Player of impending media playback.

5:	The Media Session Handler requests the establishment of a streaming session with the Media AF which confirms the streaming session establishment.

6:	The media processing pipeline is set up between the Media Access Function, the Media AS and the Application Provider.

7:	Service Access Information is retrieved by the Media AS via reference point M3, including the endpoint address of the Energy Information AF to be used for interacting with it at reference point E3.

8:	The Media AS retrieves its Media Application Service Energy Metrics Reporting configuration from the Energy Information AF via reference point E3.

9:	Service Access Information is retrieved by the Media Session Handler via reference point M5, including the endpoint address of the Energy Information AF to be used for interacting with it at reference point E5.

10: The Energy Information Collector retrieves its Media Application Service Energy Metrics Reporting configuration from the Energy Information AF via reference point E5.

11: Using the Metrics reporting procedures specified in clause 4.9.2 of TS 26.512 [26512] at reference point M11, the Energy Information Collector queries the Media Access Function on its capability to perform metrics collection in accordance with Media Application Service Energy metrics scheme as defined by its Metrics Reporting configuration. At this step it is checked if the Media Access Function can support the reporting of Media Application Service Energy metrics.

12:	The Media Access Function acknowledges its support for the collection of the required metrics of the configured scheme.

13:	The Media Access Function notifies the Media Session Handler of the start of media delivery, causing the Media Session Handler to initialize and begin countdown of separate metrics reporting interval timers for the Media Application Service Energy metrics scheme.

During the course of media playback, steps 14–29 below may be repeated, depending on the duration of the playback and the frequency of metrics reporting as set by the metrics configuration for the Media Application Service Energy metrics scheme.

14:	Causing the Media AS to initialize and begin countdown of a metrics reporting interval timer for the Media Application Service Energy metrics scheme.

15:	Causing the Energy Information Collector to initialize and begin countdown of a metrics reporting interval timer for Media Application Service Energy metrics scheme.

16:	The Media Access Function accesses media content at the Media AS.

17:	Upon expiration of the timer (associated with the Media Application Service Energy metrics scheme), the Media AS retrieves the logged metrics according to that scheme.

18:	The Media AS reports the logged metrics to the Energy Information AF.

19:	The Energy Information AF processes the information according to the Media Application Service Energy metrics scheme.

20: The timer associated with the Media Application Service Energy metrics scheme expires

21:	The UE Energy Information Collector retrieves the energy metrics from the Media Player according to Media Application Service Energy metrics scheme.

22:	The Energy Information Collector processes the information according to the Media Application Service Energy metrics scheme

23:	The EIF publishes an NF Energy-Information Report to the Energy Information AF.

24:	The Energy Information AF publishes a Media Application Service Energy metrics reports to the Media Application Provider.

The following steps may occur to expose Media Application Service Energy metrics to the Media-aware Application:

25:	The Energy Information AF publishes a  Media Application Service Energy metrics reports to the UE Energy Information Collector at reference point E5 reference point to allow the Media Session Handler to report to the Media Aware Application.

26:	The Energy Information Collector aggregates the Media Application Service Energy Metric reports of the Media Client and the ones received from the Energy Information AF and publishes the final Media Application Service Energy metrics reports to the Application at reference point E6.

26a:	The…

27:	The Media Aware Application may publish Media Application Service Energy Metrics reports to the Media Application Provider via M8 reference point

Optionally, if exposure of Media Application Service Energy events is provisioned in the Media AF:

28:	The Energy Information AF shares Media Application Service Energy metric reports with the Data Collection AF instantiated in the same Media AF.

29:	The Data Collection AF exposes the Media Application Service Energy Metrics events to the Event Consumer AF via reference point R6 as specified in TS 26.531 [26531].

Upon the termination of media playback (as notified by the Media-Aware Application to the Media Access Function) a final round of metrics collection and reporting is performed.

### 7.13.4	Impacts on existing services, entities and interfaces

#### 7.13.4.1	Media AF

The Media AF would be required to support a Media Application Service Energy Metrics Reporting Configuration as part of the Metrics Reporting Configuration.

#### 7.13.4.2	Energy Information AF

The Energy Information AF is instantiated as a subfunction of the Media AF to collect energy-related information per application, per media session, and per media component, at multiple granularities and across  CN and AS domains at reference points E3 and E12.

Collected information can be reported by the existing mechanisms of the Release 19 architecture.

Collected information can be reported to the UE Energy Information Collector instantiated within the Media Session Handler at reference point E5 to be propagated to the Media-aware Application at reference point E6 and, optionally, to the Media Application Provider at reference point M8.

Optionally, collected information can be reported to a Data Collection AF instantiated within the Media AF via the reference point E4 prior to exposure to authorized third parties at reference point R6, privacy-preserving post-processing being applied.

#### 7.13.4.3	Reference point M1

At reference point M1, the Media Application Provider shall instantiate a Metrics Reporting Configuration resource associated with the target Provisioning Session to enable --Media Application Service Energy- collection and reporting.

(NEW text)

#### 7.13.5	Application Server Energy Report

This is the metrics report generated periodically by the Application Server describing its energy consumption over the last sampling period.

Table 7.13.5-1: Baseline parameters of the Application Server energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| sampleTimestamp | The start date of the Energy measure |
| Duration | The duration of the energy measure |
| Media Delivery Session Identifier | A string that uniquely identifies a media delivery session in a Media Delivery System for the purpose of collating information from different system functions |
| Energy Exposing Mode | A value that indicates the type of energy information reported by the Application Server, energy consumption or carbon equivalent emission |
| Delivery Session Energy | A value that indicates the Energy consumption or the Carbon equivalent emission of the Application Server to convey the media data of the Media Delivery Session. |
| Media Component list | A list of Media Component objects that indicates the Energy consumption or the Carbon equivalent emission of each Media component of the Media Delivery Session |
| Renewable ratio | A value that indicates the energy renewable source ratio for the Media Delivery Session |
| Mix Energy sources | A value that indicates energy mix source percentages : Petroleum, Coal, Nuclear, Solar, Wind, Hydro. |

(NEW text)

#### 7.13.6	Processed EIF Energy Report

This is the metrics report generated periodically by the Energy Information AF function describing the energy consumption of the 5G Core over the last sampling period.

Table 7.13.6-1: Baseline parameters of the Processed EIF energy metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |
| sampleTimestamp | The start date of the Energy measure |
| Duration | The duration of the energy measure |
| Media Delivery Session Identifier | A string that uniquely identifies a media delivery session in a Media Delivery System for the purpose of collating information from different system functions |
| Energy Exposing Mode | A value that indicates the type of energy information reported by the 5G Core, energy consumption or carbon equivalent emission |
| Delivery Session Slice Energy | A value that indicates the Energy consumption or the Carbon equivalent emission of a slice of the 5G Core to convey the media data of the Media Delivery Session. |
| Delivery Session Energy | A value that indicates the Energy consumption or the Carbon equivalent emission of the 5G Core to convey the media data of the Media Delivery Session. |
| Media Component list | A list of Media Component objects that indicates the 5G Core Energy consumption or the Carbon equivalent emission to convey each Media component of the Media Delivery Session. |
| Radio Access Network Energy | A value that indicates the Energy consumption or the Carbon equivalent emission of the Radio Access Network to convey the media data of the Media Delivery Session. |
| Core Network Energy (UPF) | A value that indicates the Energy consumption or the Carbon equivalent emission of the Core Network to convey the media data of the Media Delivery Session. |
| Renewable ratio | A value that indicates the energy renewable source ratio for the Media Delivery Session. |
| Mix Energy sources | A value that indicates energy mix source percentages : Petroleum, Coal, Nuclear, Solar, Wind, Hydro. |

(NEW text)

#### 7.13.7	Processed Reporting entity aggregated metrics report

The aggregated processed metrics report is a set of Media Application Service Energy information aggregated by the UE based on the processed metrics report received from the network (see clause 7.11.2.4 above) and Media Application Service Energy metrics received from the UE itself.

Table 7.13.7-1: Baseline parameters of the processed metrics report

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

(NEW text)

#### 7.13.8	Media Application Service Energy metrics event

The Media Application Service metrics event is a collection of Media Application Service metrics reports exposed by the Data Collection AF to subscribing event consumers. It is reported on a per-UE basis. Similar to the processed metrics report described in clause 7.11.2.4 above, the information conveyed is based on per-UE NF Energy Reports received from the EIF and per-UE aggregated Application Service Energy Reports received from the Application Server.

Table 7.13.8-1: Baseline parameters of the Media Application Service Energy metrics event

| Abstract element | Semantics / constraints (abstract) |
| --- | --- |

End of changes