3GPP SA4-e (AH) MBS SWG post 134	S4aI250208

# Online, 18 December 2025	revision of S4-2520199

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0007 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI5 Media Application Server Energy management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-12-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Few steps are missing in the procedures in 7.6.3.2. Addition of a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of steps 10a to 10d in the procedures in 7.6.3.2. Addition of a clause 7.12 adding a solution to Key Issue 5 on Media Application Server Energy management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.1, 7.6.3.2, 7.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

…

[87]	Accubattery: https://play.google.com/store/apps/details?id=com.digibites.accubattery

[88]	3GPP TS 26.512: "5G Media Streaming (5GMS); Protocols".

[89]	ETSI TS 103 998: "Content Steering for DASH".

2nd Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #11 |  |  |  |  | X |  |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

3rd Change

#### 7.6.3.2	5GMS high-level procedures for collection and exposure of energy-related information

Figure 7.6.3.2-1 below details the different steps for energy-related information collection and reporting in the context of the 5GMS System defined in TS 26.501 [23] and as extended in clause 7.6.2.3 of the present document.

Figure 7.6.3.2-1: Procedures for energy-related information collection and reporting
in the 5GMS System

A first step is required to provision Energy Information Collection:

1.	The 5GMS Application Provider provisions the 5GMS AF via reference point M1, including a Provisioning Session resource, any other required resources and an Energy Information exposure configuration intended for the Energy Information AF instantiated in the 5GMS AF. The Energy Information exposure configuration may identify application(s) that fall within its scope.

2.	The Energy Information AF subscribes to receive NF Energy Information reporting from the Energy Information Function via reference point E12, if relevant.

3.	The 5GMS AF configures the 5GMS AS via reference point M3, including details of the Energy Information AF instantiated in the 5GMS AF.

4.	Based on configuration received from the 5GMS AF in the previous step, the 5GMS AS obtains an AS Energy Information collection configuration from the Energy Information AF via reference point E3, if relevant. This includes a callback endpoint on the Energy Information AF for submitting AS Energy Information reports.

At some later point, some initial Network Energy Information is collected, reported and processed per steps 16 to 18 below.

At some later point, a media delivery session is initiated:

5.	The 5GMS-Aware Application initiates a new media delivery session with the Media Session Handler via reference point M6, including a request to enable energy-related information collection and reporting.

6.	The Media Session Handler obtains Service Access Information from the 5GMS AF, including access details of the Energy Information AF.

7.	As a consequence of the previous step, the Media Session Handler creates a new energy-related information collection and reporting context with the Energy Information Collector instantiated in it.

8.	The Energy Information Collector subscribes to Network Energy Information reporting from Energy Information AF via reference point E5, if relevant, and receives in response a UE Energy Information collection configuration.

9.	The Energy Information Collector shares the UE Energy Information collection configuration with the Media Session Handler.

10.	Based on the UE Energy Information collection configuration received in the previous step, the Media Session Handler configures UE Energy Information collection and reporting in the Media Stream Handler via reference point M11.

11.	The Media Session Handler provides the Media Entry Point to the 5GMS-Aware Application.

12.	The 5GMS-Aware Application invokes the Media Stream Handler with the Media Entry Point to start media access.

13.	The Media Stream Handler establishes a transport session at reference point M4 for acquiring the Media Entry Point.

14.	The Media Stream Handler requests the Media Entry Point from the 5GMS AS.

After this initialisation phase, reporting can be done:

15.	The Energy Information Function may submit an NF Energy Information report to the Energy Information AF via reference point E12. The NF Energy Information report may include energy consumption information of different granularities, e.g. UE, PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

16.	The 5GMS AS may submit an AS Energy Information report to the Energy Information AF via reference point E3 using the callback endpoint supplied in step 4. The AS Energy Information report may include energy consumption information of different granularities, e.g. UE, PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

17.	The Energy Information AF processes the energy information report(s) it has received.

18.	The Energy Information AF exposes a processed Network Energy Information report about the UE to the Energy Information Collector subscriber established in step 8 via reference point E5. The Network Energy Information report may include energy consumption information of different granularities, e.g. PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

19.	The Energy Information Collector shares the report received in the previous step with the Media Session Handler.

In parallel:

20.	The Media Session Handler may collect additional UE Energy Information about the media delivery session from the Media Stream Handler via reference point M11. Based on the UE Energy Information collection configuration obtained in step 9, the UE Energy Information report may include energy consumption information of different granularities, e.g. PDU Session and/or QoS Flow, as described in clause 5.51.2.3 of TS 23.501 [72].

21.	The Media Session Handler processes the UE Energy Information it has obtained in the previous step.

22.	Based on the energy-related information received in step 17, the Media Session Handler may reconfigure the Media Stream Handler to change its streaming behaviour (e.g. change its streaming bit rate).

NOTE:	Details of Media Stream Handler reconfiguration in response to energy-related information are for further study.

23.	Based on the Energy Information received in steps 19 and 20, the Media Session Handler may instantiate a Dynamic Policy with a different energy usage profile.

NOTE 2:	Details of Dynamic Policy changes in response to energy-related information are for further study.

24.	The Media Session Handler exposes energy-related information to the subscribed 5GMS-Aware Application via reference point M6. Based on the UE Energy Information collection configuration obtained in step 6, the energy-related information may include energy consumption information of different granularities, e.g. PDU Session and/or QoS flow, as described in clause 5.51.2.3 of TS 23.501 [72] and/or for individual media delivery sessions.

25.	The 5GMS-Aware Application may expose the received energy-related information to the Application Service Provider via reference point M8 using methods beyond the scope of 3GPP standardisation.

In parallel:

26.	The Media Stream Handler may establish a new transport session with an appropriate service location on the 5GMS AS.

27.	Media is streamed between the Media Stream Handler, the 5GMS AS service location and the 5GMS Application Provider, as required by the media streaming session.

4th change
(All new text)

## 7.12	Solution #11: Selection of downlink media streaming service locations driven by content steering server based on energy characteristics

### 7.12.1	Key Issue mapping

This solution candidate addresses Key Issue #5 (Media Application Server Energy management) described in clause 6.5.

### 7.12.2	Functional description

The main media streaming formats such as MPEG-DASH and HLS have been updated to include additional information in their manifests to specify a server (called a "steering server") capable of dynamically selecting the service location to use. Currently, this selection is primarily based on Quality of Service (QoS) criteria.

From Release 19, this feature is already supported in the 5G Media Streaming System whereby the Media Player supports content steering for downlink media streaming as described in clause 10.3A.4 of TS 26.512 [88].

In parallel, networks have collectors of energy-related characteristics, including information that characterizes the network's power supply in terms of energy consumption, energy source, carbon footprint, capacity, and availability. These collectors provide insights into environmental impact at different granularities (per user, per service, etc.). This is notably the case with the Energy Information Function (EIF) defined in the 5G architecture.

However, there is currently no method for using these network energy characteristics to select the video streaming delivery path with the lowest environmental impact.

The proposed solution consists of defining a process that allows steering servers to make this choice based on environmental impact, through the sharing of network energy-related characteristics. Since content steering is exclusively defined for downlink media streaming, as specified in clause 10.3A.4 of TS 26.512 [88], this solution is applicable solely within this context.

The specific method for making this decision is implementation-dependent, but it could, for example, involve the kgCO2e per Joule of the energy supplying the AS. The content is available on 2 Media AS service locations having different energy supply mix which can vary according to the conditions of the environment (e.g. wind condition, solar power condition, etc.). To reduce its environmental impact, the MNO adapt steering information to maximize the use of the service location with the lower kgCO2e per Joule.

### 7.12.3	Reference architecture

Figure 7.12.3-11 depicts a reference architecture that realises this Candidate Solution in the downlink media streaming architecture defined in TS 26.501 [23] and TS 26.506 [59]. It is based on the solution for collecting and exposing energy-related characteristics instantiated in the 5G Media Streaming System (Solution #5) outlined in clause 7.6.3.2 of the present document.

NOTE:	The Energy Information Collector and reference point E5 are not instantiated in this reference architecture because they are not required by the Candidate Solution.

Figure 7.12.3-1: Reference architecture for media streaming service locations driven by content steering server based on energy characteristics

The core principle of this architecture revolves around the Energy Information AF instantiated within the 5GMSd AF. Its responsibilities vary depending on its current provisioning state, as determined by the 5GMSd AF (which includes an Energy Information Processing Configuration provisioned in the 5GMsd AF by the 5GMSd Application Provider). These responsibilities include:

-	Subscribing to and consuming NF Energy Information from the Energy Information Function as defined in clause 5.51 of TS 23.501 [72]) with required granularities (e.g. per UE) in accordance with the parameters of the provisioned Energy Information Exposure Specification.

-	Subscribing to and consuming AS Energy Information from the 5GMSd AS in accordance with the parameters of the provisioned Energy Information Exposure Specification.

Unlike other solutions described in the present document that utilise the Energy Information AF (e.g. Solution #5 or #10), this approach does not require an Energy Information Collector to be instantiated within the 5GMSd Client in the UE. Instead, the 5GMSd AF is the decision-making function. In the context of this solution, its primary role is to select the most appropriate service location based on energy characteristics that align with service provider configuration. Regardless of the chosen delivery path, the energy characteristics are assumed to remain unchanged on the UE. Therefore, the service location selection relies solely on the energy information provided by the network through the Energy Information AF.

A second difference from other solutions is the use of reference point M4d, already defined in TS 26.501 [23], to communicate service modification recommendations to the 5GMSd Client in the form of steering information.

While this approach may be less efficient than solutions that also incorporate energy-related data from the UE, it offers the advantage of having no impact on the UE itself, ensuring compatibility with legacy devices.

### 7.12.4	Procedures

Figure 7.12.4-1 below illustrates the various steps involved in the steering server-driven selection of service location, based on energy characteristics outlined in clause 7.12.3. This process combines the procedure for content steering as specified in clause 10.3A.4 of TS 26.512 [88] with the baseline procedure for energy-related information sharing for 5G Media Streaming outlined in clause 7.6.3.2 (Solution #5) of the present document, omitting the interactions with the Energy Information Collector which is not required by this Candidate Solution.

Figure 7.12.4-1: Procedures for selection of downlink media streaming service locations driven by content steering server based on energy characteristics

The procedure is based on the steps outlined in Solution #5 in clause 7.6, except for those involving the Energy Information Collector, which is not used in the context of this solution. Instead, it integrates the typical content steering procedure, such as the one described for DASH as specified in ETSI TS 103 998 [89]. A steering server is instantiated as part of the (logical) 5GMSd AS to enable to decide the service location based on energy characteristics of each path. The details of content steering have been highlighted in boldface within the steps below.

The steps are as follows:

0.	Downlink media streaming is provisioned by the 5GMSd Application Provider creating a Provisioning Session and Content Hosting Configuration in the 5GMSd AF. The Content Hosting Configuration declares multiple service locations in different Distribution Configurations, and declares that these are eligible for content steering. In response, the 5GMSd AF provides the endpoint of the content steering service at reference point M4d as well as the base URLs of the provisioned 5GMSd AS service locations at reference point M4d so that these can be included in Service Access Information advertised to the 5GMSd-Aware Application via reference point M8d.

Steps 1 to 4 are as described in the baseline procedure (see clause 7.6.3.2).

Initial network energy reporting may occur after the passage of some time:

15.	The Energy Information Function may submit a Network Energy Information report to the Energy Information AF via reference point E12.

16.	The 5GMSd AS may submit an AS Energy Information report to the Energy Information AF via reference point E3.

17.	The Energy Information AF processes the energy-related information reports it has received and provides this information to the 5GMSd AF.

Steps 18 to 25 of the baseline call flow are omitted because the processed energy-related information is not shared with the 5GMSd Client in this solution. Instead:

17a.	The 5GMSd AF makes a content steering prioritisation decision, taking into account a number of factors, including the load on the available service locations as well as energy-related information about the service locations provided in the previous step. The processing done by the Energy Information AF in the previous step allows the 5GMSd AF to identify the appropriate service location. This service location is then configured in the steering instructions with the higher priority to ensure its selection by the Media Player.

17b.	The 5GMSd AF provides content steering configuration via reference point M3d to the 5GMSd AS providing the content steering service.

Then, time passes until a downlink media streaming session is initiated by the 5GMsd-Aware Application.

Steps 5 and 6 (acquiring Service Access Information) of the baseline call flow proceed as normal. The Service Access Information may include Media Player Entry URLs.

NOTE:	The endpoint address of the Energy Information AF described in the baseline procedure is not required to be provided in the Service Access Information because the Energy Information Collector is not instantiated in the Media Session Handler in this Candidate Solution.

Steps 7, 8 and 9 of the baseline call flow are omitted because the Energy Information Collector is not instantiated in the Media Session Handler.

Steps 10 of the baseline call flow are omitted because the Energy Information Collector is not instantiated in the Media Session Handler.

11.	The Media Session Handler provides the Media Player Entries to the 5GMSd-Aware Application.

12.	The 5GMSd-Aware Application selects a Media Player Entry and the Media Player is invoked with the selected Media Player Entry to start media access and playback.

13.	The Media Player establishes a transport session at reference point M4d for acquiring the Media Player Entry from any suitable 5GMSd AS service location (in this case, Service Location #1).

14.	The Media Player requests the Media Player Entry from the 5GMSd AS (in this case from Service Location #1).

14a.	The 5GMSd AS conditions the Media Player Entry returned by the 5GMSd Application Provider to add the endpoint of its content steering service if the 5GMSd Application Provider has not already done so before returning it to the Media Player.

Steps 15 to 18 are repeated during the course of the media streaming session allowing energy-related information about the network to be updated. If it detects a change in this information, the 5GMSd AF may decide to update steering instructions by repeating steps 17a and 17b.

Steps 18 to 25 of the baseline call flow are omitted because (as before) the processed energy-related information is not shared with the 5GMSd Client in this solution. Instead:

25a.	The Media Player requests a steering instruction from the content steering service location indicated in the Media Player Entry obtained in step 14. No specific energy information is described in the steering instructions.

26 or 26bis.	If necessary, the Media Player establishes a new transport session at reference point M4d for acquiring media content from either Service Location #1 or Service Location #2, according to the content steering instruction obtained in the previous step.

27 or 27bis.	The Media Player requests media content from the selected 5GMSd AS service location according to the transport session established or reused in the previous step.

### 7.12.5	Summary

This candidate solution proposes a mechanism allowing 5GMSd Application Providers to steer the 5GMSd Client towards the service location on the 5GMSd AS with the lowest environmental impact, based on:

1.	The collection of network energy-related characteristics by the Mobile Network Operator. This information is provided by the Energy Information Function to a new component called Energy Information AF, included in the 5GMSd AF.

2.	Standardised interfaces between the 5GMSd AF and the 5GMSd Application Provider allowing the 5GMSd Application Provider to provision the use of the content steering mechanism with the goal of reducing the environmental impact of downlink media streaming.

3.	The use of this information by a 5GMSd AS (acting as a content steering server) to dynamically steer 5GMSd Clients toward the service locations with the desired energy characteristics.

This approach optimizes the energy efficiency of multimedia content delivery, thereby contributing to the reduction of the environmental footprint associated with media streaming services.

The solution applies to any 5GMSd Application Provider delivering content via a 5GMSd System and integrates naturally into existing adaptive streaming architectures such as MPEG-DASH and HLS.

The solution has no impact on the UE because the content steering mechanisms employed are part of adaptive streaming protocols such as MPEG-DASH and HLS. The selection of the service location with the desired environmental impact is achieved by giving a higher priority to this service location in the steering instructions provided by the content steering service.

End of changes