3GPP SA4-e (AH) MBS SWG post 134	S4aI250210

# Online, 18 December 2025	revision of S4aI250200

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.942 | CR | 0006 | rev | 4 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [FS_Energy_Ph2_MED] Solution for KI6 Client-driven management of media delivery service energy optimisation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_Energy_Ph2_MED |  |  |  |  |  | Date: |  |  | 2025-12-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addtion of a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of a clause 7.11 adding a a solution to Key Issue 6 on Client-driven management of media delivery service energy optimisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No solution proposed for the KI. |  |  |  |  |  |  |  |  |
| Q |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1, 7.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST Change

## 7.1	Mapping of Solutions to Key Issues

Table 7.1-1: Mapping of solutions to Key Issues

| Solutions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #1 |  |  | X |  |  |  |
| #2 |  |  | X |  |  |  |
| #3 |  | X |  |  |  |  |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  |  |  |  |
| #6 | X | X |  |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 | X |  |  |  |  |  |
| #10 |  |  |  |  |  | X |

These solutions are simply candidate solutions. Their inclusion in the following clauses does not imply that they have been agreed upon or endorsed. Any decisions and work to be done for the normative work will be detailed in the conclusions of this Technical Report.

2nd change
(All new text)

## 7.11	Solution #10: Client-driven selection of Media Entry Point in the generalised Media Delivery System based on energy characteristics

### 7.11.1	Key Issue mapping

This Candidate Solution addresses Key Issue #6.

### 7.11.2	Functional description

Currently, media players lack the capability to select Media Entry Points based on their environmental impact, in addition to Quality of Service (QoS) considerations. This limitation hinders the ability to optimize energy efficiency during media playback while still maintaining the QoE at the level configured by the user.

The proposed solution introduces a mechanism for sharing energy-related characteristics with the UE, throughout the technical delivery chain. This enables more energy-efficient media playback by allowing client devices to make informed decisions about Media Entry Point selection, considering their environmental impact alongside QoS.

An example of energy-related characteristics that could enable a Media Client to select the most energy-efficient Media Entry Point is the energy consumption of media delivery session related to the different Media Entry Points or each media representation over a given sampling period and for a specified QoS. Additionally, a media client could be configured to avoid playing media representations that exceed a certain energy consumption threshold, such as a specific kWh/h level.

The decision-making process for optimizing media delivery is detailed here based on selecting different Media Entry Points, each corresponding to different service locations with varying delivery paths and/or media characteristics (e.g., different maximum resolutions, video codecs, multicast capabilities, etc.). But, once the energy-related characteristics of the media delivery are shared with the Media Client, this approach can also be extended to any 5G media delivery optimizations that the client can initiate.

This solution also takes into consideration end-user energy preferences configured in the UE Application. Three different modes are available in the UE Application to simplify the configuration:

-	Eco mode: Prioritizes energy efficiency, even at the slight expense of Quality of Experience.

-	Standard mode: Balance between energy efficiency and Quality of Experience.

-	Performance mode: Prioritizes Quality of Experience over energy consumption.

### 7.11.3	Reference architecture

Figure 7.11.3-1 depicts a reference architecture that realises this Candidate Solution in the generalised Media Delivery architecture defined in TS 26.501 [23] and TS 26.506 [59]. The core principle of this architecture is to leverage the existing method for collecting and exposing energy-related characteristics, as already outlined in Solution #5 (see clause 7.6.2.4). Additionally, it extends this approach by enabling the Media Client to select Media Entry Point based on this information.

Figure 7.11.3-1: Reference architecture for client-driven selection of Media Entry Point based on energy characteristics in the generalised Media Delivery System

The solution is reusing functions already defined for collection and exposure of energy-related information in clause 7.6.2.4:

-	The Energy Information AF instantiated in the Media AF.

-	The Energy Information Collector instantiated in the Media Session Handler of the Media Client communicates with the Media Access Function of the Media Client via existing reference point M11.

The solution also reuses reference points M1, M3, E3, M4, M5, E5, M6, M8, M11 and E12 already defined for collection and exposure of energy-related information in the instantiation in generalised media delivery architecture presented in clause 7.6.2.4.

### 7.11.4	Procedures

Based on the procedures outlined in clause 7.6.3, figure 7.11.4-1 below details additional steps supporting client-driven selection of Media Entry Points based on energy characteristics outlined in clause 7.11.3.

Figure 7.11.4-1: Procedures for client-driven selection of Media Entry Points based on energy characteristics in the generalised Media Delivery System

Steps 1 to 10 are not detailed since they are already described in the baseline procedure in clause 7.6. The energy-related information gathered during those steps allows energy-related information (e.g. energy consumption per kWh/GB or kWh/h for a given QoE) to be calculated for each media delivery session of each Media Entry Point and/or each media representation in each Media Entry Point before the start of the media deliver. Steps 20 to 22 are not required by this Candidate Solution at this stage.

Then, the collected information is used to initiate the media delivery using the desired Media Entry Point:

11.	The Media Session Handler provides the Media Entry Points to the Media-aware Application. The Media Entry Point descriptions are embellished by the Media AF with energy related information based on the information it received in steps 15.

11a.	The Media-aware Application selects the appropriate Media Entry Point according to its energy characteristic’s and the energy mode configured by the end-user.

12.	The Media Access Function is invoked with the selected Media Entry Point to start media delivery, per the baseline call flow for the Media Delivery System in question.

13/13bis.	The Media Access Function establishes a transport session with one of the available Media AS service locations.

14/14bis.	The Media Access Function acquires the Media Entry Point from one of the available Media AS service locations.

Steps 15 to 25 inclusive are repeated in parallel with media delivery (steps 26 and 27). In this case, step 23 may be omitted:

22.	If the configuration selected by the Media Session Handler in the previous step is different, the Media Session Handler reconfigures the Media Access Function.

23.	The Media Session Handler may instantiate a new Dynamic Policy (or modify an existing one) on the Media AF.

Steps 24 and 25 (exposing energy-related information) are identical to the baseline procedure.

26/26bis.	The Media Access Function establishes a transport session with one of the available Media AS service locations.

27/27bis.	Media content is delivered between the Media Access Function and one or more Media AS service locations according to the current configuration of the Media Access Function.

### 7.11.5	Summary

The proposed solution enables the integration of energy-related characteristics into multimedia content delivery systems. It empowers Media Clients to choose the Media Entry Point based on their environmental impact, while also considering user preferences that balance the trade-off between Quality of Experience and environmental sustainability.

This solution is based on Network Energy Information available from the Energy Information Function and the Application Server, as well as the two new functional entities defined in Solution #5 (see clause 7.6): the Energy Information AF and the Energy Information Collector. Leveraging existing mechanisms of the media delivery system, like the procedure for media delivery from multiple service locations illustrated in this clause, the Media Client is able to make use of energy-related information provided by these functions to optimize energy efficiency during media delivery sessions.

This solution offers several advantages:

1.	Allows the environmental impact to be reduced: The energy consumption and consequent CO2 emissions associated with multimedia content delivery may be used in the process of determining the Media Entry Point used.

2.	User transparency: The environmental impact of content consumption may be made visible without impacting privacy.

3.	Flexibility: The approach is compatible with any existing 5G System optimizations which can be initiated by the Media Client.

4.	Customization: Thanks to the energy optimization modes, end users can be given the freedom to choose their level of environmental commitment.

5.	Global optimization: The entire delivery chain, from distribution to consumption, is taken into account.

End of changes