3GPP TSG-SA WG4 Post #134 RTC AHG Call	Sa4R250216

# Online, 17th December 2025

Source:	Qualcomm Inc.

Title:	[AIML_IMS-MED] Scope in-light of the SA2 LS

Agenda item:	4.5

Document for:	Agreement

1	Introduction

This contribution discusses the AIML_IMS-MED work direction in SA4 based on the LS response from SA2 (S2-2510954).

2	Discussion on Scope

In their response, SA2 indicates that model handling in the Media Function (MF) and enriching the MF with inference capability are currently not supported by IMS specifications and are not in scope of the IMS RTC study for Release 20.

SA2 also requests clarification on what new functionality is expected in the UE and IMS entities (e.g. MF, DC Application Server) compared to what is specified in TS 23.228 Annex AC.

To keep progress in SA4 while respecting SA2 scope, this contribution proposes a phased approach.

- Phase 1 focuses on UE-only inference and related media handling in SA4. For network hosted inference, Phase 1 also considers inference in the DC Application Server (DC AS) using the existing data channel framework in Annex AC, without processing the conversational RTP media streams in the MF.

- Remote inference in the MF and split inference are proposed for Phase 2 and depend on progress and guidance from SA2.

3	Impacts on Call Flow and Procedures

Based on SA2 feedback, Phase 1 is structured to keep IMS conversational media procedures unchanged and to leverage the existing IMS data channel framework for AI-related exchanges. The key impacts are limited to data channel establishment and application-layer messaging. No MF model handling or MF inference is introduced in Phase 1.

Conversational RTP media streams are established and handled as usual. The UE performs inference locally and any media processing remains within the UE.

Where AI task discovery, configuration, or result exchange is needed, the UE establishes a bootstrap data channel and/or application data channel using existing procedures. These channels carry configuration, model retrieval references, and inference results or metadata.

Any additional protocol details for configuration messages, model identifiers, and result formats are SA4 matters and can be specified without changing SA2 procedures.

In the network-hosted option, the DC AS hosts the inference function as application logic. The UE remains responsible for generating inference inputs such as selected segments, features, or embeddings from locally received and/or captured media.

An application data channel is established using the existing procedures with the DC AS as the endpoint. The MF role is limited to anchoring and relaying the data channel media towards the DC AS under IMS AS and DCSF control, consistent with Annex AC. Conversational RTP media continues to be routed end-to-end and is not terminated, forked, or modified in the MF for AI purposes.

Remote inference in the MF and split inference between UE and MF would require new IMS procedures beyond Annex AC, including mechanisms for MF media stream processing of conversational RTP, in-network model handling, and related policy and privacy controls. SA2 indicates these are not supported by current IMS specifications and are out of scope for the Release 20 IMS RTC study.

These impacts motivate treating remote and split inference as Phase 2 items that depend on further SA2 guidance and potentially new work in SA2 for a future release.

4	Proposal

We propose to prioritize the work on phase 1 and finalize this work as soon as possible. We need to clarify assumptions and functionality of remote and split inference to SA2 and pause this work until the topic is addressed in SA2.