3GPP TSG- <TSG/WG> Meeting # <MTG_SEQ><MTG_TITLE>	<TDoc#>

# <Location>,  <Country>,  <Start_Date> - <End_Date>

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | <Title> |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | <Source_if_WG> |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Within the scope of the work item HInT, it is intended to add new test methods to TS 26.132 for analogue and digital interfaces of UE. As a preparation for these, a detailed specification and description of the introduced interfaces has to be included. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of new clauses for analogue and digital interfaces, editorial changes in the existing clauses regarding measurement equipment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of the headset interface not considered. Evaluation of a mobile phone without an associated headset not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Introduction, 1, 2, 3.1, 4, 4.1 (new), 4.2 (new), 4.3 (new), 5.1.6 (new), 5.2, 7.2.6 (new), 7.3.3 (new), 7.3.4 (new), 7.4.7 (new), 7.4.8 (new), 7.5.3a (new), 7.5.4, 7.7.5 (new), 7.8.3 (new), 7.8.4 (new), 7.10.1b (new), 7.10.2b (new), 7.10.3b (new), 7.11.1, 7.12.3 (new), 7.13.1, 7.13.2, Annex D.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 26.131 CR TBD |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

- ------------------------- START OF CHANGE 1 -------------------------

# Introduction

The present document specifies test methods to allow the minimum performance requirements for the electro-acoustic characteristics of GSM, 3G, LTE, NR and WLAN terminals when used to provide narrowband, wideband, super-wideband or fullband telephony to be assessed.

The objective for narrowband services is to reach a quality as close as possible to ITU-T standards for PSTN circuits. However, due to technical and economic factors, there cannot be full compliance with the general characteristics of international telephone connections and circuits recommended by the ITU-T.

The performance requirements are specified in TS 26.131; the test methods and considerations are specified in the main body of the text.

# 1	Scope

The present document is applicable to any terminal capable of supporting narrowband, wideband, super-wideband or fullband telephony, either as a stand-alone service or as the telephony component of a multimedia service. The present document specifies test methods to allow the minimum performance requirements for the electro-acoustic characteristics of GSM, 3G, LTE, NR and WLAN terminals when used to provide narrowband, wideband, super-wideband or fullband telephony to be assessed.

- ------------------------- END OF CHANGE 1 -------------------------

- ------------------------- START OF CHANGE 2 -------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[…]

[53]	ITU-T Recommendation P.381: "Technical requirements and test methods for the universal wired headset or headphone interface of digital mobile terminals".

- ------------------------- END OF CHANGE 2 -------------------------

- ------------------------- START OF CHANGE 3 -------------------------

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document the terms narrowband, wideband, super-wideband and fullband refer to signals associated with the corresponding operating modes of the speech codecs specified in 5.2.

For the purposes of the present document, the terms dB, dBr, dBm0, dBm0p and dBA, shall be interpreted as defined in ITU-T Recommendation G.100 [51]; the term dBPa shall be interpreted as the sound pressure level relative to 1 pascal expressed in dB (0 dBPa is equivalent to 94 dB SPL).

A 3GPP softphone is a telephony system running on a general purpose computer or PDA complying with the 3GPP terminal acoustic requirements (TS 26.131 and 26.132).

For the purposes of the present document the term clock skew is defined as the difference between the clock of the device under test (CDUT) and the clock of the reference client (CREF). The skew of CDUT relative to CREF is defined in parts per million (PPM) as: (CDUT -CREF).106/ CREF.

For the purposes of the present document, the term electrical interface is defined as an analogue or digital access to an UE, which allows injecting and capturing signals electrically instead of an acoustical interface. The interface can be either wired (analogue, digital) or wireless (digital). The purpose of this interface is to connect a separate device (typically a headset), which provides a receiver and transmitter for telephony.

- ------------------------- END OF CHANGE 3 -------------------------

- ------------------------- START OF CHANGE 4 -------------------------

# 4	Interface definitions

## 4.1	General

The interfaces required to define terminal electro-acoustic characteristics are the acoustical interfaces, the air interface and the point of interconnect (POI), see Figure 1.

MTSI speech aspects are specified by TS 26.114 [17].

## 4.2	Air interfaces

The Air Interfaces for GSM, 3G, LTE and NR are specified by GSM 05, 3GPP 45, 3GPP 25, 3GPP 36 and 3GPP 38 series specifications, and the Air Interface for WLAN access to EPC is specified by WLAN access to EPC as defined in 3GPP TS 23.402 [48] and TS 24.302 [49]. MTSI speech aspects are specified by TS 26.114 [17].

Measurements can be made using a system simulator (SS) comprising the appropriate radio terminal equipment and speech transcoder. The delays, losses and gains introduced by the test equipment shall be accounted for.

## 4.3	Acoustical interfaces

The POI with the public switched telephone network (PSTN) is considered to have a relative level of 0 dBr.

Five classes of acoustical interface are considered in this specification:

-	Handset UE including softphone UE used as a handset;

-	Headset UE including softphone UE used with headset;

-	Vehicle Mounted Hands-free UE including softphone UE mounted in a vehicle;

-	Desktop-mounted hands-free UE including softphone UE with external loudspeaker(s) used in hands-free mode;

-	Hand-held hands-free UE including softphone UE with internal loudspeaker(s) used in hands-free mode.

(See definition of softphone in Clause 3.1)

NOTE:	The test setup for a softphone UE shall be derived according to the following rules:

-	When using a softphone UE as a handset: the test setup shall correspond to handset mode.

-	When using a softphone UE with headset: the test setup shall correspond to headset mode.

-	When a softphone UE is mounted in a vehicle: the test setup shall correspond to vehicle-mounted hands-free mode.

-	When using a softphone UE in hands-free mode:

-	When using internal loudspeaker(s), the test setup shall correspond to hand-held hands-free.

-	When using external loudspeaker(s), the test setup shall correspond to desktop-mounted hands-free.

## 4.3	Electrical interfaces

An electrical interface is considered in this specification and details on standardized analogue (wired) and digital (wired and wireless) headset interfaces can be found in TS 26.132. For the electrical interface, the POI in sending / receiving direction is respectively defined as the input / output of the reference speech coder of the system simulator.

Any of the UE types mentioned in clause 4.2 providing an electrical headset interface can be considered as Electrical Interface UE.

- ------------------------- END OF CHANGE 4 -------------------------

- ------------------------- START OF CHANGE 5 -------------------------

### 5.1.6	Test setup for electrical interfaces

#### 5.1.6.1	Wired analogue connection

UE testing via analogue connection shall be carried out with a universal wired headset interface, which complies with the electrical and physical characteristics described in clause 6 of ITU-T P.381 [53]. Different types of socket designs can be used as well. Other implementations of analogue electrical interfaces (wired or wireless) are out of scope.

Figure 15a5b illustrates the setup required for testing analogue electrical interfaces. The electric output impedance of the reference interface of the test equipment shall be in the range of 1 Ω and 10 kΩ. The corresponding electric input impedance shall be 32 Ω +/- 2 Ω. The common ground impedance (between sending and receiving sides) for the test system shall be ≤ 0.05 Ω.

If not specified otherwise, the nominal average signal level in send direction (output of the interface) is -60 dBV, which corresponds to a typical acoustic level of -4.7 dBPa at the MRP. In receive direction, the average signal level for a nominal volume setting (if present) is expected to be approximately -39 dBV.

For testing echo and double talk scenarios, an artificial feedback of the receive signal into the sending path shall be used. This echo path can be realized in an analogue (e.g, a stand-alone device) or digital (e.g., part of the test system) way. For measurements without artificial echo loss, the feedback path is disabled.

NOTE:	It is assumed that mainly passive third-party devices are connected via analogue headset interface to the UE, which do not contain any typical signal processing capabilities (like e.g., echo cancellation or noise reduction). Thus, all tests specified for this interface are comparable to handset UE or headset UE, i.e. they expect that any possible signal processing is applied in the UE.

Figure 15a5b: Test setup for analogue electrical headset interface

#### 5.1.6.2	Digital connection

Figure 15a5c illustrates the setup required for testing digital electrical interfaces of a UE. Such testing is possible with different types of wired and wireless technologies and requires a digital reference interface in the test system. Only interfaces with standardized protocols and definitions (e.g., USB, Bluetooth, etc.) shall be used, i.e. digital interfaces of proprietary hardware or protocols are out of scope. The digital connection type (wired/wireless, protocol and hardware) shall be reported.

For testing echo and double talk scenarios, an artificial feedback of the receive signal into the sending path shall be used. This echo path shall be realized as part of the test system. The received and decoded signal from the UE is fed back into the sending direction, in advance to the encoding/protocol/hardware layer. For measurements without artificial echo loss, the feedback path is disabled.

Digital levels for send and receive direction are specified in dBm0, referring to the same definition as for the input/output of the terminal and the system simulator (see clause 5.2).

In contrast to the analogue interface, devices connected to the UE via digital interface may either provide active (includes signal processing for send and/or receive direction) or passive (comparable to analogue headsets, see clause 5.1.6.1) functionalities. In order to be compatible with the analogue headset interface, it is assumed for all tests in the following clauses that typical signal processing for telecommunication (e.g., noise reduction, echo cancellation) takes places only in the UE and not in the connected equipment.

However, in some cases, the digital interface and its protocol supports the exchange of commands/meta-information. A connected device may communicate that it provides capabilities to perform its own signal processing. In this case, the UE shall behave as a transparent gateway, i.e. shall not apply any signal processing.

Figure 15a5c: Test setup for digital electrical headset interface

- ------------------------- END OF CHANGE 5 -------------------------

- ------------------------- START OF CHANGE 6 -------------------------

## 5.2	Setup of the electrical interfaces of test equipment

### 5.2.1	Codec approach and specification

In this approach, a codec is used to convert the digital input/output bit-stream of the system simulator to the equivalent analogue values. With this approach a system simulator simulating the radio link to the terminal under controlled and error-free conditions is required, unless otherwise specified for the respective test. The system simulator has to be equipped with a high-quality codec with characteristics as close as possible to ideal.

Definition of 0 dBr point:

D/A converter -	a Digital Test Sequence (DTS) representing the codec equivalent of an analogue sinusoidal signal with an RMS value of 3,14 dB below the maximum full-load capacity of the codec shall generate 0 dBm across a 600 ohm load;

A/D converter -	a 0 dBm signal generated from a 600 ohm source shall give the digital test sequence (DTS) representing the codec equivalent of an analogue sinusoidal signal with an RMS value of 3,14 dB below the maximum full-load capacity of the codec.

Rename of clause 5.2 just to be confused with the electrical (headset) interface.

- ------------------------- END OF CHANGE 6 -------------------------

- ------------------------- START OF CHANGE 7 -------------------------

## 7.2	Overall loss/loudness ratings

### 7.2.1	General

The SLR and RLR values for GSM, 3G, LTE, NR or WLAN networks apply up to the POI. However, the main determining factors are the characteristics of the UE, including the analogue to digital conversion (ADC) and digital to analogue conversion (DAC). In practice, it is convenient to specify loudness ratings to the Air Interface. For the normal case, where the GSM, 3G, LTE, NR or WLAN network introduce no additional loss between the Air Interface and the POI, the loudness ratings to the PSTN boundary (POI) will be the same as the loudness ratings measured at the Air Interface.

### 7.2.2	Connections with handset UE

[...]

### 7.2.3	Connections with desktop and vehicle-mounted hands-free UE

[...]

### 7.2.4	Connections with hand-held hands-free UE

[...]

### 7.2.5	Connections with headset UE

Same as for handset.

### 7.2.6	Connections with electrical interface UE

TBD

- ------------------------- END OF CHANGE 7 -------------------------

- ------------------------- START OF CHANGE 8 -------------------------

## 7.3	Idle channel noise (handset, headset and electrical interface UE)

### 7.3.0	Overview

For idle noise measurements in sending and receiving directions, care should be taken that only the noise is windowed out by the analysis and the result is not impaired by any remaining reverberation or by noise and/or interference from various other sources. Some examples are air-conducted or vibration-conducted noise from sources inside or outside the test chamber, disturbances from lights and regulators, mains supply induced noise including grounding issues, test system and system simulator inherent noise as well as radio interference from the UE to test equipment such as ear simulators, microphone amplifiers, etc.

### 7.3.1	Sending

[...]

### 7.3.2	Receiving

[...]

### 7.3.3	Sending (electrical interface UE)

TBD

### 7.3.4	Receiving (electrical interface UE)

TBD

- ------------------------- END OF CHANGE 8 -------------------------

- ------------------------- START OF CHANGE 9 -------------------------

## 7.4	Sensitivity/frequency characteristics

### 7.4.1	Handset and headset UE sending

[...]

### 7.4.2	Handset and headset UE receiving

[...]

### 7.4.3	Desktop and vehicle-mounted hands-free UE sending

[...]

### 7.4.4	Desktop and vehicle-mounted hands-free UE receiving

[...]

### 7.4.5	Hand-held hands-free UE sending

[...]

### 7.4.6	Hand-held hands-free UE receiving

[...]

### 7.4.7	Electrical interface UE sending

TBD

### 7.4.8	Electrical interface UE receiving

TBD

- ------------------------- END OF CHANGE 9 -------------------------

- ------------------------- START OF CHANGE 10 -------------------------

## 7.5	Sidetone characteristics

### 7.5.1	Connections with handset UE

[...]

### 7.5.2	Headset UE

[...]

### 7.5.3	Hands-free UE (all categories)

[...]

### 7.5.3a	Electrical interface UE

TBD

### 7.5.4	Sidetone delay for handset, headset or electrical interface UE

[...]

- ------------------------- END OF CHANGE 10 -------------------------

- ------------------------- START OF CHANGE 11 -------------------------

## 7.7	Acoustic echo control

### 7.7.1	General

The echo loss (EL) presented by the GSM, 3G, LTE or NR networks at the POI should be at least 46 dB during single talk. This value takes into account the fact that UE is likely to be used in a wide range of noise environments.

### 7.7.2	Acoustic echo control in a hands-free UE

[...]

### 7.7.3	Acoustic echo control in handset UE

[...]

### 7.7.4	Acoustic echo control in a headset UE

[...]

### 7.7.5	Acoustic echo control in a electrical interface UE

TBD

- ------------------------- END OF CHANGE 11 -------------------------

- ------------------------- START OF CHANGE 12 -------------------------

## 7.8	Distortion

### 7.8.1	Sending distortion (excluding electrical interface UE)

[...]

### 7.8.2	Receiving distortion (excluding electrical interface UE)

[...]

### 7.8.3	Sending distortion (electrical interface UE)

TBD

### 7.8.4	Receiving distortion (electrical interface UE)

TBD

- ------------------------- END OF CHANGE 12 -------------------------

- ------------------------- START OF CHANGE 13 -------------------------

## 7.10	Delay

### 7.10.0	UE Delay Measurement Methodologies

[...]

### 7.10.1	Delay in sending direction (Handset UE)

[...]

### 7.10.1a	Delay in sending direction (headset UE)

[...]

### 7.10.1b	Delay in sending direction (electrical interface UE)

TBD

### 7.10.2	Delay in receiving direction (handset UE)

[...]

### 7.10.2a	Delay in receiving direction (headset UE)

[...]

### 7.10.2b	Delay in receiving direction (electrical interface UE)

TBD

### 7.10.3	Delay in sending + receiving direction using "echo" method (handset UE)

[...]

### 7.10.3a	Delay in sending + receiving direction using "echo" method (headset UE)

[...]

### 7.10.3b	Delay in sending + receiving direction using "echo" method (electrical interface UE)

[...]

### 7.10.4	Delay and speech quality in conditions with packet arrival time variations and packet loss

[...]

### 7.10.5	UE send clock accuracy

[...]

### 7.10.6	UE receiving with clock skew

For further study.

- ------------------------- END OF CHANGE 13 -------------------------

- ------------------------- START OF CHANGE 14 -------------------------

## 7.11	Echo control characteristics

### 7.11.1	Test set-up and test signals

The device is set up according to clause 5. The ambient noise level shall be ≤ -64 dBPa(A).

The test shall be performed with the British-English "long" double-talk and conditioning speech sequences from ITU-T Recommendation P.501 [22], with the signals in the receiving direction band limited according to clause 5.4.

A description of the test stimuli is presented in Table 2a and Table 2b. The test sequence is composed of an initial conditioning sequence of 23,5 s and a double talk sequence of 35 s. For the analysis, the double talk sequence is divided into two segments, a first double-talk sequence with single short near-end words (0 – 20 s), and a second double-talk sequence with continuous double talk (20 – 35 s).

The sending speech during double-talk and the "near-end speech only" are recorded individually, with the "near-end speech only" sequence recorded with silence in the receiving direction. The time-alignment of the two recorded sequences is performed off-line during the analysis.

Table 2a: Test stimuli for recording of Echo Canceller operation

|  | Conditioning | Single words (segment 1) and full sentence (segment 2) double talk |
| --- | --- | --- |
| Far-end signal | FB_female_conditioning_seq_long.wav | FB_male_female_single-talk_seq.wav |
| Artificial mouth signal | FB_male_conditioning_seq_long.wav | FB_male_female_double-talk_seq.wav |

Table 2b: Test stimuli for reference "near-end speech only" recording.

|  | Conditioning | Single words (segment 1) and full sentence (segment 2) double talk |
| --- | --- | --- |
| Far-end signal | FB_female_conditioning_seq_long.wav | silence |
| Artificial mouth signal | FB_male_conditioning_seq_long.wav | FB_male_female_double-talk_seq.wav |

The active speech level of the signal of the artificial mouth shall be -4,7 dBPa measured at the MRP. For electrical interface UE, the active speech level of the signal shall be calibrated to -60 dBV for analogue and to -16 dBm0 for digital connections. In order to obtain a reproducible time alignment as seen by the UE, the artificial mouth signal shall be delayed by the amount of the receiving direction delay. For the purpose of this alignment, the receiving direction delay for handset and headset modes is defined from the system simulator input to the artificial ear. For hands-free modes, the downlink delay is defined from the system simulator input to the acoustic output from the UE loudspeaker.

The level of the downlink signal shall be -16 dBm0 measured at the digital reference point or the equivalent analogue point.

[...]

- ------------------------- END OF CHANGE 14 -------------------------

- ------------------------- START OF CHANGE 15 -------------------------

## 7.12	Quality (speech quality, noise intrusiveness) in the presence of ambient noise

### 7.12.1	Handset UE

[...]

### 7.12.2	Hand-held hands-free UE

[...]

### 7.12.3	Electrical interface UE

TBD

- ------------------------- END OF CHANGE 15 -------------------------

- ------------------------- START OF CHANGE 16 -------------------------

## 7.13	Jitter buffer management behaviour

### 7.13.0	General

[...]

### 7.13.1	Delay histogram

[...]

[Measurement description needs slight adaptation for electrical interface UE]

[...]

### 7.13.2	Speech quality loss histogram

[...]

[Measurement description needs slight adaptation for electrical interface UE]

[...]

- ------------------------- END OF CHANGE 16 -------------------------

- ------------------------- START OF CHANGE 17 -------------------------

Annex D (normative):
Clock skew measurement

This Annex describes a method to measure the clock skew between the reference client and the device under test.

# D.1	Test procedure

As speech test signal, the second sentence of the first female speaker (female1.wav) of the English test sentences according to ITU-T P.501 is used. When measuring in receiving direction the signal is pre-filtered according to the used bandwith and normalized to an active speech level of -16dBm0. When measuring in sending direction the signal is calibrated to an active speech level of -4.7 dBPa at MRP. For electrical interface UE, the active speech level is calibrated to -60 dBV for analogue and to -16 dBm0 for digital connections. The sequence is centered within a 4.0s time window and repeated 40 times, resulting in a test file of 160.0s length. Alternatively CSS signals may be used with the corresponding adaptation of the step size to 333ms. The test signal used shall be reported.