Agenda item: 	6.1

Source: 	InterDigital

Title: 	[5G Video] Cloud Gaming anchors and data points

Document for	Information

# Introduction

In the context of the study on 5G Video codecs characteristics (FS_5GVideo) a limited number of service scenarios have been identified for the evaluation of video codecs already defined in 3GPP specifications, namely AVC and HEVC. One of these service is the Cloud Gaming scenario.

This contribution provides some anchor results for the sequences described in annex C of 26.955 and of the newly proposed sequences described in S4aV200542 using HEVC HM reference software in LD-P and LD-B configurations with an intra period of 1second and infinite.

# Configuration

To generate the anchor bitstream, the following HEVC reference software was used:

- https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/tags/HM-16.22

Setting:

- https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/blob/HM-16.22/cfg/encoder_lowdelay_main10.cfg

- https://vcgit.hhi.fraunhofer.de/jct-vc/HM/-/blob/HM-16.22/cfg/encoder_lowdelay_P_main10.cfg

- with the following changes:

- IntraPeriod such that:

  - 1 second is achieved

  - no fix interval

- DecodingRefreshType:

  - CRA value = 1

- GOPSize: 8

- -QP: [22, 27, 32, 37]

- Coding structure: LD-B* and LD-P . *Bi-predition with no future reference frame.

# Results

Low Delay B HEVC Main 10

|  |  |  | Reference |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps | Y psnr | U psnr | V psnr | Enc T [s] | Dec T [s] |  | Enc T [h] |
| Class Gaming Kingston | HeroesOfTheStorm_1920x1080_30Hz_8bit_P420_Part1 | 22 | 4700,22 | 42,68 | 43,99 | 44,05 | 9916,56 | 16,06 | 2,75 |  |
|  |  | 27 | 2183,24 | 39,01 | 41,14 | 41,25 | 8838,40 | 13,91 | 2,46 |  |
|  |  | 32 | 1011,54 | 35,72 | 39,21 | 39,30 | 8473,09 | 12,67 | 2,35 |  |
|  |  | 37 | 467,85 | 32,81 | 37,49 | 37,56 | 7796,32 | 12,02 | 2,17 |  |
|  | ProjectCars_1920x1080_30Hz_8bit_P420_Part1 | 22 | 12497,23 | 41,46 | 48,46 | 48,59 | 17693,30 | 31,03 | 4,91 |  |
|  |  | 27 | 4591,05 | 38,09 | 46,16 | 46,28 | 14218,29 | 25,86 | 3,95 |  |
|  |  | 32 | 1908,36 | 35,32 | 44,56 | 44,60 | 11620,16 | 19,58 | 3,23 |  |
|  |  | 37 | 857,41 | 32,87 | 43,15 | 43,02 | 10011,25 | 15,56 | 2,78 |  |
|  | WoW_1920x1080_30Hz_8bit_P420_Part1 | 22 | 10741,40 | 41,04 | 44,86 | 47,34 | 15980,09 | 27,84 | 4,44 |  |
|  |  | 27 | 4354,24 | 37,08 | 42,26 | 44,99 | 13052,74 | 20,03 | 3,63 |  |
|  |  | 32 | 1851,53 | 33,98 | 40,43 | 43,30 | 11087,28 | 17,11 | 3,08 |  |
|  |  | 37 | 789,39 | 31,43 | 38,76 | 41,87 | 9801,05 | 15,26 | 2,72 |  |
| Class Gaming | AOV5_1920x1080_60Hz_8bit_420 | 22 | 14094,41 | 42,04 | 43,72 | 44,41 | 26439,64 | 44,26 | 7,34 |  |
| HD Tencent |  | 27 | 5554,60 | 38,40 | 40,77 | 41,23 | 22171,27 | 35,79 | 6,16 |  |
|  |  | 32 | 2521,45 | 35,34 | 38,71 | 39,01 | 19154,43 | 32,29 | 5,32 |  |
|  |  | 37 | 1254,49 | 32,60 | 36,98 | 37,12 | 17833,77 | 30,35 | 4,95 |  |
|  | baolei_1920x1080_60Hz_8bit_420_01 | 22 | 24718,22 | 41,35 | 42,40 | 44,24 | 32947,78 | 73,23 | 9,15 |  |
|  |  | 27 | 10390,05 | 37,46 | 39,42 | 41,45 | 26441,54 | 46,69 | 7,34 |  |
|  |  | 32 | 4786,58 | 34,15 | 37,52 | 39,48 | 22736,82 | 34,18 | 6,32 |  |
|  |  | 37 | 2333,91 | 31,25 | 35,95 | 37,85 | 20185,58 | 29,44 | 5,61 |  |
|  | baolei_1920x1080_60Hz_8bit_420_02 | 22 | 16250,48 | 43,28 | 44,42 | 44,40 | 31290,80 | 58,00 | 8,69 |  |
|  |  | 27 | 7018,48 | 39,42 | 41,16 | 41,19 | 25964,40 | 39,93 | 7,21 |  |
|  |  | 32 | 3436,90 | 36,01 | 38,96 | 39,04 | 22699,53 | 36,54 | 6,31 |  |
|  |  | 37 | 1814,61 | 32,88 | 37,15 | 37,33 | 20810,11 | 29,93 | 5,78 |  |
|  | jianling_1920x1080_60Hz_8bit_420_02 | 22 | 76393,36 | 39,03 | 40,74 | 42,34 | 46484,95 | 96,59 | 12,91 |  |
|  |  | 27 | 33676,64 | 34,10 | 37,78 | 39,78 | 36649,35 | 72,43 | 10,18 |  |
|  |  | 32 | 13747,03 | 30,25 | 36,11 | 38,18 | 28525,70 | 49,04 | 7,92 |  |
|  |  | 37 | 4930,92 | 27,42 | 34,78 | 36,75 | 23511,75 | 35,51 | 6,53 |  |
|  | jianling_1920x1080_60Hz_8bit_420_04 | 22 | 12652,66 | 43,63 | 48,64 | 48,25 | 25792,67 | 44,04 | 7,16 |  |
|  |  | 27 | 5574,06 | 39,92 | 45,86 | 45,32 | 21447,80 | 31,03 | 5,96 |  |
|  |  | 32 | 2444,80 | 36,48 | 43,61 | 43,10 | 18593,65 | 25,64 | 5,16 |  |
|  |  | 37 | 1073,39 | 33,36 | 41,43 | 40,95 | 17618,91 | 26,03 | 4,89 |  |
| Class Gaming | baolei_4096x2160_60Hz_8bit_420_01 | 22 | 43219,47 | 46,20 | 44,81 | 45,46 | 132042,14 | 185,74 | 36,68 |  |
| 4k Tencent |  | 27 | 20274,77 | 42,50 | 41,58 | 42,31 | 116609,10 | 155,78 | 32,39 |  |
|  |  | 32 | 11042,00 | 39,13 | 39,50 | 40,22 | 103641,31 | 139,11 | 28,79 |  |
|  |  | 37 | 6399,08 | 36,06 | 37,75 | 38,48 | 94816,27 | 126,81 | 26,34 |  |
|  | baolei_4096x2160_60Hz_8bit_420_02 | 22 | 45957,09 | 45,82 | 44,90 | 46,94 | 138692,60 | 206,31 | 38,53 |  |
|  |  | 27 | 20903,82 | 41,88 | 41,58 | 44,11 | 119039,14 | 174,59 | 33,07 |  |
|  |  | 32 | 10970,47 | 38,46 | 39,53 | 42,29 | 102100,55 | 138,44 | 28,36 |  |
|  |  | 37 | 6179,24 | 35,44 | 37,79 | 40,74 | 90775,56 | 129,64 | 25,22 |  |
| Class Gaming | CSGO_1920x1080_60Hz_8bit_P420 | 22 | 14193,62 | 45,66 | 51,42 | 53,36 | 25759,11 | 42,50 | 7,16 |  |
| HD |  | 27 | 5307,95 | 42,25 | 48,81 | 51,21 | 21820,32 | 31,75 | 6,06 |  |
| DERF_TWITCH |  | 32 | 2259,69 | 38,95 | 46,86 | 49,67 | 19176,51 | 31,64 | 5,33 |  |
|  |  | 37 | 1057,86 | 35,63 | 45,40 | 48,74 | 17639,48 | 22,57 | 4,90 |  |
|  | MINECRAFT_1920x1080_60Hz_8bit_P420 | 22 | 51161,82 | 41,74 | 43,63 | 44,57 | 42359,32 | 116,53 | 11,77 |  |
|  |  | 27 | 20289,52 | 36,29 | 40,78 | 42,04 | 32096,67 | 71,98 | 8,92 |  |
|  |  | 32 | 7835,62 | 32,75 | 39,05 | 40,42 | 26364,83 | 55,76 | 7,32 |  |
|  |  | 37 | 3418,98 | 30,24 | 37,69 | 39,15 | 22651,55 | 35,52 | 6,29 |  |
|  | STARCRAFT_1920x1080_60Hz_8bit_P420 | 22 | 7576,08 | 43,05 | 47,33 | 47,97 | 19157,29 | 30,00 | 5,32 |  |
|  |  | 27 | 3160,07 | 39,24 | 43,94 | 44,59 | 16976,84 | 25,47 | 4,72 |  |
|  |  | 32 | 1263,76 | 35,71 | 41,46 | 41,94 | 16047,50 | 24,06 | 4,46 |  |
|  |  | 37 | 520,51 | 32,54 | 39,59 | 39,79 | 15242,12 | 21,40 | 4,23 |  |

Low delay P results:

|  |  |  | Reference |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps | Y psnr | U psnr | V psnr | Enc T [s] | Dec T [s] | Enc T [h] |
|  | HeroesOfTheStorm_1920x1080_30Hz_8bit_P420_Part1 | 22 | 4827,82 | 42,64 | 43,96 | 44,02 | 7626,53 | 17,13 | 2,12 |
|  |  | 27 | 2240,28 | 38,96 | 41,10 | 41,21 | 6795,09 | 13,93 | 1,89 |
|  |  | 32 | 1040,98 | 35,68 | 39,16 | 39,24 | 6295,15 | 15,69 | 1,75 |
|  |  | 37 | 480,01 | 32,77 | 37,44 | 37,51 | 5827,08 | 14,37 | 1,62 |
| Class Gaming Kingston | ProjectCars_1920x1080_30Hz_8bit_P420_Part1 | 22 | 12866,93 | 41,39 | 48,32 | 48,47 | 14801,91 | 31,00 | 4,11 |
|  |  | 27 | 4734,97 | 38,02 | 45,99 | 46,16 | 11313,66 | 22,28 | 3,14 |
|  |  | 32 | 1974,78 | 35,23 | 44,42 | 44,50 | 9325,22 | 18,51 | 2,59 |
|  |  | 37 | 887,11 | 32,80 | 43,05 | 43,00 | 7642,03 | 15,84 | 2,12 |
|  | WoW_1920x1080_30Hz_8bit_P420_Part1 | 22 | 10888,18 | 41,00 | 44,81 | 47,30 | 13095,04 | 27,96 | 3,64 |
|  |  | 27 | 4402,83 | 37,04 | 42,21 | 44,93 | 10129,91 | 19,24 | 2,81 |
|  |  | 32 | 1879,05 | 33,96 | 40,37 | 43,23 | 8462,60 | 15,98 | 2,35 |
|  |  | 37 | 805,43 | 31,41 | 38,71 | 41,82 | 7097,32 | 13,95 | 1,97 |
| Class Gaming | AOV5_1920x1080_60Hz_8bit_420 | 22 | 14508,07 | 42,02 | 43,64 | 44,31 | 21285,50 | 45,41 | 5,91 |
| HD Tencent |  | 27 | 5696,69 | 38,39 | 40,67 | 41,13 | 17346,95 | 40,57 | 4,82 |
|  |  | 32 | 2579,17 | 35,34 | 38,61 | 38,91 | 14463,03 | 26,98 | 4,02 |
|  |  | 37 | 1285,81 | 32,61 | 36,86 | 37,01 | 13060,14 | 24,08 | 3,63 |
|  | baolei_1920x1080_60Hz_8bit_420_01 | 22 | 25755,58 | 41,24 | 42,36 | 44,21 | 27651,59 | 73,24 | 7,68 |
|  |  | 27 | 10653,92 | 37,37 | 39,35 | 41,38 | 21764,01 | 45,89 | 6,05 |
|  |  | 32 | 4901,72 | 34,09 | 37,43 | 39,41 | 18018,72 | 36,06 | 5,01 |
|  |  | 37 | 2389,60 | 31,20 | 35,86 | 37,78 | 15366,25 | 27,64 | 4,27 |
|  | baolei_1920x1080_60Hz_8bit_420_02 | 22 | 16965,38 | 43,15 | 44,34 | 44,34 | 25910,82 | 54,10 | 7,20 |
|  |  | 27 | 7249,21 | 39,29 | 41,09 | 41,11 | 20809,25 | 42,64 | 5,78 |
|  |  | 32 | 3534,39 | 35,89 | 38,86 | 38,95 | 17472,25 | 33,07 | 4,85 |
|  |  | 37 | 1857,19 | 32,81 | 37,05 | 37,23 | 15571,13 | 32,86 | 4,33 |
|  | jianling_1920x1080_60Hz_8bit_420_02 | 22 | 77826,59 | 38,99 | 40,70 | 42,29 | 40528,25 | 93,56 | 11,26 |
|  |  | 27 | 34278,93 | 34,08 | 37,72 | 39,72 | 30552,96 | 70,76 | 8,49 |
|  |  | 32 | 14114,69 | 30,25 | 36,04 | 38,10 | 23273,94 | 51,84 | 6,46 |
|  |  | 37 | 5139,54 | 27,37 | 34,69 | 36,65 | 18218,79 | 37,30 | 5,06 |
|  | jianling_1920x1080_60Hz_8bit_420_04 | 22 | 12880,16 | 43,56 | 48,55 | 48,18 | 21344,39 | 47,25 | 5,93 |
|  |  | 27 | 5691,42 | 39,84 | 45,77 | 45,26 | 17056,01 | 30,79 | 4,74 |
|  |  | 32 | 2499,10 | 36,41 | 43,54 | 43,02 | 14646,01 | 27,09 | 4,07 |
|  |  | 37 | 1095,99 | 33,28 | 41,35 | 40,90 | 12715,84 | 23,03 | 3,53 |
| Class Gaming | baolei_4096x2160_60Hz_8bit_420_01 | 22 | 45481,63 | 45,99 | 44,70 | 45,35 | 113189,67 | 188,58 | 31,44 |
| 4k Tencent |  | 27 | 21173,82 | 42,28 | 41,44 | 42,16 | 93316,09 | 178,55 | 25,92 |
|  |  | 32 | 11445,67 | 38,92 | 39,33 | 40,07 | 85264,76 | 158,06 | 23,68 |
|  |  | 37 | 6580,56 | 35,88 | 37,58 | 38,31 | 75217,28 | 118,43 | 20,89 |
|  | baolei_4096x2160_60Hz_8bit_420_02 | 22 | 47864,79 | 45,61 | 44,81 | 46,84 | 120161,40 | 237,73 | 33,38 |
|  |  | 27 | 21656,46 | 41,66 | 41,48 | 43,99 | 98542,35 | 162,60 | 27,37 |
|  |  | 32 | 11328,11 | 38,26 | 39,43 | 42,16 | 85066,87 | 148,27 | 23,63 |
|  |  | 37 | 6346,34 | 35,28 | 37,67 | 40,60 | 73084,47 | 144,43 | 20,30 |
| Class Gaming | CSGO_1920x1080_60Hz_8bit_P420 | 22 | 14434,80 | 45,62 | 51,43 | 53,42 | 20591,17 | 47,97 | 5,72 |
| HD |  | 27 | 5378,21 | 42,20 | 48,78 | 51,18 | 16787,51 | 29,47 | 4,66 |
| DERF_TWITCH |  | 32 | 2287,30 | 38,88 | 46,81 | 49,65 | 15055,67 | 26,91 | 4,18 |
|  |  | 37 | 1071,41 | 35,57 | 45,35 | 48,71 | 13444,99 | 24,29 | 3,73 |
|  | MINECRAFT_1920x1080_60Hz_8bit_P420 | 22 | 51399,15 | 41,77 | 43,61 | 44,54 | 34882,41 | 82,63 | 9,69 |
|  |  | 27 | 20613,08 | 36,33 | 40,74 | 41,98 | 26513,69 | 59,48 | 7,36 |
|  |  | 32 | 7933,13 | 32,77 | 39,01 | 40,35 | 20555,73 | 43,93 | 5,71 |
|  |  | 37 | 3503,46 | 30,25 | 37,66 | 39,06 | 17374,04 | 44,14 | 4,83 |
|  | STARCRAFT_1920x1080_60Hz_8bit_P420 | 22 | 7866,39 | 42,95 | 47,29 | 47,96 | 14924,29 | 29,59 | 4,15 |
|  |  | 27 | 3272,86 | 39,17 | 43,87 | 44,55 | 12751,82 | 25,23 | 3,54 |
|  |  | 32 | 1310,37 | 35,66 | 41,43 | 41,92 | 11606,36 | 24,34 | 3,22 |
|  |  | 37 | 533,54 | 32,51 | 39,54 | 39,73 | 11166,28 | 21,75 | 3,10 |

Low Delay B with CRA=1

|  |  |  | Reference |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps | Y psnr | U psnr | V psnr | Enc T [s] | Dec T [s] | Enc T [h] |
|  | HeroesOfTheStorm_1920x1080_30Hz_8bit_P420_Part1 | 22 | 6333,00 | 43,04 | 44,24 | 44,31 | 9079,85 | 18,75 | 2,52 |
|  |  | 27 | 3117,30 | 39,26 | 41,33 | 41,43 | 8119,17 | 14,15 | 2,26 |
| Class Gaming Kingston |  | 32 | 1549,86 | 35,92 | 39,36 | 39,45 | 7793,26 | 15,20 | 2,16 |
|  |  | 37 | 766,59 | 32,99 | 37,52 | 37,58 | 7237,21 | 11,40 | 2,01 |
|  | ProjectCars_1920x1080_30Hz_8bit_P420_Part1 | 22 | 12901,20 | 41,52 | 48,57 | 48,69 | 16555,58 | 37,58 | 4,60 |
|  |  | 27 | 4849,25 | 38,15 | 46,22 | 46,33 | 13018,93 | 22,49 | 3,62 |
|  |  | 32 | 2076,66 | 35,37 | 44,52 | 44,60 | 10885,96 | 18,15 | 3,02 |
|  |  | 37 | 962,34 | 32,94 | 42,91 | 42,83 | 9372,25 | 16,22 | 2,60 |
|  | WoW_1920x1080_30Hz_8bit_P420_Part1 | 22 | 11932,50 | 41,21 | 45,04 | 47,52 | 14600,09 | 27,36 | 4,06 |
|  |  | 27 | 5111,02 | 37,24 | 42,44 | 45,15 | 11777,95 | 19,03 | 3,27 |
|  |  | 32 | 2301,71 | 34,14 | 40,59 | 43,47 | 9908,78 | 15,48 | 2,75 |
|  |  | 37 | 1041,53 | 31,57 | 38,79 | 41,87 | 8695,57 | 13,28 | 2,42 |
| Class Gaming | AOV5_1920x1080_60Hz_8bit_420 | 22 | 15384,95 | 42,29 | 43,93 | 44,58 | 25893,40 | 64,42 | 7,19 |
| HD Tencent |  | 27 | 6362,82 | 38,63 | 40,93 | 41,35 | 21035,71 | 38,50 | 5,84 |
|  |  | 32 | 3009,49 | 35,55 | 38,86 | 39,14 | 18430,04 | 33,66 | 5,12 |
|  |  | 37 | 1533,69 | 32,80 | 36,97 | 37,14 | 16849,83 | 27,66 | 4,68 |
|  | baolei_1920x1080_60Hz_8bit_420_01 | 22 | 26011,17 | 41,47 | 42,60 | 44,46 | 33017,94 | 98,07 | 9,17 |
|  |  | 27 | 11214,24 | 37,59 | 39,66 | 41,71 | 25612,84 | 53,28 | 7,11 |
|  |  | 32 | 5316,81 | 34,32 | 37,76 | 39,78 | 22082,52 | 55,88 | 6,13 |
|  |  | 37 | 2650,21 | 31,46 | 35,99 | 37,96 | 19420,40 | 31,30 | 5,39 |
|  | baolei_1920x1080_60Hz_8bit_420_02 | 22 | 17200,24 | 43,48 | 44,66 | 44,65 | 30386,54 | 57,55 | 8,44 |
|  |  | 27 | 7657,09 | 39,64 | 41,41 | 41,45 | 24798,69 | 43,12 | 6,89 |
|  |  | 32 | 3862,36 | 36,26 | 39,26 | 39,36 | 21786,51 | 48,61 | 6,05 |
|  |  | 37 | 2083,72 | 33,18 | 37,26 | 37,46 | 19559,87 | 31,93 | 5,43 |
|  | jianling_1920x1080_60Hz_8bit_420_02 | 22 | 77855,29 | 39,10 | 40,81 | 42,43 | 46370,24 | 104,13 | 12,88 |
|  |  | 27 | 34830,82 | 34,16 | 37,86 | 39,87 | 35295,28 | 71,97 | 9,80 |
|  |  | 32 | 14562,53 | 30,31 | 36,19 | 38,27 | 28671,00 | 65,82 | 7,96 |
|  |  | 37 | 5437,50 | 27,46 | 34,78 | 36,75 | 22758,36 | 38,01 | 6,32 |
|  | jianling_1920x1080_60Hz_8bit_420_04 | 22 | 13561,85 | 43,77 | 48,78 | 48,41 | 25733,35 | 48,91 | 7,15 |
|  |  | 27 | 6225,10 | 40,02 | 45,92 | 45,46 | 20637,91 | 34,90 | 5,73 |
|  |  | 32 | 2898,17 | 36,59 | 43,65 | 43,23 | 18232,48 | 30,96 | 5,06 |
|  |  | 37 | 1371,34 | 33,49 | 41,41 | 41,01 | 16558,79 | 32,79 | 4,60 |
| Class Gaming | baolei_4096x2160_60Hz_8bit_420_01 | 22 | 45516,37 | 46,30 | 44,93 | 45,57 | 130901,55 | 194,15 | 36,36 |
| 4k Tencent |  | 27 | 21759,76 | 42,60 | 41,72 | 42,43 | 111801,17 | 156,60 | 31,06 |
|  |  | 32 | 12010,40 | 39,24 | 39,67 | 40,39 | 98924,72 | 154,10 | 27,48 |
|  |  | 37 | 6980,61 | 36,20 | 37,83 | 38,56 | 89712,99 | 128,82 | 24,92 |
|  | baolei_4096x2160_60Hz_8bit_420_02 | 22 | 47694,54 | 45,89 | 45,01 | 47,06 | 131256,43 | 208,17 | 36,46 |
|  |  | 27 | 22003,46 | 41,94 | 41,68 | 44,23 | 112511,81 | 165,21 | 31,25 |
|  |  | 32 | 11668,84 | 38,52 | 39,64 | 42,39 | 99791,19 | 166,99 | 27,72 |
|  |  | 37 | 6611,04 | 35,51 | 37,84 | 40,74 | 90262,76 | 129,34 | 25,07 |
| Class Gaming | CSGO_1920x1080_60Hz_8bit_P420 | 22 | 14717,91 | 45,87 | 51,96 | 54,05 | 25045,75 | 42,44 | 6,96 |
| HD |  | 27 | 5643,09 | 42,41 | 49,25 | 51,77 | 21268,04 | 39,69 | 5,91 |
| DERF_TWITCH |  | 32 | 2465,65 | 39,08 | 47,27 | 50,07 | 18984,54 | 33,54 | 5,27 |
|  |  | 37 | 1173,96 | 35,79 | 45,44 | 48,72 | 17050,87 | 27,09 | 4,74 |
|  | MINECRAFT_1920x1080_60Hz_8bit_P420 | 22 | 51723,17 | 41,82 | 43,70 | 44,62 | 41459,52 | 103,27 | 11,52 |
|  |  | 27 | 20850,50 | 36,36 | 40,84 | 42,07 | 32103,25 | 64,88 | 8,92 |
|  |  | 32 | 8211,32 | 32,81 | 39,10 | 40,43 | 25961,38 | 54,12 | 7,21 |
|  |  | 37 | 3607,02 | 30,29 | 37,64 | 39,04 | 21931,18 | 36,97 | 6,09 |
|  | STARCRAFT_1920x1080_60Hz_8bit_P420 | 22 | 8838,14 | 43,76 | 47,88 | 48,57 | 19044,83 | 55,81 | 5,29 |
|  |  | 27 | 3995,44 | 39,91 | 44,50 | 45,17 | 16919,13 | 32,62 | 4,70 |
|  |  | 32 | 1786,75 | 36,39 | 42,04 | 42,52 | 15427,68 | 29,08 | 4,29 |
|  |  | 37 | 818,60 | 33,25 | 39,83 | 40,10 | 14649,09 | 26,69 | 4,07 |

Low Delay-P with CRA=1

|  |  |  | Reference |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QPISlice | kbps | Y psnr | U psnr | V psnr | Enc T [s] | Dec T [s] | Enc T [h] |
| Class Gaming Kingston | HeroesOfTheStorm_1920x1080_30Hz_8bit_P420_Part1 | 22 | 6432,16 | 43,01 | 44,21 | 44,29 | 7131,32 | 17,95 | 1,98 |
|  |  | 27 | 3160,98 | 39,23 | 41,30 | 41,40 | 6265,80 | 14,14 | 1,74 |
|  |  | 32 | 1573,40 | 35,89 | 39,34 | 39,43 | 5769,63 | 12,22 | 1,60 |
|  |  | 37 | 775,31 | 32,95 | 37,50 | 37,55 | 5316,06 | 10,92 | 1,48 |
|  | ProjectCars_1920x1080_30Hz_8bit_P420_Part1 | 22 | 13249,78 | 41,46 | 48,47 | 48,60 | 13956,68 | 40,42 | 3,88 |
|  |  | 27 | 4978,82 | 38,08 | 46,11 | 46,25 | 10741,28 | 23,81 | 2,98 |
|  |  | 32 | 2133,61 | 35,29 | 44,45 | 44,55 | 8381,32 | 17,71 | 2,33 |
|  |  | 37 | 987,83 | 32,88 | 42,85 | 42,76 | 7074,81 | 14,97 | 1,97 |
|  | WoW_1920x1080_30Hz_8bit_P420_Part1 | 22 | 12059,95 | 41,18 | 45,01 | 47,49 | 12263,54 | 29,65 | 3,41 |
|  |  | 27 | 5147,90 | 37,22 | 42,40 | 45,12 | 9371,72 | 22,16 | 2,60 |
|  |  | 32 | 2328,32 | 34,13 | 40,56 | 43,44 | 7716,03 | 18,45 | 2,14 |
|  |  | 37 | 1054,53 | 31,55 | 38,77 | 41,85 | 6510,48 | 12,91 | 1,81 |
| Class Gaming | AOV5_1920x1080_60Hz_8bit_420 | 22 | 15781,16 | 42,27 | 43,88 | 44,53 | 20657,31 | 54,21 | 5,74 |
| HD Tencent |  | 27 | 6491,25 | 38,62 | 40,88 | 41,30 | 16895,19 | 53,88 | 4,69 |
|  |  | 32 | 3057,17 | 35,54 | 38,81 | 39,09 | 14091,23 | 37,85 | 3,91 |
|  |  | 37 | 1556,98 | 32,78 | 36,92 | 37,10 | 12909,09 | 27,77 | 3,59 |
|  | baolei_1920x1080_60Hz_8bit_420_01 | 22 | 26996,76 | 41,36 | 42,57 | 44,43 | 27738,78 | 74,88 | 7,71 |
|  |  | 27 | 11461,54 | 37,52 | 39,61 | 41,66 | 21370,99 | 60,04 | 5,94 |
|  |  | 32 | 5420,80 | 34,27 | 37,71 | 39,74 | 17467,72 | 51,74 | 4,85 |
|  |  | 37 | 2703,41 | 31,43 | 35,96 | 37,93 | 14818,63 | 41,68 | 4,12 |
|  | baolei_1920x1080_60Hz_8bit_420_02 | 22 | 17868,81 | 43,36 | 44,60 | 44,59 | 26457,38 | 59,12 | 7,35 |
|  |  | 27 | 7866,79 | 39,52 | 41,34 | 41,39 | 20808,90 | 61,51 | 5,78 |
|  |  | 32 | 3946,84 | 36,16 | 39,20 | 39,29 | 17163,16 | 34,75 | 4,77 |
|  |  | 37 | 2119,44 | 33,12 | 37,20 | 37,39 | 15337,07 | 35,26 | 4,26 |
|  | jianling_1920x1080_60Hz_8bit_420_02 | 22 | 79273,50 | 39,07 | 40,78 | 42,39 | 40706,55 | 94,97 | 11,31 |
|  |  | 27 | 35402,04 | 34,15 | 37,82 | 39,82 | 30219,74 | 69,76 | 8,39 |
|  |  | 32 | 14907,58 | 30,32 | 36,14 | 38,21 | 23393,46 | 81,07 | 6,50 |
|  |  | 37 | 5630,09 | 27,44 | 34,73 | 36,69 | 18308,82 | 38,01 | 5,09 |
|  | jianling_1920x1080_60Hz_8bit_420_04 | 22 | 13773,06 | 43,74 | 48,73 | 48,37 | 21031,32 | 72,42 | 5,84 |
|  |  | 27 | 6320,45 | 40,00 | 45,87 | 45,42 | 16785,91 | 42,83 | 4,66 |
|  |  | 32 | 2943,40 | 36,56 | 43,61 | 43,19 | 14024,66 | 31,26 | 3,90 |
|  |  | 37 | 1391,31 | 33,46 | 41,36 | 40,97 | 12380,17 | 28,04 | 3,44 |
| Class Gaming | baolei_4096x2160_60Hz_8bit_420_01 | 22 | 47617,75 | 46,12 | 44,83 | 45,48 | 108812,82 | 193,33 | 30,23 |
| 4k Tencent |  | 27 | 22552,68 | 42,41 | 41,59 | 42,31 | 92448,26 | 155,65 | 25,68 |
|  |  | 32 | 12343,67 | 39,07 | 39,53 | 40,26 | 80521,54 | 153,13 | 22,37 |
|  |  | 37 | 7129,43 | 36,05 | 37,69 | 38,43 | 71416,92 | 132,82 | 19,84 |
|  | baolei_4096x2160_60Hz_8bit_420_02 | 22 | 49514,84 | 45,70 | 44,93 | 46,97 | 111781,62 | 202,12 | 31,05 |
|  |  | 27 | 22692,48 | 41,75 | 41,60 | 44,13 | 92948,52 | 179,58 | 25,82 |
|  |  | 32 | 11988,31 | 38,35 | 39,56 | 42,29 | 80481,44 | 149,70 | 22,36 |
|  |  | 37 | 6752,59 | 35,37 | 37,75 | 40,64 | 70833,56 | 126,40 | 19,68 |
| Class Gaming | CSGO_1920x1080_60Hz_8bit_P420 | 22 | 14966,11 | 45,85 | 51,92 | 54,04 | 20799,00 | 76,19 | 5,78 |
| HD |  | 27 | 5710,74 | 42,38 | 49,23 | 51,75 | 16437,13 | 45,26 | 4,57 |
| DERF_TWITCH |  | 32 | 2489,14 | 39,04 | 47,27 | 50,04 | 14534,32 | 47,87 | 4,04 |
|  |  | 37 | 1186,62 | 35,76 | 45,38 | 48,71 | 13127,03 | 25,04 | 3,65 |
|  | MINECRAFT_1920x1080_60Hz_8bit_P420 | 22 | 51974,08 | 41,85 | 43,68 | 44,59 | 35445,28 | 81,68 | 9,85 |
|  |  | 27 | 21160,56 | 36,39 | 40,80 | 42,02 | 26378,03 | 73,27 | 7,33 |
|  |  | 32 | 8311,15 | 32,83 | 39,06 | 40,39 | 20097,09 | 42,62 | 5,58 |
|  |  | 37 | 3691,97 | 30,31 | 37,60 | 38,99 | 16888,52 | 53,63 | 4,69 |
|  | STARCRAFT_1920x1080_60Hz_8bit_P420 | 22 | 9120,01 | 43,72 | 47,85 | 48,55 | 14924,17 | 34,64 | 4,15 |
|  |  | 27 | 4101,71 | 39,88 | 44,47 | 45,15 | 12728,08 | 45,78 | 3,54 |
|  |  | 32 | 1831,13 | 36,37 | 42,01 | 42,50 | 11490,95 | 31,98 | 3,19 |
|  |  | 37 | 830,15 | 33,23 | 39,81 | 40,09 | 10832,37 | 27,99 | 3,01 |