Agenda item: 	4.1

Source: 	Qualcomm Incorporated

Title: 	[FS_5GSTAR] Architecture Considerations

Document for	Discussion and Agreement

# Introduction

In SP-200399, the new study on “Support of 5G Glass-type AR/MR Devices” has been agreed.

The objective of this study item includes the following:

- Provide formal definitions for the functional structures of AR glasses, classified as device types of XR5G-A4 (standalone) and XR5G-A2, A5 (wirelessly tethered) in TR 26.928, including their capabilities and constraints with respect to communication, computing and graphics processing, tracking, sensors, display, and power consumption

NOTE 1: Device type of XR5G-A3 (video see-through HMD) are not the primary scope of this study, but are not excluded per se.

- Describe key use cases for AR services over 5G based on those in Table 4.10 of TR 26.928, map each use case to suitable device types, and define relevant processing functions and reference architectures for the devices that are required to support the use cases, taking into account the constraints and capabilities identified under the first objective.

NOTE 2: The preferred cases will be those capable of delivering experiences previous or existing media services could not support, e.g., those requiring features for AR/MR 3D download, communication or conversational (related to use cases 1, 2, 7, 8, 11, 15, 16, 17, 19, 20, or 23). The study also allows new use cases not covered in TR 26.928.

- Describe the architecture for media flow relevant to the use cases identified in the second objective. Identify media (exchange) formats and profiles relevant to the use cases identified in the second objective that can be processed on AR platforms as defined under the first objective. Identify where media processing functions occur and which type of media formats are used for exchange between these elements to the described architecture.

NOTE 3: Exchange formats include both, formats and signals consumed on AR devices (e.g. overlays, scenes, animations) as well as those that are generated (and shared) on/by those (for example to support spatial localization, object recognition and tracking).

- Identify necessary content delivery transport protocols and capability exchange mechanisms, as well as suitable 5G system functionalities (e.g., device, edge, network) and QoS (including radio access and core network technologies) required for the use cases. If existing technologies and protocols cannot serve the cases sufficiently, describe the necessary functionalities and features.

- Identify key performance indicators and quality of experience factors (such as immersiveness, presence, localization, and world tracking accuracy) for the AR use cases based on the initial considerations in TR26.928, clause 4.2 with additional emphasis on the use cases, device platforms and exchanges formats for AR as identified in earlier objectives.

NOTE 4: As there can be several approaches for the optical implementation of displays, this study does not prefer any display types, nor will it define any detailed criteria or quality requirements.

- Identify relevant radio and system parameters (required bitrates, latencies, loss rates, range, etc.) to support the identified AR use cases and the required QoE, in particular when the AR device is connected via 5G sidelink interfaces.

NOTE 5: Depending on the use cases, other types of peripheral devices, such as external cameras and sensors, can also be connected to the UEs in a similar fashion.

- For each of the identified use cases and AR device platforms, provide a detailed overall power analysis for media AR related processing and communication building on the information in TR26.928, clause 4.8. Different design options should be considered, for example if media processing is carried out on the device, on a puck/smartphone or in the edge/cloud. It is recommended to contact relevant 3GPP WGs, in particular, and RAN4, on expected modem power consumption for certain use cases.

NOTE 6: Support other 3GPP WGs and other SDOs on relevant aspects related to their work and responsibilities for these new design options and form factors on a need basis, in particular when requested for processing functions in scope of SA4.

- Identify potential areas for normative work as the next phase and communicate with other 3GPP WGs on relevant aspects related to the study.

This contribution primarily addresses objective 2, 3 and 7.

# Glass Types

For AR, we believe that different types of glass design factors need to be taken into account. A good overview is provided here (Thanks to Ralf Schaefer): https://www.softwaretestinghelp.com/best-augmented-reality-glasses/ (see copyrights of images there).

1) Heads Up Displays Or HUDs

It is a transparent display that presents data to the user’s screen in front of their eyes, hence the user need not look away from their usual viewpoints. Additional data displayed could be routes, location, plans, black spots, chats with other device users, and even 3D images and videos.

2) Holographic Displays

Augmented Reality glasses based on this technology display 3D holograms overlaid on the real world where the user is located to render a mixed reality experience to the user. The hologram image is generated using light diffraction techniques.

Examples include Microsoft HoloLens.

The below image explains the Holographic display headset on use:

#3) Smart Glasses

AR smart glasses are wearable computer-capable glasses that add extra information, ideally 3D images and information such as animations and videos, to the user’s real-world scenes by overlaying the computer-generated or digital information on the user’s real-world. It can retrieve information from computers, smartphones, or other devices and can support WiFi, Bluetooth, and GPS.

Examples include Google Glass Explorer Edition and Vuzix M100. North Focals

Google Glass Explorer Edition

#4) Handheld

Handheld AR is using handheld devices such as smartphones on which AR apps are installed to access and apply AR. They contrast with the AR headsets that are worn on the head and are easy to use and cheap.

Examples include using your smartphone to try out virtual models of furniture on your house floor, on the IKEA app, or playing Pokemon Go on an AR app on your smartphone.

Further types include:

Smartphone AR headsets: These AR headsets employ the smartphone to generate AR environments. A smartphone can be slotted on a holder on the wearable AR headset that also has visors through which the user can view the real-world even as smartphone generated projections are overlaid above.

Smartphone-based AR

While we believe that each of the device may be interesting, we consider primarily the focus in priority order as follows:

- Smart Glasses

- Holographic Displays

- Smartphone-based AR

- Heads-up Display

An idea for high-level specifications for a Smart Glass is provided below:

| High level Tech Specs |  |
| --- | --- |
| Battery | Specs: For 18-20 hours of battery life based on average use |
| Sensors | 9-axis IMU, Ambient Light Sensor, Proximity sensor |
| Weight | 65 - 75 grams (depending on size, style, and lenses) |
| Connectivity | Be able to connect to internet and peripheral devices |
| Display | Dual see-through display (minimum 1080p 60fps, but likely more) |
| Audio | Microphone (speech-to-text capable), Directional speaker |
| Colors | Various styles including prescription lenses. |
| Features | Hands free experience, Voice dictation, virtual objects interaction, See through display and heads-up display |

We believe it is important to clearly differentiate the device types above when discussing architectures, signal flows and so on.

# Smart Glasses – Simple Use Cases

We believe that for Smart glasses, simple use cases should matter. Examples are below.

- Room Design and virtual shopping

- Games

- Navigation

- AR Video Calls

For each of the simple use cases a clear understanding of the needed functions is required. A list of typical smart glass functions are provided in the following:

- Content Processing:

  - Video Capturing

  - Video decoding,

  - Video encoding

  - Composition

  - Rendering

- Display: Fiber Scanning Display (FSD) world locked

  - https://kguttag.com/2018/01/06/magic-leap-fiber-scanning-display-fsd-the-big-con-at-the-core/

- Tracking

  - User 6DOF

  - Eye

  - Hand

- Interaction:

  - Gestures

  - Hand Tracking

- Scene Understanding:

  - Object Detection

  - 3DR-based Plane Detection

- Communication:

  - 5G, WiFi

This also allows to do an idea of budget power analysis per use case.

# Architecture

An architecture that relies on splitting functions across the network is shown below. As steps for each of the use cases, the following is needed

- Analysis of different split options, with different communication links, trade off latency and power KPIs, proposals to improve existing systems

- Media-Centric Processing Options

- End to end use-case latencies computed for different system configuration

- Extending analysis to 5G link

# Device Type 1

Editor’s Note:

- It should be clarified that the 5G Modem is in the AR glass

- We need different architectures as a refinement of the above architecture

  - There are different types of connectivity

    - Uu to gNB

    - FR (sidelink) – device to device

  - There are different glass types

    - Design glasses: AR glasses designed a regular glasses not weighing more than 70 grams (Michael Abrash, Oculus Connect 5 in 2018, the human head can comfortably carry significantly more weight than those 70g - if that weight is well distributed.)

      - As an example, in such glass types rendering can not be done in the device. Available power is very different.

    - Enterprise glasses: hololense like glasses for which design is less relevant.

# Proposal

Based on the discussion in this document, it is proposed:

- Provide a clear definition for the 4 different AR Glass types.

- Focus on smart glasses and possibly holographics displays

- Develop different architectures for smart glasses for which the 5G modem is included in the smart glass taking into account

  - Uu based communication

  - FR2 based communication

- Identify power budgets for each of the use cases and identify exchange formats, KPIs, etc.

- Use this analysis to progress other objectives