3GPP TSG SA WG4 #112-e	S4aV200650

23rd February 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.955 | CR | <CR#> | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 4K-HDR Test Material Selection for FS_5GVideo |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ATEME, InterDigital, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA4 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_5GVideo |  |  |  |  |  | Date: |  |  | 2021-02-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | No test sequences are currently selected for 4KTV material |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test material selection process applied on SDR is applied to HDR case. HDR test material is provided. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No HDR test material for 4KTV scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

===== CHANGE 1 =====

### 6.3.7	Test Sequences

The Annex C.3 describes in details the selection process conducted to build the test sequences considered for this scenario, the outcome of this process is reported in the tables below.

Table 6.3-3 : Selected SDR test material for 4K-TV scenario

| Name | Resolution | Frame-rate | ColorGamut | Number of Frames / Duration | Scene Cut |
| --- | --- | --- | --- | --- | --- |
| 4EVER_Brest_Sedof | 3840 x 2160 | 60 | BT.2020 | 600 | 0 |
| Rain_fruits | 3840 x 2160 | 50 | BT.709 | 600 | 0 |
| ParkJoy | 3840 x 2160 | 50 | BT.709 | 500 | 0 |
| Indoor Soccer | 3840 x 2160 | 23.98 | BT.709 | 17s | 4 |
| Netflix_ToddlerFountain | 4096 x 2160 | 59.94 | BT.709 | 1199 | 0 |
| Netflix_TunnelFlag | 4096 x 2160 | 59.94 | BT.709 | 600 | 0 |
| RiverBank | 3840 x 2160 | 50 | BT.709 | 12s | 0 |
| Netflix_Boat | 4096 x 2160 | 59.94 | BT.709 | 300 | 0 |

Table 6.3-4 : Selected HDR test material for 4K-TV scenario

| Name | Resolution | Frame-rate | ColorGamut | Number of Frames / Duration | Scene Cut |
| --- | --- | --- | --- | --- | --- |
| LifeUntouched1 | 4096 x 2160 | 59.94 | P3-D65 | 8s | 0 |
| meridian_aom_22412-22738 | 3840 x 2160 | 59.94 | P3-D65 | 324 | 0 |
| cosmos_aom_12149-12330 | 3840 x 2160 | 24 | P3-D65 | 181 | 0 |
| sol_levante_aom_2268-2412 | 3840 x 2160 | 24 | P3-D65 | 144 | 0 |
| sparks_aom_2024-2455 | 4096 x 2160 | 59.94 | P3-D65 | 431 | 0 |
| sparks_aom_5764-6024 | 4096 x 2160 | 59.94 | P3-D65 | 260 | 0 |
| nocturne_aom_27740-28109 | 3840 x 2160 | 60 | P3-D65 | 369 | 0 |

===== End of CHANGE 1 =====

===== CHANGE 2 =====

#### C.3.1.2.3	SDR Sequence Selection

The first step consists in classifying the sequences in two categories : easy and challenging. This reflects the complexity of the sequences from an encoding perspective. Based on the R-D data, the QP points covering the application range [10-40]Mbps are identified. For the lower (10Mbps) and higher (40Mbps) boundaries, a linear interpolation is achieved between the two closest QP points to estimate the distortion at the boundaries: D_10 and D_40. If the distortion difference D_DIFF=(D_40-D_10) is less than or equal to a threshold Thr=1.5, then the sequence is classified as follows:

- Low-dynamic: the PSNR dynamic on the application bitrate range is limited

- High-dynamic: the PSNR dynamic on the application bitrate range is high

The results of this first classification step is reported in the table below.

Table C.3-8 : SDR sequence classification based on R-D information in the [10-40] Mbps range, Thr=1.5

|  | Name | D_10 | D_40 | D_DIFF | Dynamic |
| --- | --- | --- | --- | --- | --- |
| JVET | Tango2 | 40,33 | 41,44 | 1,12 | Low |
|  | FoodMarket4 | 40,28 | 42,37 | 2,09 | High |
|  | Campfire | 35,06 | 37,29 | 2,23 | High |
|  | Catrobots1 | 38,36 | 39,85 | 1,50 | Low |
|  | Daylightroad2 | 37,42 | 38,61 | 1,19 | Low |
|  | Parkrunning3 | 31,17 | 33,65 | 2,48 | High |
| UVG | CityAlley | 43,37 | 43,49 | 0,12 | Low |
|  | FlowerFocus | 41,13 | 41,33 | 0,20 | Low |
|  | FlowerKids | 40,92 | 42,78 | 1,86 | High |
|  | FlowerPan | 39,60 | 39,85 | 0,25 | Low |
|  | RaceNight | 37,82 | 38,62 | 0,79 | Low |
|  | RiverBank | 36,62 | 38,84 | 2,22 | High |
|  | SunBath | 43,85 | 46,98 | 3,13 | High |
|  | Twilight | 42,92 | 43,24 | 0,33 | Low |
| Netflix | Netflix_Aerial | 38,62 | 41,00 | 2,37 | High |
|  | Netflix_DinnerScene | 42,85 | 43,08 | 0,23 | Low |
|  | Netflix_RollerCoaster | 42,61 | 44,73 | 2,11 | High |
|  | Netflix_DrivingPOV | 37,27 | 39,54 | 2,27 | High |
|  | Netflix_BarScene | 41,39 | 41,62 | 0,23 | Low |
|  | Netflix_ToddlerFountain | 33,56 | 36,01 | 2,45 | High |
|  | Netflix_Dancers | 41,96 | 42,14 | 0,18 | Low |
|  | Netflix_PierSeaside | 41,05 | 43,10 | 2,06 | High |
|  | Netflix_WindAndNature | 43,35 | 45,09 | 1,75 | High |
|  | Netflix_Boat | 35,27 | 38,64 | 3,37 | High |
|  | Netflix_FoodMarket | 37,62 | 38,95 | 1,32 | Low |
|  | Netflix_Tango | 40,31 | 41,27 | 0,97 | Low |
|  | Netflix_BoxingPractice | 40,97 | 42,14 | 1,17 | Low |
|  | Netflix_Narrator | 45,33 | 46,36 | 1,03 | Low |
|  | Netflix_TunnelFlag | 39,47 | 42,06 | 2,59 | High |
|  | Netflix_RitualDance | 40,25 | 43,46 | 3,21 | High |
|  | Netflix_FoodMarket2 | 39,11 | 41,27 | 2,16 | High |
|  | Netflix_SquareAndTimelapse | 39,43 | 42,15 | 2,72 | High |
| EBU | Rain_fruits | 40,04 | 41,69 | 1,65 | High |
|  | Lupo_candlelight | 39,81 | 40,01 | 0,20 | Low |
| SVT | CrowdRun | 31,41 | 33,94 | 2,53 | High |
|  | ParkJoy | 29,86 | 32,13 | 2,27 | High |
|  | DucksTakeOff | 29,31 | 30,75 | 1,44 | Low |
|  | IntoTree | 35,06 | 35,45 | 0,39 | Low |
|  | OldTownCross | 33,93 | 34,06 | 0,13 | Low |
| 4ever | 4EVER_Brest_Sedof | 28,66 | 31,42 | 2,76 | High |
|  | 4EVER_Paris_Montmartre | 30,33 | 33,19 | 2,86 | High |
| CableLabs | LiftingOff | 45,01 | 48,31 | 3,30 | High |
|  | Soccer | 43,31 | 44,89 | 1,58 | High |
|  | ElDorado | 39,41 | 44,29 | 4,88 | High |
|  | MomentsOfIntensity | 43,09 | 44,66 | 1,57 | High |

It is observed that 26 sequences are classified as “High” while 19 are “Low”. In order to keep sequences that are relevant for a codec characterization, the low-dynamic sequences are removed. Then, the next step consists in removing sequences having some similarities, using SI-TI information computed in Section C.3.1.2.2. The sequences are represented below in the SI-TI plane with a k-means clustering to help the decision.

Figure C.3-9 : SI-TI clustering for the Challenging  SDR sequences (10 clusters)

In order to remain representative in the sense of sources origin, format, and SI-TI values, the following approach was taken:

- 1 sequence (out of 6) from the C2 cluster (EBU: Rain-fruits)

- 1 sequence (out of 4) from the C3 cluster (CableLabs: Soccer)

- 2 sequences (out of 8) from the C4 cluster (Netfllix: ToddlerFountain, Boat)

- 1 sequence (out of 1)  from the C5 cluster (UVG: Riverbank)

- 1 sequence (out of 2) from the C6 cluster (Netflix: TunnelFlag)

- 1 sequence (out of 1) from the C8 cluster (SVT: ParkJoy)

- 1 sequence (out of 1) from the C10 cluster (4ever: Brest-Sedof)

The following eight sequences are selected:

- 1 From 4ever: Brest-Sedof (3840x2160p 60fps)

- 1 From EBU: RainFruits (3840x2160p 50fps)

- 1 From SVT: ParkJoy (3840x2160p 50fps)

- 1 From CableLabs: Soccer (3840x2160p 23.98fps)

- 3 From Netflix:, TunnelFlag, Boat and Toddlerfountain (4096x2160p 59.94fps)

- 1 From UVG: RiverBank (3840x2160p 50fps)

### C.3.2 	HDR Category

#### C.3.2.1	HDR Candidate Test Sequences

In this Annex, a wide range of  HDR test material is reported. The candidate sequences are extracted from test material used in video processing community. The following sections describe the test material in terms of format characteristics and copyright.

#### C.3.2.1.1	AOM Sequences

The AOM sequences are used for the purpose of AV1 and AV2 development. The sequences are available for download at https://media.xiph.org/video/av2/ and an overview is provided in the table below. The following copyrights are applicable:

- Sparks : http://creativecommons.org/licenses/by-nc-nd/4.0/

- Meridian : https://creativecommons.org/licenses/by-nc-nd/4.0/

- Sol Levante : https://creativecommons.org/licenses/by-nc-nd/4.0/

- Cosmos Laundromat : https://creativecommons.org/licenses/by/3.0/

- Nocturne : https://creativecommons.org/licenses/by-nc-nd/4.0/

Table C.3-9 : AOM test material description

| Name | Resolution | Frame-rate | ColorGamut | TF | duration |
| --- | --- | --- | --- | --- | --- |
| sparks_aom_30-511 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 481 |
| sparks_aom_2024-2455 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 431 |
| sparks_aom_5363-5763 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 400 |
| sparks_aom_5764-6024 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 260 |
| sparks_aom_6026-6502 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 476 |
| sparks_aom_8396-8941 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 545 |
| sparks_aom_9774-10071 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 297 |
| sparks_aom_11198-11570 | 4096 x 2160 | 59.94 | P3-D65 | PQ | 372 |
| meridian_aom_1782-2163 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 381 |
| meridian_aom_11872-12263 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 391 |
| meridian_aom_12264-12745 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 481 |
| meridian_aom_15932-16309 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 377 |
| meridian_aom_20988-21412 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 424 |
| meridian_aom_22412-22738 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 324 |
| meridian_aom_24058-24550 | 3840 x 2160 | 59.94 | P3-D65 | PQ | 492 |
| sol_levante_aom_289-453 | 3840 x 2160 | 24 | P3-D65 | PQ | 164 |
| sol_levante_aom_519-649 | 3840 x 2160 | 24 | P3-D65 | PQ | 130 |
| sol_levante_aom_2268-2412 | 3840 x 2160 | 24 | P3-D65 | PQ | 144 |
| sol_levante_aom_3282-3874 | 3840 x 2160 | 24 | P3-D65 | PQ | 592 |
| sol_levante_aom_4123-4545 | 3840 x 2160 | 24 | P3-D65 | PQ | 422 |
| cosmos_aom_1573-1749 | 3840 x 2160 | 24 | P3-D65 | PQ | 176 |
| cosmos_aom_8686-8826 | 3840 x 2160 | 24 | P3-D65 | PQ | 176 |
| cosmos_aom_9561-9789 | 3840 x 2160 | 24 | P3-D65 | PQ | 228 |
| cosmos_aom_11589-11752 | 3840 x 2160 | 24 | P3-D65 | PQ | 163 |
| cosmos_aom_12025-12075 | 3840 x 2160 | 24 | P3-D65 | PQ | 50 |
| cosmos_aom_12149-12330 | 3840 x 2160 | 24 | P3-D65 | PQ | 181 |
| cosmos_aom_12916-13078 | 3840 x 2160 | 24 | P3-D65 | PQ | 162 |
| cosmos_aom_13446-13649 | 3840 x 2160 | 24 | P3-D65 | PQ | 203 |
| nocturne_aom_2370-2539 | 3840 x 2160 | 60 | P3-D65 | PQ | 169 |
| nocturne_aom_8540-9009 | 3840 x 2160 | 60 | P3-D65 | PQ | 469 |
| nocturne_aom_9010-9349 | 3840 x 2160 | 60 | P3-D65 | PQ | 339 |
| nocturne_aom_17140-17709 | 3840 x 2160 | 60 | P3-D65 | PQ | 569 |
| nocturne_aom_18013-18315 | 3840 x 2160 | 60 | P3-D65 | PQ | 302 |
| nocturne_aom_23820-24322 | 3840 x 2160 | 60 | P3-D65 | PQ | 502 |
| nocturne_aom_27740-28109 | 3840 x 2160 | 60 | P3-D65 | PQ | 369 |
| nocturne_aom_32660-32799 | 3840 x 2160 | 60 | P3-D65 | PQ | 139 |

The clip cosmos_aom_12025-12075 being fifty frames duration, it is removed from the selection process and is not considered. In order to use these sequences for the selection process, a convertion into the appropriate format is conducted using FFmpeg and HDRTools software v0.21. The following convertion is conducted:

Figure C.3-10 : Processing pipeline for AOM sequence conversion

In the first step FFMPEG was used to decode JPEG 2000 sequences and store in uncompressed TIFF format. The following command line was used:

FFMPEG -i $INPUT.j2k -compression_algo raw -pix_fmt rgb48le $OUTPUT.tiff

HDRConvert was used to convert the sequences from P3-D65 RGB 12 bit to Rec. BT.2100 10 bit YCbCr 4:2:0.  HDRConvert config file used for this process is attached to this contribution.

#### C.3.2.1.2	CableLabs Sequences

The CableLabs dataset is composed by one HDR sequence described in the table below. The sequence can be used under the CC BY-NC-ND 3.0 license restriction.

Table C.3-10 : Cable Labs HDR test material description

| Name | Resolution | Frame-rate | ColorGamut | TF | duration |
| --- | --- | --- | --- | --- | --- |
| Life Untouched | 4096 x 2160 | 59.94 | P3-D65 | PQ | 5:20 |

For this sequence, a selection of some critical time intervals have been conducted based on experience on compression. Especially intervals with fast or complex motion, representative of TV contents have been selected. Static pictures, or pictures with artistic intent with few spatial details have been discarded. To remain in the scope of the scenario, the following sections have been selected as final candidates:

- Life Untouched 1 : 0-8s

- Life Untouched 2 : 8-15s

The extraction of subpart from original ProRes sequences is realized with FFmpeg release 4.3.1 using the following commandline:

./ffmpeg.exe -i $IN.mov -vcodec rawvideo -ss $START_TIME -t $DURATION -pix_fmt yuv420p10le $OUT.yuv

Where $IN and $OUT are respectively the input and output sequences, $START_TIME and $DURATION respectively the start time and duration in hh:mm:ss format.

In order to map the color into a BT.2020 color container, HDRTools v2.1 has been used, using the same process as the one described in Section C.3.2.1.1. HDRConvert config file used for this process is attached to this contribution.

#### C.3.2.2	Selected Test Sequences

In order to decide wich sequences should be used in this specification a similar process as the SDR one is used. The following process is conducted:

- Run informative encoding using x265 on the converted candidate sequences

- Run SI-TI metrics computation on the converted candidate sequences

- Run codeword histogram computation on the converted candidate sequences

- Select a diverse datasets based on R-D, SI-TI and codewords distribution.

#### C.3.2.2.1	x265 Encoding

To represent candidate sequences in the R-D plane, fixed QP encodings are carried out for QPs [12,	17,22,27,32,37,42,51] using ffmpeg release 4.3.1, with the following command line:

./ffmpeg.exe -pix_fmt yuv420p10le -s $Wx$H -framerate $FR -i $IN -vcodec libx265 -qp $QP -preset fast -x265-params “psnr=1:ssim=1” $OUT.mp4

Where $W and $W are respectively the sequence width and height, $FR the sequence frame-rate, $QP the selected quantization parameter. $IN and $OUT are respectively the input and output files.

From the obtained log, the overall bitrate and PSNR are extracted to be used for plotting. The encoding result are presented below, all sequences being represented in the targeted [10-40] Mbps range (red box).

Figure C.3-11 : R-D curves for the AOM dataset (Nocturne clips)

Figure C.3-12 : R-D curves for the AOM dataset (Meridian clips)

Figure C.3-13 : R-D curves for the AOM dataset (Cosmos clips)

Figure C.3-14 : R-D curves for the AOM dataset (Sparks clips)

Figure C.3-14 : R-D curves for the AOM dataset (SolLevante clips)

TBD (cablelabs)

Figure C.3-15 : R-D curves for the Cable Labs dataset

#### C.3.2.2.2	SI-TI Metrics

The analysis is based on spatial perceptual information (SI) and temporal perceptual information (TI) metrics calculated as defined in ITU-P.910 recommendation [40]. In order to reduce the bias introduced by scene-cuts in TI metric computation, the TI at scene-cut boundaries are removed from computation.

Figure C.3-16 : SI-TI for the AOM dataset (Sparks clips)

Figure C.3-16 : SI-TI for the AOM dataset (Meridian clips)

Figure C.3-16 : SI-TI for the AOM dataset (Nocturne clips)

Figure C.3-17 : SI-TI for the AOM dataset (Cosmos clips)

TBD (cablelabs)

Figure C.3-18 : SI-TI for the AOM (SolLevante) and CableLabs dataset

#### C.3.2.2.3	Codewords distribution

For the HDR case, the distribution of codewords into the sequence is also taken into account in the decision process. The obtained histograms are provided below.

Figure C.3-19 : Codewords distribution for AOM dataset (Meridian clips)

Figure C.3-20 : Codewords distribution for AOM dataset (Cosmos clips)

Figure C.3-21 : Codewords distribution for AOM dataset (Sparks clips)

Figure C.3-22 : Codewords distribution for AOM dataset (SolLevante clips)

Figure C.3-23 : Codewords distribution for AOM dataset (Nocturne clips)

TBD (cablelabs)

#### Figure C.3-24 : Codewords distribution for CableLabs dataset

#### C.3.1.2.4	HDR Sequence Selection

The first step consists in classifying the sequences in two categories : easy and challenging. This reflects the complexity of the sequences from an encoding perspective. Based on the R-D data, the QP points covering the application range [10-40]Mbps are identified. For the lower (10Mbps) and higher (40Mbps) boundaries, a linear interpolation is achieved between the two closest QP points to estimate the distortion at the boundaries: D_10 and D_40. If the distortion difference D_DIFF=(D_40-D_10) is less than or equal to a threshold Thr=3, then the sequence is classified as follows:

- Low-dynamic: the PSNR dynamic on the application bitrate range is limited

- High-dynamic: the PSNR dynamic on the application bitrate range is high

The results of this first classification step is reported in the table below.

Table C.3-11 : HDR sequence classification based on R-D information in the [10-40] Mbps range, Thr=3

|  | Name | D_10 | D_40 | D_DIFF | Complexity |
| --- | --- | --- | --- | --- | --- |
| Sparks | sparks_aom_30-511 | 55,93 | 58,88 | 2,95 | Low |
|  | sparks_aom_2024-2455 | 38,95 | 43,28 | 4,33 | High |
|  | sparks_aom_5363-5763 | 40,24 | 44,16 | 3,92 | High |
|  | sparks_aom_5764-6024 | 33,45 | 37,29 | 3,84 | High |
|  | sparks_aom_6026-6502 | 34,54 | 38,69 | 4,16 | High |
|  | sparks_aom_8396-8941 | 43,35 | 44,46 | 1,10 | Low |
|  | sparks_aom_9774-10071 | 47,15 | 48,89 | 1,74 | Low |
|  | sparks_aom_11198-11570 | 42,45 | 44,36 | 1,91 | Low |
| Nocturne | nocturne_aom_2370-2539 | 49,81 | 52,14 | 2,33 | Low |
|  | nocturne_aom_8540-9009 | 48,50 | 51,39 | 2,89 | Low |
|  | nocturne_aom_9010-9349 | 48,50 | 51,39 | 2,89 | Low |
|  | nocturne_aom_17140-17709 | 50,83 | 53,05 | 2,23 | Low |
|  | nocturne_aom_18013-18315 | 42,57 | 44,37 | 1,80 | Low |
|  | nocturne_aom_23820-24322 | 50,84 | 55,62 | 4,78 | High |
|  | nocturne_aom_27740-28109 | 40,37 | 45,54 | 5,17 | High |
|  | nocturne_aom_32660-32799 | 45,04 | 50,25 | 5,22 | High |
| Meridian | meridian_aom_1782-2163 | 42,08 | 42,91 | 0,83 | Low |
|  | meridian_aom_11872-12263 | 41,81 | 42,44 | 0,62 | Low |
|  | meridian_aom_12264-12745 | 43,15 | 43,85 | 0,70 | Low |
|  | meridian_aom_15932-16309 | 41,81 | 42,44 | 0,62 | Low |
|  | meridian_aom_20988-21412 | 44,11 | 44,34 | 0,24 | Low |
|  | meridian_aom_22412-22738 | 40,43 | 42,03 | 1,60 | Low |
|  | meridian_aom_24058-24550 | 43,35 | 43,57 | 0,22 | Low |
| SolLevante | sol_levante_aom_289-453 | 30,57 | 38,02 | 7,45 | High |
|  | sol_levante_aom_519-649 | 24,23 | 26,81 | 2,58 | Low |
|  | sol_levante_aom_2268-2412 | 32,42 | 39,30 | 6,89 | High |
|  | sol_levante_aom_3282-3874 | 31,50 | 39,30 | 7,81 | High |
|  | sol_levante_aom_4123-4545 | 35,95 | 41,22 | 5,27 | High |
| Cosmos | cosmos_aom_1573-1749 | 43,32 | 47,37 | 4,04 | High |
|  | cosmos_aom_8686-8826 | 48,12 | 49,85 | 1,73 | Low |
|  | cosmos_aom_9561-9789 | 51,75 | 54,25 | 2,50 | Low |
|  | cosmos_aom_11589-11752 | 45,46 | 48,02 | 2,56 | Low |
|  | cosmos_aom_12149-12330 | 42,55 | 47,49 | 4,93 | High |
|  | cosmos_aom_12916-13078 | 49,76 | 52,29 | 2,53 | Low |
|  | cosmos_aom_13446-13649 | 49,84 | 52,09 | 2,25 | Low |
| CableLabs | LifeUntouched-1 | 45,26 | 49,89 | 4,63 | High |
|  | LifeUntouched-2 | 38,70 | 41,36 | 2,66 | Low |

It is observed that 14 sequences are classified as "High" while 23 are "Low". In order to keep sequences that are relevant for a codec characterization, the low-dynamic sequences are removed. To select a data set having different sources, LifeUntouched_1 is automatically selected since it is the only complex one in CableLabs set. Similarly, the sequence meridian_aom_22412-22738 from Meridian which is most challenging from an encoding perspective is selected.

Then, the next step consists in selecting a subset of remaining sequences in order to end up with a reasonnable number of sequences, from diverse sources and having different characteristics. With the target of selecting the same number of sequences as SDR, the remaining set of sequences are clustered into the SI-TI plane, using k-mean clustering to identify sequences having same spatial and temporal features. Since two sequences have already been selected, and given that eight sequences are targeted, six clusters are identified in the figure below.

Figure C.3-25 : SI-TI clustering for the Challenging  HDR sequences (6 clusters)

It is observed that clusters C3 and C4 only contain one sequence each, which are automatically selected since they present interesting codewords distribution and content. For other clusters, the codeword histograms are used to help deciding which sequence should be selected.

For C5, sparks_aom_2024-2455 is selected since it presents interesting codewords distribution compared to what is already included in the figure above.

For C1, sol_levante_aom_289-453 is rejected since it is similar to the cosmos sequence. In the two remaining sequences,  nocturne_aom_32660-32799 is too short in duration while nocturne_aom_23820-24322 does not present particular interest due to a static long dark transition part in the middle. Thus, no sequences are selected from C1.

For C6, sol_levante sequences are removed since two computer-generated sequences are already selected. sparks_aom_5764-6024 is selected to make the dataset more diverse in terms of codeword ditributions.

For C2, cosmos_aom_1573-1749 is removed since two computer-generated sequences are already selected. For the sake of having diverse sources, nocturne_aom_27740-28109 is selected since two sparks sequences are already selected.

Eventually, the selected sequences are summarized below :

- LifeUntouched1 (TBD)

- meridian_aom_22412-22738

- sol_levante_aom_2268-2412

- cosmos_12149-12330

- sparks_aom_2024-2455

- sparks_aom_5764-6024

- nocturne_aom_27740-28109

===== End of CHANGE 2 =====