Source:	NTT and NTT DOCOMO INC.

Title:	On Mixed content and Music categories for EVS Selection

Document for:	Approval

Agenda Item:	3.3

- Introduction

Mixed content and Music types shall be matched with 6 categories required for designing mixed content and music experiments.

This contribution proposes to keep 50:50 split in mixed content and music conditions.

- Outline of Mixed content and Music types for EVS Selection Testing

Mixed Content types

Three Mixed Content types are proposed as follows:

- Radio Jingle, Movie Trailer and Advertisement

- On hold content / answering machine message

- Speech over background music

The following table lists each Mixed Content type, their primary attributes and example sources for samples.

| Type | Attributes | Example of source |
| --- | --- | --- |
| Radio Jingle Movie Trailer Advertisement | 1, 2, 3 1, 3 1, 2, 3 | Radio jingle from commercial demo CD or directly from radio stations (i.e. as original, not broadcast signal). If not available, LL mixed synthetic content is acceptable, however, synthetic is much less preferred. Movie trailer could be recorded from DVD or BluRay disc or Internet (e.g. from music/movie store) in target language. Advertisement could be recorded from demo CDs. If not available, could be recorded from TV in target language. A TV recording should have a bandwidth of at least 15 kHz and have high fidelity. |
| On hold content / answering machine message | 1, 2, 4 | “Corporate” answering system. LL could synthetically create such a signal based on the observed content of actual corporate on-hold signals (i.e. transcribed speech and identified music excerpt). Alternatively, LL could use professional pre-recorded answering machine messages in target languages (e.g. http://www.onhold.on.ca/demos.php) |
| Speech over background music | 1, 2 | LL creates synthetic mix of clean speech and music signal simulating a conversional scene with present background music The only restriction on mix is that speech should be clearly intelligible. |

Attribute notes:

- Professional material recorded in controlled acoustic environment and source materials are of high quality. Material mixed from multiple source materials.

- Highly intelligible speech

- Selection and alignment of material is intentional, i.e., music and background sounds are relevant to the speech.

- Single talker

Music Types

Three Music types are proposed as follows:

- Classical (instrumental and vocal)

- Modern instrumental

- Modern vocal

The following table lists Music types and example sources for samples.

| Type | Examples |
| --- | --- |
| Classical | Symphonic, Choral, Opera (e.g. Vivaldi, Mozart, Beethoven). Record from CD |
| Modern instrumental | Modern music instrumental Record from CD |
| Modern vocal | Western pop, include vocals. Vocals could be in English if the music is known worldwide (e.g. The Beatles), otherwise vocals will be in target language. Record from CD |

- Conclusion

Definitions of mixed content and music types are proposed.

These should be reflected into EVS-8b.