Source:	Nokia Corporation

Title:	Further clarifications on generation of the MASA format

Document for:	Discussion

Agenda Item:	3

# 1. Introduction

At the SA4#97 Fukuoka meeting [1], the source first discussed mobile device capture of immersive sound fields for use cases relating to user-generated content (UGC) and mobile communications. It was proposed to agree on a parametric spatial audio format for IVAS to allow for high-quality representation of scene-based captures also from these challenging practical devices. Following extensive discussions on and development of the format, a reference implementation for analysis and synthesis of the MASA format was provided at the SA4#106 Busan meeting for IVAS standardization [2].

At SA4#106, the source also provided a demonstration of multi-microphone spatial audio capture and binaural presentation based on a mobile device form factor. During various offline discussions, it was noticed that it may be useful to further clarify certain aspects for MASA format generation.

Based on earlier discussion, it may also be useful to clarify the type of signal generation used for some of the listening tests previously conducted for IVAS standardization.

# 2. FOA or MASA?

Immersive sound scene capture on a smartphone is typically based on multiple microphones (e.g., three or four) integrated to the device design. Various audio processing such as noise suppression and equalization can be performed on these microphone signals, just like for capture targeting traditional mono signals, e.g., for EVS encoding. From the audio representation and IVAS encoding point of view, the most interesting step is however making sense of the multiple captured microphone signals!

For a spherical array or an array at least roughly approximating one (e.g., a tetrahedron setup), it is common to utilize an ambisonics representation for 3D spatial sound. A first-order ambisonics (FOA) representation is also one agreed input format for IVAS.

For an irregular array, such as one found on a flat smartphone device with a large screen, straightforward equations for ambisonics transformation do not exist. The scene interpretation is therefore achieved using parametric analysis that is carefully optimized for each device configuration. Based on knowledge of the microphone positions in the array, the analysis creates a scene representation that typically consists of one or more audio channels and spatial parameters. Figure 1 illustrates the procedure, where the output is a parametric audio signal, e.g., directly according to the MASA format.

Figure 1. Smartphone immersive audio capture and parametric audio representation.

On the other hand, as described in [2], the IVAS MASA C Reference Software utilizes for the MASA analysis primarily a set of commonly known methods based on Eigenmike-captured audio input. From this spherical array input, it is fairly easy to create a FOA signal with good quality. Based on the FOA formulation (specifically, a B-format ordering), DirAC-based analysis can be used to generate the parameters used in the MASA format. These methods are widely documented in literature.

In mobile device capture and analysis, a FOA signal is not created or otherwise utilized at all. It is, however, possible to synthesize other formats, e.g., FOA, from the MASA format. Figure 2 illustrates such processing resulting in 4 audio streams.

Figure 2. Smartphone immersive audio capture and FOA synthesis of parametric audio representation.

Thus, it can be clarified that:

- in mobile device capture, MASA analysis is based on the microphone signals, while

- in MASA C Reference Software, MASA analysis is based on a FOA signal.

In summary, FOA generation is not part of MASA format generation from smartphones.

While the capture functionality is out of scope of the IVAS codec work, it is observed from Figures 1 and 2 that maintaining a parametric format is generally less complex than going the route of format conversion. This is because the format conversion itself needs to be based on the capture analysis. The format transformation is thus an extra step that typically increases the number of audio signals at least back to the original microphone signal count (or even higher). While additional computational complexity and likely also additional latency are required, this extra step also complicates encoding of the spatial signals. Due to higher number of input channels, the required bit rate likely increases. Maintaining a parametric representation thus reduces overall implementation complexity and the complexity of the audio input itself.

It is also useful to consider the inherent quality achievable using various formats.

# 3. Listening test summary

Two types of tests have been previously performed for MASA format. The first test category relates to the format generation on mobile devices and comparison to other formats (FOA), while the second category relates to use of Eigenmike-based systems for generating the MASA format. We here summarize the results and clarify the processing methods. For convenience, the results are copied into annex of the present document.

Two listening test experiments [1, 3] were performed at Nokia Technologies specifically comparing parametric spatial audio (MASA) vs. FOA for a sound scene representation using practical mobile devices. Captures from both a commercial smartphone and a smartphone prototype device were utilized for the tests.

The input capture processing for the experiments in [1, 3] thus correspond to Figures 1 and 2.

The results [1, 3] indicate that for smartphone immersive audio capture there is a consistent, significant quality degradation caused by the format transformation when a FOA synthesis is used instead of the parametric format.

A further listening test based on Eigenmike-captured signals was performed for verifying the correctness of the functionality and completeness of the IVAS MASA C Reference Software [2]. In this case, the MASA analysis utilizes a FOA signal derived from the spherical array signals. Thus, the processing does not correspond to either Figure 1 or 2; instead a DirAC-based analysis is used. This avoids designing and implementing a new type of parametric analysis where it is not necessary to do so.

The results [2] indicate no issues with the reference software. The reference software can thus be used to generate MASA input signals. A rendering system is also provided.

# 4. Summary

In this input document, the source provides clarification on the internal differences between MASA format generation based on a mobile phone form factor and the Eigenmike-based reference software. Specifically, spatial audio capture for smartphones is based on parametric interpretation of multiple microphone signals that originate from irregular arrays. These methods do not utilize any ambisonics-based internal representation.

In the reference software, a spherical array is utilized as input. That allows for a high-quality FOA generation, where DirAC-based methods can then be employed for MASA format generation.

Either approach can be used to derive a valid MASA format input signal for the IVAS codec. The format itself allows for a wide range of analysis methods to be used (as shown also by the reference software), and, in case of practical smartphones, avoids quality losses from unnecessary format transformations. A common representation thus accommodates any microphone array (for which the implementor designs a suitable analysis) and provides an interoperable codec interface for various capture approaches and algorithms.

# References

[1]	S4-180087: “On IVAS audio format for mobile capture devices”

[2]	S4-191167: “Description of the IVAS MASA C Reference Software”

[3]	S4-180462: “On spatial metadata for IVAS spatial audio input format”

# Annex A – MASA vs. FOA listening test results based on mobile capture

Figure Annex-1. AB listening test results (n=24). (Figure 4 of [1].)

Figure Annex-2. AB listening test results, expert listeners (n=14). (Figure 5 of [1].)

Figure Annex-3. AB listening test results, naïve listeners (n=10). (Figure 6 of [1].)

Figure Annex-4. Listening test results for loudspeaker presentation (n=12). (Figure 2 of [3].)

# Annex B – MASA format test results based on Eigenmike capture

Figure Annex-5. Software verification test results (n=12). (Figure 5 of [2].)