Source:		Dolby Laboratories, Inc.

Title:			Input Audio and Session Metadata for the IVAS encoder

Document for:	Discussion and Agreement

Agenda Item:	7.5

## Preface and proposal

In a previous contribution [1], the source put forth a first proposal for an IVAS encoder input audio and session metadata specification. The focus of the contribution was on metadata syntax and semantics, rather than a representation format, which is an important but somewhat separate matter. In contribution [2], the source provides a first proposal of an audio input and metadata format that supports the suggested metadata syntax and semantics.

The contribution was initially submitted to SA4#105 and updated and submitted as document [1] to SA4#106, where it was noted. The source believes that it is crucial for the progress of the work item to have agreement on input audio and session metadata and its format. As no other or competing proposals were made and the comments received so far do not appear as severe reservations, the source herewith resubmits the proposal with a few minor adjustments. It is noted that another contribution [3] by the source suggests adding the possibility of passthrough of external metadata. Upon agreement this feature should be added to the framework of input audio and session metadata. We also note here that it may be for discussion if some energy or level parameter should be added to the track group metadata, which for instance could support server-based mixing in conferencing use cases. The source may suggest adding a corresponding parameter in one of the coming meetings.

It is proposed that the text starting with numbered Section 1 below be agreed upon as the first draft of an IVAS Encoder Metadata specification and that it should be included as an annex to Pdoc IVAS-4.

## 1. Introduction

What follows is a proposed description of IVAS encoder metadata syntax and semantics. It is assumed in this document that audio and metadata is stored and delivered in suitable container format. More specifically, it is assumed that audio input is delivered to the encoder as multiple mono audio tracks, each track having an associated metadata stream. Audio tracks are delivered to the encoder in a known order.

The metadata specified herein can be used to indicate the type of each audio track and its related properties. All metadata in this proposal is dynamic, with one metadata update provided per frame of audio. However, many fields are optional. Optional fields take a default value when not present. By convention, it is assumed that any metadata is associated with the last audio sample of the corresponding audio frame and that the metadata would be applied by cross-fading it from one frame to the next.

It should be emphasized that the present proposal in all potential metadata combinations likely comprises a superset of codec features that would be beyond what is envisioned for IVAS. This proposal should not be construed in a sense that it competes against IVAS codec design constraints. Rather, the only specification of required codec features shall remain the IVAS design constraints.

## 2. Session Metadata

Session metadata is associated with the entire encoding session rather than a specific audio track. Like audio metadata, session metadata is updated once every audio frame.

## 2.1 Device Position and Orientation

Session metadata includes optional information about device position and orientation at the time each audio frame is encoded. Device orientation is expressed in degrees of yaw, pitch and roll relative to some reference frame. Device position (translatory) is expressed in the same way as for object tracks (Section 3.2.1) The device is assumed to be in the same normalized space as the encoded audio. The definition of the reference frame is assumed to beyond the scope of the session metadata and it could for instance be set by SDP at session setup.

| deviceYaw (9 bits, optional) | Description | Data type |
| --- | --- | --- |
| Default: 0 | Yaw, in degrees, from -179 to 180, where 180 = rear, 90 = left, -90 = right, and 0 = front. | Unsigned integer |

| devicePitch (9 bits, optional) | Description | Data Type |
| --- | --- | --- |
| Default: 0 | Pitch, in degrees, from -179 to 180. | Signed integer |

| deviceRoll (8 bits, optional) | Description | Data Type |
| --- | --- | --- |
| Default: 0 | Roll, in degrees, from -90 to 90. | Signed integer |

Note: It may be for discussion if multiple Device Position and Orientation metadata elements should be enabled to better address multi-party scenarios.

## 2.2 Track Groups

Audio tracks may be combined into track groups. Track groups can be used to indicate to the encoder that a set of tracks are intended to be subjected to the same transformation(s) at the rendering end (e.g., for headtracking purposes). This information allows the encoder to combine and jointly code audio tracks in order to use available bitrate efficiently, without the expectation that independent manipulation or rendering of the individual tracks will be necessary. There can be up to [64] track groups. The number of expected track groups is given as session metadata. Group membership is provided as audio metadata (i.e., it is specified within the metadata for each track).

Note: Passthrough of single objects according to the concept suggested in [4] can be supported by specifying track groups containing single object tracks.

The numTrackGroups field specifies how many track groups are currently present in the encoding session.

| numTrackGroups ([6] bits, optional) | Description | Data type |
| --- | --- | --- |
| Default: 0 | numTrackGroups = Number of track groups present – 1. The default value of 0 indicates 1 group is present. | unsigned integer |

## 2.2.1 Track Group metadata

The following metadata is associated with each track group in the encoding session.

The bitrate field indicates the target bitrate for the track group in bits per second. The encoder cannot exceed this bitrate in the portion of the IVAS bitstream associated with this track group.

| bitrate ([TBD] bits, required) | Description | Data type |
| --- | --- | --- |
|  | Indicates bitrate for the track group. [Allowed values are TBD.] | unsigned integer |

The bw field (2 bit) indicates the audio bandwidth to be for coding the tracks in the track group.

| bw (2 bits, optional) | Description |
| --- | --- |
| 00 (default) | FB |
| 01 | SWB |
| 10 | WB |
| 11 | NB |

The dtx field (1 bit) indicates whether discontinuous transmission may be used for the tracks in the track group.

| dtx (1 bit, optional) | Description |
| --- | --- |
| 0 (default) | dtx disabled |
| 1 | dtx enabled |

The diegetic field (1 bit) indicates whether the audio tracks in the track group are diegetic or non-diegetic.

| diegetic (1 bit, optional) | Description |
| --- | --- |
| 0 (default) | non-diegetic |
| 1 | Diegetic |

The dialogue field (1 bit) indicates whether the track group consists of dialogue content. This information is useful in situations where dialogue may be treated differently from other components in the session for the purpose of prioritizing intelligibility.

| dialogue (1 bit, optional) | Description |
| --- | --- |
| Default = 0 | When set to 1, specifies that a track group contains dialogue. |

The binaural flag indicates that headphone virtualization has already been applied to the track group.

| binaural (1 bit, optional) | Description |
| --- | --- |
| Default = 0 | When set to 1, no additional spatialization should be carried out before direct presentation over headphones. |

Note: the binaural flag should be used in conjunction with a channel audio track of channelType binaural.

The directPresentation flag indicates that headphone virtualization is not to be applied to the track group when the rendering endpoint is headphones. Instead, reproduction should be the same as for a Stereo speaker endpoint.

| directPresentation (1 bit, optional) | Description |
| --- | --- |
| Default = 0 | When set to 1, specifies that headphone virtualisation should be bypassed. |

Note: the directPresentation flag has a distinct purpose from the binaural flag. The binaural flag indicates that headphone spatialization has already been applied. Whereas, the directPresentation flag is used to indicate that the content creator does not want headphone spatialization to be applied to the content. Conventional reproduction over stereo loudspeakers is, however, appropriate when directPresentation is set.

## 3. Audio Metadata

Audio metadata is associated with each audio track in the encoding session. It is updated once per input audio frame.

The trackType field (2 bits) indicates whether the audio track is part of a channel-, scene-, or object-based presentation.

| trackType (2 bits, optional) | Description |
| --- | --- |
| 00 (default) | Channel |
| 01 | Scene-based |
| 10 | Object |
| 11 | RESERVED |

The trackGroup field indicates group membership.

| trackGroup ([6] bits, optional) | Description | Data type |
| --- | --- | --- |
| Default: 0 | Indicates which track group this track belongs to. By default, a track belongs to group 0. | unsigned integer |

## 3.1. Channel Tracks

Channel audio tracks contain the following metadata, per frame:

| channelType (2 bits, optional) | Description |
| --- | --- |
| 00 (default) | Stereo |
| 01 | Standard |
| 10 | Custom |
| 11 | Binaural |

For Stereo and Binaural channelTypes, the first track that is tagged with this channelType is assumed to be the left channel. The track that immediately follows is assumed to be the right channel and its channelType and trackGroup fields, if present, will be ignored. Behaviour is undefined if a track of channelType Stereo or Binaural is the last track present.

The Standard channelType is used to specify standard channels that are given in [5]. If channelType = 01, the channelIndex field identifies one of the following standard channels.

| channelIndex (6 bits, optional) | Description (from [5]) |
| --- | --- |
| 0 (default) | M+000 |
| 1 | M+022 |
| 2 | M-022 |
| 3 | M+SC |
| 4 | M-SC |
| 5 | M+030 |
| 6 | M-030 |
| 7 | M+045 |
| 8 | M-045 |
| 9 | M+060 |
| 10 | M-060 |
| 11 | M+090 |
| 12 | M-090 |
| 13 | M+110 |
| 14 | M-110 |
| 15 | M+135 |
| 16 | M-135 |
| 17 | M+180 |
| 18 | U+000 |
| 19 | U+022 |
| 20 | U-022 |
| 21 | U+030 |
| 22 | U-030 |
| 23 | U+045 |
| 24 | U-045 |
| 25 | U+060 |
| 26 | U-060 |
| 27 | U+090 |
| 28 | U-090 |
| 29 | U+110 |
| 30 | U-110 |
| 31 | U+135 |
| 32 | U-135 |
| 33 | U+180 |
| 34 | UH+180 |
| 35 | T+000 |
| 36 | B+000 |
| 37 | B+022 |
| 38 | B-022 |
| 39 | B+030 |
| 40 | B-030 |
| 41 | B+045 |
| 42 | B-045 |
| 43 | B+060 |
| 44 | B-060 |
| 45 | B+090 |
| 46 | B-090 |
| 47 | B+110 |
| 48 | B-110 |
| 49 | B+135 |
| 50 | B-135 |
| 51 | B+180 |
| 52 | LFE1 |
| 53 | LFE2 |
| 54-63 | RESERVED |

If channelType = 10, positional metadata for the channel is indicated by the posCoordinateSystem, pos1, pos2, and pos3 fields, according to the specification for object tracks.

Note: Besides position, other object metadata such as extent, diffuseness, etc., cannot be used with channels.

## 3.2 Object Tracks

Object audio tracks contain the following metadata, per frame:

| objectType (2 bits, optional) | Description |
| --- | --- |
| 00 (default) | Mono |
| 01 | Planar FoA |
| 10 | Full FoA |
| 11 | Mono with panning gain |

FoA object tracks are used for representing objects with directivity (Section 3.2.3). For FoA objectTypes, the first track tagged with this objectType is assumed to be the W channel. The tracks that immediately follow are assumed to be the (Z)YX channels (in that order) and their trackGroup and object metadata fields, if present, will be ignored (i.e., all the object metadata must be provided with the W track). Behaviour is undefined if a track of FoA objectType is not followed by the correct number of tracks for the planar or full FoA representation. Normalization for FoA object tracks follows the SN3D scheme.

The objectID field can be used to identify the object, per frame. One out of 63 unique object identifiers may be chosen out of the set from 1…63. The default objectID 0 (zero) is used to signal that there is no particular object identification.

| objectID ([6] bits, optional) | Description | Data type |
| --- | --- | --- |
| 0 (default) | Identification index | unsigned integer |

3.2.1 Position/Panning gain

Minimally, an object (except for mono objects with panning gain) needs an associated position in space. Cartesian and spherical coordinates are both useful in different usage scenarios and are therefore both supported. Object positions are given in normalised coordinates. The object space is either a unit sphere or a unit cube, depending on the coordinate system.

For objectTypes “00” (mono), “01” (planar FoA) and “10” (full FoA) the following definition of object position applies:

| posCoordinateSystem (1 bit, optional) | Description |
| --- | --- |
| 0 (default) | Spherical coordinates, over the unit sphere. |
| 1 | Cartesian coordinates, over the unit cube. |

| pos1 (9 bits, required) | Description | Data type |
| --- | --- | --- |
| If posCoordinateSystem = 0 | Azimuth, in degrees, from -179 to 180, where 180 = rear, 90 = left, -90 = right, and 0 = front. | Signed integer |
| If posCoordinateSystem = 1 | X-position (left-right), from -1 to 1, where -1 = left, 0 = centre, 1 = right. | Signed fractional |

| pos2 (8 bits, optional) | Description | Data Type |
| --- | --- | --- |
| If posCoordinateSystem = 0 | Elevation, in degrees, from -90 to 90, where -90 = below, 90 = above, 0 = listener ear level. Default = 0.0 | Signed integer |
| If posCoordinateSystem = 1 | Y-position (back-front), from -1 to 1, where -1 = back, 0 = centre, 1 = front. Default = 1.0; | Signed fractional |

| pos3 (8 bits, optional) | Description | Data Type |
| --- | --- | --- |
| If posCoordinateSystem = 0 | Radius from 0 to 1, where 0 = centre (at the listener position) and 1 is at the surface of the unit sphere. Default = 1.0 | Unsigned fractional |
| If posCoordinateSystem = 1 | Z-position (bottom to top), from -1 to 1, where -1 = below, 0 = centre (at ear level), 1 = above. Default = 0.0 | Signed fractional |

For objectType “11” (mono with panning gain), the following metadata element for panning gain applies:

| Panning gain (8 bits, optional) | Description | Data type |
| --- | --- | --- |
| Default = 0.5 | Gain from 0 to 1 to pan the mono object to left or right stereo channel, using the weights leftGain and rightGain, with LeftGain = Gain RightGain = 1 - Gain | Unsigned fractional |

3.2.2 Extent

Objects can represent point sources or larger sources having a specified extent over space. This property is captured by extent metadata. Extent can be uniform in all dimensions or can be specified separately for each dimension.

| extentDimensions (1 bit, optional) | Description |
| --- | --- |
| 0 (default) | One-dimensional extent is specified in extent field |
| 1 | 3-D extent is specified in extent1, extent2, and extent3 fields. |

| extent (8 bits, optional) | Description | Data Type |
| --- | --- | --- |
| default = 0.0 | One-dimensional extent (uniform in all dimensions), in normalized units, from 0 to 1. | Unsigned fractional |

| extent1 (9 bits, optional) | Description | Data Type |
| --- | --- | --- |
| If posCoordinateSystem = 0 | Azimuth extent, in degrees, from 0 to 359. Default = 0.0 | Unsigned integer |
| If posCoordinateSystem = 1 | Horizontal extent (X-dimension), in normalized units, from 0 to 1. Default = 0.0 | Unsigned fractional |

| extent2 (9 bits, optional) | Description | Data Type |
| --- | --- | --- |
| If posCoordinateSystem = 0 | Elevation extent, in degrees, from 0 to 359. Default = 0.0 | Unsigned integer |
| If posCoordinateSystem = 1 | Front-back extent (Y-dimension), in normalized units, from 0 to 1. Default = 0.0 | Unsigned fractional |

| extent3 (8 bits, optional) | Description | Data Type |
| --- | --- | --- |
| If posCoordinateSystem = 0 | Radial extent, normalized units, from 0 to 1. Default = 0.0 | Unsigned fractional |
| If posCoordinateSystem = 1 | Up-down extent (Z-dimension), in normalized units, from 0 to 1. Default = 0.0 | Unsigned fractional |

3.2.3 Directivity and Orientation

Object directivity metadata can be used to represent a time and frequency-dependent directivity pattern for an object. The source proposes either a waveform or a parametric representation to convey directivity information for an object in FoA form. A waveform approach can consist of 3 waveforms for planar directivity or 4 waveforms for full directivity. In such an approach, multiple audio tracks must be associated with the object, as stated in Section 3.2. A parametric approach, on the other hand, may consist of frequency-dependent FoA coefficients over a set of frequency bands. These metadata can be associated with a mono object audio track. The details of this approach are TBD.

Objects with directivity metadata are required to also contain orientation metadata. Orientation metadata represents object orientation along 3 axes. Orientation metadata follows the same definition as device orientation metadata, see clause 2.1.

3.2.4 Diffuseness

Diffuseness metadata specifies the balance between spatial and isotropic rendering for an object.

| diffuse (8 bits, optional) | Description | Data Type |
| --- | --- | --- |
|  | Specifies the diffuseness of an object, in normalized units from 0 to 1. A value of 1 indicates complete diffuseness (i.e., an audio source that is completely isotropic), whilst a value of 0 indicates no diffuseness. Default = 0.0 | Unsigned fractional |

## 4. Scene-based Tracks

Ambisonics tracks shall use Ambisonic Channel Number (ACN) component ordering [6]. The first track of the scene shall have a trackType value of 01 and in addition may contain a sceneOrder field.

| sceneOrder (3 bits, optional) | Description |
| --- | --- |
| 00 (default) | Planar FOA (WXY) |
| 01 – 06 | Ambisonics order |

The following N-1 tracks, where N is the number of ambisonics channels corresponding to the sceneOrder, are assumed to belong to the same audio scene and their trackType, sceneOrder and trackGroup fields will be ignored. Undefined behaviour will result if a trackType = 01 track is followed by an insufficient number of tracks for the selected sceneOrder.

Ambisonics scenes shall use Schmidt semi-normalized (SN3D) normalization of the scene components.

## 5. References

[1]	S4-191200: “Input Audio and Session Metadata for the IVAS encoder”, Dolby Laboratories, Inc.

[2]	AHEVS-507: “The IVAS Project Format”, Dolby Laboratories, Inc.

[3]	AHEVS-508: “On IVAS support of external metadata passthrough”, Dolby Laboratories, Inc.

[4]	S4-190949: “On IVAS Codec Operation of Audio Objects”, Dolby Laboratories, Inc.

[5] 	Recommendation ITU-R BS.2051-2 (07/2018): “Advanced sound system for programme production”.

[6]	https://iem-projects.github.io/ambix/apiref/format.html#format_acn