3GPP TSG-SA4 MBS Adhoc on EMM-EFEC	S4-AHI237

13 – 15 December 2011	update to S4-111048

Copenhagen, Denmark

Agenda item: 	4

Source: 	Qualcomm Incorporated

Title: 	Proposed Evaluation and Selection Criteria for enhanced MBMS FEC (EMM-EFEC)

Document for	Proposal and Approval

# Introduction

In SP-110555 (also available as S4-110792) a new work item on "Enhancement to FEC for MBMS" has been approved during SA#53.

This document addresses the following topics for the purpose to define the evaluation criteria:

- Use cases that are relevant for the selection of the application layer FEC in clause 2.

- Evaluation Criteria proposed for the evaluation of application layer FEC in clause 3.

- Benchmark codes and the results for these codes in clause 4.

- Qualification and selection criteria for a new application layer FEC for 3GPP MBMS user services in clause 5.

The concrete proposals are provided in clause 6.

Note that some simulation results for RFC5053 performance over LTE are still to be completed and will be made available later this week before the start of SA4#66.

# Use Cases

## Services

### Streaming Delivery

#### Introduction

The purpose of the MBMS streaming delivery method is to deliver continuous multimedia data (i.e. speech, audio, video and DIMS) over an MBMS bearer. The streaming delivery method is particularly useful for multicast and broadcast of scheduled streaming content. RTP is the transport protocol for MBMS streaming delivery. RTP provides means for sending real-time or streaming data over UDP.

TS26.346 defines a generic mechanism for applying Forward Error Correction to streaming media. The mechanism consists of three components:

(i)	construction of an FEC source block from the source media packets belonging to one or several UDP packet flows related to a particular segment of the stream(s) (in time). The UDP flows include RTP, RTCP, SRTP and MIKEY packets.

(ii)modification of source packets to indicate the position of the source data from the source packet within the source block

(iii)definition of repair packets, sent over UDP, which can be used by the FEC decoder to reconstruct missing portions of the source block.

The details on transport for the streaming delivery service are provided in section 2.1.1.2.

An alternative way to deliver streaming services over MBMS is the use of DASH and FLUTE. This is use case is discussed in section 2.1.2.4.

#### Transport in streaming delivery service

The MBMS streaming framework operates on RTP packets or more precisely UDP payloads, incoming at same or different UDP ports. According to TS26.346, clause 8.2.2, the FEC layer for streaming delivery is based on top of the UDP layer. The legacy RTP packets and the UDP port information are used in order to generate FEC repair symbols. Original UDP payloads become FEC source packets by appending a 3 byte FEC source payload ID field at the end of each UDP payload. These packets are then UDP encapsulated and transported on the IP multicast bearer.

According to Figure 1 a copy of these packets is forwarded to the FEC encoder and is arranged in a source block with row width T bytes at the first empty row. The encoding symbol starts at the beginning of a new row, but it is preceded by a 3 byte field containing the UDP flow ID (1 byte) and the length field (2 bytes). In case the length of the packet is not an integer of the symbol the remaining bytes in the last row are filled up with zero bytes. The source block is filled up to k rows whereby k is flexible and can be changed dynamically for each source block. The selection of k depends on the desired delay, the available terminal memory and also might depend on aspects such as desired zapping time in mobile TV applications. Typically for a streaming service a protection period is defined and the value of the protection period dynamically determines the source block size.

Figure 1 MBMS Streaming Framework

After processing all packets to be protected within one source block, the FEC encoder generates n-k FEC repair symbols of size T by applying FEC. The generated FEC repair symbols can be transmitted individually or as blocks of symbols as payload of a single UDP packet. Each FEC source and repair packet contains sufficient information such that the receiver can correctly insert them in the receiver source and repair block.

#### Examples

Examples are audio streaming applications or video streaming applications with bitrates ranging from 32 kbit/s to one or several MBit/s.

### Download Delivery

#### Introduction

According to TR26.946, the MBMS Download Delivery Method allows the error-free transmission of files via the unidirectional MBMS Bearer Services. The files are "downloaded" and stored in the local files-system of the user equipment. Files may contain multimedia content or any other binary data. The MBMS Download Delivery Method allows the transmission of an arbitrary number of files within a single data transfer phase.

Figure 1: Definition of MBMS Download Sessions

Figure 1 is an example of an MBMS User Service based on the Download Delivery Method. The file transmission events are organized in MBMS Download Sessions. Each session is started with a File Delivery Table (FDT) instance, which describes in this example each file within the MBMS Download Session in terms of file name and file type (MIME Content Type). The service operator and the actual service determine the timing of MBMS Download Sessions. Depending on the service type, the MBMS Download session may require strict or more relaxed time-constraint delivery of content.

#### Transport in download delivery service

This clause explains briefly how files are constructed for and transported during a FLUTE session. The BM-SC takes a file, e.g. a video clip or a still image, which is used as the transport object for FLUTE (see figure 2). The BM-SC constructs source block by breaking the file into contiguous portions of approximately equal size. Each source block is broken into source symbols. One or more encoding symbols are carried as the payload of a FLUTE packet, thus the FLUTE packet size must be divisible by the encoding symbol size. The target FLUTE packet size is configured by the BM-SC and, together with the file size, is used to determine the encoding symbol length. When FEC is used it may be beneficial to include several symbols in each FLUTE packet. Based on the transport object size, the encoding symbol size and the maximum source block length, FLUTE calculates the source block structure (i.e., the number of source blocks and their length).

Figure 2: Constructing FLUTE packets

The BM-SC communicates the transport object size, the encoding symbol size and the file size to the receivers within the FLUTE session transmission such that the receiver can also calculate the source block structure in advance of receiving a file.

The FLUTE packet is constructed from FLUTE header and payload containing one or more encoding symbols.

The distinction between file and transport object is that the file is the object provided to the BM-SC and played-out or stored at the MBMS UE. Within the scope of FLUTE sessions, content encoding may be used, for instance to compress the file with gzip for delivery. In the presence of FLUTE session content encoding, the file and the transport object will be different binary objects, and in the absence of content encoding the transport object will be identical to the file. Any symbol calculations (including FEC) are performed on transport objects.

#### Download Examples

In a typical use case, multimedia files typically in 3GP or MP4 format are distributed through download delivery method. In this case the delivery rate and the media rate may be completely different as no real-time consumption is considered.

Table 1 shows some typical examples of file sizes for different types of multimedia content.

Table 1 Examples for Download delivery use cases

| Number | File Size | Example |
| --- | --- | --- |
| 1 | 1 MByte (1 048 576 bytes) | AAC encoded audio clip |
| 2 | 3 MByte (3 145 728 bytes) | MP3 audio clip |
| 3 | 128 MByte (134 217 728) bytes | 30 min SD movie coded at 500 kbit/s |
| 4 | 1.8 GByte (1 887 436 800) bytes | 2 hours HD movie coded at 2 MBit/s |

#### DASH over FLUTE

In another use case as indicated in TS26.346, section 5.6, the download delivery method may be used to distribute DASH formatted content over MBMS. MBMS is designed to serve large receive groups with same content. The MBMS Download Delivery Method is designed to deliver an arbitrary number of (binary) files via MBMS to a large receiver population. MBMS Download defines several methods to increase reliability such as file repair. The download delivery method supports the delivery of media segments and even media presentation descriptions. Media segment URIs are described using the FDT in FLUTE.

In this case the media bitrate and the delivery bitrate are typically the same to maintain the real-time delivery capabilities.

Table 2 shows some typical examples of DASH media segment files for live services.

Table 2 Examples for DASH segments

| Number | File Size | Example |
| --- | --- | --- |
| 1 | 50 KByte (51 200 bytes) | 1 sec DASH segment 250 kbit/s stream |
| 2 | 128 KB (131 072 bytes) | 4 sec DASH segment 250 kbit/s stream |
| 3 | 512 kByte (524 288 bytes) | 4 sec DASH segment for 1 Mbit/s stream |
| 4 | 2.5 MByte (2 621 440 bytes) | 20 sec DASH segment for 1 Mbit/s stream |

## Radio Access

### UTRAN

The MBMS Bearer service reuses most of the legacy UMTS protocol stack in the packet-switched domain. Only minor modifications are introduced to support MBMS. The IP packets are processed in the Packet Data Convergence Protocol (PDCP) layer where for example header compression might be applied. In the Radio Link Control (RLC) the resulting PDCP- Protocol Data Units (PDUs), generally of arbitrary length, are mapped to fixed length RLC-PDUs. The RLC layer operates in unacknowledged mode as feedback links on the radio access network are not available for point-to-multipoint bearers. Functions provided at the RLC layer are for example segmentation and reassembly, concatenation, padding, sequence numbering, reordering and out-of-sequence and duplication detection. The Medium Access Control (MAC) layer maps and multiplexes the RLC-PDUs to the transport channel and selects the transport format depending on the instantaneous source rate. The MAC layer and physical layer appropriately adapt the RLC-PDU to the expected transmission conditions by applying, among others, channel coding, power and resource assignment, and modulation.

During the MBMS definition, appropriate for UTRAN bearer settings for the simulation of FEC parameters had been defined and are summarized in Table 3.

Table 3 Typical UTRAN bearer parameters

| UTRAN Bearer parameters |  |  |
| --- | --- | --- |
|  | Bearer rates | 64 kbit/s, 128 kbit/s, 256 kbit/s |
|  | RLC PDU size | 640 bytes, 1 280 bytes, 1 280 bytes respectively |
|  | RLC BLER | 1%, 5%, 10%, 15%, 20%, 30% |
|  | RLC block loss pattern | Independent random loss |

### LTE eMBMS

To obtain some representative numbers for the performance of an FEC code in an LTE MBMS environment, some simple models are necessary for AL-FEC evaluation.

Figure 2 shows the mapping of RLC-SDUs to RLC-PDUs. RLC-SDUs in the context of MBMS are IP packets. The RLC header is 1 byte if the RLC SDU consists of 1 IP packet. The header is longer, if multiple IP packets are multiplexed in an RLC-SDU. A reasonable assumption is to use 3 byte header of the RLC-PDU assuming a 5 bit sequence number. The loss of one RLC-PDU results in the loss of all IP packets included in the RLC-PDU.

The MAC PDU consists of a number of MAC SDUs, where a MAC-SDUs is an RLC-PDU. The MAC multiplexer notifies the RLC layer of the available bits. The RLC layer would then create an RLC PDU that fits exactly into the available space in the MAC PDU. There is no need for fragmentation of MAC SDUs across subframes. Based on this, it can be assumed that the loss of one MAC-PDU results in the loss of one RLC-PDU.

Figure 2 Mapping of IP packets (RLC-SDUs) to RLC-PDUs (see TS 36.300, section 6.2.2)

LTE MBMS defines modulations and coding schemes with a MAC-PDU size ranging from 680 bit to 18336 bit for a 5 MHz bandwidth. Reasonable MCS configurations to cover different environments are between MCS 17 (for rural areas) and MCS 24 (for dense urban areas). For 5 MHz, this results in MAC-PDU payload sizes of 7992 bits and 14112 bits, respectively.

Each MAC-PDU is mapped to a subframe. At allocation level 1, LTE MBMS can use up to 6 out of the 10 subframes of a 10ms frame. Each subframe is 1ms.

The interleaving for MBMS in LTE is the same as for regular unicast LTE delivery of 1ms. As a first order approximation, it can be assumed that at most one MAC-PDU per 10ms frame is delivered and it is assumed that MAC-PDUs encounter iid random losses.

As a reasonable configuration proposal for the simulations in this document, we propose to use

- a typical value of MCS 21 with 1335 bytes for the MAC-PDU payload size. This results then in a maximum RLC-PDU payload size of 1332 bytes.

- This results in a maximum RLC layer net bitrate of 1065.6 kbit/s for each subframe within the 10ms frame.

Typical loss rates of MAC-PDUs are in the range of 1-2%, but higher loss rates may occur occasionally up 10% and very bad conditions up to 20%. The latter also permits to give some indication on the performance in case losses are more correlated.

Typical LTE MBMS bearer parameters are provided in Table 4.

Table 4 Typical LTE MBMS bearer parameters

| LTE eMBMS Download |  |  |
| --- | --- | --- |
|  | Bearer bitrates | 266.4 kbit/s, 532.8 kbit/s, 1.0656 Mbit/s |
|  | RLC-SDU size | 1332 byte |
|  | RLC-SDU frequency | 40ms, 20ms, 10ms |
|  | MAC PDU loss pattern | iid random |
|  | MAC-PDU loss probability | 1%, 2%, 5%, 10%, 20% |

<<ED NOTE: Changes and updates had been requested on this section. For this purpose, communication with RAN has been initiated in S4-111014. Additional input contributions are provided to the adhoc meeting to address this subject.>>

## Simulation conditions and assumptions (UTRAN)

The simulation conditions for UTRAN-based MBMS are provided in Table 5.

Additional details on the simulation methodology are provided in contribution S4-AHI236.

Table 5 Simulation Conditions for UTRAN-based MBMS

| UTRAN Download |  |  |
| --- | --- | --- |
|  | Bearer rates | 64 kbit/s, 128 kbit/s, 256 kbit/s |
|  | RLC-PDU size | 640 bytes, 1 280 bytes, 1 280 bytes respectively |
|  | RLC-PDU BLER | 1%, 5%, 10%, 15%, 20%, 30% |
|  | RLC-PDU block loss pattern | Independent random loss |
|  | Number of trials | At least 10,000 for files 512 KB, 3,000 for 3 072 KB |
|  | File sizes | 50 KB, 512 KB, 3 072 KB |
|  | FLUTE payload size | 456 bytes |
|  | ROHC | No |
|  | IPv4/UDP header | 28 bytes |
|  | FLUTE header | 16 bytes |
|  | FEC overhead | Varied in steps of X packets, where X=ceil(0.005N) and N is the number of packets containing source data |
| UTRAN Streaming |  |  |
|  | Bearer rates | 64 kbit/s, 128 kbit/s and 256 kbit/s |
|  | RLC PDU size | 640 bytes (for 64 kbit/s bearer) 1280 bytes (for 128 kbit/s bearer) 1280 bytes (for 256 kbit/s bearer) |
|  | RLC BLER | 1 %, 5 %, 10 %, 15 %, 20 %, 30 % |
|  | RLC block loss pattern | Independent random loss |
|  | Simulation duration | 24 hours |
|  | Media rates | Varied by steps of 1 % of bearer rate, assuming only a single media stream with constant bitrate (see note 1) |
|  | FEC overhead | Varied to sum FEC and Media to equal bearer rate |
|  | Source packet RTP payload size | 64 kbit/s: 456 bytes 128 kbit/s: 456 bytes 256 kbit/s: 768 bytes |
|  | Repair packet RTP payload size | Minimum value supported by the FEC code which is not less than 470 (for 64 kbit/s and 128 kbit/s) and 782 (for 256 kbit/s) - (see note 2) |
|  | Protection period | 5 s, 20 s |
|  | ROHC | No |
|  | IPv4/UDP/RTP header | 40 |
| NOTE 1: In practice, multiple media streams may be carried within a single MBMS bearer. However, only a single media stream is considered for FEC simulation purposes for simplicity. NOTE 2: The last repair packet of a block may be shorter if supported by the FEC code in order to fit within the protection period. |  |  |

## Simulation conditions and assumptions (LTE eMBMS)

The simulation conditions for LTE-based MBMS are provided in Table 6.

Additional details on the simulation methodology are provided in contribution S4-AHI236.

Table 6 Simulation Conditions for LTE-based MBMS

| LTE eMBMS Download |  |  |
| --- | --- | --- |
|  | RLC-SDU | 266.4 kbit/s, 532.8 kbit/s, 1.0656 Mbit/s |
|  | RLC-SDU size | 1332 byte |
|  | RLC-SDU frequency | 40ms, 10ms |
|  | MAC PDU loss pattern | iid random |
|  | MAC-PDU loss probability | 1%, 2%, 5%, 10%, 20% |
|  | Number of trials | At least 10,000 for files 1 MB, at least 3,000 otherwise |
|  | File sizes | 1MB, 3MB, 128MB, 1.8GB |
|  | FLUTE payload size | 1288 bytes |
|  | ROHC | No |
|  | IPv4/UDP header | 28 bytes |
|  | FLUTE header | 16 bytes |
|  | FEC overhead | Varied in steps of X packets, where X=ceil(0.005N) and N is the number of packets containing source data |
| LTE eMBMS Streaming (based on DASH) |  |  |
|  | Bearer rates | 266.4 kbit/s, 1.0656 Mbit/s |
|  | RLC payload size | 1332 byte |
|  | RLC-SDU frequency | 40ms, 10ms |
|  | MAC PDU loss pattern | iid random |
|  | MAC-PDU loss probability | 1%, 2%, 5%, 10%, 20% |
|  | Simulation duration | 24 hours |
|  | Media rates | Varied by steps of FLUTE payload sizes, but constant |
|  | FEC overhead | Varied to sum FEC and Media to equal bearer rate |
|  | FLUTE payload size | 1288 bytes |
|  | Segment duration | 1s, 4s, 20s |
|  | Protection period | matches segment duration |
|  | ROHC | No |
|  | IPv4/UDP/FLUTE header | 44 |

# Evaluation Criteria

## Code Performance

### Introduction

Codes are based on source blocks, in both, streaming and download delivery. A source block consists of K source symbols and in lossy delivery environments, typically not all K source symbols are received, but some of them are lost or they are not available at the receiver for other reasons, for example they have not even been sent. The amount of lost source symbols depends on different aspects, such as the loss rate, the sending strategy, etc. In case source symbols are lost, repair symbols may be used to reconstruct lost source symbols. The total number of required symbols (source or repair) are at least K, but due to code inefficiency more than K symbols, namely K+O symbols, may be required in for certain permutations of received encoding symbols. The value O expresses the code overhead for this specific experiment. The distribution of the code overhead O for different permutations of received symbols is a relevant measure for the code performance. Specifically, the failure probability distribution defined as Pf(O)= Pr{decoding with O overhead symbols or less fails} is relevant and may be used to determine the code performance.

### Evaluation Procedure

To obtain a the distribution Pf(O) a statistical evaluation procedure is proposed based on the following four parameters:

- the source block size K providing the total number of source symbols

- the number of lost source symbols L in the source block K

- the maximum encoding symbol ID (ESI) M for any repair symbol

Given these numbers the following procedure is proposed to obtain the O for one experiment:

- Generate a source block with K symbols

- Randomly drop L of the K symbols

- Generate L repair symbols with ESI randomly chosen between K+1 and M

- Set O to 0

- Attempt decoding using the K-L source symbols and the L+O repair symbols

- If decoding is not successful then

  - generate one additional repair symbol randomly chosen between K+1 and M

  - Set O to O+1

  - goto 5

- Report O as the overhead result for this experiment

To obtain the distribution for the necessary overhead O at least 10,000 of the above experiments shall be carried out.

### Test Cases

The following test cases are determined for the purpose of evaluating the code performance.

Table 7 Test Cases for Code Performance

| Number | K | L | M |
| --- | --- | --- | --- |
| CP1 | 32 | 3 | 1024 |
| CP2 | 32 | 16 | 1024 |
| CP3 | 32 | 32 | 1024 |
| CP4 | 256 | 26 | 8192 |
| CP5 | 256 | 128 | 8192 |
| CP6 | 256 | 256 | 8192 |
| CP7 | 1024 | 100 | 16384 |
| CP8 | 1024 | 512 | 16384 |
| CP9 | 1024 | 1024 | 16384 |
| CP10 | 8192 | 819 | 30000 |
| CP11 | 8192 | 4096 | 30000 |
| CP12 | 8192 | 8192 | 30000 |

### Performance Metrics

For each of the above test cases the following performance metrics shall be reported.

- The probability that decoding is not successful with O = 0 symbols Pf(O=0),

- The probability that decoding is not successful with O <= 1 symbols Pf(O=1),

- The probability that decoding is not successful with O <= 2 symbols Pf(O=2),

- The probability that decoding is not successful with O <= 3 symbols Pf(O=3),

- The probability that decoding is not successful with O <= 4 symbols Pf(O=4),

- The probability that decoding is not successful with O <= 5 symbols Pf(O=5),

- The probability that decoding is not successful with O <= 6 symbols Pf(O=6),

- The average symbol over head E{O} for the test case.

Table 8 Reporting format for Code Performance

| Case | Pf(O=0) | Pf(O=1) | Pf(O=2) | Pf(O=3) | Pf(O=4) | Pf(O=5) | Pf(O=6) | E{O} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP1 |  |  |  |  |  |  |  |  |
| CP2 |  |  |  |  |  |  |  |  |
| CP3 |  |  |  |  |  |  |  |  |
| CP4 |  |  |  |  |  |  |  |  |
| CP5 |  |  |  |  |  |  |  |  |
| CP6 |  |  |  |  |  |  |  |  |
| CP7 |  |  |  |  |  |  |  |  |
| CP8 |  |  |  |  |  |  |  |  |
| CP9 |  |  |  |  |  |  |  |  |
| CP10 |  |  |  |  |  |  |  |  |
| CP11 |  |  |  |  |  |  |  |  |
| CP12 |  |  |  |  |  |  |  |  |

## Primary Performance Metrics

### Download Delivery

For download delivery, the required FEC Overhead required for 99% of recovery of the entire file for a specific simulation case provides a very good indication for the system level performance.

Results following the mode as provided in Annex A of TR26.946 are expected.

Table 9 provides a reporting format for UTRAN test cases.

Table 9 FEC Overhead required for 99 % probability for UTRAN download test cases

| Test Case | Error rates | File size | Fec Overhead for 128/256 kbit/s | Fec Overhead for 64 kbit/s |
| --- | --- | --- | --- | --- |
| UD1 | Low (1 % BLER) | Small (50 KB) |  |  |
| UD2 |  | Medium (512 KB) |  |  |
| UD3 |  | Large (3 072 KB) |  |  |
| UD4 | Medium (5 % BLER) | Small (50 KB) |  |  |
| UD5 |  | Medium (512 KB) |  |  |
| UD6 |  | Large (3 072 KB) |  |  |
| UD7 | High (10 % BLER) | Small (50 KB) |  |  |
| UD8 |  | Medium (512 KB) |  |  |
| UD9 |  | Large (3 072 KB) |  |  |
| UD10 | 15 % BLER | Small (50 KB) |  |  |
| UD11 |  | Medium (512 KB) |  |  |
| UD12 |  | Large (3 072 KB) |  |  |
| UD13 | 20 % BLER | Small (50 KB) |  |  |
| UD14 |  | Medium (512 KB) |  |  |
| UD15 |  | Large (3 072 KB) |  |  |
| UD16 | 30 % BLER | Small (50 KB) |  |  |
| UD17 |  | Medium (512 KB) |  |  |
| UD18 |  | Large (3 072 KB) |  |  |

Table 10 provides a reporting format for LTE test cases.

Table 10 FEC Overhead required for 99 % probability for LTE download delivery test cases

| Test Case | Error rates | File size | FEC Overhead |
| --- | --- | --- | --- |
| LD1 | Very Low (1 % BLER) | Audio (1 MB) |  |
| LD2 |  | Clip (3 MB) |  |
| LD3 |  | SD (128 MB) |  |
| LD4 |  | HD (1.8 GB) |  |
| LD5 | Low (2 % BLER) | Audio (1 MB) |  |
| LD6 |  | Clip (3 MB) |  |
| LD7 |  | SD (128 MB) |  |
| LD8 |  | HD (1.8 GB) |  |
| LD9 | Medium (5 % BLER) | Audio (1 MB) |  |
| LD10 |  | Clip (3 MB) |  |
| LD11 |  | SD (128 MB) |  |
| LD12 |  | HD (1.8 GB) |  |
| LD13 | High (10 % BLER) | Audio (1 MB) |  |
| LD14 |  | Clip (3 MB) |  |
| LD15 |  | SD (128 MB) |  |
| LD16 |  | HD (1.8 GB) |  |
| LD17 | 20 % BLER | Audio (1 MB) |  |
| LD18 |  | Clip (3 MB) |  |
| LD19 |  | SD (128 MB) |  |
| LD20 |  | HD (1.8 GB) |  |

### RTP-based Streaming Delivery over UTRAN

For RTP-based streaming delivery, as a suitable measure it was considered to evaluate the maximum supported Media Rate (kbit/s) for Mean Time Between FEC Block Loss of 1 hour.

Results following the mode as provided in Annex A of TR26.946 are expected.

Table 11 provides a reporting format for UTRAN streaming test cases.

Table 11 Maximum supported Media Rate (kbit/s)
for Mean Time Between FEC Block Loss of 1 hour for UTRAN streaming test cases

| Test Case | Error rates | Bearer rate | Protection Period | Performance |
| --- | --- | --- | --- | --- |
| US1 | Low (1 % BLER) | Low (64 kbit/s) | 5 sec |  |
| US2 |  |  | 20 sec |  |
| US3 |  | Medium (128 kbit/s) | 5 sec |  |
| US4 |  |  | 20 sec |  |
| US5 |  | High (256 kbit/s) | 5 sec |  |
| US6 |  |  | 20 sec |  |
| US7 | Medium (5 % BLER) | Low (64 kbit/s) | 5 sec |  |
| US8 |  |  | 20 sec |  |
| US9 |  | Medium (128 kbit/s) | 5 sec |  |
| US10 |  |  | 20 sec |  |
| US11 |  | High (256 kbit/s) | 5 sec |  |
| US12 |  |  | 20 sec |  |
| US13 | High (10 % BLER) | Low (64 kbit/s) | 5 sec |  |
| US14 |  |  | 20 sec |  |
| US15 |  | Medium (128 kbit/s) | 5 sec |  |
| US16 |  |  | 20 sec |  |
| US17 |  | High (256 kbit/s) | 5 sec |  |
| US18 |  |  | 20 sec |  |

### DASH-based Streaming Delivery over LTE

For DASH-based streaming delivery, as a similarly suitable measure it is considered to evaluate the media rate to support a Mean Time Between FEC Block Loss of 1 hour.

Test cases are considered for Low Latency (LL) with 1 sec segment duration as well as Medium Latency (ML) of 4 sec segment duration and High Latency (HL) with 20 sec segment duration. Low-Bitrate (LB) at around 260 kbit/s and High-Bitrate (HB) at around 1MBit/s are considered.

Table 11 provides a reporting format for LTE streaming test cases.

Table 12 Media Bitrate in kbit/s
for Mean Time Between FEC Block Loss of 1 hour for LTE use cases

| Test Case | Error rates | Scenario | Supported Media Bitrate |
| --- | --- | --- | --- |
| LS1 | Very Low (1 % BLER) | LL-LB |  |
| LS2 |  | LL-HB |  |
| LS3 |  | ML-HB |  |
| LS4 |  | HL-HB |  |
| LS5 | Low (2 % BLER) | LL-LB |  |
| LS6 |  | LL-HB |  |
| LS7 |  | ML-HB |  |
| LS8 |  | HL-HB |  |
| LS9 | Medium (5 % BLER) | LL-LB |  |
| LS10 |  | LL-HB |  |
| LS11 |  | ML-HB |  |
| LS12 |  | HL-HB |  |
| LS13 | High (10 % BLER) | LL-LB |  |
| LS14 |  | LL-HB |  |
| LS15 |  | ML-HB |  |
| LS16 |  | HL-HB |  |
| LS17 | 20 % BLER | LL-LB |  |
| LS18 |  | LL-HB |  |
| LS19 |  | ML-HB |  |
| LS20 |  | HL-HB |  |

## Implementation-specific Performance Metrics

Codes not only differ in terms of the code efficiency but also in other performance criteria. Two important aspects are the required memory for decoding in the MBMS client as well as the complexity of the considered decoding algorithm. It is not expected that dedicated hardware is available to perform decoding of the application layer FEC in MBMS.

Therefore, to judge the complexity of a decoding algorithm, the decoding speed in terms of bit/s on top of a recognized mobile processor platform running a recognized mobile operating system can provide good insight into the feasibility of executing the code for mobile applications.

In terms of memory requirements, a reasonable measure is the required random access memory in the MBMS client to decode large files, such as considered in the video delivery use cases from above.

Another performance metric for successful integration into mobile platforms is the library footprint of the code.

## Summary

In summary, the following information is collected

- 12 test cases on code performance as documented in section 3.1.4, Table 8. The test case shall focus on the E{O} value, the remaining values should be provided for information only.

- 18 test cases on FEC Overhead for UTRAN download test cases as document in Table 9

- 20 test cases on FEC Overhead for LTE download test cases as document in Table 10

- 18 test cases on maximum supported media rate for UTRAN streaming test cases as documented in Table 11.

- 20 test cases on maximum supported media rate for LTE streaming test cases as documented in Table 12.

This results in total in 88 test cases.

# Benchmark Codes

## Ideal Code

### Description

A code is generally capable to handle one or a few or many of the following parameters

- T: source symbol size

- K: source block size and number of source symbols

- N: word length and number of encoding symbols

An ideal code with parameters (K, N, T)  can reconstruct the K source symbols from any set of K of the N encoding symbols. Ideal codes exists, but are usually very complex in encoding and decoding, especially if K is not small or if N needs to be large.

We use the property of ideal codes to determine the performance bound for any code considered in this context.

### Evaluation Criteria

#### Code Performance

| Case | Pf(O=0) | Pf(O=1) | Pf(O=2) | Pf(O=3) | Pf(O=4) | Pf(O=5) | Pf(O=6) | E{O} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP2 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP5 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP6 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP7 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP8 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP9 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP10 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP11 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| CP12 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

#### Download Delivery UTRAN

| Test Case | Error rates | File size | Fec Overhead for 128/256 kbit/s | Fec Overhead for 64 kbit/s |
| --- | --- | --- | --- | --- |
| UD1 | Low (1 % BLER) | Small (50 KB) | 7.5 % | 7.0 % |
| UD2 |  | Medium (512 KB) | 3.1 % | 3.3 % |
| UD3 |  | Large (3 072 KB) | 2.1 % | 2.4 % |
| UD4 | Medium (5 % BLER) | Small (50 KB) | 20.0 % | 21.8 % |
| UD5 |  | Medium (512 KB) | 11.2 % | 13.0 % |
| UD6 |  | Large (3 072 KB) | 8.8 % | 11.0 % |
| UD7 | High (10 % BLER) | Small (50 KB) | 35.0 % | 39.0 % |
| UD8 |  | Medium (512 KB) | 21.5 % | 25.8 % |
| UD9 |  | Large (3 072 KB) | 17.8 % | 22.6 % |
| UD10 | 15 % BLER | Small (50 KB) | 50.0 % | 56.0 % |
| UD11 |  | Medium (512 KB) | 32.0 % | 40.5 % |
| UD12 |  | Large (3 072 KB) | 28.0 % | 36.0 % |
| UD13 | 20 % BLER | Small (50 KB) | 66.0 % | 76.0 % |
| UD14 |  | Medium (512 KB) | 44.6 % | 57.0 % |
| UD15 |  | Large (3 072 KB) | 38.0 % | 52.0 % |
| UD16 | 30 % BLER | Small (50 KB) | 106.0 % | 130.0 % |
| UD17 |  | Medium (512 KB) | 72.0 % | 100.0 % |
| UD18 |  | Large (3 072 KB) | 66.8 % | 92.0 % |

#### Download Delivery LTE

| Test Case | Error rates | File size | FEC Overhead |
| --- | --- | --- | --- |
| LD1 | Very Low (1 % BLER) | Audio (1 MB) | tbd |
| LD2 |  | Clip (3 MB) | tbd |
| LD3 |  | SD (128 MB) | tbd |
| LD4 |  | GD (4 GB) | tbd |
| LD5 | Low (2 % BLER) | Audio (1 MB) | tbd |
| LD6 |  | Clip (3 MB) | tbd |
| LD7 |  | SD (128 MB) | tbd |
| LD8 |  | GD (4 GB) | tbd |
| LD9 | Medium (5 % BLER) | Audio (1 MB) | tbd |
| LD10 |  | Clip (3 MB) | tbd |
| LD11 |  | SD (128 MB) | tbd |
| LD12 |  | GD (4 GB) | tbd |
| LD13 | High (10 % BLER) | Audio (1 MB) | tbd |
| LD14 |  | Clip (3 MB) | tbd |
| LD15 |  | SD (128 MB) | tbd |
| LD16 |  | GD (4 GB) | tbd |
| LD17 | 20 % BLER | Audio (1 MB) | tbd |
| LD18 |  | Clip (3 MB) | tbd |
| LD19 |  | SD (128 MB) | tbd |
| LD20 |  | GD (4 GB) | tbd |

#### RTP-based Streaming over UTRAN

| Test Case | Error rates | Bearer rate | Protection Period | Performance |
| --- | --- | --- | --- | --- |
| US1 | Low (1 % BLER) | Low (64 kbit/s) | 5 sec | 56.8 |
| US2 |  |  | 20 sec | 60.6 |
| US3 |  | Medium (128 kbit/s) | 5 sec | 116.3 |
| US4 |  |  | 20 sec | 122.6 |
| US5 |  | High (256 kbit/s) | 5 sec | 237.4 |
| US6 |  |  | 20 sec | 246.4 |
| US7 | Medium (5 % BLER) | Low (64 kbit/s) | 5 sec | 47.4 |
| US8 |  |  | 20 sec | 54.2 |
| US9 |  | Medium (128 kbit/s) | 5 sec | 102.2 |
| US10 |  |  | 20 sec | 112.5 |
| US11 |  | High (256 kbit/s) | 5 sec | 228.0 |
| US12 |  |  | 20 sec | 224.5 |
| US13 | High (10 % BLER) | Low (64 kbit/s) | 5 sec | 39.5 |
| US14 |  |  | 20 sec | 47.5 |
| US15 |  | Medium (128 kbit/s) | 5 sec | 88.5 |
| US16 |  |  | 20 sec | 101.8 |
| US17 |  | High (256 kbit/s) | 5 sec | 182.0 |
| US18 |  |  | 20 sec | 201.5 |

#### DASH-based streaming over LTE

| Test Case | Error rates | Scenario | Supported Media Bitrate |
| --- | --- | --- | --- |
| LS1 | Very Low (1 % BLER) | LL-LB | tbd |
| LS2 |  | LL-HB | tbd |
| LS3 |  | ML-HB | tbd |
| LS4 |  | HL-HB | tbd |
| LS5 | Low (2 % BLER) | LL-LB | tbd |
| LS6 |  | LL-HB | tbd |
| LS7 |  | ML-HB | tbd |
| LS8 |  | HL-HB | tbd |
| LS9 | Medium (5 % BLER) | LL-LB | tbd |
| LS10 |  | LL-HB | tbd |
| LS11 |  | ML-HB | tbd |
| LS12 |  | HL-HB | tbd |
| LS13 | High (10 % BLER) | LL-LB | tbd |
| LS14 |  | LL-HB | tbd |
| LS15 |  | ML-HB | tbd |
| LS16 |  | HL-HB | tbd |
| LS17 | 20 % BLER | LL-LB | tbd |
| LS18 |  | LL-HB | tbd |
| LS19 |  | ML-HB | tbd |
| LS20 |  | HL-HB | tbd |

#### Implementation-specific Performance Metrics

Obviously, as the ideal code is generally "non-existing", no implementation specific performance metrics are available.

## MBMS FEC RFC5053

### Description

The code is fully specified in IETF RFC 5053 and is also used in MBMS TS26.346.

Simulation results are mostly available in Annex A of TR26.946. Additional simulation results for non-documented cases are provided below.

### Evaluation Criteria

#### Code Performance

| Case | Pf(O=0) | Pf(O=1) | Pf(O=2) | Pf(O=3) | Pf(O=4) | Pf(O=5) | Pf(O=6) | E{O} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP1 | 0.68660 | 0.40283 | 0.22085 | 0.11752 | 0.06100 | 0.03232 | 0.01744 | 1.55845 |
| CP2 | 0.76503 | 0.48823 | 0.27977 | 0.15150 | 0.08147 | 0.04289 | 0.02226 | 1.85573 |
| CP3 | 0.75538 | 0.47209 | 0.26726 | 0.14544 | 0.07638 | 0.03931 | 0.02057 | 1.79893 |
| CP4 | 0.74958 | 0.46410 | 0.26094 | 0.13781 | 0.07134 | 0.03661 | 0.01919 | 1.75906 |
| CP5 | 0.82118 | 0.55875 | 0.33788 | 0.18909 | 0.10140 | 0.05332 | 0.02763 | 2.11988 |
| CP6 | 0.83968 | 0.58973 | 0.36663 | 0.21111 | 0.11563 | 0.06192 | 0.03259 | 2.25307 |
| CP7 | 0.79502 | 0.52100 | 0.30319 | 0.16808 | 0.08781 | 0.04500 | 0.02268 | 1.96666 |
| CP8 | 0.89344 | 0.68450 | 0.46370 | 0.28737 | 0.16597 | 0.09299 | 0.05062 | 2.69554 |
| CP9 | 0.91632 | 0.74026 | 0.53655 | 0.35565 | 0.22283 | 0.13471 | 0.08039 | 3.10262 |
| CP10 | 0.90455 | 0.70908 | 0.48609 | 0.29882 | 0.17131 | 0.09376 | 0.04940 | 2.76552 |
| CP11 | 0.98622 | 0.93556 | 0.83786 | 0.70220 | 0.54892 | 0.40742 | 0.28610 | 5.24902 |
| CP12 | 0.99693 | 0.98165 | 0.94178 | 0.87250 | 0.77372 | 0.65606 | 0.53517 | 7.32096 |

#### Download Delivery UTRAN

| Test Case | Error rates | File size | Fec Overhead for 128/256 kbit/s | Fec Overhead for 64 kbit/s |
| --- | --- | --- | --- | --- |
| UD1 | Low (1 % BLER) | Small (50 KB) | 8.0 % | 8.0 % |
| UD2 |  | Medium (512 KB) | 3.4 % | 3.6 % |
| UD3 |  | Large (3 072 KB) | 2.2 % | 2.6 % |
| UD4 | Medium (5 % BLER) | Small (50 KB) | 21.0 % | 22 % |
| UD5 |  | Medium (512 KB) | 11.4 % | 13.4 % |
| UD6 |  | Large (3 072 KB) | 9.0 % | 11.2 % |
| UD7 | High (10 % BLER) | Small (50 KB) | 35.0 % | 39.0 % |
| UD8 |  | Medium (512 KB) | 21.5 % | 26.0 % |
| UD9 |  | Large (3 072 KB) | 18.1 % | 22.8 % |
| UD10 | 15 % BLER | Small (50 KB) | 50.0 % | 56.0 % |
| UD11 |  | Medium (512 KB) | 32.1 % | 41.0 % |
| UD12 |  | Large (3 072 KB) | 28.1 % | 37.0 % |
| UD13 | 20 % BLER | Small (50 KB) | 66.0 % | 76.0 % |
| UD14 |  | Medium (512 KB) | 45.0 % | 57.0 % |
| UD15 |  | Large (3 072 KB) | 38.2 % | 52.0 % |
| UD16 | 30 % BLER | Small (50 KB) | 106.0 % | 130.0 % |
| UD17 |  | Medium (512 KB) | 72.0 % | 100.0 % |
| UD18 |  | Large (3 072 KB) | 67.0 % | 92.0 % |

#### Download Delivery LTE

| Test Case | Error rates | File size | FEC Overhead |
| --- | --- | --- | --- |
| LD1 | Very Low (1 % BLER) | Audio (1 MB) |  |
| LD2 |  | Clip (3 MB) |  |
| LD3 |  | SD (128 MB) |  |
| LD4 |  | GD (4 GB) |  |
| LD5 | Low (2 % BLER) | Audio (1 MB) |  |
| LD6 |  | Clip (3 MB) |  |
| LD7 |  | SD (128 MB) |  |
| LD8 |  | GD (4 GB) |  |
| LD9 | Medium (5 % BLER) | Audio (1 MB) |  |
| LD10 |  | Clip (3 MB) |  |
| LD11 |  | SD (128 MB) |  |
| LD12 |  | GD (4 GB) |  |
| LD13 | High (10 % BLER) | Audio (1 MB) |  |
| LD14 |  | Clip (3 MB) |  |
| LD15 |  | SD (128 MB) |  |
| LD16 |  | GD (4 GB) |  |
| LD17 | 20 % BLER | Audio (1 MB) |  |
| LD18 |  | Clip (3 MB) |  |
| LD19 |  | SD (128 MB) |  |
| LD20 |  | GD (4 GB) |  |

#### RTP-based Streaming over UTRAN

| Test Case | Error rates | Bearer rate | Protection Period | Performance |
| --- | --- | --- | --- | --- |
| US1 | Low (1 % BLER) | Low (64 kbit/s) | 5 sec | 55.8 |
| US2 |  |  | 20 sec | 60.4 |
| US3 |  | Medium (128 kbit/s) | 5 sec | 115.5 |
| US4 |  |  | 20 sec | 122.4 |
| US5 |  | High (256 kbit/s) | 5 sec | 236.2 |
| US6 |  |  | 20 sec | 245.7 |
| US7 | Medium (5 % BLER) | Low (64 kbit/s) | 5 sec | 46.6 |
| US8 |  |  | 20 sec | 53.6 |
| US9 |  | Medium (128 kbit/s) | 5 sec | 100.8 |
| US10 |  |  | 20 sec | 111.8 |
| US11 |  | High (256 kbit/s) | 5 sec | 227.0 |
| US12 |  |  | 20 sec | 224.0 |
| US13 | High (10 % BLER) | Low (64 kbit/s) | 5 sec | 38.5 |
| US14 |  |  | 20 sec | 47.2 |
| US15 |  | Medium (128 kbit/s) | 5 sec | 87.5 |
| US16 |  |  | 20 sec | 101.2 |
| US17 |  | High (256 kbit/s) | 5 sec | 179.5 |
| US18 |  |  | 20 sec | 200.5 |

#### DASH-based streaming over LTE

| Test Case | Error rates | Scenario | Supported Media Bitrate |
| --- | --- | --- | --- |
| LS1 | Very Low (1 % BLER) | LL-LB |  |
| LS2 |  | LL-HB |  |
| LS3 |  | ML-HB |  |
| LS4 |  | HL-HB |  |
| LS5 | Low (2 % BLER) | LL-LB |  |
| LS6 |  | LL-HB |  |
| LS7 |  | ML-HB |  |
| LS8 |  | HL-HB |  |
| LS9 | Medium (5 % BLER) | LL-LB |  |
| LS10 |  | LL-HB |  |
| LS11 |  | ML-HB |  |
| LS12 |  | HL-HB |  |
| LS13 | High (10 % BLER) | LL-LB |  |
| LS14 |  | LL-HB |  |
| LS15 |  | ML-HB |  |
| LS16 |  | HL-HB |  |
| LS17 | 20 % BLER | LL-LB |  |
| LS18 |  | LL-HB |  |
| LS19 |  | ML-HB |  |
| LS20 |  | HL-HB |  |

#### Implementation-specific Performance Metrics

RFC5053 Raptor is designed to perform software-based encoding and decoding. The different aspects

- complexity of the decoding algorithm

- memory requirements

- footprint of the code

have been carefully designed to meet the hardware and software requirements when selecting the code for Release-6 in 2005.

In S4-0500357, section 3 provides some overview on the complexity of the Raptor decoding process. "With very low overhead, decoding a Raptor code requires approximately 18.9 XOR operations per symbol. Each of these requires at least one of its operands to be read from memory. Approximately 8.2 of these 18.9 require both operands to be read from memory. Finally, approximately 9.6 require the result to be written back to memory. The cost in weighted operations of these is:

- XOR operations (including one memory read):		4 x 18.9k = 75.4 k

- Additional memory reads:									4 x 8.2k = 32.9 k

- Memory writes:												4 x 9.6k = 38.2 k

The total weighted operations per symbol word is therefore: 146.6 k"

Due to sub-blocking, Raptor can provide very efficient memory handling. The maximum subblock size in TS26.346 is restricted to 256 kByte.

The footprint for the Raptor library is reported to be in the range of 100-150 kByte where a major part of the memory is dedicated to specifying all 8188 systematic indices.

# Qualification and Selection Criteria

## Qualification

### Introduction

TS26.346 defines an application layer FEC for streaming and download services based on IETF RFC5053. The code has very good performance for a many of the use cases. Therefore, any new should provide sufficient benefits compared to the existing code to justify the replacement of the existing code.

Only codes that

- fulfill all requirements in section 5.1.2, and

- provide sufficient information on the recommendations in 5.1.3

to qualify as a candidate to replace the existing application layer FEC.

### Requirements

To qualify as a candidate for replacing the existing application layer FEC shall fulfill the following requirements:

- The candidate code shall not perform worse for at least 84 test cases (95% of the 88 test cases) when compared to the existing MBMS FEC.

- The candidate code shall perform better for at least 44 test cases (50% of the 88 test cases) when compared to the existing MBMS FEC.

- For the candidate code object code shall be made available for verification of the performance results and for providing verification of independent encoder and decoder implementations. The code may be offered to interested parties only under conditions that are restricted to the above purpose.

- The FEC code shall be a single code and shall be applicable for download delivery services and streaming delivery services.

- The FEC code shall be systematic as defined in TS26.346, Annex B1.1.

- The new code shall be an open standard that can be referenced by 3GPP. If it is not a standard, other information shall be provided why the code is not standardized, what are the plans for standardization and when the completion of the standard is expected.

- Sufficient information shall be provided that documents that the decoding complexity of new application layer FEC is in the same range as MBMS FEC RFC5053.

- Sufficient information shall be provided that documents that the necessary decoding memory of a new application layer FEC is in the same range as MBMS FEC RFC5053.

- Sufficient information shall be provided that documents that the foot print of new application layer FEC is in the same range as MBMS FEC RFC5053.

### Recommendations

To qualify as a candidate for replacing the existing application layer FEC should fulfill the following requirements:

- Other information on difference in terms of flexibility and applicability of the new application layer should be provided

- Information on available ports and deployments should be provided

If this documentation is not provided there shall at least be justification why this information is not available.

## Selection

### Introduction

If none of the codes pass the qualification requirements, the existing Application Layer FEC shall be maintained.

If exactly one code passes the qualification requirements, the new code shall replace the existing Application Layer FEC in MBMS.

If multiple codes pass the qualification criteria defined in 5.1, then only one new code shall be selected. To select a single code the one with the higher single Figure of Merit (FoM) as defined in section 5.2.2 shall be applied.

### Definition of single Figure-of-Merit

Based on the test cases from above, a single figure of merit is generated. For all 88 test cases, the relative difference of the result of the new application layer FEC and MBMS FEC shall be summed to create a single figure-of-merit. To provide the higher importance for LTE based eMBMS, the relative difference for LTE use cases shall be weighted by a factor of 3.

# Proposal

It is proposed to use the evaluation and selection criteria in sections 2-5 for the selection of a new application layer FEC for MBMS.