3GPP TSG-SA4 Adhoc EMM-FEC	S4-AHI0263

8 March 2012	update to S4-AHI0251

Telephone Conference

Agenda item: 	5

Source: 	Qualcomm Incorporated

Title: 	Proposed Updates to Evaluation Procedures

Document for	Proposal and Approval

# Introduction

In SP-110555 (also available as S4-110792) a new work item on "Enhancement to FEC for MBMS" has been approved during SA#53.

To evaluate the performance of a candidate FEC code for certain environments a code performance evaluation is proposed as provided in section 3. Motivational background is provided in section 2. The concrete proposal is provided in section 4.

# Motivation

## Use Cases

Codes are based on source blocks, in both, streaming and download delivery. A source block consists of K source symbols and in lossy delivery environments, typically not all K source symbols are received, but some of them are lost or they are not available at the receiver for other reasons, for example they have not even been sent.

Use cases requiring different sending strategies are for example

- Streaming: In this case the source data is sent and a certain amount of overhead, typically in the range of 5-30% is added to compensate errors observed in the source data.

- Scheduled download from single transmitter: In this case the receivers join at the start of the delivery and accumulate the first symbols they have received until decoding is successful. If decoding is not successful with the amount of sent symbols, a repair session may be initiated.

- Carousel background download: In this case a download delivery service is offered over longer time, possibly over a long time, at a low transmission rate. For more details refer to section 2.2.

- Scheduled download with CDN-based repair: In this case the transmitter delivers only repair symbols on the multicast delivery. If the file cannot be recovered from the multicast delivery, then the associated repair procedures permit to recover the data in a repair session. The receiver requests an arbitrary set of source symbols to recover, for an ideal code typically only the first K-R with R the number of symbols received in the multicast session. The request may even be simplified to request only byte range of the file that is made available on an HTTP-server or an HTTP-cloud. This operation avoids the complex repair architectures, in the latter case only a regular HTTP-server is necessary and a regular CDN-cache download can be offered in parallel on a multicast session with zero cost for the repair procedure and maximum delivery efficiency. For more details on this use case refer to S4-120047.

- other use cases not yet considered

## Typically Observed Loss Characteristics

In addition to the sending and reception strategy, during an active reception also some encoding symbols may be solved. Without completeness, some of the following loss characteristics may occur:

- statistically independent symbol losses at high speed and line-of-sight reception

- burst-packet losses because of short-term fading

- burst-packet losses due to shadowing and obstruction, the latter possibly driving through a tunnel or below a bridge.

- antenna obstruction when holding the mobile phone, etc.

## Code Performance

The first two use cases (streaming and download delivery) from above with the first two channel models are considered relevant in the evaluation of the code and they are covered in the permanent document in the primary performance metrics.

However, for the other use cases and channel models the combination of the sending and reception strategy as well as the different reception conditions results on a significant amount of variations on which data is available at the receiver for the recovery of the source block. Therefore, not only the primary performance metrics should to be assessed, but also the flexibility of the code for different use cases as introduced in section 2.1 as well as for loss characteristics in section 2.2.

In any case non-available source symbols may be reconstructed by repair symbols. The total number of required symbols (source or repair) are at least K, but due to code inefficiency more than K symbols, namely K+O symbols, may be required for certain permutations of received encoding symbols. The value O expresses the code overhead (aka reception overhead) for this specific experiment.

The distribution of the code overhead O for different permutations of received symbols is a relevant measure for the code performance. Specifically, the failure probability distribution defined as Pf(O)= Pr{decoding with less than O overhead symbols fails} is relevant and may be used to determine the code performance.

# Code Performance

## Evaluation Procedure

The distribution of the code overhead O for different permutations of received symbols is a relevant measure for the code performance. Specifically, the failure probability distribution defined as Pf(O)= Pr{decoding with less than O overhead symbols fails} is relevant and may be used to determine the code performance.

To obtain a the distribution Pf(O) a statistical evaluation procedure is proposed based on the following four parameters:

- the source block size K providing the total number of source symbols

- the maximum encoding symbol ID (ESI) N for any repair symbol

Given these numbers the following procedure is proposed to obtain the O for one experiment:

- Generate a source block with K symbols

- Generate N-K repair symbols with ESI=K+1, ..., N

- Randomly pick K of the N symbols

- Set O to 0

- Attempt decoding using the K+O encoding symbols

- If decoding is not successful then

  - pick one additional not yet included encoding symbol randomly chosen from the N symbols.

  - Set O to O+1

  - goto 5

- Report O as the overhead result for this experiment

To obtain the distribution for the necessary overhead O at least 10,000 of the above experiments shall be carried out.

## Test Cases

The following test cases are determined for the purpose of evaluating the code performance.

Table 1 Test Cases for Code Performance

| Number | K | N |
| --- | --- | --- |
| CP1 | 32 | 34 |
| CP2 | 32 | 38 |
| CP3 | 32 | 1024 |
| CP4 | 256 | 269 |
| CP5 | 256 | 307 |
| CP6 | 256 | 8192 |
| CP7 | 1024 | 1075 |
| CP8 | 1024 | 1229 |
| CP9 | 1024 | 16384 |
| CP10 | 8192 | 8601 |
| CP11 | 8192 | 9830 |
| CP12 | 8192 | 30000 |

Note that code does not necessarily have to provide N different repair symbols, but the code may have less symbols N'. To use such codes in an environment were N symbols are distributed, the N' are repeatedly sent.

## Performance Metrics

For each of the above test cases the following performance metrics shall be reported.

- The probability that decoding is not successful with less than O < 0 symbols Pf(O=0),

- The probability that decoding is not successful with O < 1 symbols Pf(O=1),

- The probability that decoding is not successful with O < 2 symbols Pf(O=2),

- The probability that decoding is not successful with O < 3 symbols Pf(O=3),

- The probability that decoding is not successful with O < 4 symbols Pf(O=4),

- The probability that decoding is not successful with O < 5 symbols Pf(O=5),

- The probability that decoding is not successful with O < 6 symbols Pf(O=6),

- The average symbol over head E{O} for the test case.

Table 2 Reporting format for Code Performance

| Case | Pf(O=0) | Pf(O=1) | Pf(O=2) | Pf(O=3) | Pf(O=4) | Pf(O=5) | Pf(O=6) | E{O} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP1 |  |  |  |  |  |  |  |  |
| CP2 |  |  |  |  |  |  |  |  |
| CP3 |  |  |  |  |  |  |  |  |
| CP4 |  |  |  |  |  |  |  |  |
| CP5 |  |  |  |  |  |  |  |  |
| CP6 |  |  |  |  |  |  |  |  |
| CP7 |  |  |  |  |  |  |  |  |
| CP8 |  |  |  |  |  |  |  |  |
| CP9 |  |  |  |  |  |  |  |  |
| CP10 |  |  |  |  |  |  |  |  |
| CP11 |  |  |  |  |  |  |  |  |
| CP12 |  |  |  |  |  |  |  |  |

# Proposal

It is proposed to add the Code Performance Evaluation to the evaluation criteria for this work item.