3GPP TSG-<TSG/WG> Meeting # <MTG_SEQ><MTG_TITLE>	S4-AHI822

# <Location>, <Country>,  <Start_Date> - <End_Date>

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pseudo CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | <Spec#> | CR | <CR#> | rev | <Rev#> | Current version: | <Version#> |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for 5G Media Streaming Architecture |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | <Source_if_TSG> |  |  |  |  |  |  |  |  |  |
| Work item code: | <Related_WIs> |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This contribution proposes a first set of procedures for 5G Media Streaming. The procedures focuses on two type of player models, one fully integrated and one decomposed. Further, the contribution raises the issue of using the Media Session Handler function with other infrastructure functions. |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

### 4.2.1	Standalone – Non-Roaming

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.2.1-1: Media Architecture for unicast media downlink streaming

NOTE 1: The yellow filled boxes can be defined by SA4. The red lines can be in scope of SA4 specifications. It is ffs, which of the in-scope interfaces and functions will be defined by SA4. The green lines are under SA2 control and SA4 may provide some usage guidelines. The black lines are not in scope of SA4.

NOTE 2: 	Red oval indicate API provider functions.

NOTE 3:	The Media AF may also interact with NEF for NEF provided API access. However, until Release 16, only interactions with Policy and Charging (PCF) is considered in SA4 specifications.

Editor's note: The interface between Media AF and Media AS may be in scope of SA4 (currently not shown on the architecture picture)

Functions:

-	5G Media Player Functions on UE: Receiver of the unicast downlink media streaming service.

Editor's note: It might be good to break down the 5G Media Functions into more subfunctions

-	External Media Functions (e.g. App): A 5G Media Function on a UE is typically controlled by an App, which is implementing the content provider desired behaviour. The App is not defined within 5G Media Streaming.

- 	Media AS: An Application Server, which hosts 5G media functions. Note, there may be different realizations of Media ASes.

-	External Media Functions: For example, functions related to Media Creation.

-	Media AF (e.g. Network Assistance (NA) Application Function (AF)): A Media AF provides different control functions to 5G Media Functions on the UE. It may relay / trigger request for different Policy or Charging principles (PCT) or to other types of network functions.

-	Media Control Plane and Media User Plane API functions: API provided for exposing media services to external media functions.

NOTE: 	There may be several Media AFs among the Trusted Media Functions.

The following subfunctions are identified as more detailed breakdown of the Trusted Media Functions.

-	ABR Encoder, Encryption and Encapsulator

-	Manifest (MPD) Generator and Segment (DASH) Packager

-	Origin Server

-	CDN Server (Edge Servers)

-	DRM Server (DRM License)

-	Metrics Server

-	Service Directory

-	Content Guide Server

-	Ad or Replacement content server

-	MPD Proxy

The following additional functions may be relevant:

-	App Server

-	Session Management Server

The following subfunctions are identified as more detailed breakdown of the UE based Trusted Media Functions:

-	Application

-	DASH access client

-	Manifest and Segment Parser

-	Decryptor and DRM client

-	Media Decoders for audio, video, media

-	Renderer (GPU, etc.)

-	Metrics Collector

*** Next Change ***

### 4.2.2	UE Media Functions

Figure 4.2.2-1 shows the functional components of a UE to access network-based media functions.

Figure 4.2.2-1: UE Media Functions

NOTE 1: 	SBA refers to principles of 5G Service based Architecture (SBA) realizations of control transactions.

NOTE 2: 	The white color indicate here that the 3GPP has created specifications for the function.

NOTE 3: 	A UE is a logical device, allowing for tethering or other type of realizations.

NOTE4	: There are two types of UE realizations:

UE Media Functions Type 1: Monolytic Player, which just takes an URL in. The player exposes some basic controls such as play, pause, seek, stop. The player is rendering the media on the provided output devices, such as a surface in case of video.

UE Media Functions Type 2: A decomposed player (like a JavaScript Player), where the App can control each and every component.

The following subfunctions are identified as more detailed breakdown of the UE based Trusted Media Functions:

-	Operator App or Third party App: Uses APIs to access media services

-	Media Access Client: Accesses e.g. DASH Formatted Media

-	DRM Client (Optional): When present, the DRM client realizes the policies.

-	Media Decoders: Decodes the media like audio or video

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

-	Media Session Handler: Realizes a session concept, spanning optionally over multiple stateless sessions. May optionally interact with network based Media AFs.

-	Metrics collection and reporting: executes metrics collection and reporting.

Interfaces:

NOTE 4: 	The following device internal APIs may exist inside the UE.

- MC1: Media Control Interface(s) to configure and interact with the different UE media functions.

- MC2: Media Control Interface for media session management.

- MU1: Decoded media samples are handed over to the media renderer.

- MU2: Decrypted, compressed media samples are handed over to a trusted media decoder.

- MU3: In case of encryption, the encrypted, compressed media samples are handed over to the DRM Client.

Editor's Note: An interface to the Metrics Collection and Reporting function is TBD.

Editor's Note: Roaming, Non-3GPP Access and EPC-5GC interworking aspects are TBD

[

Editor's Note: The text in square brackets is not agreed.

*** Next Change ***

# 5	Procedures for Downlink Streaming

## 5.1	General

<Some general intro>

## 5.2	Establish a Unicast Media Downlink Streaming Session

This procedure indicates the establishment of a Unicast Downlink Streaming Session. A streaming session may use DASH format, 3GP File Format (Progressive Streaming), 3GP Time Text  or other (potentially non-3GPP defined) formats.

Assumption: The procedures include the service and content discovery (although these procedures are often customized)

High Level considering 5G Media Function Type 1.

Steps

1: 	The Application (App) discovers available services and content. Optionally, the application establishes an application session with the external application server (Ext AS).

2: 	The application receives a trigger, e.g. a user presses a play button.

3:	When the application receives a trigger, the application invokes the start media playback function at the 5G Media Function (Trusted), passing on an Entry URL. The Entry URL may be a URL to a DASH Manifest resource or an URL to a Progressive Streaming resource (formated according to 3GP File Format) or to other resources.

4:	The 5G Media Function (Trusted) establishes a media session with a Media AF. The media session may be used for different purposes, e.g. influence charging policies or request other network assistance support.

Editor's Note: It should be clarified, how the Media Functions within the UE get the addresses of the Media AF.

5:	The 5G Media Function establishes one or more transport sessions to acquire the 5G Media Content.

Editor's Note: The relation of the media session and the transport session establishment is ffs. Many existing players open multiple transport layer sessions (like TCP). In case of progressive streaming, only a single transport is needed.

6:	The Media AS acknolwedges the Media Content request and starts streaming the data

7:	The Media AS streams the media data.

High Level considering 5G Media Function Type 2.

<Steps>

1: The App triggeres the Service and Content Discovery sequence. The Media Session handler establishes a session with the Media AF and / or with the external AS. Note, it is not clear, whether the Media AF is involved in the initial Service and Content Discovery Procedure.

2:	A certain media item is selected.

3: The App triggeres the Media Session handler to start the playback. It is assumed here that the Media Session Handler always has an overview about a number Transport Sessions, the involved and instantiated media components, etc.

4: The Media Session Handler triggers the Media Access Client to start the session.

….

Only the media access client knows the manifest and can determine the needed information to setup a QoS flow. But a DANE may know the manifest and the associated bitrates. However, for QoS control, the PCF needs to have the (wildcarded) 5-Tuple of the Transport Sessions.

## 5.3	Releasing a Unicast Media Downlink Streaming Session

## 5.4	Ingesting Content for Unicast Downlink Streaming

## 5.5	Interacting with a Media AF

# 6	Procedures for Uplink Streaming

*** Last Change ***