3GPP TS 26.501 V1.1.0 (2019-04)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

5G Media Streaming (5GMS);

General description and architecture

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<5G, Media, Streaming, Live, Uplink, Codec, Multimedia, transport>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definition of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Media Streaming General Service Architecture	6

4.1	Overall Media Architecture	6

4.2	5G Unicast Media Downlink Streaming Architecture	6

4.2.1	Standalone – Non-Roaming	6

4.2.2	UE Media Functions	7

4.2.3	Non-Standalone LTE NR Dual Connectivity for Unicast Distribution	9

4.2.4	NR Single Connectivity for Unicast Distribution	10

4.3	5G Media Uplink Streaming Architecture	10

4.3.1	Media Architecture	10

4.3.2	LTE NR Dual Connectivity for Uplink Streaming	11

4.3.3	NR Single Connectivity for Uplink Streaming	12

4.4	Network Slicing	12

4.5	Edge Computing	12

Annex A (informative):	Usage Guidelines for collaboration scenarios	13

Annex B (informative):	Change history	14

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the 5G Media Streaming (5GMS) architecture. The 5GMS supported services include MNO and 3rd party Media Downlink Streaming Services, and MNO and 3rd party Media Uplink Streaming Services. The 5GMS supports related network and UE functions and APIs, backwards compatible functions for EUTRAN deployments (with and without MBMS) and 5G specific features.

NOTE:	Support of 5G Media streaming over MBMS with 5GC is not considered in the current version of this specification.

The 5GMS architecture is functionally divided into independent components enabling different deployments with various degrees of integration between 5G MNOs and Content Providers. It is specified as a set of extensions to TS 23.501 "System Architecture for the 5G System".

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[3]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[4]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

# 3	Definition of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G Media streaming Client (5GMS Client): Device-side function that includes at least a 5G Media Streaming Player and a Media Session Handler.

Media Player Entry: a document or a pointer to a document that defines a media presentation e.g. MPD for DASH content or URL to a video clip file.

Presentation Entry: a document or a pointer to a document that defines an application presentation e.g. an HTML5 page as defined in e.g. TS 26.307.

5G Media Streaming Player (5GMS Player) Device-side function that includes at least a Media Acces Client, Media Decoders, Media rendering/presentation, and may include DRM Client and Metrics Collection and Reporting. The 5GMS Player’s Media Access Client receives a Media Player Entry. The 5GMS Player exposes some basic controls such as play, pause, seek, stop to the App via MC1. The 5GMS player renders the media on the provided output devices, such as a tablet or a touchscreen-capable PC in case of video. Note that the 5GMS Player is functionally similar to a TS 26.247 3GP-DASH Player.

Third-Party Player: A Third-Party Player is a part of an application and this Player uses MU1, MU2 and MU3 APIs to exercise selected 5GMS Media Functions to play-back the Media content. Note that such APIs are for example defined in TS 26.307 when using the Media Source Extensions for media playback. This type of Player is downloaded by or built-in to an application, or it is downloaded with the Presentation Entry (e.g. as a Javascript library).

Note: a.k.a. Type-3 player in other contexts.

Media Streamer Entry: a pointer, e.g., in the form of  a URL that defines an entry point of a media uplink streaming session.

5G Media Uplink Streamer (5GMUS) includes at least a Media Upstream Client, Media Encoder(s), Media Capturer(s) and may include Media Configuration, Remote Control, and Assistance Handling functions. The 5GMUS receives a Media Streamer Entry to initiate an uplink streaming session. The 5GMUS captures the media on the provided input devices. The 5GMUS exposes some basic controls such as capture, pause, and stop to the App. via MUC1 and MUC2.

Third-Party Uplink Streamer: The Third-Party Uplink Streamer is a part of an application and the application uses MF1, MF2 and MUC2 APIs to exercise selected 5GMUS functions to capture and stream the media content. This type of Streamer is typically implemented as downloadable software.

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

5GMS	5G Media Streaming

5GS	5G Systems

AF	Application Function

ABR	Adaptive BitRate

API	Application Programming Interface

App	Application

AS	Application Server

CDN	Content Delivery Network

DASH	Dynamic and Adaptive Streaming over HTTP

DRM	Digital Rights Management

GPU	Graphics Processing Unit

MNO	Mobile Network Operator

MPD	Media Presentation Description

NA	Network Assistance

NR	New Radio

NEF	Network Exposure Function

PCF	Policy and Charging Function

QoE	Quality of Experience

SBA	Service based Architecture

# 4	Media Streaming General Service Architecture

## 4.1	Overall Media Architecture

Streaming in the context of the specification is defined as the delivery of time-continous media as the predominant media. Streaming points to the fact that the media is predominantly sent only into a single direction and hence, latency is not determined by conversational service requirements.

This specification addresses two dominant use cases:

- Downlink streaming: The network is the origin of the media and the UE acts as the consumption device

- Uplink streaming: The UE is the origin of the media and the network acts as the consumption entity,

Clause 4.2 introduces the 5G Unicast Media Downlink Streaming Architecture.

## Clause 4.3 introduces the 5G Unicast Media Uplink Streaming Architecture.

## 4.2	5G Unicast Media Downlink Streaming Architecture

### 4.2.1	Standalone – Non-Roaming

This architecture represents the media architecture connecting UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.2.1-1: Media Architecture for unicast media downlink streaming

NOTE 1: The functions indicated by the yellow filled boxes are in scope of stage 3 specifications for 5GMS. The interfaces indicated by the red lines are in scope of stage 3 for 5GMS specifications. The stage 2 and 3 for the interfaced indicated in green lines are specified elsewhere and 5GMS stage 3 may include usage guidelines. The interfaces indicated with black lines are not in scope of 5GMS stage 3, but are considered as part of informative call flows.

NOTE 2: 	Red ovals indicate API provider functions.

NOTE 3:	The Media AF may also interact with the NEF for NEF-enabled API access. However, within Release 16, only interactions with the Policy and Charging Function (PCF) are considered in 5GMS specifications.

The following

functions are defined:

-	5G Media Functions (Trusted) on UE: Receiver of a unicast downlink media streaming service that are specified in 3GPP and may be accessed through well-defined interfaces/APIs.

-	(UE-resident) External Media Functions (e.g. an App): The 5G Media Functions component in the UE is typically controlled by the External Media Functions component, e.g., an App, which implements external application or content service provider specific logic. The App is not defined within 5G Media Streaming.

- 	Media AS: An Application Server which hosts 5G media functions. Note that there may be different realizations of Media AS’s.

-	(Network-resident) External Media Functions: Functions that are part of the Media Service, but not part of the 5G Media Service. For example, functions related to Media Creation.

-	Media AF (e.g. Network Assistance (NA) Application Function (AF)): provides different control functions to 5G Media Functions on the UE. It may relay or initiate a request for different Policy or Charging Function (PCF) treatment or interact with other network functions.

-	Media Control Plane and Media User Plane API functions: APIs which enable the exposure or control of media services to External Media Functions in the UE or in the network.

NOTE: 	There may be multiple Media AFs residing within the Trusted Media Functions entity.

The following subfunctions are identified as as part of a more detailed breakdown of the Trusted Media Functions entity.

-	Adaptive Bit Rate (ABR) Encoder, Encryption and Encapsulator

-	Manifest (e.g., MPD) Generator and Segment (e.g., DASH) Packager

-	Origin Server

-	CDN Server (e.g., Edge Servers)

-	DRM Server (e.g., DRM License Server)

-	Metrics Server

-	Service Directory

-	Content Guide Server

-	Replacement content server (e.g. Ad content server)

-	Manifest Proxy, i.e. MPD modification server

-	App Server

-	Session Management Server

The following subfunctions are identified as more detailed breakdown of the UE-based Trusted Media Functions:

-	Application

-	DASH access client

-	Manifest and Segment Parser

-	Decryptor and DRM client

-	Media Decoders for audio, video, media

-	Renderer (Display, GPU, Audio Rendering, etc.)

-	Metrics Collector

### 4.2.2	UE Media Functions

Figure 4.2.2-1 shows the functional components of a UE to access network-based media functions.

Figure 4.2.2-1: UE 5G Media Streaming Functions

NOTE 1: 	SBA refers to principles of 5G Service based Architecture (SBA) realizations of control transactions.

NOTE 2: 	The white color indicate here that the 3GPP has created specifications for the function.

NOTE 3: 	A UE is a logical device which may correspond to the tethering of multiple physical devices or other types of realizations.

The following subfunctions are identified as part of a more detailed breakdown of the UE based Trusted Media Functions:

-	Operator App or Third-party App: Application that uses APIs to access media services.

-	Media Access Client: Accesses Media Content such as DASH-formatted Media Segments.

-	DRM Client (Optional): When present, the DRM client realizes the content protection related policies of its associated DRM system.

-	Media Decoders: Decodes the media like audio or video

-	Media Rendering / Presentation: Renders the media using an appropriate output device.

-	Media Session Handler: Realization of a “session” concept for media communications, spanning optionally over multiple stateless sessions. May optionally interact with network-based Media AFs.

-	Metrics collection and reporting: executes the collection and reporting of metrics such as QoE, interactive consumption, etc..

- 	Consumption reporting: It reports to an AF about the currently consumed media within the available presentation and may include information on UE capabilities and the environment of the media session for potential transport optimizations by the network.

Interfaces:

NOTE 4: 	The following device internal APIs may exist inside the UE.

- MC1: Media Control Interface(s) to configure and interact with the different UE media functions.

- MC2: Media Control Interface for media session management.

- MC3: Control interface for metrics collection and reporting

- MU1: Decoded media samples are handed over to the media renderer.

- MU2: Decrypted, compressed media samples are handed over to a trusted media decoder.

- MU3: In case of encryption, the encrypted, compressed media samples are handed over to the DRM Client.

Editor's Note: Roaming, Non-3GPP Access and EPC-5GC interworking aspects are TBD

[

Editor's Note: The text in square brackets is not agreed.

### 4.2.3	Non-Standalone LTE NR Dual Connectivity for Unicast Distribution

Editor's Note: The new RAN architecture separates between single Connectivity and dual connectivity according to http://www.3gpp.org/news-events/3gpp-news/1929-nsa_nr_5g. Options 3 and 3a is based on EPC and the other options on NGC (aka 5GC)

Editor's Note: The SA2 architectures should be referenced.

Editor's Note: The O&M path for QoE is defined in TS 28.404, 28.405 and 28.406

Figure 4.2.3-1: Network Architecture for Unicast Media Downlink Streaming with LTE NR Dual Connectivity

NOTE 1: 	Rx is the reference point between an AF and the PCRF. T8 is the reference point between an AS/AF and the SCEF.

NOTE 2: 	Figure 4.2.3-1 depicts an untrusted Application Function (AF), which is located outside of EPC.

NOTE 3: 	The QoE reports in EPC may be forwarded via RRC to an eNB, which forwards the reports via O&M to the QoE Reporting server.

### 4.2.4	NR Single Connectivity for Unicast Distribution

Figure 4.2.4-1: Network Architecture for Unicast Media Downlink Streaming with NR Single Connectivity

NOTE 1: 	Interface between AF/Network Assistance and PCF/NEF is realized by Interfaces Nnef, Npcf, Naf, N5 or N33.

NOTE 2: 	The figure depicts an untrusted Application Function (AF), which is located outside of 5GC.

NOTE 3: 	Nnef, Npcf and Naf are 5GC service-based interfaces (SBIs). N5 and N33 are the same interfaces in reference point representation.

NOTE 4: 	MNO trusted functions can use Npcf or N6 interfaces. External functions should use Nnef or N33 interfaces.

]

## 4.3	5G Media Uplink Streaming Architecture

### 4.3.1	Media Architecture

This architecture represents the media architecture between UE internal functions and related infrastructure functions. The next sub-clauses depict the according network architectures for EPC and 5GC.

Figure 4.3.1-1: Media Architecture for Uplink Streaming

NOTE 1: 	The yellow filled boxes are in scope of stage 3 specifications for 5GMS. The red lines are in scope of stage 3 specifications for 5GMS.. The stage 2 and 3 for green lines are specified elsewhere and 5GMS stage 3 may include usage guidelines. The black lines are not in scope of 5GMS stage 3.

NOTE 2: 	It is currently assumed that interfaces between Trusted UL Media Functions (i.e. in the box labelled Trusted UL Media Functions) are not specified in 5GMS stage 3.

NOTE 3: 	Red ovals indicate that the function exposes an API Provider. API invokers may access services, provided by this function.

Functions:

-	5G UL Media Functions (Trusted): Sender of the uplink streaming data.

Editor's note: The 5G UL Media Functions (Trusted) will be broken into smaller subfunctions.

-	External Media Functions (e.g. App): A 5G UL Media Function (Trusted) is controlled by an App, which implements external application or content service provider specific logic. The App is not defined within 5G Media Streaming.

-	UL Media AS: Receiver of the media stream, originating from the 5G UL Media Functions in the UE. The UL Media AS may forward the data to other UL Media AS’s and even to External UL Media Functions.

- 	External UL Media Function: External Receiver of the uplink media data.

-	UL Media AF, e.g. Network Assistance (NA AF): A Media Application Function, which provides different kinds of Services.

NOTE 4: 	There may be multiple UL Media AFs among the Trusted UL Media Functions.

Reference Points:

### 4.3.2	UE Media Functions

Figure 4.3.2-1: UE 5G Media Uplink Streaming Functions

NOTE 1: A UE is a logical device which may correspond to the tethering of multiple physical devices or other types of realizations.

The following subfunctions are identified as part of a more detailed breakdown of the UE 5G Media Uplink Streaming Functions:

- Operator App. or Third-party App.: application which is out of scope of the present specification and uses the UE 5G Media Uplink Streaming Functions and APIs.

- Media Capturing(s): Devices such as video camera or microphone that transform an analog media signal into digital media data.

- Media Encoder(s): Compress the media data.

- Media Upstream Client: encapsulates encoded media data and pushes it upstream.

Note: this maps to the FLUS media function in the FLUS Source specified in TS 26.238 Uplink Streaming stage 3.

- Assistance Handler: requests and/or subscribes to receive assistance information from one or more Media AF in support of the streaming process.

Note: this is expected to map to the FLUS Assistance function specified in TS 26.238 Uplink Streaming stage 3.

- Media Remote Control Handler: receives control commands from a Media AF.

Note: this is expected to map to the FLUS Remote control function specified in TS 26.238 Uplink Streaming stage 3.

- Media Configuration: configures the Media AS for uplink streaming reception.

Note: this maps to the FLUS control function in the FLUS source specified in TS 26.238 Uplink Streaming stage 3.

Here are the role of the different APIs of the UE 5G Media Uplink Streaming Functions:

MUC1: Control Plane API used to configure, activate and stop the Media Capturing(s), Media Encoding(s) and Media Upstream Client functions.

MUC2: Control Plane API used to control Media Configuration, Assistance Handler, Media Remote Control Handler functions.

MF1: User Plane API between Media Capturing(s) and Media Encoder(s) functions that carries Media data to be encoded. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Encoder(s) function.

MF2: User Plane API between Media Encoder(s) and Media Upstream Client functions to carry encoded Media data. Can also be used between a Third-Party Uplink Streamer in an App. and the Media Upstream Client function.

[

Editor's Note: The text is square brackets is not agreed,

### 4.3.2	LTE NR Dual Connectivity for Uplink Streaming

Figure 4.3.2-1: Media Uplink Streaming Architecture with LTE NR Dual Connectivity

NOTE 1: 	Interface between AF/Network Assistance and PCF/NEF is realized by Interfaces Rx and / or T8. Rx is the reference point towards PCRF. T8 is the reference point towards SCEF.

NOTE 2: 	The figure 4.3.2-1 depicts an untrusted Application Function (AF), which is located outside of EPC.

### 4.3.3	NR Single Connectivity for Uplink Streaming

Figure 4.3.3-1: Media Uplink Streaming Architecture with NR Single Connectivity

NOTE 1: 	Interface E is realized by Interfaces Nnef, Npcf, Naf, N5 or N33.

NOTE 2: 	Nnef, Npcf and Naf are 5GC service-based interfaces (SBIs). N5 and N33 are the same interfaces in reference point representation

NOTE 3: 	MNO trusted functions can use Npcf or N6 interfaced. External functions should use Nnef or N33 interfaces

NOTE 4: 	The Application Function (AF) may be located inside or outside of the Operator's trust domain.

]

## 4.4	Network Slicing

Editor's Note: This clause is TBD

## 4.5	Edge Computing

Editor's Note: This clause is TBD

# 5	Procedures for Downlink Streaming

## 5.1	General

<Some general intro>

## 5.2	Establish a Unicast Media Downlink Streaming Session

This procedure describes the establishment of a Unicast Downlink Streaming Session. A streaming session may use DASH format, 3GP File Format (Progressive Streaming), 3GP Time Text or other (potentially non-3GPP defined) formats.

Assumption: The procedures include the service and content discovery (although these procedures are often customized).

High Level Procedure for progressive download for on-demand media.

Steps

1: The App triggers the Service and Content Discovery sequence.

The Content Discovery procedure only involves the App and the external Application Server.

2:	A media content item is selected.

3: The App triggers the Media Session handler to start the playback. The media player entry is provided to the Media Session handler.

4: The Media Session Handler triggers the 5GMS Player to start the session.

5: The 5GMSP establishes the transport session.

6: The 5GMSP sends the request for the progressive download content

7: The 5GMSP receives the initialization information of the progressive download content. The initialization information contains configuration recommendations for reception of the media, and optionally also DRM information.

8:	The 5GMSP configures the rendering pipeline for media playback.

9: The 5GMSP notifies the media session handler, providing the transport session information and some media content related information.

10: Optional: the media session handler establishes a media session with the Media AF. Additional parameters such as the transport session parameters are provided.

11: Optional: 5GMSP acquires a DRM License from an External AS.

12: The 5GMSP receivers media content and puts it into the rendering pipeline.

13: The 5GMSP continuously receives and plays back the media content.

….

High Level Procedure for DASH content.

It is assumed here that the key information to initialize the media decoding and rendering pipeline is present in the Media Player Entry (or referenced by the Media Player Entry). The intention is to provide the client with information to setup the media decoding and rendering pipeline in such a way that no pipeline reset is needed during the session. Implementations need to consider that parts of the information are provided with the initialization segments.

It is assumed that the client is enabled to use the same media decoding and rendering pipeline during the session.

Steps

1: The App triggers the Service and Content Discovery sequence. The Content Discovery procedure only involves the App and the external Application Server.

2:	A media content item is selected.

3: The App triggers the Media Session handler to start media playback. The Media Player Entry is provided to the Media Session handler.

4: The Media Session Handler triggers the 5GMS Player to start the session.

5: The 5GMSP establishes the transport session for acquiring the manifest referenced by the Media Player Entry.

6: The 5GMSP requests the manifest

7: The media access client receives the manifest

8: The 5GMSP processes the manifest. It determines for example the number of needed transport sessions for media acquisition. The 5GMSP should be able to use the manifest information to initialize the media pipelines for each media stream. The manifest should also contain information to start the DRM client initialization, when DRM is used.

9: The 5GMSP notifies the media session handler about the manifest. The notification may contain parameters from the manifest

10: Optional. The Media Session handler establishes session with the Media AF.

11: Optional: the 5GMSP acquires the necessary DRM information, for example a DRM License.

12: The 5GMSP configures the media rendering pipeline.

13: The 5GMSP establishes the necessary transport sessions for the content. For example, the media access client may establish one transport sesession for each media component (audio, video, etc) and for some bitrate representations.

14: The media session handler is notified about the established transport sessions for the content.

15: Optional: The media session handler may notify the media AF about the session session and the established transport sessions.

16: The 5GMSP requests initialization information. The 5GMSP repeats this step for each required initialization segment.

17: The 5GMSP receives the initialization information.

18: The 5GMSP requests media segments according to the manifest.

19: the 5GMSP receives media segments and puts the information into the according media rendering pipeline.

20: Previous steps are repeated according to the manifest information.

….

## 5.3	Releasing a Unicast Media Downlink Streaming Session

TBA

## 5.4	Ingesting Content for Unicast Downlink Streaming

TBA

## 5.5	Metrics collection and reporting

Metrics collection and reporting can be done in different ways, depending on the relation between the Service Provider and the Network Operator. The following figures shows simplified signalling examples for two different use-cases.

In the first use-case, shown in figure 5.x.1 below, the MNO is controlling the metric reporting, using the out-of-band configuration and reporting option. Note that in this scenario the App is not actively involved in collection or reporting of the metrics, so metrics can be collected for any App which uses the trusted media functions in the UE, even when the media server is not under control of the MNO.

Figure 5.5-1: Metrics collection using the out-of-band 5G control plane option

The different steps are explained below:

1: Overall metrics configuration is done on the network level, for instance defining which geographical areas that shall have metrics collection active.

2: The metric configuration(s) is sent from the Metrics AF to the RAN, which does not forward that information to the UE at this stage.

3: Time passes, and it assumes that the UE moves around during that period.

4: The UE enters an area (cell, location area, etc.,) which is inside the geographical constraint. This is discovered by the RAN, and it now needs to activate metric reporting for the UE.

5: The actual metric configuration is sent from the RAN to the metrics collection reporting function in the UE, via the 5G control plane.

6: Additional time passes, and it is assumed that the UE has a metrics configuration, but no streaming session has started.

7: A streaming session is started.

8: The session setup is done in conjunction with signalling transactions (not shown here). No metrics configuration is done during this setup.

9: Media is delivered and rendered, and...

10: ...more media is delivered...

11: Metrics are collected during the session according to the configuration, and reported via the 5G control plane with the frequency defined in the configuration.

12: The session continues...

13: ...more media is delivered...

14: One or more additional metrics collection and reporting transaction(s) are performed, until the session ends.

15: Time passes during which no streaming session is active.

16 to 23: Another session is started, and as the UE is still inside the collection area, this session will also be collecting metrics in the same way as in steps 7-14.

24: Time passes, the UE moves around.

25: The UE leaves the geographical area specified by the metrics configuration.

26: RAN sends metrics (de)configuration to the UE, to stop future metrics collection.

The second use-case, shown in figure 5.x.2 below, illustrates a scenario where the metrics collection is configured in-band. Thus together with the media there are metadata which describe how metrics shall be collected, and how they shall be reported. The AF/AS functions can both be either trusted or untrusted (but not mixed).

Figure 5.5-2: Metrics collection using the in-band metadata option

The different steps are explained below:

1: The content servers are set up, including any metrics configuration.

2: If the AF and AS are located in a trusted domain, metadata regarding the metrics collection and reporting are sent to the AS. The procedure for configuring the AS, when the AF and AS reside in an untrusted domain, is unspecified.

3: Time passes during which no streaming session is active...

4: A streaming session starts.

5: The App and the session handler initializes a new session.

6: The media access client requests any metadata, from the Media AS, which is needed for downloading the media content

7: Metadata is returned to the access client and also forwarded to the App.

8: The metrics parts of the metadata is sent by the App to the metrics collection and reporting function.

9: The streaming session is running.

10: Media is delivered and rendered, and...

11: ...more media is delivered...

12: Metrics are collected during the session according to the configuration, and reported with the frequency defined in the configuration. The reporting method is defined in the configuration, for instance HTTP upload.

13: The session continues...

14: ... more media is delivered...

15: ...Occurrence(s) of additional metrics reporting, until the session ends.

## 5.6	Consumption reporting

Step:

The first phase is the initialisation phase

- The App is started.

- The App instructs the Media Session Handler to start

- The Media AS initialise the parameters for the consumption report

- The Media Session Handler initialises the Consumption reporting

- The Media Session Handler starts the 5GMS player with the URL

The second phase is media playback

The user preferences may be change:

- The app select/change user preferences.

- The 5GMS player transmits user preferences

When Media is playing, the consumption report parameters may be updated

- The Media AS updates the parameters for the consumption report

- The Media Session Handler updates the Consumption reporting

When Media is playing

- The 5GMS player regularly accesses to the media content

- The consumption reporting  regularly sends report to the Media AF

The last phase is to stop the media

- The App instructs the Media Session Handler to stop the video

- The Media Session Handler stops the Consumption reporting

- The consumption reporting may send the last report to the Media AF

- The Media Session Handler stops the 5GMS player

# 6	Procedures for Uplink Streaming

## 6.1	General

## 6.2	Establishment of an Uplink Streaming Session

The procedure allows an 5G Media Uplink Streamer to establish a uplink streaming session with a Media AS.

Figure 6.2-1: Uplink Streaming Session Establishment

Steps

1: 	During provisioning, the 5G Media Uplink Streamer, the Media AS is provisioned with basic information, such as the Media AF and Media AS addresses. For some devices, only the remote control information is provisioned and all additional parameters are retrieved from Remote Controller.

2: 	The App starts the uplink media streaming.

3: 	The App instructs the 5GMUS to start providing a Media Streaming Entry.

When remote control is provisioning in step 1:

4: 	The transport session is established for the remote control

5: 	The 5GMUS starts waiting for incoming remote control commands

6: 	The 5GMUS receives a remote control command to start upstreaming (either now or with a timestamp). The Remote control command may contain details around the Media AS, etc.

7: 	The 5GMUS establishes the transport session

8: 	The 5GMUS establishes the media streaming session

When client assistance is provisioned:

9: The 5GMUS establishes the assistance channel to the provisioned Media AF(s)

When server assistance is desired (e..g for QoS or charging)

13: The Media AS establishes a assistance session with the Media AF.

## 6.3	Termination of an Uplink Streaming Session

The procedure defines the termination of an uplink streaming session.

Figure 6.3-1: Uplink Streaming Session Teardown

Steps

1: 	An Uplink streaming session is active and should be terminated

When the App terminates the session, e.g. triggered by user input

2: 	The App sends a Stop commands to the 5G Media Uplink Streamer

When remote control is used and the remote control session is established

3:	The 5G Media Uplink Streamer receives a stop command.

4: The 5G Media Uplink Streamer stops the capturing process

5: The uplink media streaming process is terminated

6: The transport session is released

When client assistance was established

7: The Media AF is notified that the media uplink streaming session is terminated.

When server assistance was established

8: The Media AS is notified the Media AF that the media uplink streaming session is terminated.

## 6.4	Providing Network Assistance

The procedure defines the network assistance from a Media AF.

Figure 6.4-1: Providing Network Assistance

Steps

1: 	An Uplink streaming session is active. The Assistence Session is established with the Media AF.

2: 	The Media AF determines the need to send an assistance message.

3:	The assistance message is sent.

4: The 5G Media Uplink Streamer takes an appropriate action to the message.

## 6.5	Providing Remote Control

The procedure defines the remote control from a Media AF. Note that the Media AF for remote control may be deployed as a network based function or a UE based function.

Figure 6.5-1: Uplink Streaming Session Establishment

Steps

1: 	An Uplink streaming session is active. The Remote Control Session is established with the Media AF.

2: 	The Media AF for Remote Control determines the need to send a remote control message.

3:	The remote control message is sent.

4: The 5G Media Uplink Streamer takes the defined action to the message.

## 6.6	RAN Signaling based Support for Uplink Network Assistance

The diagram in Figure 4.3.4-1 depicts an uplink streaming architecture where RAN signaling is employed to support uplink network assistance functionality..

The RAN in the architecture contains control plane and user plane entities that interact with peer control and user plane entities in the UE, in the request/response for boost of the uplink streaming data rate, and subsequent media transport. The user plane functionality as shown in Figure 4.3.4-1 (solid line going from UE to RAN to Media AS) illustrates the scheduling/pass-through functionality associated with user plane communications, governed by the RAN, as result of the uplink assistance messaging over the control plane (shown by dotted line between the UE and RAN). It is assumed in this case that application-level network assistance, i.e., as represented by the ‘NA AF’ sub-function of the “UL Media AF” is not utilized.

It should be noted that although the NA AF is not utilized when RAN signaling based network assistance is performed, it is assumed that there is a higher level network entity which coordinates and tracks network assistance performed using RAN signaling vs. application signaling for individual UEs.

Figure 6.6-1: RAN Signaling based Uplink Network Assistance

Annex A (informative): Usage Guidelines for collaboration scenarios

Editor's Note: The intention of this annex is to describe the media architecture instances based on various collaboration scenarios between e.g. MNOs and Third-Party Service providers.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 03-2019 | SA-83 | SP-190037 |  |  |  | Presented to TSG SA#83 (for information) | 1.0.0 |
| 04-2019 | SA4#103 | S4-190357 |  |  |  | pCR on Editorial updates | 1.0.1 |
| 04-2019 | SA4#103 | S4-190452, S4-190459, S4-190470, S4-190455, S4-190471 |  |  |  | Downlink procedures, uplink procedures, metrics collection an reporting, RAN Signaling based Uplink Assistance, consumption reporting. | 1.1.0 |