3GPP TSG-WG SA4 MBS Call	S4-AHI939

# Telco

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 26.512 | CR |  | rev | - | Current version: | 0.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Provisioning Interface Updates |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMS3 |  |  |  |  |  | Date: |  |  | <Res_date> |
| Category: | <Cat> |  |  |  |  |  | Release: |  |  | <Release> |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | This pCR provides updates to the Provisioning and Ingest procedures and APIs. It also provides updated OpenAPI specifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The pCR fixes a few mistakes and typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The content ingest for 5GMS3 will not be properly defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR … CR … |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR … CR … |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR … CR … |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

### Change #1

## 4.2 Procedures of the M1d (5GMS Provisioning) interface

Editor’s Note: This clause should contain control procedures between Network External Media Application Servers and the Media AF.

### 4.2.1 General

A 5GMSd-aware Application Provider may use the procedures in this clause to provision the network for downlink media sessions that are operated by the application provider.  These sessions may be DASH streaming sessions, progressive download sessions, or any other type of media streaming or distribution (e.g. HLS).

The M1d offers three different sets of procedures:

-	Media Ingest for CDN distribution: designed as an API that matches the functionality of public CDNs.

-	Configuration for dynamic policy: allows the configuration of policy templates that can be applied to downlink sessions of the application provider.

-	Configuration for reporting: permits the MNO to collect QoE and consumption reports about the downlink sessions.

A 5GMSd-aware Application Provider may use any of these procedures and in any combination it wishes, to support its downlink media sessions.

### Change #2

#### 4.2.2.2 Create Provisioning

This procedure is used by the external application provider to create a new Provisioning session. A Provisioning session may be used to configure CDN distribution, QoS and charging rules, and/or QoE and consumption reporting for streaming sessions of that application provider.

#### The application provider shall use the HTTP POST method to create a new Provisioning. Upon successful creation, the resource Id of the new created Provisioning session shall be returned in the body of the reply.

#### 4.2.2.3 Read Provisioning

#### This procedure is used by the application provider to obtain the properties of the Provisioning session from the Media AF. The application provider uses the GET method for this purpose.

#### 4.2.2.4 Update Provisioning

#### The Update operation is not allowed on Provisioning sessions.

#### 4.2.2.5 Delete Provisioning

This operation is used by the application provider to delete a Provisioning. The AF will release any associated resources, purge any cached data, delete all QoS and reporting configurations associated with this Provisioning. The Media AF shall use the HTTP Delete method for this purpose.

### Change #3

#### 4.2.3.2	Create Ingest

This procedure is used by the application provider to provision a new Ingest session. The application provider shall use the HTTP POST method for this purpose. Upon successful creation, the resource Id of the newly created Ingest session shall be returned in the body of the reply.

If the ingest uses the Push method, i.e. the Pull attribute is set to False, then the Path and entryPoint shall not be provided and are read-only. The TargetDomain is assigned by the Ingest Media AF and is read-only.

### Change #4

#### 4.2.3.5	Delete Ingest

This operation is used by the application provider to end the ingest session and the related distribution. The AF will release any associated resources, purge any cached data, and delete any corresponding configurations. The Media AF shall use the HTTP Delete method for this purpose.

### Change #5

### 5.2.2	Data Model

The Ingest Object for media ingest is provided below:

| Name | Type | Cardinality | Description |
| --- | --- | --- | --- |
| origin | Object | 1 .. 1 | Describes the origin that will be used to ingest the content for distribution. |
| name | string | 1 .. 1 | A name associated with this origin |
| path | string | 1 .. 1 | The relative path which will be used to address the media resources. This Path is provided by the 5GMSd AF in the case of Push. |
| pull | boolean | 1 .. 1 | Indicates whether to use Pull or Push for ingesting the content. |
| protocol | string | 1 .. 1 | Identifier of the ingest protocol. The set of supported protocols are defined in clause 5.4.2.3. |
| entryPoint | string |  | An entry point to ingest the content. The semantics of the entry point are dependent on the selected Protocol. In case of Push (Pull flag is set to False), this parameter is returned by the Provisioning AF to the content provider and provides the entry point for pushing the content. In case of Pull (Pull flag is set to True), the EntryPoint shall be provided by the origin to the Media AF to indicate the location from which content is to be pulled. The URL shall be used as the base URL, in the case of Pull. A request received by the Media AS is mapped to a URL using the provided base URL to fetch the content from the origin server. |
| cache | [Object] | 1 .. 1 | Defines the Cache configuration for the content of this Distribution. |
| pattern | string |  | A pattern that will be used to match the object’s URL to the Cache settings. The pattern shall be defined as a regular expression as specified by [5]. |
| originCacheHeaders | boolean |  | Indicates whether to apply the cache directives for the object at ingest to the distribution. This is only applicable if the Ingest protocol is HTTP. |
| cacheDirectives | Object |  | If the cache directives are not inherited from the origin, this provides instructions on how to cache the content that matches the pattern. |
| noCache | boolean |  | If set to true, this indicates that the objects that match this pattern are not be cached. |
| httpMethodFilter | [string] |  | The methods to which the Cache rules apply. The filter shall be provided as a regular expression as specified in [5]. |
| statusCodeFilter | [integer] |  | The status codes to which the Cache rules apply. The filter shall be provided as a regular expression as specified in [5]. |
| cdnTTL | [integer] |  | The TTL value to set to the objects that match the pattern and the other filters. |
| Configuration | Object | 1 .. 1 | Specifies the distribution method and configuration for the ingested content |
| contentPreparation | Object | 0..1 | Defines the requested content preparation. A set of templates for content preparation may be provided for selection. |
| mimeType | string |  | This field provides the MIME type of the content preparation description document. If the MIME type is not understood, the Media AF shall reject the provisioning request. |
| Content | string |  | This field provides the content preparation description document. The document shall be Base64 encoded. |
| targetDomain | string | 1 | The target domain name, which will be used to access the content from the CDN. This will replace the domain name of the |
| originDomain | string | 1 | The origin domain that is redirected to the target domain. This domain name can be exposed to the Ues instead of the target domain. |
| geoFencing | Object | 0..1 | Limit access to the content to the indicated geographic areas. |
| locationType | string | 1 | The type of the location information. |
| Locations | [string] |  | Array of locations from which access to the resources is to be allowed. |
| urlSignature | Object | 0..1 | Defines the URL signing scheme. Only correctly signed and valid URLs will be allowed to access the content. |
| Pattern | string |  | A pattern that shall be used to match the object’s URL for which URL signing is activated. An object of which the URL does not match this pattern will not be URL signed. The pattern shall be defined as a regular expression as specified by [5]. |
| tokenName | string |  | The name of the query parameter that carries the token. |
| passphraseName | string |  | The name of the query parameter that carries the passphrase. |
| Passphrase | string |  | The shared secret between the eMAS and the MAS for this Distribution. |
| ttlName | string |  | The name of the query parameter that carries the TTL field. |
| useIPAddress | boolean |  | If set to true, the IP address of the UE is matched to the IP address in the signature. Access to the object is only allowed when they match. |
| ipAddressName | string |  | The name of the query parameter that carries the IP address of the client. |
| Certificate | string |  | When content is distributed using TLS, the certificate for the Origin Domain is shared with the 5GMSd AF. This provides the id of the certificate to use. |

### Change #6

### 5.2.3	Resource structure

The Ingest and Distribution API is accessible through this URL paths:

{apiRoot}/3gpp-m1d/v1/provisionings/{provisioning-id}/

The following operations and the corresponding HTTP methods are supported:

| Operation | Resource URI | HTTP Method | Description |
| --- | --- | --- | --- |
| Create a new Ingest Session | ingests | POST | This is used to create a new Ingest resource. |
| Fetch Ingest SessionConfiguration | ingests/{ingest-id} | GET | This operation is used to read the Ingest configuration. |
| Update Ingest Session | ingests/{ingest-id} | PUT/PATCH | This operation is used to update the Ingest configuration. |
| Delete Ingest Session | ingests/{ingest-id} | DELETE | This operation is used to delete an existing Ingest. |
| Purge Cache | ingests/{ingest-id}/purge | POST | This operation is used to invalidate some or all cached objects of this Ingest. |
| Fetch list of supported ingest protocols | protocols | GET | This operation is used to retrieve a list of supported Ingest Protocols. |
| Create a new Content Preparation Template | preparation-templates | POST | This operation is used to create a new Content Preparation Template that defines how the ingested content is to be prepared for ingest sessions. |
| Create a new Certificate | certificates | POST | This operation is used to add a new certificate that is used for the distribution of the content using TLS. |
| Delete a certificate | certificates/{certificate-id} | DELETE | This operation removes a certificate from the list of certificates that are owned by the Application Provider and that are associated with the current Provisioning. |